3GPP TSG-RAN5 Meeting #104	R5-244806

# Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Title:							MU discussion on EVM for FR2 PC5

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the previous meeting, MTSU and TT were discussed for EVM testing for PC5 FR2a [1]. This paper provides our testability analysis on the EVM MU.

# 2.	Discussion

Based on TS 38.521-2 [2], FR2a, the minimum possible UL level of EVM testing for PC5 FR2a is calculated by minimum peak EIRP of 30 dBm – MPR – T(MPR) – MBR as shown below.

Table 1 Minimum possible UL level in EVM testing for PC5 FR2a

| Test configuration |  |  | MPRnarrow [dB] | MPRWT [dB] |  | MPR [dB] |  | T(MPR) [dB] |  | MBR [dB] | Min. peak EIRP [dBm] | UL level [dBm] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Modulation | RB allocation |  | ≤ 200 MHz | 400 MHz | ≤ 200 MHz | 400 MHz | ≤ 200 MHz | 400 MHz |  |  | ≤ 200 MHz | 400 MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.7 | 30 | 29.3 | 29.3 |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0 | 2 | 3 | 2 | 3 | 1.5 | 2 | 0.7 | 30 | 25.8 | 24.3 |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.7 | 30 | 29.3 | 29.3 |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0 | 2 | 3 | 2 | 3 | 1.5 | 2 | 0.7 | 30 | 25.8 | 24.3 |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0 | 3 | 4.5 | 3 | 4.5 | 2 | 4 | 0.7 | 30 | 24.3 | 20.8 |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0 | 3.5 | 4.5 | 3.5 | 4.5 | 3 | 4 | 0.7 | 30 | 22.8 | 20.8 |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 0 | 5 | 6.5 | 5 | 6.5 | 4 | 5 | 0.7 | 30 | 20.3 | 17.8 |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 0 | 5.5 | 6.5 | 5.5 | 6.5 | 5 | 5 | 0.7 | 30 | 18.8 | 17.8 |
| 9 | CP-OFDM QPSK | Inner_Full | 0 | 3.5 | 5 | 3.5 | 5 | 3 | 4 | 0.7 | 30 | 22.8 | 20.3 |
| 10 | CP-OFDM QPSK | Outer_Full | 0 | 4 | 5 | 4 | 5 | 3 | 4 | 0.7 | 30 | 22.3 | 20.3 |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0 | 5 | 6.5 | 5 | 6.5 | 4 | 5 | 0.7 | 30 | 20.3 | 17.8 |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0 | 5 | 6.5 | 5 | 6.5 | 4 | 5 | 0.7 | 30 | 20.3 | 17.8 |
| 13 | CP-OFDM 64 QAM | Inner_Full | 0 | 7.5 | 9 | 7.5 | 9 | 5 | 5 | 0.7 | 30 | 16.8 | 15.3 |
| 14 | CP-OFDM 64 QAM | Outer_Full | 0 | 7.5 | 9 | 7.5 | 9 | 5 | 5 | 0.7 | 30 | 16.8 | 15.3 |

For the above test points, we assume the total EVM MU as below.

Table 2 EVM MU for PC5 FR2a

| Test configuration |  |  | EVM MU |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Modulation | RB allocation | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.47% | 3.50% | 4.95% | 7.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full |  |  |  |  |
| 3 | DFT-s-OFDM QPSK | Inner_Full |  |  |  |  |
| 4 | DFT-s-OFDM QPSK | Outer_Full |  |  |  |  |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full |  |  |  |  |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full |  |  |  |  |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full |  |  |  |  |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full |  |  |  |  |
| 9 | CP-OFDM QPSK | Inner_Full |  |  |  |  |
| 10 | CP-OFDM QPSK | Outer_Full |  |  |  |  |
| 11 | CP-OFDM 16 QAM | Inner_Full |  |  |  |  |
| 12 | CP-OFDM 16 QAM | Outer_Full |  |  |  |  |
| 13 | CP-OFDM 64 QAM | Inner_Full | 2.60% | 3.67% | 5.20% | 8.73% |
| 14 | CP-OFDM 64 QAM | Outer_Full |  |  |  |  |

Proposal 1: Adopt Table 2 as MTSU of EVM for PC5 FR2a.

The above EVM MU is expressed as RSS of EVM due to available SNR and additional TE EVM MU, and these MU elements are shown in the following tables. Note that it assumes polarization mismatch of -3 dB and TE noise floor of -10.6 dBm/400 MHz.

Table 3 EVM due to available SNR for PC5 FR2a

| Test configuration |  |  | Available SNR [dB] |  |  |  | EVM due to available SNR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Modulation | RB allocation | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 45.9 | 42.9 | 39.9 | 36.9 | 0.51% | 0.71% | 1.01% | 1.43% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 42.4 | 39.4 | 36.4 | 31.9 | 0.76% | 1.07% | 1.51% | 2.54% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 45.9 | 42.9 | 39.9 | 36.9 | 0.51% | 0.71% | 1.01% | 1.43% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 42.4 | 39.4 | 36.4 | 31.9 | 0.76% | 1.07% | 1.51% | 2.54% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 40.9 | 37.9 | 34.9 | 28.4 | 0.90% | 1.27% | 1.80% | 3.80% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 39.4 | 36.4 | 33.4 | 28.4 | 1.07% | 1.51% | 2.14% | 3.80% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 36.9 | 33.9 | 30.9 | 25.4 | 1.42% | 2.01% | 2.85% | 5.37% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 35.4 | 32.4 | 29.4 | 25.4 | 1.69% | 2.39% | 3.38% | 5.37% |
| 9 | CP-OFDM QPSK | Inner_Full | 39.4 | 36.4 | 33.4 | 27.9 | 1.07% | 1.51% | 2.14% | 4.03% |
| 10 | CP-OFDM QPSK | Outer_Full | 38.9 | 35.9 | 32.9 | 27.9 | 1.13% | 1.60% | 2.26% | 4.03% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 36.9 | 33.9 | 30.9 | 25.4 | 1.42% | 2.01% | 2.85% | 5.37% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 36.9 | 33.9 | 30.9 | 25.4 | 1.42% | 2.01% | 2.85% | 5.37% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 33.4 | 30.4 | 27.4 | 22.9 | 2.13% | 3.01% | 4.26% | 7.16% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 33.4 | 30.4 | 27.4 | 22.9 | 2.13% | 3.01% | 4.26% | 7.16% |

Table 4 Additional TE EVM for PC5 FR2a

| Test configuration |  |  | Additional TE EVM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Modulation | RB allocation | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.42% | 3.43% | 4.85% | 6.85% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.36% | 3.33% | 4.71% | 6.52% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.42% | 3.43% | 4.85% | 6.85% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.36% | 3.33% | 4.71% | 6.52% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.31% | 3.26% | 4.61% | 5.88% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 2.23% | 3.16% | 4.47% | 5.88% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.02% | 2.86% | 4.05% | 4.49% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 1.81% | 2.55% | 3.61% | 4.49% |
| 9 | CP-OFDM QPSK | Inner_Full | 2.23% | 3.16% | 4.47% | 5.73% |
| 10 | CP-OFDM QPSK | Outer_Full | 2.20% | 3.11% | 4.40% | 5.73% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 2.02% | 2.86% | 4.05% | 4.49% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 2.02% | 2.86% | 4.05% | 4.49% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 1.49% | 2.10% | 2.97% | 5.00% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 1.49% | 2.10% | 2.97% | 5.00% |

Finally, TT values are calculated from the minimum requirement and MTSU. Note that the formula to calculate TT is clarified in Table F.3.2-1 [2].

Table 5 TT for EVM testing for PC5 FR2a

| Test configuration |  |  | Minimum requirement | TT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Modulation | RB allocation |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 30.0% | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 30.0% | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 17.5% | 0.00% | 0.00% | 0.00% | 1.35% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 17.5% | 0.00% | 0.00% | 0.00% | 1.35% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 12.5% | 0.00% | 0.00% | 0.94% | 1.83% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 12.5% | 0.00% | 0.00% | 0.94% | 1.83% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 8.0% | 0.00% | 0.73% | 1.41% | 2.63% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.0% | 0.00% | 0.73% | 1.41% | 2.63% |
| 9 | CP-OFDM QPSK | Inner_Full | 17.5% | 0.00% | 0.00% | 0.00% | 1.35% |
| 10 | CP-OFDM QPSK | Outer_Full | 17.5% | 0.00% | 0.00% | 0.00% | 1.35% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 12.5% | 0.00% | 0.00% | 0.94% | 1.83% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 12.5% | 0.00% | 0.00% | 0.94% | 1.83% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 8.0% | 0.00% | 0.80% | 1.54% | 3.84% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 8.0% | 0.00% | 0.80% | 1.54% | 3.84% |

Proposal 2: Adopt Table 5 as TT of EVM for PC5 FR2a.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals for FR2a, PC5, CBW  400 MHz, NTC, IFF, max device size  40 cm.

Proposal 1: Adopt Table 2 as MTSU of EVM for PC5 FR2a.

Proposal 2: Adopt Table 5 as TT of EVM for PC5 FR2a.

# 4.	References

[1] R5-242264, “Discussion on MU and TT analysis for PC5”, Keysight Technologies, RAN5#103

[2] TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.3.0 (2024-06)