3GPP TSG-RAN5 Meeting #104	R5-244807

# Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Title:							QoQZ for Spurious test cases for QZ size 40 cm

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the previous meeting, the action point AP#103.21 was created to define spurious emissions QoQZ MU for QZ size = 40 cm and NTC. This paper repeats our measurement results and proposal of [1] to proceed the discussion.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#103.21 | RF | Define Spurious Emissions QoQZ MU for 40cm QZ and NTC | Anritsu, Keysight, R&S | R5-243002 | RAN5#104 | Open |

# 2.	Discussion

Following table shows our measurement results of spurious emissions QoQZ MU for QZ size = 40 cm and NTC.

Table 1: Standard Deviations for Spurious measurement (QZ size = 40 cm, NTC)

| Positions | QoQZ MU [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 6.000 GHz | 12.750 GHz | 23.450 GHz | 40.800 GHz | 66.000 GHz | 87.000 GHz |
| P1-P7 | 0.99 | 0.58 | 0.61 | 0.38 | 0.50 | 0.48 |

Considering the above measurement results and commonality with the already endorsed MU values for QZ size ≤ 30 cm, we proposee the following spurious emissions QoQZ MU for QZ size = 40 cm and NTC.

Table 2: QoQZ MU for Spurious measurement

| Condition | QoQZ MU [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 6 GHz to 12.75 GHz | 12.75 GHz to 23.45 GHz | 23.45 GHz to 40.8 GHz | 40.8 GHz to 66 GHz | 66 GHz to 87 GHz |
| QZ size ≤ 30 cm, NTC | 0.7 | 0.6 | 0.6 | 0.6 | 0.6 |
| QZ size = 40 cm, NTC | 1.0 | 0.6 | 0.6 | 0.6 | 0.6 |
| Note 1: Black text indicates endorsed values, and green text indicates proposed values in this paper. |  |  |  |  |  |

Proposal 1: Adopt the QoQZ MU values in Table 2.

# 3.	Conclusion

RAN5 is asked to endorse the following proposals

Proposal 1: Adopt the QoQZ MU values in Table 2.

# 4.	References

[1] R5-243002, “QoQZ for QZ size 40 cm”, Anritsu, RAN5#103