3GPP TSG-RAN5 Meeting #104 	R5-244837

# Maastricht, Netherlands, 19th Aug. 2024 - 23rd Aug. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3369 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test requirements in FR1 L1-RSRP absolute measurement for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TCs 16.7.4.2.1, 16.7.4.3.1, 16.7.4.4.1: - Noc values for config 3 do not align with Annex F values. - Test requirement values for config 3 do not align with Annex F values |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected Noc values and test requirements according to Annex F. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test could not be properly performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.7.4.2.1, 16.7.4.3.1, 16.7.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

16.7.4.2.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 2 Rx UE

16.7.4.2.1.1	Test purpose

To verify that the SSB based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.4.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.2.

16.7.4.2.1.4	Test description

16.7.4.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.1.1.4.1 with following exceptions:

-	Table 6.7.4.1.1.4.1-1 is replaced by Table 16.7.4.2.1.4.1-1;

-	Table 6.7.4.1.1.4.1-2 is replaced by Table 16.7.4.2.1.4.1-2;

Table 16.7.4.2.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.2.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.1.1.4.2.

16.7.4.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.1.1.4.3.

16.7.4.2.1.5	Test requirement

Same as the thest requirements given in clause 6.7.4.1.1.5 with following exceptions:

-	Table 6.7.4.1.1.5-1 is replaced by Table 16.7.4.2.1.5-1;

-	Table 6.7.4.1.1.5-2 is replaced by Table 16.7.4.2.1.5-2;

Table 16.7.4.2.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2,3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD Configuration | 1,4 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | 3 |  | 20: NRB,c = 51 |  |
| PDSCH RMC | 1,4 |  | SR.1.1 FDD |  |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | 1,4 |  | CR.1.1 FDD |  |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | 1,4 |  | CCR.1.1 FDD |  |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |  |
|  | 3 |  | SSB.2 RedCap FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  |
| TRS configuration | 1,4 |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  |
| reportConfigType | 1~4 |  | periodic |  |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | 1,2,4 | dBm/ 15kHz | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -96.00 |  |
| Note2 | 1,2,4 | dBm/ SCS | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -91.65 | -114+ ΔBG_offset |
|  | 1~4 | dB | 10 | -2.2 |
| SS-RSRP Note3 | 1,2,4 | dBm/ SCS | -84.65 | -119.2 + ΔBG_offset |
|  | 3 |  | -81.65 | -116.2 + ΔBG_offset |
| Io Note3 | 1,2,4 | dBm/ 9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  | 3 | dBm/ 18.36 MHz | -53.36 | -84.07 + ΔBG_offset |
|  | 1~4 | dB | 10 | -2.2 |
| Propagation condition | 1~4 |  | AWGN |  |
| Antenna configuration and Correlation matrix | 1~4 |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table 16.7.4.2.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Band group | Test 2 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_62 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_31 |
|  |  | NR_FDD_FR1_B | RSRP_31 |
|  |  | NR_TDD_FR1_C | RSRP_32 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_32 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_33 |
|  |  | NR_FDD_FR1_F | RSRP_33 |
|  |  | NR_FDD_FR1_G | RSRP_34 |
|  |  | NR_FDD_FR1_H | RSRP_34 |
| Highest reported value (Cell 1) | RSRP_82 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_44 |
|  |  | NR_FDD_FR1_B | RSRP_45 |
|  |  | NR_TDD_FR1_C | RSRP_45 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_46 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_46 |
|  |  | NR_FDD_FR1_F | RSRP_47 |
|  |  | NR_FDD_FR1_G | RSRP_47 |
|  |  | NR_FDD_FR1_H | RSRP_48 |
| Extreme Conditions | Test 1 All bands | Band group | Test 2 |
| Lowest reported value (Cell 1) | RSRP_59 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_26 |
|  |  | NR_FDD_FR1_B | RSRP_27 |
|  |  | NR_TDD_FR1_C | RSRP_27 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_28 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_28 |
|  |  | NR_FDD_FR1_F | RSRP_29 |
|  |  | NR_FDD_FR1_G | RSRP_29 |
|  |  | NR_FDD_FR1_H | RSRP_30 |
| Highest reported value (Cell 1) | RSRP_85 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_49 |
|  |  | NR_FDD_FR1_B | RSRP_49 |
|  |  | NR_TDD_FR1_C | RSRP_50 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_51 |
|  |  | NR_FDD_FR1_F | RSRP_51 |
|  |  | NR_FDD_FR1_G | RSRP_52 |
|  |  | NR_FDD_FR1_H | RSRP_52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 16.7.4.2.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Band group | Test 2 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_63 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_34 |
|  |  | NR_FDD_FR1_B | RSRP_34 |
|  |  | NR_TDD_FR1_C | RSRP_35 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_36 |
|  |  | NR_FDD_FR1_F | RSRP_36 |
|  |  | NR_FDD_FR1_G | RSRP_37 |
|  |  | NR_FDD_FR1_H | RSRP_37 |
| Highest reported value (Cell 1) | RSRP_84 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_47 |
|  |  | NR_FDD_FR1_B | RSRP_48 |
|  |  | NR_TDD_FR1_C | RSRP_48 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_49 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_49 |
|  |  | NR_FDD_FR1_F | RSRP_50 |
|  |  | NR_FDD_FR1_G | RSRP_50 |
|  |  | NR_FDD_FR1_H | RSRP_51 |
| Extreme Conditions | Test 1 All bands | Band group | Test 2 |
| Lowest reported value (Cell 1) | RSRP_60 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_29 |
|  |  | NR_FDD_FR1_B | RSRP_30 |
|  |  | NR_TDD_FR1_C | RSRP_30 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_31 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_31 |
|  |  | NR_FDD_FR1_F | RSRP_32 |
|  |  | NR_FDD_FR1_G | RSRP_32 |
|  |  | NR_FDD_FR1_H | RSRP_33 |
| Highest reported value (Cell 1) | RSRP_87 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_52 |
|  |  | NR_FDD_FR1_B | RSRP_52 |
|  |  | NR_TDD_FR1_C | RSRP_53 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_53 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_54 |
|  |  | NR_FDD_FR1_F | RSRP_54 |
|  |  | NR_FDD_FR1_G | RSRP_55 |
|  |  | NR_FDD_FR1_H | RSRP_55 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

<<Unchanged sections skipped>>

16.7.4.3.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 1 Rx UE

16.7.4.3.1.1	Test purpose

To verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.4.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.4.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.3.

16.7.4.3.1.4	Test description

16.7.4.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.1.4.1 with following exceptions:

-	Table 6.7.4.2.1.4.1-1 is replaced by Table 16.7.4.3.1.4.1-1;

-	Table 6.7.4.2.1.4.1-2 is replaced by Table 16.7.4.3.1.4.1-2;

Table 16.7.4.3.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.3.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.3.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.1.4.2.

16.7.4.3.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.1.4.3.

16.7.4.3.1.5	Test requirement

Same as the thest requirements given in clause 6.7.4.2.1.5 with following exceptions:

-	Table 6.7.4.2.1.5-1 is replaced by Table 16.7.4.3.1.5-1;

-	Table 6.7.4.2.1.5-2 is replaced by Table 16.7.4.3.1.5-2;

Table 16.7.4.3.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2,3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD Configuration | 1,4 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | 3 |  | 20: NRB,c = 51 |  |
| PDSCH RMC | 1,4 |  | SR.1.1 FDD |  |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | 1,4 |  | CR.1.1 FDD |  |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | 1,4 |  | CCR.1.1 FDD |  |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |  |
|  | 3 |  | SSB.2 RedCap FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  |
| TRS configuration | 1,4 |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  |
| CSI-RS configuration | 1,4 |  | CSI-RS 1.2 FDD |  |
|  | 2 |  | CSI-RS 1.2 TDD |  |
|  | 3 |  | CSI-RS 2.2 TDD |  |
| reportConfigType | 1~4 |  | periodic |  |
| reportQuantity | 1~4 |  | cri-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | 1,2,4 | dBm/ 15kHz | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -96.00 |  |
| Note2 | 1,2,4 | dBm/ SCS | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -91.65 | -114+ ΔBG_offset |
|  | 1~4 | dB | 10 | -2.2 |
| CSI-RSRP Note3 | 1,2,4 | dBm/ SCS | -84.65 | -119.2 + ΔBG_offset |
|  | 3 |  | -81.65 | -116.2 + ΔBG_offset |
| Io Note3 | 1,2,4 | dBm/ 9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  | 3 | dBm/ 18.36 MHz | -53.36 | -84.07 + ΔBG_offset |
|  | 1~4 | dB | 10 | -2.2 |
| Propagation condition | 1~4 |  | AWGN |  |
| Antenna configuration | 1~4 |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table 16.7.4.3.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Band group | Test 2 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_62 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_31 |
|  |  | NR_FDD_FR1_B | RSRP_31 |
|  |  | NR_TDD_FR1_C | RSRP_32 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_32 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_33 |
|  |  | NR_FDD_FR1_F | RSRP_33 |
|  |  | NR_FDD_FR1_G | RSRP_34 |
|  |  | NR_FDD_FR1_H | RSRP_34 |
| Highest reported value (Cell 1) | RSRP_82 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_44 |
|  |  | NR_FDD_FR1_B | RSRP_45 |
|  |  | NR_TDD_FR1_C | RSRP_45 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_46 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_46 |
|  |  | NR_FDD_FR1_F | RSRP_47 |
|  |  | NR_FDD_FR1_G | RSRP_47 |
|  |  | NR_FDD_FR1_H | RSRP_48 |
| Extreme Conditions | Test 1 All bands | Band group | Test 2 |
| Lowest reported value (Cell 1) | RSRP_59 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_26 |
|  |  | NR_FDD_FR1_B | RSRP_27 |
|  |  | NR_TDD_FR1_C | RSRP_27 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_28 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_28 |
|  |  | NR_FDD_FR1_F | RSRP_29 |
|  |  | NR_FDD_FR1_G | RSRP_29 |
|  |  | NR_FDD_FR1_H | RSRP_30 |
| Highest reported value (Cell 1) | RSRP_85 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_49 |
|  |  | NR_FDD_FR1_B | RSRP_49 |
|  |  | NR_TDD_FR1_C | RSRP_50 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_51 |
|  |  | NR_FDD_FR1_F | RSRP_51 |
|  |  | NR_FDD_FR1_G | RSRP_52 |
|  |  | NR_FDD_FR1_H | RSRP_52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 16.7.4.3.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Band group | Test 2 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_63 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_34 |
|  |  | NR_FDD_FR1_B | RSRP_34 |
|  |  | NR_TDD_FR1_C | RSRP_35 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_36 |
|  |  | NR_FDD_FR1_F | RSRP_36 |
|  |  | NR_FDD_FR1_G | RSRP_37 |
|  |  | NR_FDD_FR1_H | RSRP_37 |
| Highest reported value (Cell 1) | RSRP_84 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_47 |
|  |  | NR_FDD_FR1_B | RSRP_48 |
|  |  | NR_TDD_FR1_C | RSRP_48 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_49 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_49 |
|  |  | NR_FDD_FR1_F | RSRP_50 |
|  |  | NR_FDD_FR1_G | RSRP_50 |
|  |  | NR_FDD_FR1_H | RSRP_51 |
| Extreme Conditions | Test 1 All bands | Band group | Test 2 |
| Lowest reported value (Cell 1) | RSRP_60 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_29 |
|  |  | NR_FDD_FR1_B | RSRP_30 |
|  |  | NR_TDD_FR1_C | RSRP_30 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_31 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_31 |
|  |  | NR_FDD_FR1_F | RSRP_32 |
|  |  | NR_FDD_FR1_G | RSRP_32 |
|  |  | NR_FDD_FR1_H | RSRP_33 |
| Highest reported value (Cell 1) | RSRP_87 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_52 |
|  |  | NR_FDD_FR1_B | RSRP_52 |
|  |  | NR_TDD_FR1_C | RSRP_53 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_53 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_54 |
|  |  | NR_FDD_FR1_F | RSRP_54 |
|  |  | NR_FDD_FR1_G | RSRP_55 |
|  |  | NR_FDD_FR1_H | RSRP_55 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

<<Unchanged sections skipped>>

16.7.4.4.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 2 Rx UE

16.7.4.4.1.1	Test purpose

To verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.4.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.4.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.4.4.

16.7.4.4.1.4	Test description

16.7.4.4.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.1.4.1 with following exceptions:

-	Table 6.7.4.2.1.4.1-1 is replaced by Table 16.7.4.4.1.4.1-1;

-	Table 6.7.4.2.1.4.1-2 is replaced by Table 16.7.4.4.1.4.1-2;

Table 16.7.4.4.1.4.1-1: Supported test configurations

| Test Config | Description |
| --- | --- |
| 16.7.4.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.4.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.4.4.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.4.4.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.4.4.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.4.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

16.7.4.4.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.4.2.1.4.2.

16.7.4.4.1.4.3	Message contents

Same as the message contents given in clause 6.7.4.2.1.4.3.

16.7.4.4.1.5	Test requirement

Same as the thest requirements given in clause 6.7.4.2.1.5 with following exceptions:

-	Table 6.7.4.2.1.5-1 is replaced by Table 16.7.4.4.1.5-1;

-	Table 6.7.4.2.1.5-2 is replaced by Table 16.7.4.4.1.5-2;

Table 16.7.4.4.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2,3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD Configuration | 1,4 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | 3 |  | 20: NRB,c = 51 |  |
| PDSCH RMC | 1,4 |  | SR.1.1 FDD |  |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC | 1,4 |  | CR.1.1 FDD |  |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | 1,4 |  | CCR.1.1 FDD |  |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1,2,4 |  | SSB.3 FR1 |  |
|  | 3 |  | SSB.2 RedCap FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  |
| TRS configuration | 1,4 |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  |
| CSI-RS configuration | 1,4 |  | CSI-RS 1.2 FDD |  |
|  | 2 |  | CSI-RS 1.2 TDD |  |
|  | 3 |  | CSI-RS 2.2 TDD |  |
| reportConfigType | 1~4 |  | periodic |  |
| reportQuantity | 1~4 |  | cri-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | 1,2,4 | dBm/ 15kHz | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -96.00 |  |
| Note2 | 1,2,4 | dBm/ SCS | -94.65 | -117+ ΔBG_offset |
|  | 3 |  | -91.65 | -114+ ΔBG_offset |
|  | 1~4 | dB | 10 | -2.2 |
| CSI-RSRP Note3 | 1,2,4 | dBm/ SCS | -84.65 | -119.2+ ΔBG_offset |
|  | 3 |  | -81.65 | -116.2 + ΔBG_offset |
| Io Note3 | 1,2,4 | dBm/ 9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  | 3 | dBm/ 18.36 MHz | -53.36 | -84.07 + ΔBG_offset |
|  | 1~4 | dB | 10 | -2.2 |
| Propagation condition | 1~4 |  | AWGN |  |
| Antenna configuration and Correlation matrix | 1~4 |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table 16.7.4.4.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Band group | Test 2 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_62 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_31 |
|  |  | NR_FDD_FR1_B | RSRP_31 |
|  |  | NR_TDD_FR1_C | RSRP_32 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_32 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_33 |
|  |  | NR_FDD_FR1_F | RSRP_33 |
|  |  | NR_FDD_FR1_G | RSRP_34 |
|  |  | NR_FDD_FR1_H | RSRP_34 |
| Highest reported value (Cell 1) | RSRP_82 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_44 |
|  |  | NR_FDD_FR1_B | RSRP_45 |
|  |  | NR_TDD_FR1_C | RSRP_45 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_46 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_46 |
|  |  | NR_FDD_FR1_F | RSRP_47 |
|  |  | NR_FDD_FR1_G | RSRP_47 |
|  |  | NR_FDD_FR1_H | RSRP_48 |
| Extreme Conditions | Test 1 All bands | Band group | Test 2 |
| Lowest reported value (Cell 1) | RSRP_59 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_26 |
|  |  | NR_FDD_FR1_B | RSRP_27 |
|  |  | NR_TDD_FR1_C | RSRP_27 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_28 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_28 |
|  |  | NR_FDD_FR1_F | RSRP_29 |
|  |  | NR_FDD_FR1_G | RSRP_29 |
|  |  | NR_FDD_FR1_H | RSRP_30 |
| Highest reported value (Cell 1) | RSRP_85 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_49 |
|  |  | NR_FDD_FR1_B | RSRP_49 |
|  |  | NR_TDD_FR1_C | RSRP_50 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_51 |
|  |  | NR_FDD_FR1_F | RSRP_51 |
|  |  | NR_FDD_FR1_G | RSRP_52 |
|  |  | NR_FDD_FR1_H | RSRP_52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 16.7.4.4.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Band group | Test 2 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | RSRP_63 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_34 |
|  |  | NR_FDD_FR1_B | RSRP_34 |
|  |  | NR_TDD_FR1_C | RSRP_35 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_35 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_36 |
|  |  | NR_FDD_FR1_F | RSRP_36 |
|  |  | NR_FDD_FR1_G | RSRP_37 |
|  |  | NR_FDD_FR1_H | RSRP_37 |
| Highest reported value (Cell 1) | RSRP_84 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_47 |
|  |  | NR_FDD_FR1_B | RSRP_48 |
|  |  | NR_TDD_FR1_C | RSRP_48 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_49 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_49 |
|  |  | NR_FDD_FR1_F | RSRP_50 |
|  |  | NR_FDD_FR1_G | RSRP_50 |
|  |  | NR_FDD_FR1_H | RSRP_51 |
| Extreme Conditions | Test 1 All bands | Band group | Test 2 |
| Lowest reported value (Cell 1) | RSRP_60 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_29 |
|  |  | NR_FDD_FR1_B | RSRP_30 |
|  |  | NR_TDD_FR1_C | RSRP_30 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_31 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_31 |
|  |  | NR_FDD_FR1_F | RSRP_32 |
|  |  | NR_FDD_FR1_G | RSRP_32 |
|  |  | NR_FDD_FR1_H | RSRP_33 |
| Highest reported value (Cell 1) | RSRP_87 | NR_FDD_FR1_A, NR_TDD_FR1_A | RSRP_52 |
|  |  | NR_FDD_FR1_B | RSRP_52 |
|  |  | NR_TDD_FR1_C | RSRP_53 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | RSRP_53 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | RSRP_54 |
|  |  | NR_FDD_FR1_F | RSRP_54 |
|  |  | NR_FDD_FR1_G | RSRP_55 |
|  |  | NR_FDD_FR1_H | RSRP_55 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

<<End of change>>