3GPP TSG-RAN5 Meeting #104	R5-244847

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1538 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS WLAN test case G.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case G.8.2 (“Initial registration on new P-CSCF / WLAN”) needs to be added as per P-CSCF Restoration work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case G.8.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage doesnot exist for P-CSCF restoration procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, G.8.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0342 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact xxx and yyy in References and Specific Message Contents to be resolved. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[4]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[5]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

…

[150]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[151]	GSMA PRD NG.108: "IMS Profile for Voice and SMS for UE category M1"

[152]	3GPP TS 24.390: " Unstructured Supplementary Service Data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem IMS; Stage 3"

[153]	IETF RFC 5646: " Tags for Identifying Languages"

[154]	IETF RFC 7315: "P-Access-Network-Info ABNF Update"

[155]	3GPP TS 23.237: " IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[156]	3GPP TS 34.229-5: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)".

[157]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS"

[158]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

[xxx]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[yyy]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

<End of modified section>

<Start of modified section>

## G.8.2		Initial registration on new P-CSCF / WLAN

### G.8.2.1	Definition

Test to verify that the UE supporting P-CSCF reselection can correctly perform initial registration for IMS services when it receives new P-CSCF addresses in IKEv2 INFORMATIONAL Exchange request.  The process consists of sending initial registration to the newly received P-CSCF address, authenticating the user and finally subscribing the registration event package for the registered default public user identity using WLAN access and is described in TS 24.229[10].

### G.8.2.2	Conformance requirement

[TS 24.229, clause R.2.2.1C]:

A UE supporting the P-CSCF restoration procedure performs one of the following procedures:

A)	if the UE used method IV for P-CSCF discovery, the UE has previously sent the "P-CSCF Re-selection support" PCO indicator (in trusted non-3GPP access network) or the P-CSCF_RESELECTION_SUPPORT IKEv2 attribute (in untrusted non-3GPP access network) during the PDN connection establishment, and the UE receives one or more P-CSCF address(es) during the TWAG initiated PDN connectivity modification procedure (in trusted non-3GPP access network) or the ePDG initiated modification (in untrusted non-3GPP access network), then the UE shall acquire a P-CSCF address from the one or more P-CSCF addresse(s). If more than one P-CSCF addresses of the same IP address type are included, then the UE shall assume that the more than one P-CSCF addresses of the same IP address type are prioritised with the first P-CSCF address within the Protocol Configuration Options information element or within the IKEv2 configuration payload as the P-CSCF address having the highest priority; and

…

When the UE has acquired the P-CSCF address, the UE shall perform an initial registration as specified in subclause 5.1.

NOTE:	For UEs using procedure A described above, the network ensures that P-CSCF address(es) received in the Protocol Configuration Options information element during TWAG initiated PDN connectivity modification procedure (in trusted non-3GPP access network), and P-CSCF address(es) received in the P-CSCF_IP6_ADDRESS attribute, the P-CSCF_IP4_ADDRESS attribute or both during ePDG initiated modification (in untrusted non-3GPP access network) are sent only during P-CSCF restoration procedures as defined in subclause 5 of 3GPP TS 23.380 [7D].

[TS 24.302, clause 7.2.3.2]:

This procedure is used if P-CSCF restoration extension for untrusted WLAN is supported as specified in 3GPP TS 23.380 [66].

If the UE receives the P_CSCF_IP6_ADDRESS attribute, the P_CSCF_IP4_ADDRESS attribute or both as specified in IETF RFC 7651 [64] in the CFG_REQUEST configuration payload within the INFORMATIONAL request from the ePDG and the UE supports P-CSCF restoration extension for untrusted WLAN as specified in 3GPP TS 23.380 [66], the UE shall reply with an INFORMATIONAL response and proceed as specified in subclause 5.6.5.2 of 3GPP TS 23.380 [66]. The INFORMATIONAL response shall include the received P_CSCF_IP6_ADDRESS attribute or the P_CSCF_IP4_ADDRESS attribute or both in the CFG_REPLY Configuration Payload. The P_CSCF_IP6_ADDRESS attribute shall contain no value and the length field shall be set to 0. The P_CSCF_IP4_ADDRESS shall contain no value and the length field shall be set to 0.

Reference(s)

3GPP TS 24.229 [10], clause R.2.2.1C and TS 24.302[143], clause 7.2.3.2.

### G.8.2.3	Test purpose

To verify that after receiving new P-CSCF address from the network in INFORMATIONAL request message, the UE acquires a P-CSCF address from the one or more P-CSCF addresse(s) and performs initial IMS registration.

### G.8.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated the IP-CAN to the Evolved Packet Core (EPC) via Wireless Local Access Network (WLAN). The UE has registered to IMS according to the procedures C.2c.

Test procedure

1	SS sends an IKEv2 INFORMATIONAL Exchange request message containing new P-CSCF addresses in P_CSCF_IP4_ADDRESS attribute, the P_CSCF_IP6_ADDRESS attribute or both in CFG_REQUEST Configuration payload.

2	UE responds with IKEv2 INFORMATIONAL Exchange response message and then acquires the first P-CSCF address having the highest preference.

3	SS waits for the UE to send a REGISTER request using the new P-CSCF address received at Step 1.

4	Continue test execution with the Generic test procedure in Annex C.2c from Step 3.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | IKEv2 INFORMATIONAL Exchange request | The SS sends two new P-CSCF addresses different from the ones received at preamble in the P_CSCF_IP4_ADDRESS attribute, the P_CSCF_IP6_ADDRESS attribute or both in the CFG_REQUEST configuration payload within the INFORMATIONAL request. |
| 2 |  |  | IKEv2 INFORMATIONAL Exchange response | UE responds with INFORMATIONAL response including the received P_CSCF_IP6_ADDRESS attribute or the P_CSCF_IP4_ADDRESS attribute or both in the CFG_REPLY Configuration Payload. The P_CSCF_IP6_ADDRESS attribute shall contain no value and the length field shall be set to 0. The P_CSCF_IP4_ADDRESS shall contain no value and the length field shall be set to 0. |
|  |  |  | Below messages are to be observed at the first P-CSCF address sent at Step 1. |  |
| 3 |  |  | REGISTER | UE sends a new REGISTER request using the first P-CSCF address received at step 1. |
| 4-10 |  |  | Continue with Annex C.2c step 3. | Execute the Generic test procedure Annex C.2c steps 3-9 in order to get the UE in a stable registered state. |

Specific Message Contents

IKE_AUTH Request (Preamble, Step 2 of TS 33.402, Fig 8.2.2-1)

Use the default message as TS 33.402[xxx] Fig 8.2.2-1 with the following exception:

| Header/param | Value/remark | Reference |
| --- | --- | --- |
| Notify payload |  |  |
| Protocol ID | 0 | RFC 7296[yyy] |
| SPI Size | 0 |  |
| Notify Message type | 41304 (P-CSCF_RESELECTION_SUPPORT) | TS 24.302[143] |

### G.8.2.5	Test requirements

The UE shall send a REGISTER request and perform initial registration using the new P-CSCF address received in P-CSCF_IP6_ADDRESS attribute, the P-CSCF_IP4_ADDRESS attribute or both of IKEv2 INFORMATIONAL request configuration payload.

<End of modified section>