3GPP TSG-RAN5 Meeting #104	R5-244851

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3303 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of RRC message for Further NR mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Further NR mobility enhancements test cases, the RRCReconfiguration message needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRCReconfiguration message was updated according to TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of LTM configuration for Further NR mobility enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.6.1	Contents of RRC messages

–	CounterCheck

[Skip unchanged text]

–	RRCReconfiguration

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR, NR-DC |
|  |  | RadioBearerConfig with conditions SRB2 and REEST |  | REEST |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  |  | CellGroupConfig with condition NR-DC_SCG | OCTET STRING (CONTAINING CellGroupConfig) | NR-DC_SCG |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_HO |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_MEAS |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_SCell_add |
|  |  | Not present |  |  |
| measConfig |  | Not present |  |  |
|  |  | MeasConfig | Measurements configuration | NR_MEAS, EN-DC_MEAS, IRAT_MEAS |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR, SCell_add, NR_MEAS, NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC, REEST, L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | SCell_add |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | NR_MEAS |
|  |  | CellGroupConfig with condition REEST | OCTET STRING (CONTAINING CellGroupConfig) | REEST |
|  |  | CellGroupConfig with condition RELAY | OCTET STRING (CONTAINING CellGroupConfig) | RELAY |
| fullConfig |  | Not present |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  | NR |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |
| } |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  | MasterKeyChange |
| keySetChangeIndicator |  | true |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| nas-Container |  | Not present |  |  |
|  |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  | Inter_Sys_HO |
| } |  |  |  |  |
| dedicatedSIB1-Delivery |  | Not present |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC, NE-DC,L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1540 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig |  | Not present |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  | NR-DC, NE-DC |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG | OCTET STRING (CONTAINING RRCReconfiguration) | NR-DC |
| eutra-SCG |  | RRCConnectionReconfiguration according TS 36.508 [2], table 4.6.1-8 with condition NE-DC | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | NE-DC |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioBearerConfig2 |  | Not present |  | NE-DC |
|  |  | RadioBearerConfig with condition DRBn and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) | NR-DC |
| sk-Counter |  | SK-Counter |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SIDELINK, DAPS_HO_ReleaseSource, CHO, CPA, CPC,L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1610 |  | Not present |  |  |
| bap-Config-r16 |  | Not present |  |  |
| iab-IP-AddressConfigurationList-r16 |  | Not present |  |  |
| conditionalReconfiguration-r16 |  | Not present |  |  |
|  |  | ConditionalReconfiguration |  | CHO, CPA, CPC |
| daps-SourceRelease-r16 |  | Not present |  |  |
|  |  | true |  | DAPS_HO_ReleaseSource |
| t316-r16 |  | Not present |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |
| onDemandSIB-Request-r16 |  | Not present |  |  |
| dedicatedPosSysInfoDelivery-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  | SIDELINK |
| setup |  | SL-ConfigDedicatedNR-r16 |  |  |
| } |  |  |  |  |
| sl-ConfigDedicatedEUTRA-Info-r16 |  | Not present |  |  |
| smtc-r16 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RelayUE, L2RemoteUE, SHRT304, SHRT310, SHRT312, SHRRLF, DeactivatedSCG |
| otherConfig-v1700 |  | Not present |  |  |
| otherConfig-v1700 SEQUENCE{ |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| ul-GapFR2-PreferenceConfig-r17 |  | Not present |  |  |
| musim-GapAssistanceConfig-r17 |  | Not present |  |  |
| musim-LeaveAssistanceConfig-r1 |  | Not present |  |  |
| successHO-Config-r17 |  | Not present |  |  |
| successHO-Config-r17 CHOICE { |  |  |  | SHRT304, SHRT310, SHRT312, SHRRLF |
| setup SEQUENCE { |  |  |  |  |
| thresholdPercentageT304-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT304 |
| thresholdPercentageT310-r17 |  | Not present |  |  |
|  |  | p40 |  | SHRT310 |
| thresholdPercentageT312-r17 |  | Not present |  |  |
|  |  | p20 |  | SHRT312 |
| sourceDAPS-FailureReporting-r17 |  | Not present |  |  |
|  |  | true |  | SHRRLF |
| } |  |  |  |  |
| } |  |  |  |  |
| maxBW-PreferenceConfigFR2-2-r17 |  | Not present |  |  |
| maxMIMO-LayerPreferenceConfigFR2-2-r17 |  | Not present |  |  |
| minSchedulingOffsetPreferenceConfigExt-r17 |  | Not present |  |  |
| rlm-RelaxationReportingConfig-r17 |  | Not present |  |  |
| bfd-RelaxationReportingConfig-r17 |  | Not present |  |  |
| scg-DeactivationPreferenceConfig-r17 |  | Not present |  |  |
| rrm-MeasRelaxationReportingConfig-r17 |  | Not present |  |  |
| propDelayDiffReportConfig-r17 |  | Not present |  |  |
| } |  |  |  |  |
| sl-L2RelayUE-Config-r17 |  | Not present |  |  |
| sl-L2RelayUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RelayUE-Config with condition SL-SRB0 and SL-SRB1 |  | L2RelayUE |
| } |  |  |  |  |
| sl-L2RemoteUE-Config-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  | L2RemoteUE |
| setup |  | SL-L2RemoteUE-Config with condition SL-SRB2 and SL-DRB1 |  |  |
| } |  |  |  |  |
| dedicatedPagingDelivery-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigNR-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 |  | Not present |  |  |
| musim-GapConfig-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 CHOICE { |  |  |  | NR-FR2- Uplink-Gaps |
| setup |  | UL-GapFR2-Config-r17 |  |  |
| } |  |  |  |  |
| scg-State-r17 |  | Not present |  |  |
|  |  | deactivated |  | DeactivatedSCG |
| appLayerMeasConfig-r17 |  | Not present |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | LTM |
| ltm-Config-r18 CHOICE { |  |  |  |  |
| setup |  | LTM-Config |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| EN-DC_MEAS | An EN-DC measurement is configured |
| IRAT_MEAS | An IRAT measurement is configured in the NR PCell |
| NR_MEAS | A NR measurement is configured |
| NR | NR connected to 5GC |
| NR-DC | NR-NR Dual Connectivity is configured |
| NR-DC_SCG | Add SCG side configuration (NR-DC) |
| EN-DC_HO | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| EN-DC_SCell_add | Add SCell (EN-DC) |
| MasterKeyChange | MasterKeyUpdate when performing ReconfigurationWithSync and indicating a change of the AS security algorithms associated to the master key |
| Inter_Sys_HO | Used during inter-system handover to NR procedure |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure |
| SIDELINK | For NR sidelink dedicated configuration |
| DAPS_HO_ReleaseSource | The source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released. |
| CHO | Conditional handover |
| CPA | Conditional PSCell addition |
| CPC | Conditional PSCell change |
| NE-DC | NR E-UTRA Dual Connectivity |
| L2RelayUE | For L2 U2N Relay UE test cases. |
| L2RemoteUE | For L2 U2N Remote UE test cases. |
| RELAY | Add Uu Relay RLC channel configuration |
| NR-FR2-Upink-Gaps | Configure Uplink Gaps for NR FR2 |
| SHRT304 | Report the successful handover information to the network for the elapsed time of the timer T304 over the threshold |
| SHRT310 | Report the successful handover information to the network for the elapsed time of the timer T310 over the threshold |
| SHRT312 | Report the successful handover information to the network the ror elapsed time of the timer T312 over the threshold |
| SHRRLF | Report the successful handover information to the network for radio link failure |
| DeactivatedSCG | Indicate that SCG is in deactivated state |
| LTM | L1/L2-triggered mobility |

[End of change]