3GPP TSG RAN5 Meeting #104	 	R5-244893

Maastricht, Netherlands, 19th - 23rd August 2024

Source:		Rohde & Schwarz

Title:	On the MU for n259

Agenda Item:		5.4.20

Document for:	Discussion and Endorsement

# Introduction

In previous meetings, the MU for FR2c has been discussed [1 to 14]. However, some MU values have not been finalized. We continue the discussion with this contribution.

# Discussion

In this paper, we focus our analysis on PC3, CHBW ≤ 400 MHz, IFF with a maximum device size of 40cm.

## ACS and In-band Blocking

In the RAN5 #102 meeting, a value of [9.46 dB] has been agreed for the total MU of the in-band blocking and ACS TCs for CHBW ≤ 100 MHz, IFF and PC3. In addition relaxations for the interferer power levels have been agreed within square brackets. In order to progress, we propose to remove these square brackets.

Proposal 1: Remove the square brackets around the MU value and relaxation of the interferer power level for in-band blocking and ACS for FR2c, CHBW ≤ 100 MHz, IFF and PC3.

The caption of Table B.21.2-4 states that the relaxation values for the interferer are for a CHBW of 200 MHz. However, 200 MHz has been considered as not testable as documented in Note 5 in Table 7.5.5-2 in TS 38.521-2 [15].

Table B.21.2-4: Adjacent channel selectivity requirement relaxation considered in MU assessment for 100 MHz EIRP measurement (f=23.45GHz, 32.125GHz, 40.8GHz, 44.3GHz, Quiet Zone size ≤ 30 cm)

| Power Class | Frequency Range | Relaxation (dB) |
| --- | --- | --- |
| PC3 | FR2a | 0 |
|  | FR2b | 4.8 |
|  | FR2c | [6.8] |
| PC1 | FR2a | 0 |
|  | FR2b | 0 |
| PC5 | FR2a | 0 |

Table 7.5.5-2: Test parameters for adjacent channel selectivity, Case 1, PC3

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| < some rows omitted > |  |  |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB NOTE 5 | REFSENS +35.5dB NOTE 5 |
| PInterferer for band n260 | dBm | REFSENS + 34.5 - 1.8 dB NOTE 4 | REFSENS +34.5 - 4.8 dB NOTE 4 | REFSENS +34.5dB NOTE 5 | REFSENS +34.5dB NOTE 5 |
| PInterferer for band n259 | dBm | REFSENS + 34.5 - [3.8] dB NOTE 4 | REFSENS +34.5 - [6.8] dB NOTE 4 | REFSENS +34.5 dB NOTE 5 | REFSENS +34.5 dB NOTE 5 |
| PInterferer for band n262 | dBm | REFSENS + 34.5 - TBD dB NOTE 4 | REFSENS +34.5 - TBD dB NOTE 4 | REFSENS +34.5dB NOTE 5 | REFSENS +34.5dB NOTE 5 |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in subclause 7.3.2.5. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: Core requirement cannot be tested due to testability issue and test requirement for wanted signal and interferer includes relaxation to achieve feasible interferer power level. NOTE 5: Core requirement cannot be tested due to testability issue. NOTE 6: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 7: For PC7 RedCap UEs only 50MHz and 100MHz Test Channel Bandwidths are applicable |  |  |  |  |  |

Proposal 2: Correct the caption of Table B.21.2-4 to 100 MHz.

## Minimum Output power

In the RAN5 #102 meeting, relaxation values within square brackets for minimum output power have been agreed. We propose to remove the square brackets in order to progress.

Proposal 3: Remove the square brackets around the relaxation value, influence of noise and MU value for minimum output power for FR2c and PC3.

## MPR

In the RAN5 #102 meeting, a value of [0.55 dB] has been agreed for the influence of noise. In order to progress, we propose to remove these square brackets.

Proposal 4: Remove the square brackets around the influence of noise and MU value for MPR for FR2c, IFF and PC3.

## Occupied Bandwidth

Based on our MU assessment, we propose 1.3% CBW for FR2c 200 MHz. We consider 400 MHz with the current test procedure as not testable. As a remedy, we  propose to measure the EIRP spectrum distribution within 1.3-times over the requirement for 400 MHz. With this update, we propose an OBW MU of 1.3% CBW for 400 MHz.

Proposal 5: Apply a MU of 1.3% CBW for FR2c 200 MHz, IFF, PC3 and SISO.

Proposal 6: Update the test procedure to measure the EIRP spectrum over 1.3-times over the requirement for FR2c 400 MHz and apply a MU of 1.3% CBW for FR2c 400 MHz, IFF, PC3 and SISO.

Accompanying CR in Ref. [16] documents the MU assessment in TR 38.903 [17].

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2c |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | Default: 400 MHz ACS & IBB: 100 MHz ACLR: 200 MHz |

# Conclusion

In summary, we have assessed the uncertainty in the frequency range of FR2c and make the following proposals:

Proposal 1: Remove the square brackets around the MU value and relaxation of the interferer power level for in-band blocking and ACS for FR2c, CHBW ≤ 100 MHz, IFF and PC3.

Proposal 2: Correct the caption of Table B.21.2-4 to 100 MHz.

Proposal 3: Remove the square brackets around the relaxation value, influence of noise and MU value for minimum output power for FR2c and PC3.

Proposal 4: Remove the square brackets around the influence of noise and MU value for MPR for FR2c, IFF and PC3.

Proposal 5: Apply a MU of 1.3% CBW for FR2c 200 MHz, IFF, PC3 and SISO.

Proposal 6: Update the test procedure to measure the EIRP spectrum over 1.3-times over the requirement for FR2c 400 MHz and apply a MU of 1.3% CBW for FR2c 400 MHz, IFF, PC3 and SISO.

# References

- R5-233174, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #99, May 2023

- R5-232983, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #99, May 2023

- R5-235140, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #100, August 2023

- R5-235753, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #100, August 2023

- R5-237725, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-237723, Discussion on MU and TT analysis for FR2c, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-237724, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #101, November 2023

- R5-241344, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #102, February 2024

- R5-241858, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #102, February 2024

- R5-240401, Discussion on MU and TT analysis for FR2c, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #102, February 2024

- R5-243100, On the MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- R5-243724, Discussion on MU and TT analysis for FR2c, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- R5-243725, MU discussion on FR2c, Anritsu, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- R5-242998, On testability issue of FR2c OBW, Anritsu, 3GPP TSG RAN WG 5 Meeting #103, May 2024

- TS 38.521-2, V18.3.0, 3GPP TSG RAN WG5, June 2024

- R5-244894, Documentation of MU for n259, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #104, August 2024

- TR 38.903, V18.3.0, 3GPP TSG RAN WG5, June 2024