3GPP TSG-RAN5 Meeting #104	R5-244906

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1817 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case for FR2 additional spurious emissions for UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case FR2 additional spurious emissions for UL MIMO is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case 6.5B.4.4D Additional Spurious Emissions for Inter-band including FR2 for UL MIMO has been introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test coverage cannot be fulfilled and the work plan will not progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 6.5B.4.4D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522 CR 0465 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Applicability update in CR R5-244899, update of MU and TT in CR R5-244898 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.5B.4.4_1.3	Additional Spurious Emissions for Inter-band including FR2 (4 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Test procedure only includes the testing of smartphone and is FFS for laptop and FWA.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5B.4.4_1.3.1	Test purpose

Same test purpose as in clause 6.5A.3.3.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.4.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4_1.3.4	Test description

6.5B.4.4_1.3.4.1	Initial condition

Same test description as in clause 6.5A.3.3.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.3.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.3.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.3.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.3.3.3.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.5B.4.4D	Additional Spurious Emissions for Inter-band including FR2 for UL MIMO

Editor’s note: This clause is complete for Band n257 and n258 and PC3. The following aspects of the clause are for future consideration:

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5B.4.4D.1	Test purpose

Same test purpose as in clause 6.5D.3.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4D.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 and UL MIMO.

6.5B.4.4D.3	Minimum conformance requirements

The additional spurious emissions requirements specified for E-UTRA in clause 6.6.3.3 and 6.6.3.3A of TS 36.101 [5] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.5.3.3, 6.5A.3.3 and 6.5D.3 of TS 38.101-2 [3] apply for each component carrier.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

Same minimum conformance requirements as in clause 6.5D.3.3.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.4.4D.4	Test description

Same test description as in clause 6.5D.3.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5D.3.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5D.3.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5D.3.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4D.5	Test Requirements

Same test requirement as in clause 6.5D.3.3.5 in TS 38.521-2 [9] for the NR carrier.

### 6.5B.5	Transmit intermodulation for DC

< End of changes >