3GPP TSG-RAN WG5 Meeting # 104	R5-244926

# Maastricht, Netherlands,  August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2942 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on 6.2D for test requirements for UL MIMO transmitter power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_bands_UL_MIMO_PC3_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Pcmax lower tolerance requirements for UL MIMO have been corrected in the core spec from 6dB to 5dB for the case of 20 ≤ PCMAX < 21 due to the rule of the tolerance changes in a monotonic way with the variation of output power. The test requirements for the transmitter power for UL MIMO should be corrected accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the Pcmax lower tolerance requirements for UL MIMO in the case of 20 ≤ PCMAX < 21. - Correct T(PCMAX_L,c) and Lower limit for UE Power Class test requirements for UL MIMO in other cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirements for UL MIMO transmitter power will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1.3, 6.2D.1.5, 6.2D.2.5, 6.2D.2_1.5, 6.2D.3.5, 6.2D.3_1.5, 6.2D.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2D	Transmitter power for UL MIMO

6.2D.1	UE maximum output power for UL MIMO

Editor’s Note:

- No test points are defined for 2-layer UL MIMO since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- No test points are defined for Power Class 1.5 non-FWA UEs supporting UFPTx since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- TP analysis needs to be updated regarding Note 3 in Table 6.2D.1.4.1-2

6.2D.1.1	Test purpose

To verify that the error of the UE maximum output power for UL MIMO does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2D.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 FWA UEs, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n2 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n5 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n8 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n24 |  |  |  |  |  |  | 23 | +2/-41 |  |  |
| n25 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n28 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n38 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n40 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n41 |  |  | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n70 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n97 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.1.3-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UEs supporting uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1.3-1 shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1.3-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [9]. NOTE 3: For PUSCH configured with ULFPTx Mode set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2 apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling with the following exception: for UEs indicating txDiversity-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass.

A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.1.

6.2D.1.4	Test description

6.2D.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1.4.1-1: Test Configuration Table for 2-layer UL MIMO

NOTE:	No test points are defined since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2D.1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx) for Power Class 3, Power Class 2 and Power Class 1.5 FWA UEs

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Only Test IDs 1, 2 and 3 apply to Power Class 2 UEs if TxD is indicated. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.1.4.3.

6.2D.1.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO:

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

Sub-test for ULFPTx

4.	If the UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on the UE supported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

6.2D.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1.5-1.

Table 6.2D.1.5-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-3-TT |  |  |
| n2 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n3 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n5 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n7 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n8 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n24 |  |  |  |  |  |  | 23 | +2+TT/-41-TT |  |  |
| n25 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n28 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n30 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n34 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n38 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n39 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n40 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n41 |  |  | 29 | _+2+TT/-31-TT | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n48 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n66 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n70 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n71 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n77 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n78 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n79 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n97 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.1.5-2 NOTE 3: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<<Unchanged texts are omitted>>

6.2D.2	UE maximum output power reduction for UL MIMO

6.2D.2.1	Test purpose

To verify that the power reduction of UE due to higher order modulations and transmit bandwidth configuration does not exceed the specified maximum power reduction.

6.2D.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed.

<<Unchanged texts are omitted>>

6.2D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1, Table 6.2D.2.5-2, Table 6.2D.2.5-2a, Table 6.2D.2.5-3, or Table 6.2D.2.5-4. The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1a, Table 6.2D.2.5-2b, Table 6.2D2.5-2c, Table 6.2D.2.5-3a, or Table 6.2D.2.5-4a.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n24, n25, n28, n30, n34, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79, n97) for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. NOTE 4: Applicable for band n24. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-1a: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n24, n25, n28, n30, n34, n38, n41, n48, n66, n70, n71, n77, n78, n79) for Power Class 3 supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （14.02 - TT） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. NOTE 4: Only applicable for band n24. NOTE 5: For band n24 when transmission bandwidths are not confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged texts are omitted>>

Table 6.2D.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

6.2D.2_1	UE maximum output power reduction for SUL with UL MIMO

6.2D.2_1.1	Test purpose

Same test purpose as in clause 6.2D.2.1

6.2D.2_1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1_1 is executed.

6.2D.2_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The minimum requirements are same as in 6.2D.1.3 with following exceptions:

Instead of Table 6.2D.1.3-1  use Table 6.2D.1_1.3-1

6.2D.2_1.4	Test description

6.2D.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.2D.2_1.4.1-1 ~ 6.2D.2_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2_1.4-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2_1.4.1-2: Test Configuration Table for Power Class 2

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2_1.4.1-1 ~ Table 6.2D.2_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2_1.4.3.

6.2D.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2_1.4.1-1 and Table 6.2D.2_1.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

6.2D.2_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2_1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2_1.5-1~6.2D.2_1.5-2.

Table 6.2D.2_1.5-1: UE Power Class test requirements (for Band n80, n84, n95, n97) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 25.0 + TT | 16.5 - TT | （14.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2D.2_1.5-1a: UE Power Class test requirements for Band n99 for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 4 | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 4 | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-2: UE Power Class test requirements (for Band n80, n84, n95, n97) for Power Class 2

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16 - TT2） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （222） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17 - TT2） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16 - TT2） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14 - TT2） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14 - TT2） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14 - TT2） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16,52） | 5.0 | （5.02） | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 | （5.02） | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 | （5.02） | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80 and n84, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-3: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

<<Unchanged texts are omitted>>

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 2 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 3 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 4 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 6 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 7 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 8 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 9 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 10 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 14 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 15 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 16 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 17 | 26 | 4.5 | 0 | 1.52 | 20 | 6 | 3 | 28+TT | 14.0-TT |
| 18 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 19 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 20 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 21 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 22 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 23 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 24 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-1a: UE Power Class 2 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 10 | 26 | 1 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 11 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 12 | 26 | 1 | 0 | 0 | 25 | 3 | 3 | 28+TT | 22.-TT |
| 13 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 16 | 26 | 2 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 17 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 18 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.-TT |
| 19 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 22 | 26 | 2.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 23 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 24 | 26 | 2.5 | 0 | 0 | 23.5 | 3 | 3 | 28+TT | 20.5-TT |
| 25 | 26 | 3.5 | 6.5 | 1.52 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 28 | 26 | 3 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 29 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 30 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.-TT |
| 31 | 26 | 5.5 | 8 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 33 | 26 | 5.5 | 0 | 0 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 34 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 35 | 26 | 5.5 | 0 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 36 | 26 | 5.5 | 0 | 0 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 37 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 38 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 39 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 40 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 41 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 42 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 43 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 44 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 45 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 46 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 47 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 48 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 49 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 50 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 51 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 52 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 53 | 26 | 4.5 | 0 | 1.52 | 20 | 6 | 3 | 28+TT | 14.-TT |
| 54 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 55 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 56 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 57 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 58 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 59 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 60 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 2 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 4 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 7 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 8 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 13 | 23 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 14 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 16 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.0-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 20 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.0-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2a: UE Power Class 3 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 5.5 | 1.52 | 19 | 5 | 3 | 28+TT | 14-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 3 | 3 | 28+TT | 21.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.52 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 17 | 23 | 0 | 1 | 0 | 1.52 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 19 | 23 | 0 | 2 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 23 | 23 | 0 | 2 | 0 | 1.52 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.52 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.52 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 2 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 4 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 5 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 7 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 8 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 9 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 10 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 11 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 12 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-4: UE Power Class 3 test requirements (NS_05) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-5: UE Power Class 3 test requirements (NS_05U) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-6: UE Power Class 3 test requirements (NS_48) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 4 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 9 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 11 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 16 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 17 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 18 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 21 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 27 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 29 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 30 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-7: UE Power Class 3 test requirements (NS_49) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 4 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 5 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 9 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 11 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 16 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 17 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 18 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 20 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 30 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 31 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 32 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 35 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 36 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 37 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 38 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 39 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 40 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 41 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 42 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9a: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_100 | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_100 | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_100 | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_100 | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_100 | 23 | 2 | 2.5 | 1.5 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | NS_100 | 23 | 2 | 2.5 | 0 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 10, 11 | NS_100 | 23 | 2.5 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | NS_100 | 23 | 2.5 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 13, 14 | NS_100 | 23 | 4.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | NS_100 | 23 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 16, 17 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_100 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 24 | NS_100 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 25, 26 | NS_100 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 27 | NS_100 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10a: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 6 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 12 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 13, 14 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16, 17 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 18 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 21 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 24 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 25, 26 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 27 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11a: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 1.5 | 20 | 5 | 3 | 25+TT | 14-TT |
|  | NS_03U | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_03; NS_03U | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_03; NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_03; NS_03U | 23 | 2 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03; NS_03U | 23 | 2 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 10, 11 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 1.5 | 18 | 5 | 3 | 25+TT | 13-TT |
| 12 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 13, 14 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 1.5 | 16 | 5 | 3 | 25+TT | 11-TT |
| 15 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16, 17 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 25, 26 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 27 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-12: UE Power Class 3 test requirements (NS_46) for band n7

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 3 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 4 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 10 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 11 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 12 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-13: UE Power Class 3 test requirements (NS_21) for band n30

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4,5 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 6,7 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9,10 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 11,12 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14,15 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 16,17 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 18 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19,20 | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-14: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 2 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 3 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 5 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 7 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 8 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 11 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 14 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 16 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 17 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 0 | 6.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 22 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-15: UE Power Class 3 test requirements (NS_27) for band n48

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 7 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 8 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 9 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 12 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 14 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 15 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 17 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 18 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 19 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 21 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 22 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 23 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 25 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 27 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 28 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 29 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 32 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 33 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 35 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 37 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 39 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 40 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 43 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 44 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 45 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 49 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 52 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 53 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 56 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 57 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 58 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 59 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 60 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 61 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 62 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 63 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 64 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 65 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 66 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 67 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 68 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 71 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 72 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 73 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 74 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 75 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 76 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 77 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 78 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 79 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 80 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 81 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 82 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 83 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 85 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 86 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 87 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 88 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 89 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 90 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 91 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 92 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 93 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 94 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 95 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 96 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 97 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 98 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 102 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 103 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 104 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 105 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 106 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 107 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 108 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 109 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 110 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

Table 6.2D.3.5-16: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 2 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 3 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 4 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 5 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 6 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 7 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 8 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 9 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14 -TT |
| 10 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 11 | 23 | 3.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 12 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 13 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 14 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 15 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 16 | 23 | 6.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 17 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 18 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 19 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 20 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 21 | 23 | 3 | 12 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 22 | 23 | 3 | 8 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 23 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 24 | 23 | 1.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 25 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 26 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 27 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 28 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 29 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 30 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 31 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 32 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 33 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 34 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 35 | 23 | 2 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 36 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 37 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 38 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 39 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 40 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 41 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 42 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 43 | 23 | 3.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 44 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 45 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 46 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 47 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 48 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 49 | 23 | 3.5 | 2 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 50 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 51 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 52 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 53 | 23 | 3.5 | 3 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 54 | 23 | 6.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 55 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 56 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 57 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 58 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 59 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 60 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 61 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 62 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 63 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 64 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-17: UE Power Class 3 test requirements (NS_50) for band n39

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 5.0 |  | 3 | 25.0 + TT | 13.0 - TT |  |
| 2 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 3 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 5.0 |  | 3 | 25.0 + TT | 15 - TT |  |
| 4 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 5.0 |  | 3 | 25.0 + TT | 15 - TT |  |
| 5 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 6 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 5.0 |  | 3 | 25.0 + TT | 15 - TT |  |
| 7 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 5.0 |  | 3 | 25.0 + TT | 15 – TT |  |
| 8 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 9 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT |  |
| 10 | 23 | 0 | 3 | 3.5 | 0 |  | 19.5 |  | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT |  |
| 11 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 5.0 |  | 3 | 25.0 + TT | 13.0 - TT |  |
| 12 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 13 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 14 | 23 | 0 | 6.5 | 6 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 15 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 16 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 17 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 – TT |  |
| 18 | 23 | 0 | 6.5 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 19 | 23 | 0 | 6.5 | 4..5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 20 | 23 | 0 | 6.5 | 8 | 0 |  | 15 |  | 6.0 |  | 3 | 25.0 + TT | 9.0 - TT |  |
| 21 | 23 | 0 | 6.5 | 3.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 22 | 23 | 0 | 6.5 | 5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2D.3_1	UE additional maximum output power reduction for SUL with UL MIMO

6.2D.3_1.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1_1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3_1.2	Test applicability

The requirements of this test apply in test case 6.5D.2.4.2_1 UTRA Adjacent channel leakage ratio for network signalling values NS_05U to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

The requirements of this test apply in test case 6.5D.3_2.3 Additional Spurious Emissions for network signalling value NS_05, NS_05U and NS_56 to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

NOTE:	Test execution is not necessary if 6.5D.2.4.2_1 and 6.5D.3_2.3 are executed.

6.2D.3_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1_1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1_1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4_1.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.1.

<<Unchanged texts are omitted>>

6.2D.3_1.5	Test requirement

The measured maximum output power shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable tables from Table 6.2D.3_1.5-1 to Table 6.2D.3_1.5-3. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2D.1_1.3-1 apply.

Table 6.2D.3_1.5-1: UE Power Class 3 test requirements (NS_05) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-2: UE Power Class 3 test requirements (NS_05U) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-3: UE Power Class 3 test requirements (NS_56) for band n99

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 2 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 3 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 4 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 5 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 6 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 7 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 8 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 9 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 10 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 11 | 23 | 3.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 12 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 13 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 14 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 15 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 16 | 23 | 6.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 17 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 18 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 19 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 20 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 21 | 23 | 3 | 12 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 22 | 23 | 3 | 8 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 23 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 24 | 23 | 1.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 25 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 26 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 27 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 28 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 29 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 30 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 31 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 32 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 33 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 34 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 35 | 23 | 2 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 36 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 37 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 38 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 39 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 41 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 42 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 43 | 23 | 3.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 44 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 45 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 46 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 47 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 48 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 49 | 23 | 3.5 | 2 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 50 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 51 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 52 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 53 | 23 | 3.5 | 3 | 0 | 19.5 | 5 | 4 | 25+TT | 15.5-TT |
| 54 | 23 | 6.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 55 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 56 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 57 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 58 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 59 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 60 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 61 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 62 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 63 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 64 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n99, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

6.2D.4	Configured transmitted power for UL MIMO

6.2D.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c for UL MIMO is within the specified bounds.

6.2D.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.4.3	Minimum conformance requirements

For UE supporting UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.4 shall apply to UE supporting UL MIMO, where

PPowerClass, ΔPPowerClass and TC,c are specified in subclause 6.2D.1.3;

MPRc is specified in subclause 6.2D.2.3;

A-MPRc is specified in subclause 6.2D.3.3.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1.3-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2D.4.3-1. The requirements shall be met with UL MIMO configurations specified in Table 6.2D.1.3-2.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the tolerance is specified in Table 6.2D.4.3-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.4.3-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2D.1.3 apply for the power class as indicated by the ue-PowerClass field in capability signalling.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.4.

6.2D.4.4	Test description

6.2D.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Table 6.2D.4.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.4.4.3.

6.2D.4.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.4.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.2D.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO and following exception.

Table 6.2D.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 0 |  |  |

Table 6.2D.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 14 |  |  |

Table 6.2D.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2D.4.4.3-4: ServingCellConfig: ULFPTx

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | ULFPTx Test ID 1, 2 |
|  | 1 |  | ULFPTx Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2D.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2D.4.5-1.

Table 6.2D.4.5-1: PCMAX configured UE output power

| Configured transmitted power |  |
| --- | --- |
| Measured UE output power test point 1 | 0 dBm ± (7+TT) |
| Measured UE output power test point 2 | 14 dBm ± (6+TT) |
| Measured UE output power test point 3 | 20 dBm + (4+TT) / - (6+TT) |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2D.4.5-2. Note 2: In addition note 2 in Table 6.2D.1.3-1 shall apply to the tolerances. |  |

Table 6.2D.4.5-2: Test Tolerance (Configured transmitted power for UL MIMO)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

==============================================================

<< End of changes >>