3GPP TSG-RAN5 Meeting #104	R5-244945

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:	5.3.31.9

Source: 	Apple

Title: 	Discussion on FR2 beam correspondence during initial access

Document for:	Discussion and Endorsement

# Introduction

During FR2 Beam correspondence in Initial access [1] discussion, UE beamlock functionality is reviewed and following Option a) and Option b) were discussed.

As specified in TS 38.213 section 7.4[4], UE determines physical random-access channel (PRACH) transmission power (PPRACH) for preambles transmission based on the minimum of two, a)  PCMAX or  b) [ PPRACH,target + Pathloss (PL)]. PCMAX is the maximum configured power derived from regulatory requirements [2] and specified in TS 38101-2 Table 6.1.1.3-2 for power class 3[3]. UE uses Open Loop power control which requires the indication of PRACH target power ( PPRACH,target), calculated based on preamble power related parameters provided by higher layers. The UE computes the downlink pathloss (PL) based on the SS/PBCH block associated with the PRACH transmission.

This paper also discusses the approach used for the SS response to the UE’s transmitted preambles, mentioned in TS 38.521 section 6.4.2.1. This approach can also be used to track the UE’s preambleTransMax counter on the SS to improve the synchronization between UE and SS during beam correspondence test procedure.

The goal is

- To allow UE to determine preamble power based on  PPRACH power formula as defined in TS 38.213 section 7.4 [4] and the purpose of beam lock function should be limited to only perform the UE beam lock activation/deactivation as specified in TS 38.509 section 5.4[5].

- To improve the synchronization between UE and SS during RACH. This objective can be accomplished by tracking the SS response not corresponding to the UE’s transmitted preambles.

This paper provides Apple’s view on the beam lock functionality and tracking of preambleTransMax counter on SS.

# Discussion

## On UE Open Loop UL Power settings

The intention of this topic is to further discuss and promote the proposal 4 Option b-1[1], to use Beam Lock function exclusively for UE beamlock activation/deactivation and allows UE to handle open loop UL power control using power ramping for PPRACH.

Following factor should be considered to evaluate the impact of using UE beam lock function to force UE at Pcmax:

- The UE beamlock test function already in place only activate/deactivate beamlock as specified in TS 38.509 section 5.4. Using beam lock function to control power settings along with the beamlock activation does require the additional step in beamlock activation procedure on UE side.

- UE side implementation is required to perform a) memorizing the beamlock command b) power settings c) activate UE beamlock. All these needs to be executed by UE within a specific timeframe. Any process that avoids UE implementation overhead will be preferable and needs to be pursued.

- Setting up the  PPRACH = Pcmax can bias the PRACH spherical coverage measurements result by ignoring/discarding UE’s pathloss computation for PPRACH.

- There is always an opportunity for UE to achieve PPRACH= Pcmax using power ramping procedure and PL computation within beam refinement duration.

- Setting PPRACH= Pcmax to avoid power ramping, have minimum impact on test time. Beam refinement time and RAR holding period have major impact on test time duration.

Observation 1: UE beamlock test function only perform activation/deactivation as defined in TS 38.509 section 5.4[5] and UE is allowed to handle UL power control as specified in TS 38.521-2 section 6.2.1.2[6] test procedure for UE maximum output power- Spherical coverage.

Observation 2: Any process that avoids UE implementation overhead will be preferable and needs to be pursued.

Proposal 1:  Agree that the beam lock function need not control the UE’s power setting, and the power ramping can be left to be handled on the UE side.

## On SS Tracking of UE preambleTransMax Counter

The intention of this topic is to propose a solution for SS to be aware of UE’s preambleTransMax counter. The preambleTransMax counter starts on the UE with the transmission of first preamble and the UE will keep incrementing counter with the transmission of preambles. As specified in TS 38.521 section 6.4.2.1 Error vector magnitude test procedure for PRACH[6], the SS shall transmit a random-access response (MSG2) not corresponding to the UE transmitted random-access preamble (MSG1). The SS tracking of responses which are not corresponding to UE transmitted preambles allows SS to track the UE’s preambleTransMax counter.

This approach has several benefits:

- SS can track the UE’s preambleTransMax counter.

- SS is informed of the RACH occasions available to perform measurements.

- The expiry of preambleTransMax counter is avoidable.

- SS can complete preambles measurements before UE declares RACH failure.

UE sends RACH Preamble (MSG1)

TE sends response (MSG2) with NOT corresponding to the transmitted preamble

UE starts: preambleTransMax counter

TE perform Measurements

UE Ends: preambleTransMax counter

TE sends valid response (MSG2), corresponding to the transmitted preamble

Figure 1 Procedure to track premable counter on SS Side

Observation 3: SS needs to be synchronized with UE to ensure measurements are completed before the expiry of preambleTransMax counter.

Proposal 2: Incorporate a procedure step for SS to track the preambleTransMax counter when the UE’s transmitted preambles are not corresponding to the UE transmitted preambles.

# Conclusion

The following observation and proposal were made in this contribution.

Observation 1: UE beamlock test function only perform activation/deactivation as defined in TS 38.509 section 5.4[5] and UE is allowed to handle UL power control as specified in TS 38.521-2 section 6.2.1.2[6] test procedure for UE maximum output power- Spherical coverage.

Observation 2: Any process that avoids UE implementation overhead will be preferable and needs to be pursued.

Proposal 1:  Agree that the beam lock function need not control the UE’s power setting, and the power ramping can be left to be handled on the UE side.

Observation 3: SS needs to be synchronized with UE to ensure measurements are completed before the expiry of preambleTransMax counter.

Proposal 2: Incorporate a procedure step for SS to track the preambleTransMax counter when the UE’s transmitted preambles are not corresponding to the UE transmitted preambles.

# References

- R5-243823, “Discussion on FR2 beam correspondence for initial access”, RAN4#103, May 20 – May 24, 2024

- 47 CFR Part 30, "UPPER MICROWAVE FLEXIBLE USE SERVICE, §30.202 Power limits", FCC.

- TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”, V18.5.0 (2024-03)

- TS 38.213, “Physical layer procedures for control”, V18.2.0 (2024-03)

- TS 38.509, “Special conformance testing functions for User Equipment (UE)”, V17.5.0 (2024-03)

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.2.0 (2024-03)