3GPP TSG-RAN5 Meeting #104	R5-244958

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0369 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New Generic Test Case 5.3C.19 Message Store Function Post Notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A new generic test case for HTTP messages during Message Store Function procedures was needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test cases 5.3C.19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | References from new test cases in TS 36.579-7 would not be valid and the test cases could not be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3C.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 5.3C.19	Message Store Function Post Notification using HTTP

5.3C.19.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3C.19.2	Procedure

Table 5.3C.19.2-1: Message Store Function Retrieve using HTTP

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection. | - | - | - | - |
| 2 | The SS (MCData server) sends an HTTP POST message about changes in the message store using the message store function. | <-- | HTTP POST | - | P |
| 3 | Check: Does the UE (MCData client) respond to the HTTP POST with a HTTP 204 (No Content) message? | --> | HTTP 204 (No Content) | - | - |
| 4 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |

5.3C.19.3	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

<END OF CHANGES>