3GPP TSG-RAN5 Meeting #104	R5-244961

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0372 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of clause 5.5.15 Default MCData call control messages and other information elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There were no default MCData call control messages |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clause 5.5.15 Default MCData call control messages and other information elements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases that refer to the default messages may not be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 5.5.15	Default MCData call control messages and other information elements

#### 5.5.15.1	General

The control messages specified in the present document are based on those specified in TS 24.582 [89] which in term are based on the RTCP Application Packets (RTCP: APP), as defined in IETF RFC 3550 [76].

#### 5.5.15.2	Map Group To Bearer

Table 5.5.15.2-1: Map Group To Bearer

| Derivation Path: 24.582 [89], Table 11.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00000 | Map Group To Bearer |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the participating MCData function |  |
| name | MCDM |  |  |
| MCData Group ID | px_MCData_Group_A_ID | The group ID of the call |  |
| TMGI |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [69] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |
| MBMS Subchannel |  |  |  |
| Appl. m-line Number | "1" | The number of the " m=application " m-line in the SIP MESSAGE request announcing the MBMS bearer |  |
| IP version | "0" | '0' = IP version 4 '1' = IP version 6 All other values are reserved for future use |  |
| Media Port Number | "9" |  |  |
| IP Address | "0.0.0.0" |  |  |

#### 5.5.15.3	Unmap Group To Bearer

Table 5.5.15.16-1: Unmap Group To Bearer

| Derivation Path: 24.582 [89], Table 11.2.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00001 | Unmap Group To Bearer |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the participating MCData function |  |
| name | MCDM |  |  |
| MCData Group ID | px_MCData_Group_A_ID | The group ID of the call |  |
| TMGI |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [69] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |
| MBMS Subchannel |  |  |  |
| Appl. m-line Number | "1" | The number of the " m=application " m-line in the SIP MESSAGE request announcing the MBMS bearer |  |
| IP version | "0" | '0' = IP version 4 '1' = IP version 6 All other values are reserved for future use |  |
| Media Port Number | "9" |  |  |
| IP Address | "0.0.0.0" |  |  |

#### 5.5.15.4	Application Paging

Table 5.5.15.17-1: Application Paging

| Derivation Path: 24.582 [89], Table 11.2.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00010 | Application Paging |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the participating MCData function |  |
| name | MCDM |  |  |
| MCData Group ID | px_MCData_Group_A_ID | The group ID of the call |  |

#### 5.5.15.5	Bearer Announcement

Table 5.5.15.18-1: Bearer Announcement

| Derivation Path: 24.582 [89], Table 11.2.7-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | 00011 | Bearer Announcement |  |
| name | MCDM |  |  |
| TMGI |  |  |  |
| MBMS Service ID | "0F0F0F" | The selected value is randomly chosen - a 6 digit hexadecimal number between 000000 and FFFFFF (see TS 23.003 [69] clause 15.2. The coding of the MBMS Service ID is the responsibility of each administration |  |
| MCC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Country Code |  |
| MNC | The same value as for PLMN1 specified in Table 5.5.8.1-x | Mobile Network Code |  |
| Alternative TMGI | Not present |  |  |
| Monitoring State | '1' | The <Monitoring State> value is a binary value where the following values are defined: '0' Monitoring is inactive '1' Monitoring is active |  |

#### 5.5.15.6	GROUP EMERGENCY ALERT

##### 5.5.15.6.1	GROUP EMERGENCY ALERT from the UE

Table 5.5.15.6.1-1: GROUP EMERGENCY ALERT from the UE

| Derivation Path: TS 24.282 [87] Table 15.1.14.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCDAta_ID_User_A |  |  |
| Organization name | Any allowed value |  |  |
| User location | Not Present |  |  |

##### 5.5.15.6.2	GROUP EMERGENCY ALERT from the SS

Table 5.5.15.6.2-1: GROUP EMERGENCY ALERT from the SS

| Derivation Path: TS 24.282 [87] Table 15.1.14.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCData_ID_User_B |  |  |
| Organization name | px_MCData_Group_A_Owner_Organization |  |  |
| User location | Not Present |  |  |

#### 5.5.15.7	GROUP EMERGENCY ALERT ACK

##### 5.5.15.7.1	GROUP EMERGENC ALERT ACK from the UE

Table 5.5.15.7.1-1: GROUP EMERGENCY ALERT ACK from the UE

| Derivation Path: TS 24.282 [87] Table 15.1.15.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCData_ID_User_B |  |  |
| Sending MCData user ID | px_MCData_ID_User_A |  |  |

##### 5.5.15.7.2	GROUP EMERGENC ALERT ACK from the SS

Table 5.5.15.7.2-1: GROUP EMERGENCY ALERT ACK from the SS

| Derivation Path: TS 24.282 [87] Table 15.1.15.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCData_ID_User_A |  |  |
| Sending MCData user ID | px_MCData_ID_User_B |  |  |

#### 5.5.15.8	GROUP EMERGENCY ALERT CANCEL

##### 5.5.15.8.1	GROUP EMERGENCY ALERT CANCEL from the UE

Table 5.5.15.8.1-1: GROUP EMERGENCY ALERT CANCEL from the UE

| Derivation Path: TS 24.282 [87] Table 15.1.16.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCData_ID_User_A |  |  |

##### 5.5.15.8.2	GROUP EMERGENCY ALERT CANCEL from the SS

Table 5.5.15.8.2-1: GROUP EMERGENCY ALERT CANCEL from the SS

| Derivation Path: TS 24.282 [87] Table 15.1.16.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCData_ID_User_B |  |  |

#### 5.5.15.9	GROUP EMERGENCY ALERT CANCEL ACK

##### 5.5.15.9.1	GROUP EMERGENCY ALERT CANCEL ACK from the UE

Table 5.5.15.9.1-1: GROUP EMERGENCY ALERT CANCEL ACK from the UE

| Derivation Path: TS 24.282 [87] Table 15.1.17.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCData_ID_User_B |  |  |
| Sending MCData user ID | px_MCData_ID_User_A |  |  |

##### 5.5.15.9.2	GROUP EMERGENCY ALERT CANCEL ACK from the SS

Table 5.5.15.9.2-1: GROUP EMERGENCY ALERT CANCEL ACK from the SS

| Derivation Path: TS 24.282 [87] Table 15.1.17.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCData group ID | px_MCData_Group_A_ID |  |  |
| Originating MCData user ID | px_MCData_ID_User_A |  |  |
| Sending MCData user ID | px_MCData_ID_User_B |  |  |

<END OF CHANGES>