3GPP TSG-RAN5 Meeting #104	R5-244971

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0071 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New Test Case 7.3.1 Emergency Alert CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no Off-Network Emergency Alert test cases for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Off-Network Emergency Alert CO test case 7.3.1 and added clause 7.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Off-Network Emergency Alert CO would not be tested for MCData |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 7.3	Emergency Alert

### 7.3.1	Off-network / Group Call / Emergency Alert / Emergency Alert Retransmission / Cancel Emergency Alert / Client Originated (CO)

7.3.1.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorised for MCData Service, including authorised to initiate emergency alerts in off-network environment, and the UE (MCData Client) is in an off-network environment }

ensure that {

when { the MCData User requests to initiate an MCData emergency alert }

then { UE (MCData Client) sends a GROUP EMERGENCY ALERT message and enters the "E2: Emergency state" state }

}

(2)

with { UE (MCData Client) in the "E2: Emergency state" state, and the UE (MCData Client) is in an off-network environment }

ensure that {

when { the timer TFE2 (emergency alert retransmission) expires }

then { UE (MCData Client) retransmits the MCData emergency alert by sending a GROUP EMERGENCY ALERT message}

}

(3)

with { UE (MCData Client) in the "E2: Emergency state" state, and the UE (MCData Client) is in an off-network environment }

ensure that {

when { the MCData User requests to cancel the MCData emergency alert }

then { UE (MCData Client) sends a GROUP EMERGENCY ALERT CANCEL message and enters the "E1: Not in emergency state" state }

}

7.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282 clauses 16.3.3.1, 16.3.3.2, 16.3.3.5. Unless otherwise stated these are Rel-16 requirements.

[TS 24.282, clause 16.3.3.1]

When in state "E1: Not in emergency state", upon receiving an indication from the MCData user to transmit an emergency alert for an MCData group ID and the value of "/<x>/<x>/Common/AllowedActivateAlert" leaf node present in the user profile as specified in 3GPP TS 24.483 [42] is set to "true", the MCData client:

1)	shall set the stored emergency state as "true";

2)	shall set the stored MCData group ID to the indicated MCData group ID;

3)	shall generate a GROUP EMERGENCY ALERT message as specified in subclause 15.1.14. In the GROUP EMERGENCY ALERT message, the MCData client:

a)	shall set the MCData group ID IE to the stored MCData group ID;

b)	shall set the Originating MCData user ID IE to own MCData user ID;

c)	may set the Organization name IE to own organization name; and

d)	may set the User location IE with client's current location, if requested;

4)	shall send the GROUP EMERGENCY ALERT message as specified in subclause 9.3.1.2;

5)	shall start timer TFE2 (emergency alert retransmission); and

6)	shall enter "E2: Emergency state" state.

[TS 24.282, clause 16.3.3.2]

When in state "E2: Emergency state", upon expiry of timer TFE2 (emergency alert retransmission), the MCData client:

1)	shall generate a GROUP EMERGENCY ALERT message as specified in subclause 15.1.14. In the GROUP EMERGENCY ALERT message, the MCData client:

a)	shall set the MCData group ID IE to the stored MCData group ID;

b)	shall set the originating MCData user ID IE to own MCData user ID;

c)	may set the Organization name IE to own organization name; and

d)	may set the Location IE with client's current location, if requested;

2)	shall send the GROUP EMERGENCY ALERT message as specified in subclause 9.3.1.2;

3)	shall start the timer TFE2 (emergency alert retransmission); and

4)	shall remain in the current state.

[TS 24.282, clause 16.3.3.5]

When in "E2: Emergency state", upon receiving an indication from the MCData user to cancel an emergency alert and the value of "/<x>/<x>/Common/AllowedCancelAlert" leaf node present in the user profile as specified in 3GPP TS 24.483 [42] is set to "true", the MCData client:

1)	shall set the stored emergency state as "false";

2)	shall generate a GROUP EMERGENCY ALERT CANCEL message as specified in subclause 15.1.16. In the GROUP EMERGENCY ALERT CANCEL message, the MCData client:

a)	shall set the MCData group ID IE to the stored MCData group ID; and

b)	shall set the Originating MCData user ID IE to own MCData user ID;

3)	shall send the GROUP EMERGENCY ALERT CANCEL message as specified in subclause 9.3.1.2;

4)	shall stop timer TFE2 (emergency alert retransmission); and

5)	shall enter "E1: Not in emergency state" state.

7.3.1.3	Test description

7.3.1.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCData Client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCData operation in the MCData configuration document).

NOTE 2:	The SS operation as NW (MCData server) is needed only for the preamble if the UE has to perform procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

IUT:

-	UE (MCData Client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	CFS1 (SDS retransmission) is set to the default value of 5.

-	TFS1 (SDS retransmission) is set to the default value of 40 ms.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.3.1.3.2	Test procedure sequence

Table 7.3.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE. | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCData Client Application and register User A as the MCData User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| 4 | Make the UE (MCData Client) initiate an off-network emergency alert. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCData call establishment are described in TS 36.579-1 [2] clause 5.4.5 'MCX CO communication over ProSe direct one-to-one communication out of E-UTRA coverage-establishment'. The test sequence below shows only the MCData relevant messages exchanged. | - | - | - | - |
| 5 | Check: Does the UE (MCData Client) send a GROUP EMERGENCY ALERT message? (NOTE 2) | --> | GROUP EMERGENCY ALERT | 1 | P |
| 6 | SS starts Timer=10s (TFE2 (emergency alert retransmission)) | - | - | - | - |
| 7 | The SS-UE1 (MCData client) responds to the GROUP EMERGENCY ALERT by sending a GROUP EMERGENCY ALERT ACK | <-- | GROUP EMERGENCY ALERT ACK | - | - |
| 8 | Timer=10s expires | - | - | - | - |
| 9 | Check: Does the UE (MCData Client) send a GROUP EMERGENCY ALERT message? (NOTE 2) | --> | GROUP EMERGENCY ALERT | 2 | P |
| 10 | Make the UE (MCData Client) cancel the emergency alert (NOTE 1), (NOTE 3) | - | - | - | - |
| 11 | Check: Does the UE (MCData Client) send a GROUP EMERGENCY ALERT CANCEL message? (NOTE 4) | --> | GROUP EMERGENCY ALERT CANCEL | 3 | P |
| 12 | The SS-UE1 (MCData client) responds to the GROUP EMERGENCY ALERT CANCEL by sending a GROUP EMERGENCY ALERT CANCEL ACK | <-- | GROUP EMERGENCY ALERT CANCEL ACK | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TFE2 (emergency alert retransmission) is started upon the sending of the GROUP EMERGENCY ALERT message. Timer TFE2 (emergency alert retransmission)=10s, as defined in TS 36.579-1 [2], Table 5.5.8.1-1. At the expire of TFE2, the MCData Client retransmits the GROUP EMERGENCY ALERT message. NOTE 3: If the MCData User does not perform an action before the expiry of timer TFE2 (emergency alert retransmission), then the MCData Client will retransmit the GROUP EMERGENCY ALERT message and the remaining steps will not be valid. Timer TFE2 (emergency alert retransmission) is started upon the sending of the GROUP EMERGENCY ALERT message. Timer TFE2 (emergency alert retransmission)=10s, as defined in TS 36.579-1 [2], Table 5.5.8.1-1. At the expire of TFE2, the MCData Client retransmits the GROUP EMERGENCY ALERT message. If during test execution it is found that the specified timer value is not large enough, then a new value needs to be specified. NOTE 4: The MCData Client stops the TFE2 (emergency alert retransmission) timer upon the sending of the GROUP EMERGENCY ALERT CANCEL message. |  |  |  |  |  |

7.3.1.3.3	Specific message contents

None

<END OF CHANGES>