3GPP TSG-RAN5 Meeting #104	R5-244972

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0072 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of New Test Case 7.3.2 Emergency Alert CT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no Off-Network Emergency Alert test cases for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Off-Network Emergency Alert CT test case 7.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Off-Network Emergency Alert CT would not be tested for MCData |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.3.2	Off-network / Group Call / Emergency Alert / Emergency Alert Retransmission / Cancel Emergency Alert / Client Terminated (CT)

7.3.2.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorised for MCData Service, including authorised to receive emergency alerts in off-network environment, and the UE (MCData Client) is in an off-network environment }

ensure that {

when { UE (MCData Client) receives an MCData emergency alert via a GROUP EMERGENCY ALERT message }

then { UE (MCData Client) responds by sending a GROUP EMERGENCY ALERT ACK }

}

(2)

with { UE (MCData Client) registered and authorised for MCData Service, having received an MCData emergency alert, and the UE (MCData Client) is in an off-network environment }

ensure that {

when { UE (MCData Client) receives a GROUP EMERGENCY ALERT message from the same MCData Client that sent the previous GROUP EMERGENCY ALERT message, and the timer TFE1 (Emergency Alert) has yet to expire }

then { UE (MCData Client) shall not send a GROUP EMERGENCY ALERT ACK in response }

}

(3)

with { UE (MCData Client) registered and authorised for MCData Service, having received an MCData emergency alert, and the UE (MCData Client) is in an off-network environment }

ensure that {

when { UE (MCData Client) receives a GROUP EMERGENCY ALERT message from the same MCData Client that sent the previous GROUP EMERGENCY ALERT message, and the timer TFE1 (Emergency Alert) expired }

then { UE (MCData Client) responds by sending a GROUP EMERGENCY ALERT ACK }

}

(4)

with { UE (MCData Client) registered and authorised for MCData Service, having received an MCData emergency alert, and the UE (MCData Client) is in an off-network environment }

ensure that {

when { UE (MCData Client) receives a GROUP EMERGENCY ALERT CANCEL message from the same MCData Client that sent the previous GROUP EMERGENCY ALERT message }

then { UE (MCData Client) responds by sending a GROUP EMERGENCY ALERT CANCEL ACK }

}

7.3.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282 clauses 16.3.3.3, 16.3.3.4, 16.3.3.6, 16.3.3.7. Unless otherwise stated these are Rel-16 requirements.

[TS 24.282, clause 16.3.3.3]

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon receiving a GROUP EMERGENCY ALERT message with the Originating MCData user ID IE not stored in the list of users in emergency, the MCData client:

1)	shall store the Originating MCData user ID IE and location IE in the list of users in emergency;

2)	shall generate a GROUP EMERGENCY ALERT ACK message as specified in subclause 15.1.15. In the GROUP EMERGENCY ALERT ACK message, the MCData client:

a)	shall set the MCData group ID IE to the MCData group ID IE of the received GROUP EMERGENCY ALERT message;

b)	shall set the Sending MCData user ID IE to own MCData user ID;

c)	shall set the Originating MCData user ID IE to the Originating MCData user ID IE of the received GROUP EMERGENCY ALERT message; and

3)	shall send the GROUP EMERGENCY ALERT ACK message as specified in subclause 9.3.1.2;

4)	shall start timer TFE1 (Emergency Alert); and

5)	shall remain in the current state.

NOTE:	Each instance of timer TFE1 is per MCData user ID.

Editor’s Note: [CR 0095, WI eMCData2] Use of timer TFE1 in case of several emergency alerts from multiple users is FFS.

[TS 24.282, clause 16.3.3.4]

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon receiving a GROUP EMERGENCY ALERT message with the Originating MCData user ID IE stored in the list of users in emergency and Location IE different than the stored location of the user, the MCData client:

1)	may update the stored location of the user with the received Location IE;

2)	shall restart the associated timer TFE1 (Emergency Alert); and

3)	shall remain in the current state.

[TS 24.282, clause 16.3.3.6]

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon receiving a GROUP EMERGENCY ALERT CANCEL message with the Originating MCData user ID IE stored in the list of users in emergency, the MCData client:

1)	shall remove the MCData user ID and associated location information from the stored list of users in emergency;

2)	shall generate a GROUP EMERGENCY ALERT CANCEL ACK message as specified in subclause 15.1.17. In the GROUP EMERGENCY ALERT CANCEL ACK message, the MCData client:

a)	shall set the MCData group ID IE to the MCData group ID IE of the received GROUP EMERGENCY ALERT CANCEL message;

b)	shall set the Sending MCData user ID IE to own MCData user ID; and

c)	shall set the Originating MCData user ID IE to the Originating MCData user ID IE of the received GROUP EMERGENCY ALERT message;

3)	shall send the GROUP EMERGENCY ALERT CANCEL ACK message as specified in subclause 9.3.1.2;

4)	shall stop the associated timer TFE1 (Emergency Alert); and

5)	shall remain in the current state.

[TS 24.282, clause 16.3.3.7]

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon expiry of timer TFE1 (Emergency Alert) associated with a stored MCData user ID, the MCData client:

1)	shall remove the MCData user ID and associated location information from the stored list of users in emergency; and

2)	shall remain in the current state.

7.3.2.3	Test description

7.3.2.3.1	Pre-test conditions

System Simulator:

-	SS-UE1 (MCData Client)

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the SS is behaving as SS-UE1 as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	GNSS simulator to simulate a location and provide a timing reference for the assistance of E-UTRAN off-network testing.

NOTE 1:	For operation in off-network environment, it needs be ensured that after the UE is powered up it considers the Geographical area #1 that is simulated by the GNSS simulator as being one of the geographical areas set in the USIM for operation when UE is "not served by E-UTRAN".

-	SS-NW (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCData operation in the MCData configuration document).

NOTE 2:	The SS operation as NW (MCData server) is needed only for the preamble if the UE has to perform procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

IUT:

-	UE (MCData Client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	For the underlying "transport bearer" over which the SS and the UE will communicate, the UE is behaving as a ProSe enabled UE as defined in TS 36.508 [24], configured for and operating as ProSe Direct Communication transmitting and receiving device.

-	CFS1 (SDS retransmission) is set to the default value of 5.

-	TFS1 (SDS retransmission) is set to the default value of 40 ms.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	The GNSS simulator is configured to simulate a location in the centre of Geographical area #1 and provide a timing reference, as defined in TS 36.508 [24] Table 4.11.2-2 scenario #1.

-	The UE is switched-off.

-	UE States at the end of the preamble

-	The UE is in state 'switched-off'.

7.3.2.3.2	Test procedure sequence

Table 7.3.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power up the UE. | - | - | - | - |
| 2 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 3 | Activate the MCData Client Application and register User A as the MCData User (TS 36.579-5 [5], px_MCX_User_A_username, px_MCX_User_A_password). (NOTE 1) | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCData call establishment are described in TS 36.579-1 [2] clause 5.4.10 'MCX CT communication over ProSe direct one-to-many communication out of E-UTRA coverage / Announcing/Discoveree procedure for group member discovery / One-to-many communication'. The test sequence below shows only the MCData relevant messages exchanged. | - | - | - | - |
| 4 | SS-UE1 (MCData client) sends a GROUP EMERGENCY ALERT (NOTE 2), (NOTE 3), (NOTE 4) | <-- | GROUP EMERGENCY ALERT | - | - |
| 5 | SS starts Timer=10s (TFE2 (emergency alert retransmission)) | - | - | - | - |
| 6 | SS starts Timer=15s (TFE1 (Emergency Alert)) | - | - | - | - |
| 7 | Check: Does the UE (MCData Client) respond to the GROUP EMERGENCY ALERT by sending a GROUP EMERGENCY ALERT ACK? | --> | GROUP EMERGENCY ALERT ACK | 1 | P |
| 8 | Timer=10s expires (TFE2 (emergency alert retransmission)) | - | - | - | - |
| 9 | Upon expiry of the timer TFE2 (emergency alert retransmission), the SS-UE1 (MCData client) retransmits the emergency alert by sending a GROUP EMERGENCY ALERT | <-- | GROUP EMERGENCY ALERT | - | - |
| 10 | SS starts Timer=10s (TFE2 (emergency alert retransmission)) | - | - | - | - |
| 11 | Check: Does the UE (MCData Client) respond to the GROUP EMERGENCY ALERT by sending in the next 5 seconds a GROUP EMERGENCY ALERT ACK? | --> | GROUP EMERGENCY ALERT ACK | 2 | F |
| 12 | Wait 5 seconds for GROUP EMERGENCY ALERT ACK | - | - | - | - |
| 13 | Timer=15s expires (TFE1 (Emergency Alert)) (NOTE 5) | - | - | - | - |
| 14 | Wait 5 seconds for expiration of timer TFE2 (emergency alert retransmission) | - | - | - | - |
| 15 | Timer=10s expires (TFE2 (emergency alert retransmission)) | - | - | - | - |
| 16 | SS-UE1 (MCData client) sends a GROUP EMERGENCY ALERT (NOTE 4) | <-- | GROUP EMERGENCY ALERT | - | - |
| 17 | Check: Does the UE (MCData Client) respond to the GROUP EMERGENCY ALERT by sending a GROUP EMERGENCY ALERT ACK? | --> | GROUP EMERGENCY ALERT ACK | 3 | P |
| 18 | SS-UE1 (MCData client) sends a GROUP EMERGENCY ALERT | <-- | GROUP EMERGENCY ALERT CANCEL | - | - |
| 19 | Check: Does the UE (MCData Client) respond to the GROUP EMERGENCY ALERT CANCEL by sending a GROUP EMERGENCY ALERT CANCEL ACK? | --> | GROUP EMERGENCY ALERT CANCEL ACK | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TFE1(emergency alert) is started by the MCData Client upon the receiving of the GROUP EMERGENCY ALERT message. Timer TFE1 (emergency alert)=15s, as defined as defined in the Pre-test conditions. At the expire of TFE1, the MCData Client assumes the end of the emergency state and removes the associated user from the list. NOTE 3: Timer TFE2 (emergency alert retransmission) is started by the SS upon the sending of the GROUP EMERGENCY ALERT message. Timer TFE2 (emergency alert retransmission)=10s, as defined in TS 36.579-1 [2], Table 5.5.8.1-1. At the expire of TFE2, the SS retransmits the GROUP EMERGENCY ALERT message. NOTE 4: The UE (MCData Client) stores the Originating MCData user ID IE in the list of users in emergency. NOTE 5: Upon expiry of the timer TFE1 (Emergency Alert), the UE (MCData Client) removes the MCData user ID from the stored list of users in emergency |  |  |  |  |  |

7.3.2.3.3	Specific message contents

None

<END OF CHANGES>