3GPP TSG-RAN5 Meeting #104	R5-245019

Maastricht, Netherlands, 19 Aug – 23 Aug, 2024

Title:							Discussion on FR2 beam correspondence for initial access

Source:			Huawei, HiSilicon

Agenda Item:

Document for:			Discussion

# 1.	Background

RAN4 introduced FR2 beam correspondence requirement for initial access in Rel-18. The requirement consists of EIRP spherical coverage requirement for PRACH in initial access. Considering the differences in UE behaviour between RRC_IDLE mode and RRC_CONNECTED mode, following working assumptions are made by RAN4 when deciding the minimum requirement [1].

- RAN4 has agreed to inform RAN5 to develop the beamlock function in initial access.

- UE locks the beam direction after requested by test equipment

- UE is kept in RRC_IDLE state to ensure at least 1ms EIRP measurement period for PRACH.

- Enable multiple PRACH transmissions in testing mode, including holding RAR.

- UE transmits with the optimal Tx beam that is autonomously selected by UE.

- UE achieves Pcmax before PRACH EIRP measurement.

- UE transmits PRACH with gap <=20ms.

- To accelerate the testing, it’s recommended to use the longest applicable PRACH preamble format and minimum gap between PRACH transmission.

In RAN5#103 meeting, the potential method of testing the PRACH beam correspondence requirements was discussed, and the conclusion was captured in [4]. This document continues discussion on the open issues.

# 2	Discussion

# 2.1	How to activate/deactivate beam lock function in IDLE mode

During RAN5#103, several options of activating/deactivating the beam lock function were discussed. The summary is captured below for convenient discussion.

1> How to activate/deactivate beam lock function in IDLE mode

Option a: Activation in RRC_CONNECTED mode, then UE send the activation complete message and memorize the activation status when it enters RRC_IDLE mode. Beam lock function has to be sent per DL link polarization and per measurement grid point.

Option a-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message.

Option a-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point.

Option a-3: UE automatically deactivate beam lock function when it enters RRC_CONNECTED mode.

Option a-4: Other method.

Option b: Activation in RRC_CONNECTED mode, UE memorizes the activation status when UE switches between RRC_CONNECTED mode and RRC_IDLE mode until:

i)                 deactivation command is received, or

ii)                other agreed condition among Option b-1/b-2/b-3 is met.

Beam is only locked in RRC_IDLE mode, and will be automatically unlocked in RRC_CONNECTED mode. Beam lock function is sent only once

Option b-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message.

Option b-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point.

Option b-3: Other method.

Conclusion:

Continue discussion on all options. Method without test function is not precluded.

The Option b has the advantage of saving the time of RRC message transmission between TE and UE, i.e. the Activation Command and Activation Complete messages. However, the saved time is probably not very crucial for the overall testing time. Usually the exchange of RRC message between TE and UE takes only dozens of ms, while the beam refining process on each measurement grid point will need several seconds. Skipping the message exchange could contribute very little to the saving of overall test time.

On the other hand, Option b might introduce additional uncertainty to the testing procedure. As mentioned in the summary, the UE could automatically deactivate the beam lock function when it couldn’t return to RRC_CONNECTED mode (Option b-2), or in other cases (Option b-3). Since the Activation Command will only be transmitted once, there is no other way for the UE to inform the TE its current state. If TE continues testing on other measurement grids, it’s possible UE is not working in the appropriate state and might be failed unexpectedly.

Proposal 1: Adopt Option a for activating/deactivating the beam lock function.

# 2.2	Beam refinement time needed

During RAN5#103, the beam refinement time required by the UE on each measurement grid was discussed, and 3s was agreed to be a starting point.

3>   Beam refinement time needed

Option a: 3s as a starting point

Option b: Further optimize beam refinement time to <3s

Conclusion:

Take option a as a starting point to evaluate the feasibility for preamble formats and preamble index, and the test time.

3s is used as the beam refinement time in current FR2 test cases in RRC_CONNECTED mode. In RRC_CONNECTED mode, UE already gets some prior information, and could use the narrow beam for beam refinement. While in RRC_IDLE mode, UE has no idea of the beam direction, therefore might need to perform rough beam refinement before fine beam refinement. Additional beam refinement time needs to be considered. On the other hand, too long beam refinement time will increase the overall testing time. Based on some initial evaluation, we think 4s could be a compromise.

Proposal 2: Take 4s as the beam refinement time required by UE on each measurement grid.

# 2.3	Whether the beam lock function could be extended to UE’s power setting behaviour

The UE’s power setting behaviour was also discussed in RAN5#103 and Option a and b were listed for further discussion.

4> Whether the beam lock function could be extended to UE’s power setting behavior

Option a: Extending the beam lock function to force UE to transmit at the Pcmax level.

Option b: Don’t use beam lock function to control UE’s power setting

Option b-1: With power ramping

Option b-2: Without power ramping

Conclusion:

Continue discussion on all the options.

Several companies prefer the Option b since it’s aligned with the UE’s behaviour in field. While the Option b has some drawbacks to address. The Option b-1 relies on power ramping. It’s difficult for the TE to know the exact time that the UE’s transmit power has already ramped to the Pcmax, so the start time of testing is unclear. The Option b-2 might be depending on the open loop power control process. While the power tolerance of open loop power control could be up to +/- 14dB, and UE might need certain time to converge it’s transmit power. Same as Option b-1, the starting time of measuring the PRACH preamble is unclear.

The Option a is to extend the beam lock function to force UE to transmit at the Pcmax level for any PRACH preamble transmission, regardless the ramping process and open loop power control process. This method seems introducing some deviation from the UE’s behaviour in field and got some concern during last meeting. However, considering the purpose of beam correspondence testing is to check that UE could select the correct beam, rather than UE could perform the power control process properly, the deviation will not really harm the test purpose. In the contrary, this method could simplify the testing procedure and save the testing time.

Proposal 3: The beam lock function could be extended to set UE’s transmit power.

# 2.4	Proposed flow chart for PRACH BC testing

To help the discussion, the flow chart from last meeting is copied below. It could be further revised based on the outcome of this meeting.

Figure 1 Flow chart for Beam lock function in RRC_IDLE

# 3.	Conclusion

In summary, following proposals are for RAN5 discussion:

Proposal 1: Adopt Option a for activating/deactivating the beam lock function.

Proposal 2: Take 4s as the beam refinement time required by UE on each measurement grid.

Proposal 2: Beam refinement time is needed between the time point UE enters RRC_IDLE mode and UE transmits PRACH preambles.

Proposal 3: The beam lock function could be extended to set UE’s transmit power.

# 4.	Reference

[1]	R4-2317764, LS on demodulation of FR2 UE UL by TE

[2]	R5-241402, Initial discussions for Rel-18 beam correspondence for initial access, Keysight Technologies UK Ltd, RAN5#102

[3] R5-240944, Time synchronization required for UE beamlock function during initial access, Apple AB Denmark, RAN5#102

[4] R5-243823, Discussion on FR2 beam correspondence for initial access, Huawei, RAN5#103