3GPP TSG-RAN WG5 Meeting #104	R5-245060

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	On testing correct handling of UL grant / Unused Transmission Occasion

Agenda item:	6.3.21.7

Document for:	Discussion

Introduction

The work plan for XR enhancements for NR was agreed in [1]. Among others, the following RAN protocol test purpose was agreed for UTO-UCI feature.

Agreements:

TS 38.321 5.8.2: Test Purpose 1: with { UE in RRC_Connected state with DRB established and sps-Configuration in UL is enabled } ensure that { when { for a CG occasion in slot n (CG occasion m), the UE determines that data in the buffer can be transmitted using the current CG occasion (m) and the next N CG occasions (N>1, N<nrofBitsInUTO-UCI-r18) and the time between the last symbol of CG occasion m+1 and the first symbol of CG occasion m+N+1 is smaller than a threshold (gNB processing time) } then { UE transmits in CG occasion m (in slot n) TO-UCI with first N bits set to 0 (used) and bits from N+1 to nrofBitsInUTO-UCI-r18 set to 1 (unused) } }

In this paper we discuss the feasibility of the test based on previous discussions and agreements in RAN WG1 and WG2.

Discussion

UTO-UCI can be configured in ConfiguredGrantConfig to a UE indicating support of cg-PUSCH-UTO-UCI-Ind-r18.

uto-UCI-Config-r18                      SEQUENCE {

nrofBitsInUTO-UCI-r18               INTEGER (3..8),

betaOffsetUTO-UCI-r18               INTEGER (0..31),

...

nrofBitsInUTO-UCI indicates the number of bits in the UTO-UCI bitmap (see TS 38.212 [17], clause 6.2.7, 6.3.2, TS 38.213 [13], clause 9.3.1, TS 38.214 [19], clause 5.2.3). When this field is configured, UTO-UCI is enabled for the UE.

In summary, when transmitting on CG-PUSCH resources in a CG-PUSCH transmission occasion (TO), the UE transmits a field of nrofBitsInUTO-UCI bits (UTO-UCI), where each bit corresponds to one of the next nrofBitsInUTO-UCI valid CG PUSCH TOs.  As specified in TS 38.213, clause 9.3.1, a bit value of '0' indicates that the UE may transmit CG-PUSCH, and a bit value of '1' indicates that the UE will not transmit CG-PUSCH, in a corresponding CG-PUSCH TO. Once the UE has indicated that it will not transmit in a CG-PUSCH TO, it cannot revert such decision.

During the WI phase, it was discussed in RAN WG1 whether an UTO_Offset could be configured so that, when the UE transmits UTO-UCI in a CG-PUSCH TO, then UTO-UCI is applicable to the nrofBitsInUTO-UCI valid CG PUSCH TOs starting with UTO_Offset from the end of the last symbol of the transmitted CG-PUSCH. The reason to propose the introduction of UTO_Offset was to allow for some gNB and UE processing time, e.g., for the gNB to be able to reallocate unused resources to other UEs. However, there was no consensus in RAN WG1, and UTO_Offset was finally not specified.

A similar discussion on gNB processing time also took place in RAN WG2, but still no consensus was achieved. One additional issue discussed in RAN WG2 is the somewhat unpredictable UE behaviour when UTO-UCI is configured. Current MAC specifications (TS 38.321, clause 5.8.2) states that: “The MAC entity determines if a configured uplink grant is going to be used for PUSCH transmission or not by considering the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to the lower layers regarding this decision, for use in the procedure for reporting UTO-UCI”. However, it is not specified when (not even when at the earliest or at the latest), a UE should do so. So, in practice, there are (nrofBitsInUTO-UCI-1) CG-PUSCH TOs where a CG-PUSCH TO can be signaled as “unused” by the UE for the first time. Which one of those is used, is currently up to UE implementation. In this way, a UE may indicate a CG-PUSCH TO as “unused” either too early or too late and hence gNB is not able to reschedule the unused resource to other UEs.

based on RAN1 and RAN2 specification, it is not clear when (not even when at the earliest or when at the latest) a UE should indicate a CG-PUSCH TO as “unused” for the first time.

Based on what specified in RAN1 and RAN2 specifications and on what previously discussed in RAN WG1 and RAN WG2, we envision the following possibilities for testing correct handling of UTO-UCI indication:

Option 1: Define a “gNB processing time” in RAN5 test, though this parameter is not defined in either RAN1 or RAN2 specifications at this moment. The UE shall indicate a CG-PUSCH TO as “unused” for the first time in the last available CG-PUSCH TO which is at least “gNB processing time” prior to the CG-PUSCH TO which being indicated as “unused”. This makes the UE behavior predictable, i.e. the RAN5 test could be easily defined so that it is deterministic when a UE shall indicate a CG-PUSCH TO as “unused” for the first time based on knowledge of the CG-PUSCH configuration and the amount of data in the UE buffer. The test purpose currently included in [1] is assuming this option. It should be noted that although “gNB processing time” is used here, the intention is to cover the rescheduling time which includes the processing time at both gNB side and UE side.

Option 2: Do NOT define any “gNB processing time” in RAN5 test. In this case, there may be more options for when the UE could indicate a CG-PUSCH TO as “unused” for the first time (depending on UE implementation). This may depend on what is the value of nrofBitsInUTO-UCI used for the test, what is the amount of data put in the UE buffer, etc. This makes the UE behavior less predictable, i.e. it is NOT deterministic when UE shall indicate a CG-PUSCH TO as “unused” for the first time. However, a test case may still be defined so that the UE indicates a CG-PUSCH TO as “unused” for the first time in one of (nrofBitsInUTO-UCI-1) CG-PUSCH TOs.

RAN5 to discuss and agree which option should be used to specify test case for UTO-UCI.

Conclusion

The paper initiates discussion about the UTO-UCI testing. As stated in the discussion part above, UTO-UCI functionality specification is unclear regarding when UE should indicate CG-PUSCH TO as unused. This paper introduces couple of proposals how to tackle difficulties in testing UTO-UCI due to this unclarity in core specification.

The following Observations and Proposals were made:

Observation 1: based on RAN1 and RAN2 specification, it is not clear when (not even when at the earliest or when at the latest) a UE should indicate a CG-PUSCH TO as “unused” for the first time.

Proposal 1: RAN5 to discuss and agree which option should be used to specify test case for UTO-UCI.

- References

- R5-243179, WP for XR (eXtended Reality) enhancements for NR plus CT1 aspects, Nokia, RAN5#103, Fukuoka, Japan, May 20 – May 24, 2024.