3GPP TSG-RAN5 Meeting #104	R5-245233

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4646 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR NTN test case 6.1.2.18a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 6.1.2.18a.3, redundant text and incorrect configurations exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Removed redundant text in SS configuration in clause 6.1.2.18a.3.1. 2. Corrected time instant configurations in clause 6.1.2.18a.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.18a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 6.1.2.18a	NR NTN / Cell reselection, Sintrasearch, Snonintrasearch / location based measurements

6.1.2.18a.1	Test Purpose (TP)

(1)

with { UE supporting location-based measurements in NR RRC_IDLE state on a cell which provides access by NR NTN }

ensure that {

when { Sintrasearch is non-zero in system information and distance between UE and the serving cell referenceLocation is larger than distanceThresh }

then { UE performs intra-frequency measurements and reselects the highest ranked cell up on Srxlev > SIntraSearchP }

}

(2)

with { UE supporting location-based measurements in NR RRC_IDLE state on a cell which provides access by NR NTN }

ensure that {

when { Snonintrasearch is non-zero in system information and distance between UE and the serving cell reference location referenceLocation is larger than distanceThresh }

then { UE performs inter-frequency measurements and reselects the cell which belong to the lower priority frequency upon Srxlev > Snonintrasearch }

}

6.1.2.18a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.304, clauses 5.2.4.2 and 5.2.4.7.0. Unless otherwise stated these are Rel-17 requirements.

[TS 38.304, clause 5.2.4.2]

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements;

-	Else, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its UE location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8];

-	Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

…

[TS 38.304, clause 5.2.4.7.0]

Cell reselection parameters are broadcast in system information and are read from the serving cell as follows:

…

cellReselectionPriority

This specifies the absolute priority for NR frequency or E-UTRAN frequency.

…

distanceThresh

This indicates the distance threshold from the serving cell reference location to be used in location-based measurement initiation.

…

Qqualmin

This specifies the minimum required quality level in the cell in dB.

Qrxlevmin

This specifies the minimum required Rx level in the cell in dBm.

Qrxlevminoffsetcell

This specifies the cell specific Rx level offset in dB to Qrxlevmin.

Qqualminoffsetcell

This specifies the cell specific quality level offset in dB to Qqualmin.

…

referenceLocation

This indicates the reference location of the serving cell to be used in location-based measurement initiation.

SIntraSearchP

This specifies the Srxlev threshold (in dB) for intra-frequency measurements.

SIntraSearchQ

This specifies the Squal threshold (in dB) for intra-frequency measurements.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for NR inter-frequency and inter-RAT measurements.

SnonIntraSearchQ

This specifies the Squal threshold (in dB) for NR inter-frequency and inter-RAT measurements.

…

TreselectionRAT

This specifies the cell reselection timer value. For each target NR frequency and for each RAT other than NR, a specific value for the cell reselection timer is defined, which is applicable when evaluating reselection within NR or towards other RAT (i.e. TreselectionRAT for NR is TreselectionNR, for E-UTRAN TreselectionEUTRA).

NOTE:	TreselectionRAT is not broadcast in system information but used in reselection rules by the UE for each RAT.

TreselectionNR

This specifies the cell reselection timer value TreselectionRAT for NR. The parameter can be set per NR frequency as specified in TS 38.331 [3].

TreselectionEUTRA

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN.

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, HighQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, LowQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshServing, LowQ

This specifies the Squal threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

…

6.1.2.18a.3	Test description

6.1.2.18a.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 3 as defined in TS 38.508-1 [4] Table 4.4.2-2 is used.

-	NR Cell 1 uses System information combination NR-29 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1.

-	NR Cell 2 and NR Cell 3 use System information combination NR-32 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.18a.3.2	Test procedure sequence

Table 6.1.2.18a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration T0 indicates the initial conditions for preamble. Configurations marked "T1" and"T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.18a.3.2-2.

Table 6.1.2.18a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | The power level values are assigned to ensure the UE registered on NR Cell 1 and Srxlev of NR Cell 1 is greater than Sintrasearch. |
|  | SIntraSearch | dB | 10 | - | - |  |
|  | Srxlev | dB | 22 | - | - |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | "Off" | NR Cell 2 becomes the highest ranked cell. |
|  | SIntraSearch | dB | 10 | 10 | - |  |
|  | Srxlev | dB | 16 | 22 | - |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -94 | -88 | NR Cell 3 becomes the highest ranked cell. |
|  | SnonIntraSearch | dB | - | 10 | 10 |  |
|  | Srxlev | dB | - | 16 | 22 |  |
| Note: Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 6.1.2.18a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.18a.3.2-1. | - | - | - | - |
| 2 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 2? | - | - | 1 | - |
| 4 | The SS re-adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.1.2.18a.3.2-1. | - | - | - | - |
| 5 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 3? | - | - | 2 | - |

6.1.2.18a.3.3	Specific message contents

Table 6.1.2.18a.3.3-1: Conditions for specific message contents in Tables below

| Condition | Explanation |
| --- | --- |
| NR Cell 1 | This condition applies to system information transmitted on NR Cell 1. |
| NR Cell 2 | This condition applies to system information transmitted on NR Cell 2. |
| NR Cell 3 | This condition applies to system information transmitted on NR Cell 3. |

Table 6.1.2.18a.3.3-2: SIB2 of NR Cell 1, NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.1.2.18a.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 5 | 10 dB |  |
| cellReselectionPriority | 5 |  | NR Cell 1 and NR Cell 2 |
|  | 1 |  | NR Cell 3 |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearchP | 5 | 10 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18a.3.3-3: SIB4 of NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.1.2.18a.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | Same downlink ARFCN as used for NR Cell 3 |  | NR Cell 2 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 3 |
| cellReselectionPriority | 5 |  | NR Cell 3 |
|  | 1 |  | NR Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18a.3.3-4: SIB19 of NR Cell 1, NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.1.2.18a.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| referenceLocation-r17 | '2273BA566355'H |  |  |
| distanceThresh-r17 | 500 |  |  |
| } |  |  |  |

<End of modified section>