3GPP TSG-RAN5 Meeting #104	R5-245237

# Maastricht, Netherlands, 19th - 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0557 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR RSTD measurement accuracy test case 14.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 14.3.5 for NR RSTD measurement accuracy is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message contents of test case 14.3.5 for NR RSTD measurement accuracy message content have been completed. Editor’s note has been updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message contents of test case 14.3.5 for NR RSTD measurement accuracy will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 14.3.5	NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance is missing

#### 14.3.5.1	Test purpose

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in TS 38.133 [50] clause 10.1.23.2 in an environment with AWGN propagation conditions.

#### 14.3.5.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports DL-TDOA positioning and supportedDL-PRS-ProcessingSamples.

#### 14.3.5.3	Minimum conformance requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y+Z+Δ) Tc.

X is defined in TS 38.133 [50] Table 10.1.23.2-7 for AWGN channel in FR1 provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	UE supports positioning measurement with reduced number of sample and is indicated by LMF to perform positioning measurement with reduced number of samples.

Note: The requirements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

When UE measures RSTD on PRS resources belonging to different PFLs, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs.

If the UE doesn’t support Rx TEG reporting for RSTD measurement or when the measurements of reference cell and neighbour cell belong to different Rx TEGs, Y, Z and Δ are defined as follows:

-	When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

-	When UE measures RSTD on PRS resources belonging different PFLs, Y=256 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 1280 ms.

-	Z is defined in Table 14.3.5.3-1

-	Δ is zero for single PFL, and is defined in Table 14.3.5.3-1, for dual PFL.

Table 14.3.5.3-1: Margin for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 120 |
| ≥ 52 | ≥ 24 | N/A | 72 |
| ≥ 104 | ≥ 48 | ≥ 24 | 36 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 12 |

Table 14.3.5.3-2: Margin Δ for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 128 |
| ≥ 52 | ≥ 24 | N/A | 64 |
| ≥ 104 | ≥ 48 | ≥ 24 | 32 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 8 |

Table 14.3.5.3-3: RSTD absolute accuracy in FR1 for AWGN channel with reduced number of samples

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 98 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 15 | ≥ 52 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
| 42 |  |  | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 48 |  | 30 | ≥ 48 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
| 24 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 24 |  | 60 | ≥ 64 | ≥ 1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| 10 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [49], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [53]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Void |  |  |  |  |  |  |  |

#### 14.3.5.4	Test description

##### 14.3.5.4.1	Initial conditions

The supported test configuration is listed in Table 14.3.5.4-1.

Table 14.3.5.4-1: Test Configuration

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid-Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.3.5.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.13.

2.	The general test parameter settings are set up according to Table 14.3.5.5-1.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.3.5.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured for the test.

6.	The true RSTD (which is the receive time difference for frame 0 between cell 2 and cell 1 as seen at the UE antenna connector) is set to 5898 Tc (about 3 s)

##### 14.3.5.4.2	Test procedure

The test consists of two sub-tests; the difference between the sub-tests is the PRS configuration. The test consists of a set-up period and a measurement period. Cell 1 and Cell 2 are both active during the complete test. The beginning of the measurement period shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell. The NR-DL-TDOA-RequestLocationInformation message and the DL-TDOA assistance data as defined in clause 14.3.5.4.3 shall be provided to the UE during the set-up period.

The UE is configured with measurement gap pattern ID # 24 if UE supports MG#24, otherwise #0 before T2.

- Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, according to TS 38.508-1 [45] clause 4.5.

2.	The SS shall send a RESET UE POSITIONING STORED INFORMATION message.

3.	Set the parameters according to Table 14.3.5.5-1. Propagation conditions are set according to clause 4.15.2.

4.	The SS shall transmit an LPP REQUEST CAPABILITIES message.

5.	The UE shall transmit an LPP PROVIDE CAPABILITIES message indicating the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED in the NR-DL-PRS-ProcessingCapability IE and DL-TDOA capabilities supported by the UE in the NR-DL-TDOA-ProvideCapabilities IE.

6.	The SS shall send a LPP PROVIDE ASSISTANCE DATA message, including the NR-DL-TDOA-ProvideAssistanceData IE. If the UE message at step 5 includes the ackRequested IE set to TRUE, then the SS shall send an acknowledgment in the LPP PROVIDE ASSISTANCE DATA message.

7.	The SS shall send a LPP REQUEST LOCATION INFORMATION message, including the reducedDL-PRS-ProcessingSamples-r17 in the NR-DL-TDOA-RequestLocationInformation IE such that the UE receives the message T ms before the start of the measurement period, where T = 50 ms.

8.	The UE shall transmit a LPP PROVIDE LOCATION INFORMATION message including the NR-DL-TDOA-ProvideLocationInformation IE within the response time (see clause 4.15.3).

9.	If the UE message at step 8 includes the ackRequested IE set to TRUE, the SS shall send a LPP acknowledgement message.

10.	The SS shall check the nr-RSTD-r16 value for Cell 2 in the nr-DL-TDOA-SignalMeasurementInformation-r16 according to Table 14.3.5.5-2.

11.	Repeat steps 2-10 in Table 14.3.5.4-1 until the confidence level according to Annex D is achieved.

12.	Repeat step 1-11 for the other sub-test defined in Table 14.3.5.5-1 as appropriate.

If all (applicable) sub-tests pass, the whole test passes. If one (applicable) sub-test fails, the whole test fails.

##### 14.3.5.4.3	Message contents

Table 14.2.3.7.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 0 0 1 1 1 | DL-TDOA |  |

Table 14.3.5.4.3-2: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-DL-TDOA-RequestCapabilities-r16 | TRUE |

Table 14.3.5.4.3-3: LPP RequestLocationInformation

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsPreferred |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 14.3.5.5 | Result of the response time calculation rounded up to the next second |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 0 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 | Not present |  |  |
| reducedDL-PRS-ProcessingSamples-r17 | requested |  |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTEG-Request-r17 | Not present |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| additionalPathsExt-r17 | Not present |  |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present |  |  |
| multiMeasInSameReport-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.5.4.3-4: LPP ProvideAssistanceData

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 | Not present |  |  |
| tbs-ProvideAssistanceData-r14 | Not present |  |  |
| wlan-ProvideAssistanceData-r14 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 14.3.5.4.3-5 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 SEQUENCE { |  |  | Depending on UE capabilities, i.e. support for UE-based DL-TDOA |
| nr-TRP-LocationInfo-r16 | As defined in TS 37.571-5 [20] |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| nr-TRP-BeamAntennaInfo-r17 | Not present |  |  |
| nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 | Not present |  |  |
| nr-DL-PRS-TRP-TEG-Info-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-TDOA-Error-r16 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present |  |  |
| assistanceDataValidityArea-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.5.4.3-5: NR-DL-PRS-AssistanceData

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz15 |  | Sub-test 1-1, Sub-test 1-2, Sub test 2-1 and Sub-test 2-2 |
|  | kHz30 |  | Sub-test 1-3 and Sub-test 2-3 |
| dl-PRS-ResourceBandwidth-r16 | 13 | 52 PRBs | Subtest 1 : Config 1 and Config 2 |
|  | 7 | 48 PRBs | Subtest 1 : Config 3 |
|  | 21 | 104 PRBs | Subtest 2: Config 1 and Config 2 |
|  | 28 | 132 PRBs | Subtest 2: Config 3 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  | Subtest1: config 1,2 &3 |
|  | n4 |  | Sub-test 2: Config 1,2&3 |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 92 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 153 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.3.5.4.3-6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.5.4.3-6: NR-DL-PRS-Info

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs15-r16 CHOICE { |  |  | Sub-tests 1-1, Sub-test 1-2, Sub-test 2-1 and Sub-test 2-2 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| scs30-r16 CHOICE { |  |  | Sub-tests 3-1 and Sub–test 3-2 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n1 |  | Test1 : Config 1,2&3 |
|  | n2 |  | Test2 : Config 1,2&3 |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 | 10 | Config 1,2,&3 | Cell 1 Subtest 1&2 |
|  | 01 | Config 1,2,&3 | Cell 2 Subtest 1&2 |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 01 |  | Test 1 – Config 1,2&3 |
| n4-r16 | 01 |  | Test 2 – Config 1,2&3 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  | Subtest 1 – Cell 1& 2 – Config 1,2&3 |
|  | 4 |  | Subtest 2 – Cell 1& 2 – Config 1,2&3 |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.5.4.3-7: LPP ProvideLocation Information

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present. |  |  |
| a-gnss-ProvideLocationInformation | Not present |  |  |
| otdoa-ProvideLocationInformation | Not present |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 | Not present |  |  |
| tbs-ProvideLocationInformation-r13 | Not present |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |
| dl-PRS-ReferenceInfo-r16 |  |  |  |
| nr-DL-TDOA-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 { | 2 entries |  |  |
| NR-DL-TDOA-MeasElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-Rx-TEG-ID-r17 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| NR-DL-TDOA-MeasElement-r16[2] SEQUENCE { |  | entry 2 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-Rx-TEG-ID-r17 |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  |  |  |
| } |  |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  |  |  |
| nr-DL-TDOA-Error-r16 |  |  |  |
| nr-DL-TDOA-SignalMeasurementInstances-r17 |  |  |  |
| nr-DL-TDOA-LocationInformationInstances-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 14.3.5.5	Test requirement

Table 14.3.5.5-1 defines the primary level settings including test tolerances for the test.

Table 14.3.5.5-1: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 |  |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 |  | Freq1 |  |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 20: NRB,c = 106 |  |  |  |  | 20: NRB,c = 106 |  |
|  | 2 |  | 20: NRB,c = 106 |  |  |  |  | 20: NRB,c = 106 |  |
|  | 3 |  | 50: NRB,c = 133 |  |  |  |  | 50: NRB,c = 133 |  |
| Duplex mode | 1 |  | FDD |  |  |  |  | FDD |  |
|  | 2 |  | TDD |  |  |  |  | TDD |  |
|  | 3 |  | TDD |  |  |  |  | TDD |  |
| TDD configuration | 1 |  | N/A |  |  |  |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  | - |  |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  |  |  |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  | - |  |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD |  | - |  |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD |  | - |  |  | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  | - |  |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD |  | - |  |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD |  | - |  |  | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  |  |  |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  |  |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  |  | - |  | TRS.1.1 FDD |  |
|  | 2 |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1 | s | - |  | 3 |  |  | - | 3 |
|  | 2,3 |  | - |  | 3 |  |  | - | 3 |
| SMTC configuration | 1 |  | SMTC.2 |  |  |  |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |  |  |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.3 FR1 |  |  |  |  | PRS.1.4 FR1 |  |
|  | 2 |  | PRS.1.3 FR1 |  |  |  |  | PRS.1.4 FR1 |  |
|  | 3 |  | PRS.2.3 FR1 |  |  |  |  | PRS.2.4 FR1 |  |
| PRS BW | 1,2 |  | 52 PRBs |  |  |  | - |  |  |
|  | 3 |  | 48 PRBs |  |  |  | - |  |  |
| PRS muting info | 1~3 |  | ‘10’ |  | ‘01’ |  |  | ‘10’ | ‘01’ |
| Expected RSTD | 1, 2, 3 | s | N/A |  | 3 |  |  | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A |  | 5 |  |  | N/A | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  | 0 |  |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  |  |  | -98 |  |
|  | 3 |  | -95 |  |  |  |  | -95 |  |
|  | 1~3 | dB | -3 | -6 |  |  |  | -3 | -6 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -99.44 | -101.65 |  |  |  | -99.44 | -101.65 |
|  | 3 |  | -96.44 | -98.65 |  |  |  | -96.44 | -98.65 |
| SS-RSRPNote3 | 1,2 | dBm/SCS | -99.44+TT | -101.65+TT |  |  |  | -99.44+TT | -101.65+TT |
|  | 3 |  | -96.44+TT | -98.65+TT |  |  |  | -96.44+TT | -98.65+TT |
| IoNote3 | 1,2 | dBm/ 19.08MHz | -63.63+TT | -63.63+TT |  |  |  | -63.63+TT | -63.63+TT |
|  | 3 | dBm/ 47.88MHz | -59.65+TT | -59.65+TT |  |  |  | -59.65+TT | -59.65+TT |
|  | 1~3 | dB | -1.44 | -3.65 |  |  |  | -1.44 | -3.65 |
| Propagation condition | 1~3 | - | AWGN |  |  |  |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 14.3.5.3.

The value of the LPP Time IE depends on the UE capabilities. The calculation is FFS

The resulting value shall be rounded up to the next integer and transmitted in the LPP-RequestLocationInformation (see Table 14.3.3.4.3-3).

Table 14.3.5.5-2: RSTD accuracy requirements for the reported values

| Test Configuration | Subtest | Lowest reported value | Highest reported value |
| --- | --- | --- | --- |
| 1 | Sub-test 1 | FFS | FFS |
|  | Sub-test 2 | FFS | FFS |
| 2 | Sub-test 1 | FFS | FFS |
|  | Sub-test 2 | FFS | FFS |
| 3 | Sub-test 1 | FFS | FFS |
|  | Sub-test 2 | FFS | FFSw |

The test tolerances are defined in clauses C.1.6 and C.2.5.

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.

< End of changes >