3GPP TSG-RAN5 Meeting #104	R5-245240

# Maastricht, Netherlands, 19th - 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0560 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PRS-RSRP measurement accuracy test case 16.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 16.5.2 for PRS-RSRP measurement accuracy is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message contents of test case 16.5.2 for PRS-RSRP measurement accuracy have been completed. Editor’s note has been updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message contents of test case 16.5.2 for PRS-RSRP measurement accuracy will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 16.5.2	PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 SA

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance is missing

#### 16.5.2.1	Test purpose

The purpose of the test is to verify the PRS RSRP measurement accuracy requirements specified in TS 38.133 [50] Clause 10.1.24.2 for single positioning frequency layer under AWGN propagation conditions in RRC_INACTIVE in FR1 standalone scenario.

#### 16.5.2.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports DL-AoD positioning and supports DL-PRS measurement in RRC_INACTIVE state.

#### 16.5.2.3	Minimum conformance requirements

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 16.5.2.3-2 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

Table 16.5.2.3-1: PRS-RSRP absolute accuracy for FR1 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 7 | Minimum Io Note 1 dBm / SCSPRS |  |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 5 |  | dBm/30kHz Note 5 | dBm/60kHz Note 5 |  |
| ±3.5 | ±8 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 |  | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 |  | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 |  | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 |  | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 |  | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 |  | -117.5 | -50 |
| ±8.5 | ±13 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 3 |  |  |  |  |  |
| ±6 | ±10.5 |  | 52< BW≤ 104 | All | Note 3 |  |  |  |  |  |
| ±4.5 | ±9 |  | BW >104 | All | Note 3 |  |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 7: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |  |

The relative accuracy of PRS-RSRP is defined as accuracy of the difference between two PRS-RSRP measurements.

The relative PRS-RSRP accuracy requirements apply for the cases when PRS-RSRP is measured from PRS resources in the same PRS resource set in FR1.

The accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 16.5.2.3-2 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

-	UE supports positioning measurements with reduced number of samples, and LMF indicates UE to perform positioning measurements with reduced number of samples

-	AWGN channel

Table 16.5.2.3-2: PRS-RSRP relative accuracy for FR1 with reduced sample number

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 6 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 7 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 5 | dBm/30kHz Note 5 | dBm/60kHz Note 5 |  |
| [±3.5] | ±5.0 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
| ±9.5 | ±11.0 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 3 |  |  |  |  |
| ±6.5 | ±8.0 |  | 52< BW≤ 104 | All | Note 3 |  |  |  |  |
| ±5.0 | ±6.5 |  | BW >104 | All | Note 3 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 4: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 5: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 6: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 7: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

#### 16.5.2.4	Test description

##### 16.5.2.4.1	Initial conditions

The supported test configurations in listed in Table 16.5.2.4-1.

Table 16.5.2.4-1: Test Configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 16.5.2.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.13.

2.	The general test parameter settings are set up according to Table 16.5.2.5-1 and Table 16.5.2.5-2.

3.	Propagation conditions are set according to clause 4.16.2.

4.	Message contents are defined in clause 16.5.2.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is the neighbour cell. All cells are on the same RF channel distributed in single positioning frequency layer.

##### 16.5.2.4.2	Test procedure

Same as in clause 16.5.1.4.2.

##### 16.5.2.4.3	Message contents

As defined in clause 16.4.2.4.3, with the following exceptions:

Table 16.5.2.4.3-1 replaces Table 16.4.2.4.3-6 and Table 16.5.2.4.3-2 replaces Table 16.4.2.4.3-7.

Table 16.5.2.4.3-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz1520 |  | Config 1&2 |
|  | kHz30 |  | Config 3 |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs | Sub-test 1 Config 1&2 |
|  | 1 | 24 PRBs | Sub-test 1 Config 32 |
|  | 21 | 104 PRBs | Subtest-2 Config 1&2 |
|  | 28 | 132 PRBs | Subtest 2- Config 3 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  | Sub-test 1 |
|  | n4 |  | Sub-test 2 |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 16.5.2.4.3-2 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 92 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 153 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 16.5.2.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 16.5.2.4.3-2: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs15-r16 CHOICE { |  |  |  |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| scs30-r16 CHOICE { |  |  |  |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n2 |  | Sub-test 1 |
|  | Not present |  | Sub-test 2 |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 | Not present |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 2 entries |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  | Sub-test 1 |
| n4-r16 | 0 |  | Sub-test 2 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  | Sub-test 1 Cell 1 and Sub-test 2 Cell 1 |
|  | 4 |  | Sub-test 1 Cell 2 and Sub-test 2 Cell 2 |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| NR-DL-PRS-Resource-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-DL-PRS-ResourceID-r16 | 1 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 1 |  | Sub-test 1 |
| n4-r16 | 1 |  | Sub-test 2 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  | Sub-test 1 Cell 1 and Sub-test 2 Cell 1 |
|  | 4 |  | Sub-test 1 Cell 2 and Sub-test 2 Cell 2 |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 16.5.2.5	Test requirement

Table 16.5.2.5-1: PRS-RSRP test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  | freq1 |  | freq1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1 | MHz | 20: NRB,c = 106 |  |  |  |
|  | Config 2 |  | 20: NRB,c = 106 |  |  |  |
|  | Config 3 |  | 50: NRB,c = 133 |  |  |  |
| BWP BW | Config 1 |  | 20: NRB,c = 106 |  |  |  |
|  | Config 2 |  | 20: NRB,c = 106 |  |  |  |
|  | Config 3 |  | 50: NRB,c = 133 |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle |  | s | 1.28 |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| PRS configuration | Config 1 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  | Config 2 |  | PRS.1.3 FR1 | PRS.1.3 FR1 | PRS.1.4 FR1 | PRS.1.4 FR1 |
|  | Config 3 |  | PRS.2.3 FR1 | PRS.2.3 FR1 | PRS.2.4 FR1 | PRS.2.4 FR1 |
| PRS BW | Config 1 |  | 52 PRBs | 52 PRBs |  |  |
|  | Config 2 |  | 52 PRBs | 52 PRBs |  |  |
|  | Config 3 |  | 48 PRBs | 48 PRBs |  |  |
| PRS Resource slot offset (slot) | Config 1,2,3 | slot | 0 | 4 | 0 | 4 |
| SSB configuration | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 | Config 1 | ms | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 |
| SMTC configuration | Config 1 |  | SMTC.2 |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/15Khz | -98 |  | -98 |  |
|  | Config 3 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  | -95 |  | -95 |  |
|  |  | dB | 0 | -6 | 0 | -6 |
|  |  | dB | 2.23 | -1.73 | 2.23 | -1.73 |
| PRS-RSRP Note3 | Config 1, 2 | dBm/SCS | -95.77+TT | -99.73+TT | -95.77+TT | -99.73+TT |
|  | Config 3 |  | -92.77+TT | -96.73+TT | -92.77+TT | -96.73+TT |
| IoNote3 | Config 1,2 | dBm/19.08MHz | -61.71+TT |  | -61.71+TT |  |
|  | Config 3 | dBm/47.88MHz | -57.73+TT |  | -57.73+TT |  |
| Propagation condition |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |

The value of the LPP Time IE depends on the UE capabilities. The calculation is FFS

Table 16.5.2.5-2: PRS-RSRP accuracy requirements for the reported values

| Test Configuration | Subtest | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Lowest reported value | Highest reported value | Lowest reported value | Highest reported value |
| 1 | Sub-test 1 | FFS | FFS | FFS | FFS |
|  | Sub-test 2 | FFS | FFS | FFS | FFS |
| 2 | Sub-test 1 | FFS | FFS | FFS | FFS |
|  | Sub-test 2 | FFS | FFS | FFS | FFS |
| 3 | Sub-test 1 | FFS | FFS | FFS | FFS |
|  | Sub-test 2 | FFS | FFS | FFS | FFS |

For the overall test to pass, the ratio of successful reported values in each sub-test shall be more than 90% with a confidence level of 95%.

< End of changes >