3GPP TSG-RAN5 Meeting #104	R5-245334

# Maastricht, Netherlands, 19th Aug 2024 - 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3323 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on CSI-ReportingBand for RRM testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current csi-reportingBand in Table 7.3.1-12F and Table 7.3.1-12G has a fix value for FR1 and FR2 regardless NR band and offsetToCarrier configuation. However, the number of subbands defined in TS 38.214 cl 5.2.1.4 are based on BWP start and has dependences with test frequencies parameters. Hence, number of subbands may vary depending on first and last subband sizes according to TS 38.214 cl 5.2.1.4: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated csi-reportingBand in Table 7.3.1-12F and Table 7.3.1-12G to implement a global solution that should work for all NR bands and test frequencies defined in TS 38.508 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Csi-ReportBand will not be complaint with TS 38.214 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

–	CSI-ReportConfig for CSI

Table 7.3.1-12F: CSI-ReportConfig for CSI

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-CSI |  |  |
|  | CSI-ReportConfigId-CSI with condition SCELL_CSI_ON_SPCELL |  | SCELL_CSI_ON_SPCELL |
| carrier | Not present | indicates the same serving cell as this report configuration |  |
|  | ServCellIndex of the SCell |  | SCELL_CSI_ON_SPCELL |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-CSI |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId-CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig CHOICE { |  |  |  |
| slots5 | 2 |  | SCS15 |
| slots10 | 4 |  | SCS30 |
| slots40 | 4 |  | SCS120 |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { | 1 entry |  |  |
| PUCCH-CSI-Resource [1] SEQUENCE { |  | entry 1 |  |
| uplinkBandwidthPartId | BWP-Id of active UL BWP |  |  |
| pucch-Resource | 8 | The first format 2 PUCCH resource configured in Table 4.6.3-112 is used |  |
|  | 8 | The first format 2 PUCCH resource configured in Table 4.6.3-112 is used | SCELL_CSI_ON_SPCELL |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | null |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands3 | '111'B |  | MORE_THAN_24_PRBS AND SUBBANDS3 |
| subbands4 | '1111'B |  | MORE_THAN_24_PRBS AND SUBBANDS4 |
| subbands5 | '11111'B |  | MORE_THAN_24_PRBS AND SUBBANDS5 |
| subbands6 | '111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS6 |
| subbands7 | '1111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS7 |
| subbands8 | '11111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS8 |
| subbands9 | ‘111111111’B |  | MORE_THAN_24_PRBS AND SUBBANDS9 |
| subbands10 | '1111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS10 |
| subbands11 | '11111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS11 |
| subbands12 | '111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS12 |
| subbands13 | '1111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS13 |
| subbands14 | '11111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS14 |
| subbands15 | '111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS15 |
| subbands16 | '1111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS16 |
| subbands17 | '11111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS17 |
| subbands18 | '111111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS18 |
| } |  |  |  |
| } |  |  |  |
| codebookConfig | CodebookConfig-CSI |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCELL_CSI_ON_SPCELL | For SCell CSI reporting on NR SpCell |
| MORE_THAN_24_PRBS | Total number of PRBs in the active BWP higher than 24 |
| SUBBANDS[N] | N number of subbands as defined in [21] clause 5.2.1.4 |

–	CSI-ReportConfig for BM

Table 7.3.1-12G: CSI-ReportConfig for BM

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-BM |  |  |
|  | CSI-ReportConfigId-BM with condition APERIODIC |  | APERIODIC |
| carrier | Not present |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| csi-IM-ResourcesForInterference | Not present |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic |  |  |  |
| reportSlotConfig ::= CHOICE { |  |  |  |
| slots80 | 2 |  | SCS15 |
| slots80 | 4 |  | SCS30 OR SCS120 |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF{ |  |  |  |
| PUCCH_CSI_Resource[0] SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id |  |  |
| pucch_Resource | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| aperiodic SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RSRP | NULL |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands3 | '111'B |  | MORE_THAN_24_PRBS AND SUBBANDS3 |
| subbands4 | '1111'B |  | MORE_THAN_24_PRBS AND SUBBANDS4 |
| subbands5 | '11111'B |  | MORE_THAN_24_PRBS AND SUBBANDS5 |
| subbands6 | '111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS6 |
| subbands7 | '1111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS7 |
| subbands8 | '11111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS8 |
| subbands9 | ‘111111111’B |  | MORE_THAN_24_PRBS AND SUBBANDS9 |
| subbands10 | '1111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS10 |
| subbands11 | '11111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS11 |
| subbands12 | '111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS12 |
| subbands13 | '1111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS13 |
| subbands14 | '11111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS14 |
| subbands15 | '111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS15 |
| subbands16 | '1111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS16 |
| subbands17 | '11111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS17 |
| subbands18 | '111111111111111111'B |  | MORE_THAN_24_PRBS AND SUBBANDS18 |
| } |  |  |  |
| } |  |  |  |
| codebookConfig | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For aperiodic CSI-RS resources |
| MORE_THAN_24_PRBS | Total number of PRBs in the active BWP higher than 24 |
| SUBBANDS[N] | N number of subbands as defined in [21] clause 5.2.1.4 |

–	CSI-ReportConfigId for CSI

Table 7.3.1-12H: CSI-ReportConfigId-CSI

| Derivation Path: Table 4.6.3-40 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfigId | n | n is the CSI-ReportConfigId allocated for report config of CSI-RS for CSI. Value of n is left to internal implementation |  |
|  | m | m is the CSI-ReportConfigId allocated for report config of SCell CSI on SpCell. Value of m is left to internal implementation | SCELL_CSI_ON_SPCELL |

| Condition | Explanation |
| --- | --- |
| SCELL_CSI_ON_SPCELL | For SCell CSI reporting on NR SpCell |

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>