3GPP TSG-RAN5 Meeting #104 	R5-245345

# Maastricht, Netherlands, 19th Aug 2024 – 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3426 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-NTN Re-establishment test cases 14.2.2.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Spain |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR-NTN Re-establishment test cases 14.2.2.1.1 and 14.2.2.1.2 are missing in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new core section 14.2.2.1.0 - Added new NR-NTN test cases 14.2.2.1.1 and 14.2.2.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.2.1.0, 14.2.2.1.1, 14.2.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 14.2.2	RRC Connection Mobility Control for SAN

#### 14.2.2.1	RRC Re-establishment for SAN

##### 14.2.2.1.0	Minimum conformance requirements for RRC Re-establishment for Satellite Access

In RRC_CONNECTED state the UE shall be capable of sending RRCReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection. The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay=TUE_re-establish_delay+TUL_grant

TUL_grant: It is the time required to acquire and process uplink grant from the target PCell. The uplink grant is required to transmit RRCReestablishmentRequest message.

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 of TS 38.331 [13] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

The intra-frequency target NR cell shall be considered detectable if each relevant SSB can satisfy that:

-	SS-RSRP related side conditions given in 38.133 [6] clauses 10.1.2 and 10.1.3 are fulfilled for a corresponding NR Band for FR1, and

-	the conditions of SSB_RP and SSB Ês/Iot according to 38.133 [6] Annex B.2.17 for a corresponding NR Band are fulfilled.

The inter-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in 38.133 [6] clause 10.1.4 are fulfilled for a corresponding NR Band for FR1, and

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.18 for a corresponding NR Band are fulfilled.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in Table 14.2.2.1.0-1.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell. Tidentify_inter_NR,i shall not exceed the values defined in Table 14.2.2.1.0-2.

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [13] signaling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

TSI-NR: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [13] for the target NR cell.

TPRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TPRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [8].

Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target intra-frequency NR cell is known, else Nfreq = 2 and Tidentify_intra_NR = 0 if the target inter-frequency NR cell is known.

There is no requirement if the target cell does not contain the UE context.

In the requirement defined in the below tables, the target FR1 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

The requirements in this clause apply provided that the ephemeris information provided by the serving cell for the target cell is valid during UE re-establishment delay (TUE_re-establish_delay).

Table 14.2.2.1.0-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | Kmulti_SMTC * MAX (800 ms, 10 x TSMTC) |
| < -8 | FR1 | N/A | k * 800 ms Note1, 3 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in 38.133 [6] clause 9.2C.5.1. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

Table 14.2.2.1.0-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | k * 800 ms Note1, 3 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in 38.133 [6] clause 9.3C.4. Note 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbor cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 6.2C.1.

##### 14.2.2.1.1	NR SA FR1 RRC Re-establishment for Satellite Access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Applicability may need to be updated

- Several test parameters and configuration are still TBD or need to be recalculated

- Re-establishment delay calculation needs to be aligned with legacy tests

14.2.2.1.1.1	Test purpose

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits. This test will verify the requirements in clause 14.2.2.1.0.

14.2.2.1.1.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.2.2.1.1.3	Minimum conformance requirement

The minimum requirements are specified in clause 14.2.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.2.2.1.1.

14.2.2.1.1.4	Test description

The test parameters are given in Table 14.2.2.1.1.4.1-3 and Table 14.2.2.1.1.5-1 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

The UE shall be provided with valid information about the SAN serving each cell in the test before the test.

14.2.2.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.2.2.1.1.4.1-1.

Table 14.2.2.1.1.4.1-1: Supported test configurations for NR SA FR1 RRC Re-establishment for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.2.2.1.1-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.2.2.1.1-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.2.2.1.1.4.1-2.

Table 14.2.2.1.1.4.1-2: Initial conditions for NR SA FR1 RRC Re-establishment for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.2.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 14.2.2.1.1.4.3.

2.	The power levels and settings for Cell 1 and Cell 2 are set according to Annex C.1.2 and C.1.3. Cell 1 and Cell 2 are satellite access cells.

3.	The test parameters are given in Table 14.2.2.1.1.4.1-3.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.2.2.1.1.4.1-3: General test parameters for NR SA FR1 RRC Re-establishment for Satellite Access

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| Final condition | Active cell |  | 1, 2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
| N310 |  | - | 1, 2 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.2 |  |
| DRX cycle length |  | s | 1, 2 | OFF |  |
| PRACH configuration |  |  | 1, 2 | PRACH.1 FR1 |  |
| T1 |  | s | 1, 2 | 5 |  |
| T2 |  | ms | 1, 2 | 640 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1C of 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1C.5 of 38.133 [6]) |
| T3 |  | s | 1, 2 | 2 |  |

14.2.2.1.1.4.2	Test Procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to T1 in Table 14.2.2.1.1.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.2.2.1.1.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 14.2.2.1.1.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within [FFS ms (= FFSms + FFS s) from the beginning of time period T2], then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

14.2.2.1.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 14.2.2.1.1.4.3-1: Common Exception messages for NR SA FR1 RRC Re-establishment for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

Table 14.2.2.1.1.4.3-2: RLF-TimersAndConstants for NR SA FR1 RRC Re-establishment for Satellite Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms3000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

14.2.2.1.1.5	Test Requirement

Table 14.2.2.1.1.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 RRC Re-establishment for Satellite Access.

Table 14.2.2.1.1.5-1: Cell-specific test parameters for NR SA FR1 RRC Re-establishment for Satellite Access

| Parameter | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| Satellite information |  | 1 | SSC.1 |  |  |  |  | NSC.1 |  |  |  |  |
|  |  | 2 | SSC.2 |  |  |  |  | NSC.2 |  |  |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 |  |  |  |  | OP.1 |  |  |  |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | 1, 2 | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |  |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  |  |  | SSB |  |  |  |  |
|  | dB | 1, 2 | 1.54+TT |  | -infinity |  | -infinity | -3.79+TT | 4+TT |  |  | 4+TT |
| Note2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |  |  |  |  |  |  |
|  | dB | 1, 2 | 7+TT |  | -infinity |  | -infinity | 4+TT | 4+TT |  |  | 4+TT |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -91+TT |  | -infinity |  | -infinity | -94+TT | -94+TT |  |  | -94+TT |
| Io | dBm/9.36 MHz | 1, 2 | -60.74+TT |  | -64.59+TT |  | -64.59+TT | -60.74+TT | -64.59+TT |  |  | -64.59+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known NR intra frequency cell shall be less than 1.6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 200 ms

TSI = 1280 ms, provided that SIB1 and SIB19 are scheduled with 20ms period; it is the time required for receiving all the relevant system information as defined in 38.331 [13] for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of [1545] ms, allow [1.6] s in the test case.

##### 14.2.2.1.2	NR SA FR1-FR1 RRC Re-establishment for Satellite Access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Applicability may need to be updated

- Several test parameters and configuration are still TBD or need to be recalculated

- Re-establishment delay calculation needs to be aligned with legacy tests

14.2.2.1.2.1	Test purpose

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits. This test will verify the requirements in clause 14.2.2.1.0.

14.2.2.1.2.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.2.2.1.2.3	Minimum conformance requirement

The minimum requirements are specified in clause 14.2.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.2.2.1.2.

14.2.2.1.2.4	Test description

The test parameters are given in Table 14.2.2.1.2.4.1-3 and Table 14.2.2.1.2.5-1 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

The UE shall be provided with valid information about the SAN serving each cell in the test before the test.

14.2.2.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.2.2.1.2.4.1-1.

Table 14.2.2.1.2.4.1-1: Supported test configurations for NR SA FR1-FR1 RRC Re-establishment for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.2.2.1.2-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.2.2.1.2-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.2.2.1.2.4.1-2.

Table 14.2.2.1.2.4.1-2: Initial conditions for NR SA FR1-FR1 RRC Re-establishment for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.2.2.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 14.2.2.1.2.4.3.

2.	The power levels and settings for Cell 1 and Cell 2 are set according to Annex C.1.2 and C.1.3. Cell 1 and Cell 2 are satellite access cells.

3.	The test parameters are given in Table 14.2.2.1.2.4.1-3.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.2.2.1.2.4.1-3: General test parameters for NR SA FR1-FR1 RRC Re-establishment for Satellite Access

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| Final condition | Active cell |  | 1, 2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 ms | Asynchronous cells |
| N310 |  | - | 1, 2 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.2 |  |
| DRX cycle length |  | s | 1, 2 | OFF |  |
| PRACH configuration |  |  | 1, 2 | PRACH.1 FR1 |  |
| T1 |  | s | 1, 2 | 5 |  |
| T2 |  | ms | 1, 2 | 640 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1C of 38.133 [6], T310 and the period for UE turns off transmitter defined in clause 8.1C.5 of 38.133 [6] ) |
| T3 |  | s | 1, 2 | 5 |  |

14.2.2.1.2.4.2	Test Procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to T1 in Table 14.2.2.1.2.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.2.2.1.2.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 14.2.2.1.2.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within [FFS ms (= FFSms + FFS s) from the beginning of time period T2], then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

14.2.2.1.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 14.2.2.1.2.4.3-1: Common Exception messages for NR SA FR1-FR1 RRC Re-establishment for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

Table 14.2.2.1.2.4.3-2: RLF-TimersAndConstants for NR SA FR1-FR1 RRC Re-establishment for Satellite Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms5000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

14.2.2.1.2.5	Test Requirement

Table 14.2.2.1.2.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1-FR1 RRC Re-establishment for Satellite Access.

Table 14.2.2.1.2.5-1: Cell-specific test parameters for NR SA FR1-FR1 RRC Re-establishment for Satellite Access

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 | T1 | T2 |  |  | T3 |
| Satellite information |  | 1 | SSC.1 |  |  |  | NSC.1 |  |  |  |  |
|  |  | 2 | SSC.2 |  |  |  | NSC.2 |  |  |  |  |
| RF Channel Number |  | 1, 2 | 1 |  |  |  | 2 |  |  |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 |  |  |  | OP.1 |  |  |  |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | 1, 2 | DLBWP.1.1 | N/A |  | N/A | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 | N/A |  | N/A | N/A |  | N/A | ULBWP.1.1 |  |
| SSB configuration |  | 1, 2 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  |  | SSB |  |  |  |  |
|  | dB | 1, 2 | 4+TT |  | -infinity | -infinity | -infinity | -infinity |  |  | 7+TT |
| Note2 | dBm/SCS | 1, 2 | -98+TT |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 2 | -98+TT |  |  |  |  |  |  |  |  |
|  | dB | 1, 2 | 4+TT |  | -infinity | -infinity | -infinity | -infinity |  |  | 7+TT |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -94+TT |  | -infinity | -infinity | -infinity | -infinity |  |  | -91+TT |
| Io | dBm/9.36 MHz | 1, 2 | -64.59+TT |  | -70. 05+TT | -70. 05+TT | -70. 05+TT | -70. 05+TT |  |  | -62.26+TT |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 3 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 800 ms

Tidentify_inter_NR = 800 ms

TSI = 1280 ms, provided that SIB1 and SIB19 are scheduled with 20ms period; it is the time required for receiving all the relevant system information as defined in 38.331 [13] for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of [2945] ms, allow [3] s in the test case.

<< End of Changes >>