3GPP TSG-RAN WG5 Meeting #104	R5-245392

Maastricht, Netherlands, August 19th – 23rd, 2024

Title:	On Rel-18 beam correspondence for initial access

Source:	Keysight Technologies UK Ltd

Agenda item:	5.3.31.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN4 introduced FR2 beam correspondence requirement for initial access in Rel-18 as one of the goals of Phase 3 of NR RF requirements enhancement for FR2 [1].

RAN 5 is now working on the definition of the corresponding test case definition as part of the UE conformance – NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 [2]. In RAN5#103, one CR [3] adding the update of beam correspondence minimum conformance requirements as well as the skeleton of a new test case 6.6.3 for Beam Correspondence in RRC_INACTIVE and initial access. Additionally, technical discussions already triggered in previous meetings [4], [5], continued with [6] and [7]. The outcome of related offline and online discussions was added in clause 2.5 of R5-243823 [8].

In this paper, Keysight provides additional inputs on some of the topics not concluded yet as per clause 2.5 of R5-243823.

# 2. 	Discussion

## 2.1 How to activate/deactivate beam lock function in IDLE mode

Options considered and conclusion in previous meeting were:

Option a: Activation in RRC_CONNECTED mode, then UE send the activation complete message and memorize the activation status when it enters RRC_IDLE mode. Beam lock function has to be sent per DL link polarization and per measurement grid point. Option a-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message. Option a-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point. Option a-3: UE automatically deactivate beam lock function when it enters RRC_CONNECTED mode. Option a-4: Other method. Option b: Activation in RRC_CONNECTED mode, UE memorizes the activation status when UE switches between RRC_CONNECTED mode and RRC_IDLE mode until: i) deactivation command is received, or ii) other agreed condition among Option b-1/b-2/b-3 is met. Beam is only locked in RRC_IDLE mode, and will be automatically unlocked in RRC_CONNECTED mode. Beam lock function is sent only once Option b-1: Deactivation sent in RRC_CONNECTED mode, then UE send the deactivation complete message. Option b-2: When UE couldn’t return to RRC_CONNECTED mode successfully, UE will automatically deactivate the beam lock function in FFS seconds since FFS time point. Option b-3: Other method. Conclusion: Continue discussion on all options. Method without test function is not precluded.

Considering that:

- NAS messages required to send activation and deactivation of test functions will not make much difference in total test case testing time (call establishment and positioner movements will take much longer)

- It might be challenging for UE implementations to lock the beam whenever it is required in IDLE mode if test function activation is sent only once

- Test function commands IDs are limited (as per 38.509)

Keysight thinks that a good option could be the following combination: a.3 AND a.2

Proposal 1: Regarding how to activate beam lock function in IDLE mode, activation of the initial access beam lock test function shall be sent in RRC_CONNECTED mode any time that DL link polarization or measurement grid point is modified.

Proposal 2: Regarding how to deactivate beam lock function in IDLE mode, deactivation of the initial access beam lock test function is achieved by either:

- automatically when the UE enters in RRC_CONNECTED.

- after a FFS timeout in case the UE is having difficulties to enter RRC_CONNECTED mode.

## 2.2 Whether the beam lock function could be extended to UE’s power setting behaviour

Options considered and conclusion in previous meeting were:

Option a: Extending the beam lock function to force UE to transmit at the Pcmax level. Option b: Don’t use beam lock function to control UE’s power setting Option b-1: With power ramping Option b-2: Without power ramping Conclusion: Continue discussion on all the options.

Analysing the options considered:

- With Option b-1, TE does not know when the maximum power has been achieved, i.e., the synchronization of the measurement with the UE transmission is very challenging. Hence Option b-1 shall be withdrawn.

- With option b-2, as already described in clause 2.3 of R5-243823:

“With open loop power control, the UE decides Tx power based on PL estimation from SSB. The absolute power tolerance will apply if the transmission gap is >20ms. Since the SSB periodicity for initial access is 20ms, it’s difficult to always ensure a transmission gap <=20ms.”

the tolerance impacting the UL Pcmax power could be too high so test case would be setting testing conditions misaligned with core requirements definition. Hence, option b-2 should be discarded too.

Proposal 3: Extend the initial access beam lock test function to force UE to transmit at the Pcmax level.

# 3. 	Conclusion

In this paper, Keysight provides additional inputs on some of the topics not concluded yet as per clause 2.5 of R5-243823.

The following proposals are made:

Proposal 1: Regarding how to activate beam lock function in IDLE mode, activation of the initial access beam lock test function shall be sent in RRC_CONNECTED mode any time that DL link polarization or measurement grid point is modified.

Proposal 2: Regarding how to deactivate beam lock function in IDLE mode, deactivation of the initial access beam lock test function is achieved by either:

- automatically when the UE enters in RRC_CONNECTED.

- after a FFS timeout in case the UE is having difficulties to enter RRC_CONNECTED mode.

Proposal 3: Extend the initial access beam lock test function to force UE to transmit at the Pcmax level.

# 4. 	References

- RP-232791, “WID on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi, RAN#102.

- RP-240125, “New WID on UE Conformance - NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Apple, Nokia, RAN#103.

- R5-243841, “Introducing framework for Beam Correspondence during Initial Access in IDLE related Tx Power tests”, Apple AB Denmark, RAN5#103

- R5-241402, “Initial discussions for Rel-18 beam correspondence for initial access,” Keysight Technologies UK Ltd, RAN5#102

- R5-240944, “Time synchronization required for UE beamlock function during initial access”, Apple AB Denmark

- R5-242792, “On Beam Correspondence initial access in Idle and FR2 RRM RACH Test”, Apple AB Denmark, RAN5#103

- R5-243159, “Discussion on FR2 beam correspondence for initial access”, Huawei, HiSilicon, RAN5#103

- R5-243823, “Discussion on FR2 beam correspondence for initial access”, Huawei, HiSilicon, RAN5#103