3GPP TSG-RAN5 Meeting #104	R5-245557

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3304 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RRC IEs for Further NR mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Further NR mobility enhancements test cases, new RRC IEs needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRC IEs were defined according to TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of RRC IEs for Further NR mobility enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-244852 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-244852 -> R5-245557 |  |  |  |  |  |  |  |  |

[Start of change]

4.6.3	Radio resource control information elements

–	AdditionalSpectrumEmission

[Skip unchanged text]

–	CandidateTCI-State

Table 4.6.3-15B: CandidateTCI-State

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CandidateTCI-State-r18 ::= SEQUENCE { |  |  |  |
| tci-StateId-r18 | TCI-StateId |  |  |
| qcl-Type1-r18 | LTM-QCL-Info |  |  |
| qcl-Type2-r18 | Not present |  |  |
|  | LTM-QCL-Info |  | Type 2 |
| pathlossReferenceRS-Id-r18 | PathlossReferenceRS-Id |  |  |
| tag-Id-ptr-r18 | Not present |  |  |
| ul-powerControl-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Type 2 | QCL type 2 information for the TCI state |

–	CandidateTCI-UL-State

Table 4.6.3-15C: CandidateTCI-UL-State

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CandidateTCI-UL-State-r18 ::= SEQUENCE { |  |  |  |
| tci-UL-StateId-r18 | TCI-UL-StateId |  |  |
| referenceSignal-r18 CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| pathlossReferenceRS-Id-r18 | PathlossReferenceRS-Id |  |  |
| tag-Id-ptr-r18 | Not present |  |  |
| ul-powerControl-r18 | Not present |  |  |
| } |  |  |  |

[Skip unchanged text]

–	CondReconfigToAddModList

Table 4.6.3-25C: CondReconfigToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | CondReconfigId-r16 |  |  |
| condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | identify a measurement configuration |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO | Table 4.8.1-1A |  |
|  | RRCReconfiguration-CPA | Table 4.8.1-1AB | CPA |
| condExecutionCondSCG-r17 | Not present |  |  |
| condExecutionCondPSCell-r18 | Not present |  |  |
| subsequentCondReconfig-r18 | Not present |  |  |
|  | SubsequentCondReconfig |  | SCPAC |
| securityCellSetId-r18 | Not present |  |  |
|  | 1 |  | CPAC |
| scpac-ConfigComplete-r18 | Not present |  |  |
|  | true |  | SCPAC |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CPA | Conditional PSCell addition |
| CPAC | Conditional PSCell Addition or Change |
| SCPAC | Subsequent Conditional PSCell Addition or Change |

–	ConditionalReconfiguration

Table 4.6.3-25D: ConditionalReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16 ::= SEQUENCE { |  |  |  |
| attemptCondReconfig-r16 | true |  |  |
| condReconfigToRemoveList-r16 | Not present |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList |  |  |
| scpac-ReferenceConfiguration-r18 | Not present |  |  |
| scpac-ReferenceConfiguration-r18 CHOICE { |  |  | SCPAC |
| setup | ReferenceConfiguration |  |  |
| } |  |  |  |
| servingSecurityCellSetId-r18 | Not present |  |  |
|  | 1 |  | CPAC |
| sk-CounterConfiguration-r18 | Not present |  |  |
| sk-CounterConfiguration-r18 SEQUENCE { |  |  | SCPAC |
| sk-CounterConfigToReleaseList-r18 | Not present |  |  |
| sk-CounterConfigToAddModList-r18 SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfig-r18 { | 1 entry |  |  |
| SK-CounterConfig-r18[1] SEQUENCE { |  | entry 1 |  |
| securityCellSetId-r18 | 1 |  |  |
| sk-CounterList-r18 SEQUENCE (SIZE (1..maxSK-Counter-r18)) OF SK-Counter { | 1 entry |  |  |
| SK-Counter[1] | SK-Counter | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CPAC | Conditional PSCell Addition or Change |
| SCPAC | Subsequent Conditional PSCell Addition or Change |

–	ConfiguredGrantConfig

Table 4.6.3-26: ConfiguredGrantConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| frequencyHopping | Not present |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| uci-OnPUSCH | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| rbg-Size | Not present |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 0 |  |  |
| transformPrecoder | Not present |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| repK-RV | Not present |  |  |
| periodicity | Sym40x14 | 40ms | SCS15 |
|  | Sym80x14 | 40ms | SCS30 |
|  | Sym160x14 | 40ms | SCS60 |
|  | Sym320x14 | 40ms | SCS120 |
| configuredGrantTimer | Not present |  |  |
| rrc-ConfiguredUplinkGrant | Not present |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  | CG_Config_Type1 |
| timeDomainOffset | 9 |  |  |
| timeDomainAllocation | 0 |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in clause 6.2.3.1 |  |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in clause 6.2.3.1 |  |
| antennaPort | 2 |  |  |
| dmrs-SeqInitialization | Not present |  |  |
| precodingAndNumberOfLayers | 1 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 1 |  |  |
| frequencyHoppingOffset | Not present |  |  |
| pathlossReferenceIndex | 0 |  |  |
| pathlossReferenceIndex2-r17 | Not present |  |  |
| srs-ResourceIndicator2-r17 | Not present |  |  |
| precodingAndNumberOfLayers2-r17 | Not present |  |  |
| timeDomainAllocation-v1710 | Not present |  |  |
| timeDomainOffset-r17 | Not present |  |  |
| cg-SDT-Configuration-r17 | Not present |  |  |
| cg-SDT-Configuration-r17 SEQUENCE { |  |  | pc_cg_SDT_r17 |
| cg-SDT-RetransmissionTimer | Not present |  |  |
| sdt-SSB-Subset-r17 | Not present | If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs configured by SIB1. |  |
| sdt-SSB-PerCG-PUSCH-r17 | one |  |  |
| sdt-P0-PUSCH-r17 | 0 |  |  |
| sdt-Alpha-r17 | alpha08 |  |  |
| sdt-DMRS-Ports-r17 CHOICE { |  |  |  |
| dmrsType1-r17 | ’10000000’B | Port0 |  |
| } |  |  |  |
| sdt-NrofDMRS-Sequences-r17 | 1 |  |  |
| } |  |  |  |
| srs-ResourceSetId-r18 | Not present |  |  |
| cg-LTM-Configuration-r18 | Not present |  |  |
| cg-LTM-Configuration-r18 SEQUENCE { |  |  | LTM |
| cg-RRC-RetransmissionTimer-r18 | 2 |  |  |
| cg-RRC-RSRP-ThresholdSSB-r18 | RSRP-Range |  |  |
| rrc-SSB-Subset-r18 CHOICE { |  |  |  |
| shortBitmap-r18 | 0100 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
|  | 1000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHz) |
| mediumBitmap-r18 | 01000000 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
|  | 10000000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap-r18 | 0100000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#1 |
|  | 1000000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#0 |
| } |  |  |  |
| rrc-SSB-PerCG-PUSCH-r18 | one |  |  |
| rrc-P0-PUSCH-r18 | 0 |  |  |
| rrc-Alpha-r18 | alpha04 |  |  |
| rrc-DMRS-Ports-r18 CHOICE { |  |  |  |
| dmrsType1-r18 | ’10000000’B | Port0 |  |
| } |  |  |  |
| rrc-NrofDMRS-Sequences-r18 | Not present |  |  |
| } |  |  |  |
| cg-SDT-PeriodicityExt-r18 | Not present |  |  |
| timeReferenceHyperSFN-r18 | Not present |  |  |
| cg-RRC-Configuration-r18 | Not present |  |  |
| applyIndicatedTCI-State-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CG_Config_Type1 | Configured Grant Configuration Type1 |
| LTM | L1/L2-triggered mobility |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |

[Skip unchanged text]

–	CSI-AperiodicTriggerStateList

Table 4.6.3-32: CSI-AperiodicTriggerStateList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 1 entry |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 8 |  | FR1 |
|  | 16 |  | FR2 |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | TCI-StateId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | 8 |  | FR1 |
|  | 16 |  | FR2 |
| nzp-CSI-RS-ResourcesforInterference | 8 |  | FR1 |
|  | 16 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| ltm-AssociatedReportConfigInfo-r18 | Not present |  |  |
|  | LTM-CSI-ReportConfigId |  | LTM |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LTM | L1/L2-triggered mobility |

[Skip unchanged text]

–	CSI-MeasConfig

Table 4.6.3-38: CSI-MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceToReleaseList | Not present |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToReleaseList | Not present |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  |  |
| CSI-IM-Resource[1] | CSI-IM-Resource | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToReleaseList | Not present |  |  |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceSetToReleaseList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { | 1 entry |  |  |
| CSI-SSB-ResourceSet[1] | CSI-SSB-ResourceSet | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetToReleaseList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig | entry 1 |  |
| } |  |  |  |
| csi-ResourceConfigToReleaseList | Not present |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1 |  |
| } |  |  |  |
| csi-ReportConfigToReleaseList | Not present |  |  |
| reportTriggerSize | 0 |  |  |
| aperiodicTriggerStateList CHOICE { |  |  |  |
| setup | CSI-AperiodicTriggerStateList |  |  |
| } |  |  |  |
| semiPersistentOnPUSCH-TriggerStateList | Not present |  |  |
| reportTriggerSizeDCI-0-2-r16 | Not present |  |  |
| sCellActivationRS-ConfigToAddModList-r17 | Not present |  |  |
| sCellActivationRS-ConfigToReleaseList-r17 | Not present |  |  |
| ltm-CSI-ReportConfigToAddModList-r18 | Not present |  |  |
| ltm-CSI-ReportConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18 { | 1 entry |  | LTM |
| LTM-CSI-ReportConfig-r18[1] | LTM-CSI-ReportConfig | entry 1 |  |
| } |  |  |  |
| ltm-CSI-ReportConfigToReleaseList-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LTM | L1/L2-triggered mobility |

[Skip unchanged text]

–	CSI-ReportSubConfigTriggerList

Table 4.6.3-40C: CSI-ReportSubConfigTriggerList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportSubConfigTriggerList-r18 | FFS |  |  |

–	CSI-ReportPeriodicityAndOffset

Table 4.6.3-40D: CSI-ReportPeriodicityAndOffset

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportPeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots80 | 10 |  | FR1 |
| slots320 | 40 |  | FR2 |
| } |  |  |  |

–	CSI-ResourceConfig

[Skip unchanged text]

–	CSI-SemiPersistentOnPUSCH-TriggerStateList

Table 4.6.3-46: CSI-SemiPersistentOnPUSCH-TriggerStateList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SemiPersistentOnPUSCH-TriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfSemiPersistentPUSCH-Triggers)) OF CSI-SemiPersistentOnPUSCH-TriggerState { | 1 entry |  |  |
| CSI-SemiPersistentOnPUSCH-TriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfo | CSI-ReportConfigId |  |  |
| sp-CSI-MultiplexingMode-r17 | Not present |  |  |
| csi-ReportSubConfigTriggerList-r18 | Not present |  |  |
| ltm-AssociatedReportConfigInfo-r18 | LTM-CSI-ReportConfigId |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

–	EarlyUL-SyncConfig

Table 4.6.3-56CA: EarlyUL-SyncConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EarlyUL-SyncConfig-r18 ::= SEQUENCE { |  |  |  |
| frequencyInfoUL-r18 | FrequencyInfoUL |  |  |
| rach-ConfigGeneric-r18 | RACH-ConfigGeneric |  |  |
| bwp-GenericParameters-r18 | BWP |  |  |
| ssb-PerRACH-Occasion-r18 | one |  |  |
| prach-RootSequenceIndex-r18 CHOICE { |  |  |  |
| l839 | Set according to table 4.4.2-2 |  | FR1 |
| l139 | Set according to table 4.4.2-2 |  |  |
| } |  |  |  |
| ltm-PRACH-SubcarrierSpacing-r18 | SubcarrierSpacing |  |  |
| n-TimingAdvanceOffset-r18 | n25600 |  |  |
| } |  |  |  |

[Skip unchanged text]

–	LTM-CandidateId

Table 4.6.3-67B: LTM-CandidateId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CandidateId-r18 | 1 |  |  |

–	LTM-Candidate

Table 4.6.3-67C: LTM-Candidate

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | LTM-CandidateId |  |  |
| ltm-CandidatePCI-r18 | PhysCellId |  |  |
| ltm-SSB-Config-r18 | LTM-SSB-Config |  |  |
| ltm-CandidateConfig-r18 | RRCReconfiguration with condition LTM |  |  |
| ltm-ConfigComplete-r18 | true |  |  |
| ltm-EarlyUL-SyncConfig-r18 | EarlyUL-SyncConfig |  |  |
| ltm-EarlyUL-SyncConfigSUL-r18 | Not present |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info |  |  |
| ltm-NoResetID-r18 | Not present |  |  |
| ltm-UE-MeasuredTA-ID-r18 | Not present |  |  |
| } |  |  |  |

–	LTM-Config

Table 4.6.3-67D: LTM-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-ReferenceConfiguration-r18 CHOICE { |  |  |  |
| setup | ReferenceConfiguration |  |  |
| } |  |  |  |
| ltm-CandidateToReleaseList-r18 | Not present |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate | entry 1 |  |
| } |  |  |  |
| ltm-ServingCellNoResetID-r18 | Not present |  |  |
| ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 { | 1 entry |  |  |
| LTM-CSI-ResourceConfig-r18[1] | LTM-CSI-ResourceConfig | entry 1 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfigToReleaseList-r18 | Not present |  |  |
| attemptLTM-Switch-r18 | Not present |  |  |
| ltm-ServingCellUE-MeasuredTA-ID-r18 | 1 |  |  |
| } |  |  |  |

–	LTM-CSI-ReportConfig

Table 4.6.3-67E: LTM-CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ReportConfig-r18 ::= SEQUENCE { |  |  |  |
| ltm-CSI-ReportConfigId-r18 | LTM-CSI-ReportConfigId |  |  |
| ltm-ResourcesForChannelMeasurement-r18 | LTM-CSI-ResourceConfigId |  |  |
| ltm-ReportConfigType-r18 CHOICE { |  |  |  |
| periodic-r18 SEQUENCE { |  |  | PERIODIC |
| reportSlotConfig-r18 | CSI-ReportPeriodicityAndOffset |  |  |
| pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { | 1 entry |  |  |
| PUCCH-CSI-Resource[1] | PUCCH-CSI-Resource | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| semiPersistentOnPUCCH-r18 SEQUENCE { |  |  | SP-PUCCH |
| reportSlotConfig-r18 | CSI-ReportPeriodicityAndOffset |  |  |
| pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource { | 1 entry |  |  |
| PUCCH-CSI-Resource[1] | PUCCH-CSI-Resource | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| semiPersistentOnPUSCH-r18 SEQUENCE { |  |  | SP-PUSCH |
| reportSlotConfig-r18 | CSI-ReportPeriodicityAndOffset |  |  |
| reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER | 1 entry |  |  |
| INTEGER[1] | 0 | entry 1 |  |
| } |  |  |  |
| reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER | 1 entry |  |  |
| INTEGER[1] | 0 | entry 1 |  |
| } |  |  |  |
| reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER { | 1 entry |  |  |
| INTEGER[1] | 0 | entry 1 |  |
| } |  |  |  |
| p0alpha | 0 |  |  |
| } |  |  |  |
| aperiodic-r18 SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER | 1 entry |  |  |
| INTEGER[1] | 0 | entry 1 |  |
| } |  |  |  |
| reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER | 1 entry |  |  |
| INTEGER[1] | 0 | entry 1 |  |
| } |  |  |  |
| reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER { | 1 entry |  |  |
| INTEGER[1] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-ReportContent-r18 | LTM-ReportContent |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | Aperiodic LTM report configuration |
| PERIODIC | Periodic LTM report configuration |
| SP-PUCCH | LTM report configuration of Semi-persistent on PUCCH |
| SP-PUSCH | LTM report configuration of Semi-persistent on PUSCH |

–	LTM-CSI-ReportConfigId

Table 4.6.3-67F: LTM-CSI-ReportConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ReportConfigId-r18 | 0 |  |  |

–	LTM-CSI-ResourceConfig

Table 4.6.3-67G: LTM-CSI-ResourceConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { |  |  |  |
| ltm-CSI-ResourceConfigId-r18 | LTM-CSI-ResourceConfigId |  |  |
| ltm-CSI-SSB-ResourceSet-r18 | LTM-CSI-SSB-ResourceSet |  |  |
| } |  |  |  |

–	LTM-CSI-ResourceConfigId

Table 4.6.3-67H: LTM-CSI-ResourceConfigId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ResourceConfigId-r18 | 0 |  |  |

–	LTM-CSI-SSB-ResourceSet

Table 4.6.3-67HA: LTM-CSI-SSB-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-SSB-ResourceSet-r18 ::= SEQUENCE { |  |  |  |
| ltm-CSI-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index { | 1 entry |  |  |
| SSB-Index[1] | SSB-Index | entry 1 |  |
| } |  |  |  |
| ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18 { | 1 entry |  |  |
| LTM-CandidateId-r18[1] | LTM-CandidateId | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

–	LTM-QCL-Info

Table 4.6.3-67HB: LTM-QCL-Info

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-QCL-Info-r18 ::= SEQUENCE { |  |  |  |
| referenceSignal-r18 CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| qcl-Type-r18 | typeA |  |  |
| } |  |  |  |

–	LTM-ReportContent

Table 4.6.3-67HC: LTM-ReportContent

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-ReportContent-r18 ::= SEQUENCE { |  |  |  |
| nrOfReportedCells-r18 | n1 |  |  |
| nrOfReportedRS-PerCell-r18 | n1 |  |  |
| spCellInclusion-r18 | true |  |  |
| } |  |  |  |

–	LTM-SSB-Config

Table 4.6.3-67HD: LTM-SSB-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-SSB-Config-r18 ::= SEQUENCE { |  |  |  |
| ssb-Frequency-r18 | ARFCN-ValueNR with condition DL_SSB |  |  |
| subcarrierSpacing-r18 | SubcarrierSpacing |  |  |
| ssb-Periodicity-r18 | ms20 |  |  |
| ssb-PositionsInBurst-r18 CHOICE { |  |  |  |
| shortBitmap | 0100 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHZ) |
|  | 1000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHz) |
| mediumBitmap | 01000000 |  | FR1 AND SSB#1 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
|  | 10000000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 0100000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#1 |
|  | 1000000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#0 |
| } |  |  |  |
| ss-PBCH-BlockPower-r18 | 0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |

–	LTM-TCI-Info

Table 4.6.3-67I: LTM-TCI-Info

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-TCI-Info-r18 ::= SEQUENCE { |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18 { | 1 entry |  | DL TCI OR Joint |
| CandidateTCI-State-r18[1] | CandidateTCI-State | entry 1 |  |
| } |  |  |  |
| ltm-DL-OrJointTCI-StateToReleaseList-r18 | Not present |  |  |
| ltm-UL-TCI-StateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18 { | 1 entry |  | UL TCI |
| CandidateTCI-UL-State-r18[1] | CandidateTCI-UL-State | entry 1 |  |
| } |  |  |  |
| ltm-UL-TCI-StateToReleaseList-r18 | Not present |  |  |
| ltm-NZP-CSI-RS-ResourceToAddModList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| ltm-NZP-CSI-RS-ResourceToReleaseList-r18 | Not present |  |  |
| ltm-NZP-CSI-RS-ResourceSetToAddModList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1 |  |
| } |  |  |  |
| ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18 | Not present |  |  |
| pathlossReferenceRS-ToAddModList-r18 SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17 { | 1 entry |  |  |
| PathlossReferenceRS-r17[1] | PathlossReferenceRS | entry 1 |  |
| } |  |  |  |
| pathlossReferenceRS-ToReleaseList-r18 | Not present |  |  |
| unifiedTCI-StateType-r18 | Not present |  | NOT Unified |
|  | separate |  | Separate |
|  | joint |  | Joint |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DL TCI | Downlink TCI state the UE is configured |
| UL TCI | Uplink TCI state the UE is configured |
| Unified | Unified TCI states type the UE is configured |
| Separate | LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StatesToAddModList for UL TCI states |
| Joint | LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation |

[Skip unchanged text]

–	NZP-CSI-RS-Resource

Table 4.6.3-85: NZP-CSI-RS-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId |  |  |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| powerControlOffset | -3 |  |  |
| powerControlOffsetSS | Not present |  |  |
| scramblingID | ScramblingId |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId |  |  |
| subcarrierSpacing-r18 | Not present |  |  |
|  | SubcarrierSpacing |  | LTM |
| absoluteFrequencyPointA-r18 | Not present |  |  |
|  | ARFCN-ValueNR with condition DL_SSB |  | LTM |
| cyclicPrefix-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LTM | L1/L2-triggered mobility |

–	NZP-CSI-RS-ResourceId

Table 4.6.3-86: NZP-CSI-RS-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceId | 0 |  |  |

–	NZP-CSI-RS-ResourceSet

Table 4.6.3-87: NZP-CSI-RS-ResourceSet

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId | entry 1 |  |
| } |  |  |  |
| repetition | off |  |  |
|  | Not present |  | TRS |
| aperiodicTriggeringOffset | Not present |  |  |
| trs-Info | Not present |  |  |
|  | true |  | TRS |
| resourceType-r18 | Not present |  |  |
|  | periodic |  | LTM |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |
| LTM | L1/L2-triggered mobility |

[Skip unchanged text]

–	PathlossReferenceRS

Table 4.6.3-89AA: PathlossReferenceRS

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PathlossReferenceRS-r17 ::= SEQUENCE { |  |  |  |
| pathlossReferenceRS-Id-r17 | PathlossReferenceRS-Id |  |  |
| referenceSignal-r17 CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| additionalPCI-r17 | Not present |  |  |
| } |  |  |  |

–	PathlossReferenceRS-Id

Table 4.6.3-89AB: PathlossReferenceRS-Id

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PathlossReferenceRS-Id-r17 | 0 |  |  |

[Skip unchanged text]

–	PUCCH-CSI-Resource

Table 4.6.3-113B: PUCCH-CSI-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-CSI-Resource ::= SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id |  |  |
| pucch-Resource | PUCCH-ResourceId |  |  |
| } |  |  |  |

[Skip unchanged text]

–	PUCCH-PowerControl

Table 4.6.3-115: PUCCH-PowerControl

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-PowerControl ::= SEQUENCE { |  |  |  |
| deltaF-PUCCH-f0 | 0 |  |  |
| deltaF-PUCCH-f1 | 0 |  |  |
| deltaF-PUCCH-f2 | 0 |  |  |
| deltaF-PUCCH-f3 | 0 |  |  |
| deltaF-PUCCH-f4 | 0 |  |  |
| p0-Set | Not present |  |  |
| pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS { | 1 entry |  |  |
| PUCCH-PathlossReferenceRS[1] SEQUENCE { |  | entry 1 |  |
| pucch-PathlossReferenceRS-Id | PUCCH-PathlossReferenceRS-Id |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb-Index | SSB-Index |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| twoPUCCH-PC-AdjustmentStates | Not present |  |  |
| } |  |  |  |

–	PUCCH-ResourceId

Table 4.6.3-115A: PUCCH-ResourceId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ResourceId | 0 |  |  |

–	PUCCH-SpatialRelationInfo

[Skip unchanged text]

–	ReferenceConfiguration

Table 4.6.3-138AAA: ReferenceConfiguration

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReferenceConfiguration-r18 | RRCReconfiguration with condition LTM | OCTET STRING (CONTAINING RRCReconfiguration) |  |

[Skip unchanged text]

–	SubcarrierSpacing

Table 4.6.3-188: SubcarrierSpacing

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SubcarrierSpacing | According to clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise. |  |  |

–	SubsequentCondReconfig

Table 4.6.3-188A: SubsequentCondReconfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SubsequentCondReconfig-r18 ::= SEQUENCE { |  |  |  |
| condExecutionCondToReleaseList-r18 | Not present |  |  |
| condExecutionCondToAddModList-r18 SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondExecutionCondToAddMod-r18 { | 1 entry |  |  |
| CondExecutionCondToAddMod-r18[1] SEQUENCE { |  | entry 1 |  |
| subsequentCondReconfigId-r18 | CondReconfigId |  |  |
| subsequentCondExecutionCond-r18 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  | SCEC |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| subsequentCondExecutionCondSCG-r18 | CondReconfigExecCondSCG | OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) | SCECS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCEC | Subsequent execution of a conditional reconfiguration for SN initiated intra-SN subsequent CPAC without MN involvement |
| SCECS | subsequent execution of a conditional reconfiguration for SN initiated inter-SN subsequent CPAC, SN initiated intra-SN subsequent CPAC with MN involvement, or MN initiated subsequent CPAC |

–	TAG-Config

[End of change]