3GPP TSG-RAN5 Meeting #104	R5-245615

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4643 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 2Rx XR Idle mode test case for Access not allowed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New test case for Cell Reselection 2Rx XR Idle mode for Access not allowed needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new idle mode test case 6.1.2.33 Cell reselection / 2Rx XR / Access not allowed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of test coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.33 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.2.33	Cell reselection / 2Rx XR / Access not allowed

6.1.2.33.1	Test Purpose (TP)

(1)

with { 2Rx XR UE in NR RRC_IDLE state}

ensure that {

when { 2Rx XR UE detects the cell reselection criteria are met for two cells on different frequencies and the serving cell is broadcasting accessAllowed2RxXR to "false" for the cell on the first frequency and "true" for the cell on the second frequency}

then { UE reselects the cell which belongs to the frequency for which the serving cell is broadcasting accessAllowed2RxXR "true"}

}

6.1.2.33.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.304, clause 5.2.4.6 and TS 38.331, clause 6.3.1 and 5.2.2.4.5. Unless otherwise stated these are Rel-18 requirements.

[TS 38.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

If rangeToBestCell is configured, then the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. If there are multiple such cells, the UE shall perform cell reselection to the highest ranked cell among them. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

NOTE:	If rangeToBestCell is configured but absThreshSS-BlocksConsolidation is not configured on an NR frequency, the UE considers that there is one beam above the threshold for each cell on that frequency.

[TS 38.331, clause 6.3.1]

accessAllowed2RxXR Indicates if the cells on the frequency support 2Rx XR UEs. If present, 2Rx XR UEs shall consider only these NR frequencies in cell reselection evaluation.

.

[TS 38.331, clause 5.2.2.4.5]

Upon receiving SIB4 the UE shall:

1>	if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	for each entry in the interFreqCarrierFreqList:

3>	if the UE is neither a RedCap nor an eRedCap UE nor 2Rx XR UE; or

3>	if the UE is a RedCap UE and the interFreqCarrierFreqList-v1700 is absent; or

3>	if the UE is an eRedCap UE and the interFreqCarrierFreqList-v1800 is absent; or

3>	if the UE is a 2Rx XR UE and the interFreqCarrierFreqList-v1800 is absent; or

3>	if the UE is a RedCap UE and redCapAccessAllowed is present in interFreqCarrierFreqList-v1700; or

3>	if the UE is an eRedCap UE and eRedCapAccessAllowed is present in interFreqCarrierFreqList-v1800; or

3>	if the UE is a 2Rx XR UE and accessAllowed2RxXR is present in interFreqCarrierFreqList-v1800:

4>	select the first frequency band in the frequencyBandList (or for aerial UE frequencyBandListAerial), and frequencyBandListSUL, if present, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList (or for aerial UE as indicated by nr-NS-PmaxListAerial), if present:

6.1.2.31.3	Test description

6.1.2.33.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11 have different tracking areas according to TS 38.508-1 [4] Table 4.4.2-3 and belong to NR band where 4RX operation is mandated.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR Cells.

UE:

None.

Preamble:

-	2Rx XR UE is in state 1N-A on NR Cell 1(serving cell) according to 38.508-1 [4].

6.1.2.33.3.2	Test procedure sequence

Table 6.1.2.33.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.1.2.33.3.2-3.

Table 6.1.2.33.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -78 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 23. |

Table 6.1.2.33.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 23. |

Table 6.1.2.33.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.33.3.2-1/2. | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 23 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 3 | The SS changes SIB4 of NR Cell 1 according to Table 6.1.2.33.3.3-1. The ValueTag of SIB4 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 1. | - | - | - | - |
| 4 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification on NR Cell 1. | <-- | NR RRC: Paging | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is camped on NR Cell 23? | - | - | 1 | - |

6.1.2.33.3.3	Specific message contents

Table 6.1.2.33.3.3-1: SIB4 for NR Cell 1 (step 3, Table 6.1.2.33.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v1700 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700 { | The same number of entries as the configured inter-freq carriers defined in TS38.508-1 [4] Table 6.3.1.2-1. | n denotes the index of the entry |  |
| interFreqCarrierFreqList-v1800[n] SEQUENCE { |  | entry n |  |
| accessAllowed2RxXR-r18 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.33.3.3-2: SIB1 of NR Cell 1 (step 3, Table 6.1.2.33.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| si-SchedulingInfo | SI-SchedulingInfo |  |  |
| } |  |  |  |

Table 6.1.2.33.3.3-3: SI-SchedulingInfo (si-SchedulingInfo in Table 6.1.2.33.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-173 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE(1..maxSI-Message)) OF SchedulingInfo { | 1 entry |  |  |
| SchedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| Type | sibType4 |  |  |
| valueTag | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.33.3.3-4: SIB4 for NR Cell 1 (preamble, Table 6.1.2.33.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v1700 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700 { | The same number of entries as the configured inter-freq carriers defined in TS38.508-1 [4] Table 6.3.1.2-1. | n denotes the index of the entry |  |
| interFreqCarrierFreqList-v1800 [n] SEQUENCE { |  | entry n |  |
| accessAllowed2RxXR-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |