3GPP TSG-RAN5 Meeting #104	R5-245625

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0679 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new PICS for eRedCap and R18 eDRX test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New PiCS is needed for eRedCap test case and R18 eDRX test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new PICS: pc_supportOfERedCap_r18 (eRedCap test case 7.1.1.1.25/6.1.2.34/6.1.2.35) pc_supportOf1Rxbranch_eRedCap_r18 (eRedCap test case 6.1.2.34) pc_supportOf2Rxbranches_eRedCap_r18 (eRedCap test case 6.1.2.34) pc_extendedDRX_CycleInactive_r18 (R18 eDRX 11.7.4/11.7.5) pc_eRedCapNotReducedBB-BW-r18 for RF test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_redcap_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244731r2. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.4.3.12A	eRedCap Capabilities

According to TS 38.306 [17] clause 4.2.22:

The maximum bandwidth is 20 MHz for FR1. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 are not supported by eRedCap UEs. eRedCap UEs do not support operation in FR2 and in FR1 60kHz SCS.

CA, MR-DC, DAPS, CPAC and IAB (i.e the eRedCap IE is not expected to act as IAB mode) related UE features and corresponding capabilities are not supported by the eRedCap UEs.

-	PICS associated to the following features are as below:

-	CA: PICS defined in clause A.4.3.2A;

-	MR-DC: PICS defined in clause A.4.3.2B;

-	DAPS: PICS include pc_intraFreqDAPS_r16, interFreqDAPS_r16 and other DAPS related PICS;

-	CPAC: PICS include pc_condPSCellChange_r16 and other CPAC related PICS.

Table A.4.3.12A-1: eRedCap UE Capabilities

| Item | UE Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Indicates that the UE is an eRedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1 | 38.306, 4.2.22.2 | Rel-18 | pc_supportOfERedCap_r18 | CY |  | FR1 only An eRedCap UE shall set this field to supported but shall not indicate support of supportOfRedCap-r17. |
| 2 | Indicates that the UE is an eRedCap UE without reduced baseband bandwidth in FR1 | 38.306, 4.2.22.2 | Rel-18 | pc_eRedCapNotReducedBB-BW-r18 | No |  | FR1 only. An eRedCap UE without reduced baseband bandwidth in FR1 shall set this field to supported and set pc_supportOfERedCap_r18 to supported but shall not indicate support of supportOfRedCap-r17. |
| 3 | Support of 1 Rx branch for an eRedCap UE | 38.331, 5.2.2.4.2 | Rel-18 | pc_supportOf1Rxbranch_eRedCap_r18 | No |  | FR1 only An eRedCap UE can support both 1 Rx branch and 2 Rx branches. |
| 4 | Support of 2 Rx branches for an eRedCap UE | 38.331, 5.2.2.4.2 | Rel-18 | pc_supportOf2Rxbranches_eRedCap_r18 | No |  | FR1 only An eRedCap UE can support both 1 Rx branch and 2 Rx branches. |

<End of modified section 1>

<Start of modified section 2>

A.4.3.7	General Capabilities

Table A.4.3.7-1: UE General Capabilities

| Item | UE General Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support UL transmission via either MCG path or SCG path for the split SRB as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_splitSRB_WithOneUL_Path | No |  |  |
| 2 | Support UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_splitDRB_withUL_Both_MCG_SCG | Yes |  |  |
| 3 | Support direct SRB between the SN and the UE as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_srb3 | Yes |  |  |
| 4 | Support of reflective QoS | 38.306, 4.2.2 | Rel-15 | pc_as_ReflectiveQoS | No |  |  |
| 5 | Support of NAS reflective QoS | 24.501, 6.2.5.1.4.1, 9.11.4.1 | Rel-15 | pc_nas_ReflectiveQoS | No |  |  |
| 6 | Support of SMS over NAS | 24.501, 5.5.1.2 | Rel-15 | pc_sms_over_NAS | No |  |  |
| 7 | Support of CMAS message on NR | 38.331, 5.2.2.2.2 | Rel-15 | pc_CMAS_NR | No |  |  |
| 8 | Support of ETWS message on NR | 38.331, 5.2.2.2.2 | Rel-15 | pc_ETWS_NR | No |  |  |
| 9 | The UE supports additional UE-requested PDU establishment | 24.501, 6.4.1.5 | Rel-15 | pc_Additional_PDU_establishment | No |  | pc_ExpectedNoOfPDUSessionsAtRegistration +1 |
| 10 | The UE includes the SM PDU DN request container IE in the PDU SESSION ESTABLISHMENT REQUEST message | 24.501, 6.4.1.2 | Rel-15 | pc_SM_PDU_DN_RequestContainer | No |  |  |
| 11 | Support of emergency services fallback in NR connected to 5GCN | 24.501 | Rel-15 | pc_NR_5GC_EmergencyService_fallback | No |  |  |
| 12 | Support of EPS fallback | 24.501, | Rel-15 | pc_EPS_fallback | No |  |  |
| 13 | Support of UE requested PDU session modification | 24.501, 6.4.2.2 | Rel-15 | pc_MO_PDU_Session_Modification | Yes |  |  |
| 14 | Support of emergency services in NR connected to 5GCN | 24.501 | Rel-15 | pc_NR_5GC_EmergencyServices | No |  |  |
| 15 | Support of voiceFallbackIndication | 38.306, 4.2.13 | Rel-16 | pc_voiceFallbackIndication | No |  |  |
| 16 | Support provision of referenceTimeInfo | 38.306, 4.2.2 | Rel-16 | pc_referenceTimeProvision_r16 | No |  | specifically for TSC (time sensitive communication) services |
| 17 | Support of RACS | 24.501, 9.11.3.1 | Rel-16 | pc_5GC_RACS | No |  |  |
| 18 | Support of RRC message Segmentation in the UL | 38.306, 5.4 | Rel-16 | pc_NR_UL_Segmentation | No |  | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |
| 19 | Support of RRC_INACTIVE as specified in TS 38.331 [9]. | 38.306, 4.2.2 | Rel-15 | pc_inactiveState | Yes |  |  |
| 20 | Support of UE local release when the security check is successful but SOR Transparent container indicates ACK has been NOT requested | 23.122 clause C.2 | Rel-15 | pc_SOR_ACKNotReqLocalRel | No |  |  |
| 21 | Support of RRC connection release with deprioritisation | 38.306, 5.3 | Rel-15 | pc_NR_RRC_Release_With_Deprioritisation | No |  |  |
| 22 | Support of RRC connection establishment failure with temporary offset | 38.306, 5.3 | Rel-15 | pc_NR_RRC_ConEstFail_With_TempOffset | No |  |  |
| 23 | Support of Closed Access Group | 24.501, 9.11.3.1 | Rel-16 | pc_CAG | No |  |  |
| 24 | Support of Stand-alone Non-Public Network | 23.501, 3.1 | Rel-16 | pc_SNPN | No |  |  |
| 25 | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over NR | 38.509, 5.9 | Rel-16 | pc_Set_UE_Cap_Info_NR | No |  | This test function is mandatory for UEs supporting UL RRC segmentation and whose maximum UECapabilityInformation message size is less than the allowed maximum supported size of a PDCP SDU. |
| 26 | Support of network slice-specific authentication and authorization | 24.501, 9.11.3.1 | Rel-16 | pc_5GC_NSSAA | No |  |  |
| 27 | Support of EAP-AKA’ as EAP method for network slice-specific authentication and authorization | 24.501, 5.4.7 | Rel-16 | pc_5GC_NSSAA_EAP_AKA_Prime | No |  |  |
| 28 | Support reduced control plane latency as defined in TS 38.331 [9] | 38.306, 4.2.2 | Rel-15 | pc_reducedCP_Latency | No |  |  |
| 29 | Support of release preference assistance information | 38.306, 4.2.2 | Rel-16 | pc_releasePreference_r16 | No |  |  |
| 30 | Support of user initiated SNPN reselection in automatic mode | 23.122 | Rel-16 | pc_UserInitiated_SNPN_Reselection | No |  |  |
| 31 | Support of autonomous search function to detect CAG cells on serving and non-serving frequencies | 38.304, 5.2.4.10 | Rel-16 | pc_Autonomous_search_function_nr_CAG | No |  |  |
| 32 | Support IMS voice over NR | 38.306, 4.2.13 | Rel-15 | pc_voiceOverNR | No |  | A UE supporting IMS voice over NR shall support: - IMS emergency call over NR, and - IMS voice over E-UTRA/EPC if it supports E-UTRA/EPC. |
| 33 | Support of V2X communication | 24.501, 9.11.3.1 | Rel-16 | pc_V2X | No |  | UE support V2X communication over NR-Uu and/or NR-PC5. |
| 34 | Support of V2X communication over NR-PC5 | 24.501, 9.11.3.1 | Rel-16 | pc_V2XCNPC5 | No |  |  |
| 35 | Support of Manufacturer assigned Radio Capability ID | 23.501, 5.9.10 | Rel-16 | pc_5GC_RACS_Manufacturer_URCID | No |  | UE support of Manufacturer assigned radio capability ID |
| 36 | Support of 3GPP PS data off | 24.501, 6.2.10 | Rel-15 | pc_PS_data_off | No |  | UE support of 3GPP PS data off |
| 37 | Support of Network Slice Simultaneous Registration Group | 24.501, 9.11.3.82 | Rel-17 | pc_5GC_NSSRG | No |  |  |
| 38 | Support of slice reselection information in SIB and on RRC release for slice based cell reselection in RRC _IDLE and RRC INACTIVE | 38.306, 4.2.2 | Rel-17 | pc_sliceInfoforCellReselection_r17 | No |  |  |
| 39 | Support of reception of segmented DL RRC messages | 38.306, 4.2.2 | Rel-16 | pc_NR_dl_DedicatedMessageSegmentation | No |  | The SS initiates the DL Dedicated Message Segment transfer procedure IF the encoded RRCReconfiguration or RRCResume message PDU size > maximum PDCP SDU size. |
| 40 | Support of unified access control configuration in the list of subscriber data, indicating for which access identities (see 3GPP TS 24.501 [64]) the ME is configured, when the MS accesses an SNPN | 23.122, 4.9.3.0, 24.501 4.5.2A | Rel-16 | pc_SNPN_access_control_configuration | No |  |  |
| 41 | Support of polarization signalling in NR NTN | 38.306, 5.4 | Rel-17 | pc_Polarization_Signalling_NR_NTN | No |  | UE supports polarization signalling in NR NTN |
| 42 | Supports receiving paging early indication and UE subgrouping indication with UEID | 38.306, 4.2.2 | Rel-17 | pc_pei_SubgroupingSupportBandList_r17 | No |  |  |
| 43 | Support of Rel-17 extended DRX cycle up to 10485.76 seconds | 38.306, 5.8 24.501, 5.3.16 | Rel-17 | pc_NR_eDRX | No |  |  |
| 44 | Support of (re-)configuration of an SCG during the resume procedure. | 38.306, 4.2.2 | Rel-16 | pc_resumeWithSCG_Config_r16 | No |  |  |
| 45 | Support of slice-based RACH prioritisation | 38.306, 5.4 | Rel-17 | pc_Slice_RACH_Prioiritisation | No |  |  |
| 46 | Support of slice-based RACH partitioning | 38.306, 5.4 | Rel-17 | pc_Slice_RACH_Partitioning | No |  |  |
| 47 | Support of RACH prioritisation for Access Identity 1 | 38.306, 5.4 | Rel-17 | pc_AccId1_RACH_Prioiritisation | No |  | US supporting this shall also support MPS (Access ID 1) |
| 48 | Support of ATSSS and MA PDU session | 24.501, 6.4.1.2 | Rel-16 | pc_5GC_ATSSS | No |  |  |
| 49 | Support gNB-side RTT-based PDC | 38.306, 4.2.2 | Rel-17 | pc_gNB_SideRTT_BasedPDC_r17 | No |  | A UE supporting this feature shall also support rtt-BasedPDC-CSI-RS-ForTracking-r17 and/or rtt-BasedPDC-PRS-r17. |
| 50 | Support of user plane integrity protection with EPS | 24.301, 5.5.1 33.401, 7.3.3 | Rel-17 | pc_EPS_UPIP | No |  | A UE supporting this feature shall also support EN-DC |
| 51 | Support of UAS Services | 24.501, 3.1, 4.22 | Rel-17 | pc_UAS | No |  | A UE supporting UAS services |
| 52 | Support of accessing SNPN using credentials from a Credentials Holder | 23.501 3.2, 5.30.2.9 | Rel-17 | pc_accessing_SNPN_usingCH | No |  | UE supports access using credentials assigned by a Credentials Holder separate from the SNPN |
| 53 | Support of Onboarding Stand-alone Non-Public Network | 23.501 5.30.2.10 | Rel-17 | pc_onboarding_SNPN | No |  |  |
| 54 | Support of EAP-AKA’ as EAP method for PDU session authentication and authorization | 24.501, 6.3.1 | Rel-15 | pc_5GC_PDU_EAP_AKA_Prime | No |  |  |
| 55 | Support of relaxed cell reselection on GEO. | 38.306. 5.4 | Rel-17 | pc_relaxedCellReselectionGEO | No |  |  |
| 56 | Support of emergency services in NR connected to 5GCN in SNPN Access mode | 23.501, 5.16.4.1 | Rel-17 | pc_SNPN_EmergencyService | No |  |  |
| 57 | Support of PLMN access in SNPN Access mode | 23.122, 5.2.8 | Rel-17 | pc_SNPN_PLMN | No |  |  |
| 58 | Support of being configured for No E-UTRA Disabling In 5GS | 24.301, 4.5 | Rel-17 | pc_no_eutra_disable_5GS | No |  |  |
| 59 | Support of MICO mode | 24.501, 5.3.6, 5.5.1.2.2 | Rel-15 | pc_MICO_Mode | No |  | A UE supporting MICO mode |
| 60 | Support of establishing a PDN connection as the user plane resource of an MA PDU session in 5GS | 24.501, 6.4.1.2 | Rel-17 | pc_5GC_ATSSS_PDN_connection | No |  |  |
| 61 | Support of PEIPS assistance information | 24.501, 5.5.1.2.2, 5.3.25 | Rel-17 | pc_PEIPS_assistance_information | No |  | A UE supporting this feature shall also support pc_pei_SubgroupingSupportBandList_r17 |
| 62 | Support of steering of roaming SNPN selection information | 24.501 9.11.3.1 | Rel-17 | pc_SSNPNSI | No |  |  |
| 63 | Support of steering of roaming connected mode control information | 24.501 9.11.3.51 | Rel-17 | pc_SORCMCI | No |  |  |
| 64 | Support of extended rejected NSSAI | 24.501, 5.5.1.2.4 | Rel-17 | pc_ER_NSSAI | Yes |  | A UE supporting extended rejected NSSAI |
| 65 | Support of ATG | 38.306, 4.2.2 | Rel-18 | pc_airToGroundNetwork_r18 | No |  | UE supports ATG |
| 66 | Support of UE disabling E-UTRA capability when attach attempt counter or tracking area updating attempt counter is equal to 5 | 24.301, 5.5.1.2.6, 5.5.1.3.6,5.5.3.2.6 | Rel-17 | pc_disable_EUTRA_attempt_counter_max | No |  | This PICS shall be true for UE that always disable E-UTRA capability when attach attempt counter or tracking area updating attempt counter is equal to 5. |
| 67 | Support reporting of affected NR carrier frequencies in IDC assistance information as specified in TS 38.331 [9] | 38.306, 4.2.2 | Rel-16 | pc_inDeviceCoexInd_r16 | No |  |  |
| XXX | supports of extended DRX in RRC_INACTIVE with values above 1024 radio frames | 38.306 4.2.6 | Rel-18 | pc_extendedDRX_CycleInactive_r18 | No |  | The UE may indicate support of this capability only if it supports extended DRX in RRC_IDLE. |

<End of modified section 2>