3GPP TSG-RAN5 Meeting #104	R5-245629

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4592 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new eRedCap test case 6.1.2.35 - Emergency Call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new eRedCap test case is added to test that eRedCap UE does not consider NR Cell is barred for emergency call when all the UE supported Rx is barred by cellBarred-eRedCap1Rx or/and cellBarred-eRedCap2Rx |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new eRedCap test case 6.1.2.35 Cell Selection / eRedCap / emergency calls |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_redcap_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0499 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244736. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.2.35	Cell Selection / eRedCap / emergency call

6.1.2.35.1	Test Purpose (TP)

(1)

with { UE supporting eRedCap and in NR RRC_IDLE state }

ensure that {

when {  a cell fulfils the requirements for a suitable cell except that all the UE supported Rx is barred by cellBarred-eRedCap1Rx or/and cellBarred-eRedCap2Rx in the SIB1, barringExemptEmergencyCall is set to “true” in SIB1, and user initiates an Emergency call  }

then { the UE considers the cell as not barred and successfully completes the Emergency call }

}

6.1.2.35.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.304, clause 5.3.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.304, clause 5.3.1]

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls except for the below cases:

…

-	When cellBarred-eRedCap1Rx is set to "barred" in SIB1, an eRedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, if the eRedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true"; or

-	When cellBarred-eRedCap2Rx is set to "barred" in SIB1, an eRedCap UE that supports 2Rx branches can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, if the eRedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true"; or

6.1.2.35.3	Test description

6.1.2.35.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11 have different tracking areas according to TS 38.508-1 [4] Table 4.4.2-3.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None

Preamble:

-	The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

6.1.2.35.3.2	Test procedure sequence

Table6.1.2.35.3.2-1 for FR1 illustrates the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions from preamble. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.35.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -88 |  |

Table 6.1.2.35.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Switch on UE. | - | - | - | - |
| 2 | Wait 60s to make sure that UE has camped on acceptable cell NR Cell 1. |  |  |  |  |
| 3 | Make the UE attempt an IMS emergency call.(Note 1) | - | - | - | - |
| 4 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12 on NR Cell 1? | - | - | 1 | P |
| 5 | Make the UE release the emergency call. (Note 1) | - | - | - | - |
| 6 | Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed on NR Cell 1. | - | - | - | - |
| 7 | The SS releases the RRC connection on NR Cell 1 (Note 2). | <-- | NR RRC: RRCRelease | - | - |
| Note 1: This can be done by an AT/MMI command. Note 2: The UE may optionally transmit a De-registration REQ during Step7. |  |  |  |  |  |

6.1.2.35.3.3	Specific message contents

Table 6.1.2.35.3.3.3-1: SIB1 for NR Cell 1 and NR Cell 11 (preamble and all steps, Table 6.1.2.35.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| intraFreqReselection-eRedCap-r18 | allowed |  |  |
| eRedCap-ConfigCommon-r18 SEQUENCE { |  |  |  |
| cellBarredeRedCap-r18 SEQUENCE { |  |  |  |
| cellBarredeRedCap1Rx-r18 | barred |  |  |
| cellBarredeRedCap2Rx-r18 | barred |  |  |
| } |  |  |  |
| } |  |  |  |
| barringExemptEmergencyCall-r18 | true |  | NR Cell 1 |
|  | Not present |  | NR Cell 11 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>