3GPP TSG-RAN5 Meeting #104	R5-245645

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4573 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new single DCI test case 7.1.1.7.5 - Reference cell is not scheduling cell - DCI format 0_3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test configuration: UL CA test case, DCI 0-3/1-3 is configured, and multi–Cell PDSCH/PUSCH are scheduled based on FDRA field or Scheduled Cell indicator field. In addition, scheduling cell = SpCell = NR Cell 1, reference cell = SCell = NR Cell 3, while SpCell (NR Cell 1) and SCell (NR Cell 3) are co-scheduled Cell.. According to RAN1 work assumption, DCI size/BD/CCE of the DCI format 0_3/1_3(Single DCI) is counted on the reference cell. if search space of the DCI format 0_3/1_3 is configured on the cell in addition to the scheduling cell, the reference cell is one cell of the set of cells which Ssearch space of DCI format 0_3/1_3 is configured on one cell of the set of cells and associated with the search space of the scheduling cell with the same search space ID. The DCI size for USS DCI 0_1/DCI 1_1 and USS DCI 0_3/1_3 does not need to be updated in NR Cell 1 to satisfy the requirement in 38.212 “the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell” because the USS DCI 0_3/DCI 1_3 is counted on NR Cell 3. - Before single DCI is configured: UE have to monitor CSS DCI 0_0/DCI 1_0 and USS DCI 0_1/DCI 1_1. The total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for NR Cell 1 >> FOR NR Cell 1 DCI size 1: CSS DCI 0_0/DCI 1_0 DCI size 2: USS DCI 0_1 DCI size 3: USS DCI 1_1 - After single DCI is configured: UE have to monitor CSS DCI 0_0/DCI 1_0 , USS DCI 0_1/DCI 1_1 and USS DCI 0_3/DCI 1_3. The total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for NR Cell 1 >> FOR NR Cell 1 DCI size 1: CSS DCI 0_0/DCI 1_0 DCI size 2: USS DCI 0_1 DCI size 3: USS DCI 1_1 >> FOR NR Cell 3 if only DCI 0_3 is configured in the TC based on UE capability. DCI size 1: USS DCI 0_3 Else if DCI 0_3 and DCI 1_3 are configured in the TC based on UE capability DCI size 1: USS DCI 1_3 DCI size 2: USS DCI 0_3 - Test case is added to test: - When SCell is deactivated, UE stop monitoring DCI format 0_3 on the SpCell (NR Cell 1). - When SCell is activated, UE receive DCI format 0_3 for SCell (NR Cell3) and start the sCellDeactivationTimer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new SingleDCI test case 7.1.1.7.5 Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MC_enh-UEConTest WP is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0497 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244705r3. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.7.5	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer

7.1.1.7.5.1	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.5.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SCell configured, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e the reference cell is SCell ) }

ensure that {

when { UE receives an SCell Activation/Deactivation MAC CE activating the Scell and sCellDeactivationTimer does not expire }

then { UE monitors PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell(i.e monitors DCI format 0_3 on the scheduling cell for the set of cells) }

}

(2)

with( UE in RRC_CONNECTED state with SCell activated, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e the reference cell is SCell )  )

ensure that {

when{ UE receives a DCI format 0-3 on NR SpCell PDCCH to schedule SCell }

then {  UE restarts the sCellDeactivationTimer }

}

(3)

with ( UE in RRC_CONNECTED state with SCell activated, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e the reference cell is SCell ))

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE deactivates the SCell and stops monitoring PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell (i.e stops monitoring DCI format 0_3 on the scheduling cell for the set of cells) }

}

7.1.1.7.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.9 and TS 38.212, clauses 7.3.1.0. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

…

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

…

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation;

…

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

[TS 38.213, clause 10.1]

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP. If the UE monitors PDCCH candidates for detection of one or both of DCI format 0_3 and DCI format 1_3 for scheduling on serving cells from a set of serving cells, the serving cell for counting the size of one or both DCI format 0_3 and DCI format 1_3, respectively, is

-	the scheduling cell, if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell

-	a serving cell from the set of serving cells, if search space sets with same searchSpaceId for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the serving cell and on the scheduling cell.

…

…

In the following, if a UE monitors PDCCH candidates on a scheduling cell for detection of DCI format 0_3 or DCI format 1_3 for scheduling on serving cells from a set of serving cells, the serving cell for counting the PDCCH candidates and a corresponding number of non-overlapping CCEs is

-	the scheduling cell, if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell

-	a serving cell from the set of serving cells, if search space sets with same searchSpaceId for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the serving cell and on the scheduling cell.

7.1.1.7.5.1.3	Test description

7.1.1.7.5.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.7.5.1.3.1-1 and in additionNR Cell 3 is configured as NR Active SCell.

Table 7.1.1.7.5.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.5.1.3.2	Test procedure sequence

Table 7.1.1.7.5.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message on NR SpCell (NR Cell 1) to configure NR SCell (NR Cell 3) and DCI format 0_3/1_3. (Note 1) | <-- | (RRCReconfiguration) | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 4 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 5 | 200 ms after step 4, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 2) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 6 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1 | P |
| 8 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | (UL Grant) DCI format 0_3 (only SCell is scheduled) | - | - |
| 9 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 6 on NR SCell. | --> | MAC PDU | - | - |
| 10 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 2) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 11 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 9 on NR SpCell. | <-- | MAC PDU | - | - |
| 12 | 200 ms after step 5, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 13 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1,2 | P |
| 15 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (only SpCell is scheduled) | - | - |
| 16 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 13 on NR SpCell. | --> | MAC PDU | - | - |
| 17 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 18 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 16 on NR SpCell. | <-- | MAC PDU | - | - |
| 19 | 400 ms (sCellDeactivationTimer = 320 ms) after step 8, the the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 20 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 21 | The UE transmit a Scheduling Request on PUCCH on NR SpCell. | --> | (SR) | - | - |
| 22 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (only SpCell is scheduled) | - | - |
| 23 | Check: Does the UE transmit MAC PDU on NR SpCell? | --> | MAC PDU | 3 | F |
| 24 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (only SCell is scheduled) | - | - |
| 25 | Check: Does the UE transmit MAC PDU on NR SCell? | --> | MAC PDU | 3 | F |
| 26 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 27 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 20 on NR SpCell. | --> | MAC PDU | - | - |
| 28 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 29 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 27 on NR SpCell. | <-- | MAC PDU | - | - |
| Note 1: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is NR SCell (NR Cell 3). Therefore, the single DCI (DCI format 0_3/1_3) is counted on the NR Cell 3, and DCI size alignment procedure specified in 38.212 clause 7.3.1.0 for NR Cell 1 is complete in step3. Note 2: If the pc_multiCell_PUSCH_DCI_1_3_SameSCS_r18 = True, downlink assignment is DCI format 1_3, else the downlink assignment is DCI format 1_1. |  |  |  |  |  |

7.1.1.7.5.1.3.3	Specific message contents

Table 7.1.1.7.5.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.5.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-2: CellGroupConfig (Table 7.1.1.7.5.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 7.1.1.7.5.1.3.3-6 |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.5.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.5.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-3: ServingCellConfig (Table 7.1.1.7.5.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition (DCI_0_3) OR (DCI_0_3 AND DCI_1_3) (NOTE 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.5.1.3.3-4 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.7.5.1.3.3-5 |  |
| } |  |  |  |
| mc-DCI-SetOfCellsToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18 { | 1 entry |  |  |
| MC-DCI-SetOfCells-r18[1] SEQUENCE { |  |  |  |
| scheduledCellComboListDCI-1-3 | Not present |  | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3=False AND pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3=True |
| scheduledCellComboListDCI-1-3 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| } |  |  |  |
| scheduledCellComboListDCI-0-3-r18 | Not present |  | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_0_3=False AND pc_coScheduledCellIndicationScheme_r18_fdra_DCI_0_3=True |
| scheduledCellComboListDCI-0-3-r18 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-0-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-0-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-0-3-r18 (i.e SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-0-3-r18 (i.e SpCell) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: If the pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18=True, the condition is DCI_0_3 AND DCI_1_3, else, the condition is DCI_0_3. |  |  |  |

Table 7.1.1.7.5.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.7.5.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_0_3 |  |  |
|  | PDCCH-Config with condition DCI_0_3 and DCI_1_3 |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
|  | PDSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-5: BWP-UplinkDedicated (Table 7.1.1.7.5.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-6: PhysicalCellGroupConfig (Table 7.1.1.7.5.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  | pc_harqFeedbackType1_DCI_1_3=False AND pc_harqFeedbackType2_DCI_1_3=True |
|  | semiStatic |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-7: ServingCellConfig (Table 7.1.1.7.5.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.5.1.3.3-8 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.7.5.1.3.3-9 |  |
| } |  |  |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-8: BWP-DownlinkDedicated (Table 7.1.1.7.5.1.3.3-7)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config | Table 7.1.1.7.5.1.3.3-10 |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
|  | PDSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-9: BWP-UplinkDedicated (Table 7.1.1.7.5.1.3.3-7)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-15 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-10: PDCCH-Config (Table 7.1.1.7.5.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace | entry 1 Table 7.1.1.7.5.1.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-11: SearchSpace (Table 7.1.1.7.5.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition DCI_MC |  |  |
| controlResourceSetId | Not present |  |  |
| monitoringSlotPeriodicityAndOffset | Not present |  |  |
| duration | Not present |  |  |
| monitoringSymbolsWithinSlot | Not present |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n4 |  | FR1 |
|  | n3 |  | FR2 |
| aggregationLevel4 | n2 |  |  |
| aggregationLevel8 | n1 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType | Not present |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-10: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12 and 17 for scheduledCellComboListDCI-1-3 present configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The frist entry of scheduledCellComboListDCI-1-3 | “00” |  |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 Note: NcellDL=1 | - |  |

Table 7.1.1.7.5.1.3.3-11: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12 and 17 for scheduledCellComboListDCI-1-3 notpresent configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - |  |

Table 7.1.1.7.5.1.3.3-12: Physical layer parameters for DCI format 0_3 (steps 8, 15, 22 and 24 for scheduledCellComboListDCI-0-3 present configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The frist entry of scheduledCellComboListDCI-1-3 | “00” | Step 15,22 |
|  | The second entry of scheduledCellComboListDCI-1-3 | “01” | Step 8,24 |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 Note: NcellDL=1 | - | Step 15,22 |
|  | block number 1(SCell) : dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 Note: NcellDL=1 | - | Step 8,24 |

Table 7.1.1.7.5.1.3.3-13: Physical layer parameters for DCI format 0_3 (steps 8, 15, 22 and 24 for scheduledCellComboListDCI-0-3 notpresent configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - | Step 15,22 |
|  | block number 1(SpCell): all bits set to 1 block number 2(SCell) : dependent on the Downlink TBS specificed in 38.523-3 Annex B.1 Note: NcellDL=2 | - | Step 8,24 |

7.1.1.7.5.2	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Inter-Band CA

The scope and description of the present TC is the same as test case 7.1.1.7.5.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

- 	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

7.1.1.7.5.3	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.7.5.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

<End of modified section 1>