3GPP TSG-RAN5 Meeting #104	R5-245654

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3230 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to test procedure for UUAA-MM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, ID_UAS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Registration Accept message of the 5GS UUAA-MM procedure contains a Service-level-AA pending indication. The specific message content of the Registration Accept needs to be updated in Table 4.9.44.2.2-2 with the Service-level-AA pending indication. TS 38.508-1 is self-referring in test procedure 4.9.44, which is against the 3GPP drafting rules. Several message contents are still set to FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 4.9.44.2.2-2 with the Service-level-AA pending indication with Value/remark. NOTE 2 is added for ‘UUAA parameter transport +CUUAAPT’. Also References to TS 38.508-1 have been deleted. Message contents are updated corrected font in table 4.9.44.2.2-2 Replaced FFS with values that can be used in the AT command and specific message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The generic test procedure for UUAA-MM and the specific message contents may remain incorrect. Also, The specification will not follow the 3GPP drafting rules. The test cases cannot be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.44 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 4.9.44	Test procedure for UUAA-MM

4.9.44.1	Scope

This procedure aims to cover the UUAA-MM procedure that UEs supporting UAS perform after successful registration.

4.9.44.2	Procedure description

4.9.44.2.1	Initial conditions

User Equipment:

-	The UE has performed steps 1 – 15 of table 4.5.2.2-2.

4.9.44.2.2	Procedure sequence

Table 4.9.44.2.1-1: Test procedure sequence UUAA-MM

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 0 | Configure the UE to request transport of UUAA parameters (NOTE 2) | - | - | - | - |
| 1 | SS sends authentication message based on authentication method used through DL NAS TRANSPORT message to request Authentication. (NOTE 1) | <-- | DL NAS TRANSPORT | - | - |
| 2 | UE sends authentication message through UL NAS TRANSPORT message in response to authentication request. | --> | UL NAS TRANSPORT | - | - |
| 3 | SS sends authentication and authorisation result message through DL NAS TRANSPORT message. | <-- | DL NAS TRANSPORT | - | - |
| 4 | The SS transmits a CONFIGURATION UPDATE COMMAND message including UAV authorization information. | <-- | CONFIGURATION UPDATE COMMAND | - | - |
| 5 | UE transmit a CONFIGURATION UPDATE COMPLETE message | --> | CONFIGURATION UPDATE COMPLETE | - | - |
| - | EXCEPTION: Step 6a1 describes behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 6 – 7Ba2 | Steps 18 – 19Ba2 of Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| NOTE 1: The UE may send a PDU SESSION ESTABLISHMENT REQUEST message before receiving the DL NAS TRANSPORT message NOTE 2: The +CUUAAPT command is used to request the UE to transport the UUAA parameters to the network . The values to be used for the UUAA parameters shall be the same as those included in the specific message contents |  |  |  |  |  |

4.9.44.3	Specific message contents

Table 4.9.44.2.2-1: REGISTRATION REQUEST (Preamble)

| Derivation path: Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Service-level-AA container | Present |  |  |
| Service-level-AA parameter |  | Service-level device ID |  |
| Service-level device ID IEI | ‘10’H |  |  |
| Service-level device ID length | ‘08’H | 8 octets |  |
| Service-level device ID | ‘AABBCCDDEEFFAABB’H | configured using AT command |  |
| Service-level-AA parameter |  | Service-level-AA server address |  |
| Service-level-AA server address IEI | ‘20’H |  |  |
| Service-level-AA server address length | ‘20’H | 3 octets |  |
| Service-level-AA server address type | ‘01’H | IPv4 |  |
| Service-level-AA server address | ‘A B C D’H | 2 octets |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0008’H | 8 octets |  |
| Service-level-AA payload | ‘AABBCCDDEEFFABAB’H | configured using AT command |  |

Table 4.9.44.2.2-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Service-level-AA container |  |  |  |
| Service-level-AA pending indication | ‘1010 0001’B |  |  |

Table 4.9.44.2.2-3: UL NAS TRANSPORT (Step 2, Table 4.9.44.2.1-1)

| Derivation path: Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Payload container type | ‘1001’B | Service-level-AA container |  |
| Payload container | Present |  |  |
| Length of payload container contents |  | 4 octets |  |
| Payload container contents |  |  |  |
| Service-level-AA container | Present |  |  |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0008’H | 8 octets |  |
| Service-level-AA payload | ‘AABBCCDDEEFFABAB’H | configured using AT command |  |

Table 4.9.44.2.2-4: DL NAS TRANSPORT (Step 1, Step 3, Table 4.9.44.2.1-1)

| Derivation path: Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Payload container type | ‘1001’B | Service-level-AA container |  |
| Payload container | Present |  |  |
| Length of payload container contents |  | 4 octets |  |
| Payload container contents |  |  |  |
| Service-level-AA container |  |  | Step 1, Step 3 |
| Service-level-AA payload type IEI | ‘40’H |  |  |
| Service-level-AA payload type length | ‘01’H | 1 octet |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |  |
| Service-level-AA parameter |  | Service-level-AA payload |  |
| Service-level-AA payload IEI | ‘70’H |  |  |
| Service-level-AA payload length | ‘0008’H | 8 octets |  |
| Service-level-AA payload | ‘AABBCCDDEEFFABAB’H | configured using AT command |  |

Table 4.9.44.2.2-5: CONFIGURATION UPDATE COMMAND (Step 4, Table 4.9.44.2.1-1)

| Derivation path: Table 4.7.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Configuration update indication | 0001 | Acknowledgement (ACK) requested |  |
| Service-level-AA container |  |  |  |
| Service-level-AA response | ‘00000010’B | Service level authentication and authorization was successful |  |

<End of modified section>