3GPP TSG-RAN5 Meeting #104	R5-245655

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3318 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the procedure of establishing multiple additional PDN connections in S1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Establishment of additional PDN connection is not handled correctly when testcases are executed with conditions IMS_Internet and MobileData_OFF, similar to Step0a1 in Table 4.5A.16.3-1 of TS 36.508. UE will not bring up Interenet PDN if MobileData_OFF. Hence, it has to be bring up with AT command. Same changes proposed as in R5-195308, |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce an exception step in Table 4.5A.2B.2.2-1, for establishment of additional PDN by MMI or AT command with APN “Internet”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some testcases configured with pc_noOf_PDNsSameConnection=1 will fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5A.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: IMS_Internet condition is undefined in TS38.508-1 and replaced with ExpectedNumberOfNewPDNConnections>0 This document is final version of R5-245197 and one revision. |  |  |  |  |  |  |  |  |

<Start of change>

### 4.5A.2B	Procedure to establish multiple additional PDN connections in S1

4.5A.2B.1	Scope

The present procedure is intended for test scenarios where it is desirable, due to the tested requirements (e.g. intersystem change between S1 and N1 mode), to allow the UE to set up as many as it may want, and in any order, PDN connections after the first PDN connection is established during the ATTACH to EPS procedure (i.e. when the UE is operating in S1 mode).

Table 4.5A.2B.1-1: Conditions and other parameters

| Condition | Explanation |
| --- | --- |
| ExpectedNumberOfNewPDNConnections | Usage: Parameter determining procedure sequence. A value for this parameter shall be provided when the procedure is called. Unless the test requires specific number of PDN connections to be established the value should be either pc_noOf_PDNsSameConnection or pc_noOf_PDNsNewConnection ([19]). Meaning: The number of PDN connections which are expected to happen. Depends on the UE configuration and/or the context in which the procedure is used. |
| K | Usage: Parameter determining procedure sequence. Meaning: The number of PDN CONNECTIVITY REQUEST messages already processed including the one that is currently being processed. |
| L | Usage: Parameter determining procedure sequence. Meaning: The number of PDN CONNECTIVITY REQUEST messages being received so far. |
| N | Usage: Parameter determining procedure sequence. The parameter is initialised with the value of ExpectedNumberOfNewPDNConnections Meaning: Loop (step sequence repetition) control. |
| MobileData_OFF | Mobile data associated with Internet PDN is disabled on the DUT. It is configured via setting px_MobileDataOn (TS 36.523-3[20] Table 9.1-1) = false |

4.5A.2B.2	Procedure description

4.5A.2B.2.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5.

User Equipment:

-	The UE is in Registered, RRC_CONNECTED state (State 2).

The default system information messages are used.

4.5A.2B.2.2	Procedure sequence

Table 4.5A.2B.2.2-1: Establishment of additional PDN connectivity

| St | Procedure | Message Sequence |  | Verdict |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 0 | Set K = 0, L = 0, N = ExpectedNumberOfNewPDNConnections | - | - | - |
| - | EXCEPTION: Depending upon UE configuration Step 0Aa1 may be performed | - | - | - |
| 0Aa1 | If ExpectedNumberOfNewPDNConnections > 0 AND MobileData_OFF then the request of connectivity to an additional PDN is performed by MMI or AT command with APN “Internet”. | - | - | - |
| 1 | The procedure specified in Table 4.5A.2B.2.2-2 takes place. | - | - | - |
| 2 | Set K = K +1. | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 3-6a1 below the events specified in Table 4.5A.2B.2.2-2 may take place. | - | - | - |
| 3-4 | Step 2-3 as defined in TS 36.508 [2], Table 4.5A.16.3-1, Generic Test Procedure to establish additional PDN connectivity take place. | - | - | - |
| - | EXCEPTION: IF the 'IP address allocation' for the APN for which the PDN connection is established is set to "Yes" in Table 4.8.4-1 THEN, in parallel to the event described in step 5 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 [2], subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - |
| - | EXCEPTION: IF the 'IMS registration' for the APN for which the PDN connection is established is set to "Yes" in Table 4.8.4-1, THEN in parallel to the event described in step 5 below the relevant generic procedure for IMS signalling in the U-plane specified in Table 4.8.4-1 takes place. | - | - | - |
| 5 | Step 4 as defined in TS 36.508 [2], Table 4.5A.16.3-1, Generic Test Procedure to establish additional PDN connectivity takes place. | - | - | - |
| - | EXCEPTION: Steps 6a1 to 6b1 describe behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - |
| 6a1 | IF L > K (NOTE 1) THEN repeat from step 2. | - | - | - |
| 6b1 | ELSE IF K < N (NOTE 2) THEN repeat from step 1. | - | - | - |
| NOTE 1: One (or more) further PDN CONNECTIVITY REQUEST message has been received in parallel. NOTE 2: Less PDN CONNECTIVITY REQUEST messages than expected have been received and processed so far and consequently further requests are expected from the UE. |  |  |  |  |

Table 4.5A.2B.2.2-2: Reception of PDN CONNECTIVITY REQUEST message

| St | Procedure | Message Sequence |  | Verdict |
| --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 1 | Start Wait_Timer = 10 sec. NOTE: 10 sec is an arbitrary value. | - | - | - |
| - | EXCEPTION: Steps 2a1 to 2b1 describe behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - |
| 2a1 | Step 1 as defined in Table 4.5A.16.3-1, specified in TS 36.508 [2], subclause 4.5A.16 Generic Test Procedure to establish additional PDN connectivity takes place. The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN. | --> | PDN CONNECTIVITY REQUEST | - |
| 2a2 | Stop Wait_Timer. | - | - | - |
| 2a3 | Set L = L +1. | - | - | - |
| 2a4 | Check: Does the L>N? (NOTE 1) | - | - | F |
| 2b1 | Check: Does Wait_Timer expire? (NOTE 2) | - | - | F |
| NOTE 1: The SS shall raise a fail verdict when there are more PDN CONNECTIVITY REQUEST messages received than expected (L>N) and terminate the test. The reason for such a behaviour can be e.g. wrongly set PICS with which the ExpectedNumberOfNewPDNConnections was initiated, NOTE 2: Unless this is specified explicitly otherwise, e.g. in a test procedure which calls the procedure specified in the present table, the SS shall raise a fail verdict when there are less PDN CONNECTIVITY REQUEST messages than expected received at this point (note that when this procedure is called from the main behaviour K < N). The reason for such a behaviour can be e.g. wrongly set PICS with which the ExpectedNumberOfNewPDNConnections was initiated. |  |  |  |  |

#### 4.5A.2B.3	Specific message contents

All specific message contents shall be referred to TS 36.508 [2] subclauses 4.6 and 4.7 with the exceptions specified below.

Table 4.5A.2B.3-1: Message PDN CONNECTIVITY REQUEST (step 2a1, Table 4.5A.2B.2.2-2)

| Derivation path: TS 36.508 [2], Table 4.7.3-20. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not present or One of the provided APN(s) in the Table 4.8.4-1 | If present, the SS shall initialise the APN_Default=False If not present NOTE 2 |  |
| NOTE 1: Unless explicitly specified otherwise, the SS uses the Access point name value to address the entry of Table 4.8.4-1 to be used for the subsequent signalling of the PDN connectivity establishment and verifying specific UE behaviour e.g. depending on the type of the APN the UE may perform some actions. NOTE 2: The SS uses pc_APN_Default_Configuration to address the entry of Table 4.8.4-1 to be used for the subsequent signalling of the PDN connectivity establishment. |  |  |  |

Table 4.5A.2B.3-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 3, Table 4.5A.2B.2.2-1; Step 2, Table 4.5A.16.3-1 TS 36.508 [2])

| Derivation path: TS 36.508 [2], Table 4.7.3-6 with CONDITION Interworking_with_5GS. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS QoS | The EPC default bearer context of the entry in Table 4.8.4-1 which has been determined at step 1 |  |  |
| Access point name | IF the UE provided an Access point name in step 1 THEN the SS returns this name; OTHERWISE the SS includes the DNN/APN ID value of the entry in Table 4.8.4-1 which has been determined at step 1. |  |  |
| Container ID n+3 | '001C'H |  |  |
| Length of container ID n+3 contents |  |  |  |
| Container ID n+3 contents | 5GC QoS rule of the entry in Table 4.8.4-1 which has been determined at step 1 |  |  |
| Container ID n+4 | '001F'H |  |  |
| Length of container ID n+4 contents |  |  |  |
| Container ID n+4 contents | As per the relevant QoS rule (Container ID n+3) |  |  |

<End of change>