3GPP TSG-RAN5 Meeting #104	R5-245661

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4588 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new R18 eDRX test case 11.7.4 - RAN PTW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This R18 eDRX test case is added to test that UE can receive CN paging he RAN paging when eDRX cycle in RRC_INACTIVE is larger than 10.24s and PTW for RAN is configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new eDRX test case 11.7.4 eDRX / Inactive / CN-initiated paging / RAN-initiated paging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_redcap_enh_plus_CT1-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0499 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244732r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.7.4	eDRX / Inactive / CN-initiated paging / RAN-initiated paging

11.7.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and configured eDRX, RAN Extended Paging Cycle is larger than 10.24s , RAN Extended Paging Cycle is equal to IDLE mode eDRX cycle, and PTW length configured by RAN is shorter than PTW length configured by CN }

ensure that {

when { In an overlapped PH (i.e PH includes both CN configured PTW and RAN configured PTW ), during the overlapped part of CN configured PTW and RAN configured PTW , the network sends a CN-initiated Paging message within a paging occasion }

then { UE goes to RRC_IDLE state and establishes an RRC connection }

}

(2)

with { UE in NR RRC_INACTIVE state and configured eDRX, RAN Extended Paging Cycle is larger than 10.24s , RAN Extended Paging Cycle is equal to IDLE mode eDRX cycle, and PTW length configured by RAN is shorter than PTW length configured by CN }

ensure that {

when { In an overlapped PH, during CN configured PTW and outside RAN configured PTW , the network sends a CN-initiated Paging message within a paging occasion }

then { UE goes to RRC_IDLE state and establishes an RRC connection }

}

(3)

with { UE in NR RRC_INACTIVE state and configured eDRX, RAN Extended Paging Cycle is larger than 10.24s , RAN Extended Paging Cycle is equal to IDLE mode eDRX cycle, and PTW length configured by RAN is shorter than PTW length configured by CN }

ensure that {

when { In an overlapped PH, during the overlapped part of CN configured PTW and RAN configured PTW , the network sends a RAN-initiated Paging message within a paging occasion }

then { UE resumes RRC connection }

}

(4)

with { UE in NR RRC_INACTIVE state and configured eDRX, RAN Extended Paging Cycle is larger than 10.24s , RAN Extended Paging Cycle is shorter than IDLE mode eDRX cycle, and PTW length configured by RAN is larger than PTW length configured by CN }

ensure that {

when { In an overlapped PH, outside CN configured PTW and during RAN configured PTW , the network sends a RAN-initiated Paging message within a paging occasion }

then { UE resumes RRC connection }

}

(5)

with { UE in NR RRC_INACTIVE state and configured eDRX , RAN Extended Paging Cycle is larger than 10.24s , RAN Extended Paging Cycle is shorter than IDLE mode eDRX cycle, and PTW length configured by RAN is larger than PTW length configured by CN }

ensure that {

when { In a non-overlapped PH(i.e PH includes only CN configured PTW or RAN configured PTW ), outside CN configured PTW and during RAN configured PTW , the network sends a RAN-initiated Paging message within a paging occasion }

then { UE resumes RRC connection }

}

(6)

with { UE in NR RRC_INACTIVE state and configured eDRX , RAN Extended Paging Cycle is larger than 10.24s , RAN Extended Paging Cycle is shorter than IDLE mode eDRX cycle, and PTW length configured by RAN is larger than PTW length configured by CN }

ensure that {

when { the network changes SIB1 to exclude eDRX-AllowedInactive-r18 }

then { UE applies the new system information and stops using extended DRX for RAN-initiated paging in RRC_INACTIVE state }

}

11.7.4.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 6.2.2 and TS 38.304, clause 7.1 and clause 7.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.304, clause 7.1]

The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE.

…

In RRC_INACTIVE state, if the UE operates in eDRX and eDRX is configured by RRC, i.e., TeDRX, RAN (if any), and upper layers, i.e., TeDRX, CN, as defined in clause 7.4:

….

-	If TeDRX, CN is longer than 1024 radio frames:

…

-	else if used TeDRX, RAN is longer than 1024 radio frames:

-	During the overlapped part of CN configured PTW and RAN configured PTW, T is determined by the shortest of the UE specific DRX value configured by RRC, the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information;

-	During CN configured PTW and outside RAN configured PTW, T is determined by the shortest of the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information;

-	Outside CN configured PTW and during RAN configured PTW, T is determined by the UE specific DRX value configured by RRC.

[TS 38.304, clause 7.4]

If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE operating in eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formula:

The PH for CN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX, CN= (UE_ID_H mod TeDRX, CN), where

-	TeDRX, CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX, CN = 2, …, 1024 Hyper-frames) configured by upper layers.

The PH for RAN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX_RAN= (UE_ID_H mod TeDRX_RAN), where

-	TeDRX_RAN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_RAN = 2, …, 1024 Hyper-frames) configured by RRC.

For CN configured PTW:

PTW_start denotes the first radio frame of the PH for CN that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX, CN, where

-	ieDRX, CN = floor(UE_ID_H /TeDRX, CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by upper layers

For RAN configured PTW:

PTW_start denotes the first radio frame of the PH for RAN that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX_CN, where

-	ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by RRC

[TS 38.331, clause 6.2.2]

SIB1 field descriptions

eDRX-AllowedInactive The presence of eDRX-AllowedInactive-r17 indicates that extended DRX cycle equal to or shorter than 10.24 s for RAN paging is allowed in the cell for UEs in RRC_INACTIVE. The UE shall stop using extended DRX cycle equal to or shorter than 10.24 s for RAN paging in RRC_INACTIVE if eDRX-AllowedInactive-r17 is not present. The presence of eDRX-AllowedInactive-r18 indicates that extended DRX cycle longer than 10.24 s for RAN paging is allowed in the cell for UEs in RRC_INACTIVE. The UE shall stop using extended DRX cycle longer than 10.24 s for RAN paging in RRC_INACTIVE if eDRX-AllowedInactive-r18 is not present.

11.7.4.3	Test description

11.7.4.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC.

-	System information combination NR-1 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cell 1, with SIB 1 modified as per Table 11.7.4.3.3-1.

UE:

-	UE configured to request eDRX.

Preamble:

-	UE is in state 0N-B according to TS 38.508-1 [4] Table 4.4A.2-2.

11.7.4.3.2	Test procedure sequence

Table 11.7.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-3 | Steps 19B-19C of the generic procedure for NR RRC_INACTIVE as specified in TS 38.508-1 [4] Table 4.5.3.2-1 are performed. | - | - | - | - |
| 4 | The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5-13 | Steps 19E-19M of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 14 | The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 15 | The UE transmits a REGISTRATION COMPLETE message | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 16 | Steps 19TAa1-19TCa2 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 17 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 18 | The SS transmits a CN-initiated Paging message to the UE in a valid PO which is during the overlapped part of CN configured PTW and RAN configured PTW.(Note 1) | <-- | NR RRC: Paging | - | - |
| 19 | Check: Does the UE transmit an RRCSetupRequest message indicating that UE has moved to RRC_IDLE state? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 20 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 21 | The UE transmits an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 22-25 | Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 26 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 27 | The SS transmits a CN-initiated Paging message to the UE in a valid PO which is during CN configured PTW and outside RAN configured PTW.(Note 2) | <-- | NR RRC: Paging | - | - |
| 28 | Check: Does the UE transmit an RRCSetupRequest message indicating that UE has moved to RRC_IDLE state? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 29 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 30 | The UE transmits an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 31-34 | Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 35 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 36 | The SS transmits a RAN-initiated Paging message to the UE in a valid PO which is during the overlapped part of CN configured PTW and RAN configured PTW.(Note 1) | <-- | NR RRC: Paging | - | - |
| 37 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 3 | P |
| 38 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 39 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 40 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 41 | The SS transmits a RAN-initiated Paging message to the UE in a valid PO which is outside CN configured PTW and during RAN configured PTW in an overlapped PH.(Note 3) | <-- | NR RRC: Paging | - | - |
| 42 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 4 | P |
| 43 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 44 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 45 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 46 | The SS transmits a RAN-initiated Paging message to the UE in a valid PO which is outside CN configured PTW and during RAN configured PTW in a non-overlapped PH.(Note 3) | <-- | NR RRC: Paging | - | - |
| 47 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 5 | P |
| 48 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 49 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 50 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 51 | The SIB1 is updated according to Table 11.7.4.3.3-1 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 52 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 53 | The SS transmits a RAN-initiated Paging message to the UE in a valid PO which is derived by normal DRX and not belonging to extended DRX. | <-- | NR RRC: Paging | - | - |
| 54 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 6 | P |
| 55 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 56 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| Note 1: During the overlapped part of CN configured PTW and RAN configured PTW, DRX cycle is determined by ran-PagingCycle (32rf) configured in RRCRelease. Note 2: During the CN configured PTW and outside RAN configured PTW, DRX cycle is determined by defaultPagingCycle (128rf) configured in SIB1. Note 3: During RAN configured PTW and Outside CN configured PTW, DRX cycle is determined by ran-PagingCycle (32rf) configured in RRCRelease. |  |  |  |  |  |

11.7.4.3.3	Specific message contents

Table 11.7.4.3.3-1: SIB1 (preamble and step 51, Table 11.7.4.3.2-1)

| Derivation path: TS 38.508-1 [4] , table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| hyperSFN-r17 | Current H-SFN Value |  |  |
| eDRX-AllowedIdle-r17 | true |  |  |
| eDRX-AllowedInactive-r17 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| eDRX-AllowedInactive-r18 | Not present |  | Step 51 |
|  | true |  | Preamble |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.7.4.3.3-2: REGISTRATION REQUEST (step 4, Table 11.7.4.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested extended DRX parameters |  |  |  |
| Paging Time Window | Present but contents not checked |  |  |
| eDRX value | Present but contents not checked |  |  |
| Extended Paging Time Window | Not checked | Note 1 |  |
| Note 1: Extended Paging Time Window is an optional IE. If UE request eDRX cycle larger than 10,24 seconds, UE may include Extended Paging Time Window. |  |  |  |

Table 11.7.4.3.3-3: REGISTRATION ACCEPT (step 14, Table 11.7.4.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated extended DRX parameters |  |  |  |
| Paging Time Window | Any allowed value | Note 1 |  |
| eDRX value | ‘0100’B | 40,96 seconds |  |
| Extended Paging Time Window | ‘00000001’B | 2,56 seconds |  |
| Note 1: In NR connected to 5GCN, the Paging Time Window field is ignored and the PTW value is included in the Extended Paging Time Window field. |  |  |  |

Table 11.7.4.3.3-4: RRCRelease (steps 17, 26, 35, 40, 45 and 50, Table 11.7.4.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.6.1-16 with condition NR_RRC_INACTIVE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| ran-ExtendedPagingCycle-r17 | Not present |  |  |
| ncd-SSB-RedCapInitialBWP-SDT-r17 | Not present |  |  |
| resumeIndication-r18 | Not present |  |  |
| srs-PosRRC-InactiveEnhanced-r18 | Not present |  |  |
| ran-ExtendedPagingCycleConfig-r18 SEQUENCE { |  |  |  |
| extendedPagingCycle-r18 | hf4 |  | Step 17, 26 and 35 |
|  | hf2 |  | Step 40, 45 and 50 |
| pagingPTWLength-r18 | ms1280 |  | Step 17, 26 and 35 |
|  | ms3840 |  | Step 40, 45 and 50 |
| } |  |  |  |
| multicastConfigInactive-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.7.4.3.3-5: Paging  (steps 36, 41, 46 and 53, Table 11.7.4.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

<End of modified section 1>