3GPP TSG-RAN5 Meeting #104	R5-245674

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0119 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCData Message Store Function terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was no terminology for the Message Store Function for MCData test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added terminology for the Message Store Function for MCData test cases and fixed minor typos - Corrected Figure 7.2.1-1 - Added reference for OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C: "RESTful Network API for Network Message Storage". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases involving the Message Store Function in TS 36.579-7 may not be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2, 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-244963 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

# 2	References

### …

[71]	OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C: "RESTful Network API for Network Message Storage".

### …

## 7.2	MCX functional architecture: HTTP, SIP

### 7.2.1	HTTP-Signalling

Figure 7.2.1-1 shows the functional architecture for HTTP-Signalling between the MCX-capable UE and different network entities as emulated by the test models.

Figure 7.2.1-1: Functional architecture for HTTP-Signalling

The UE is expected to establish HTTP connections to the following entities:

-	Initial Configuration Server: 
The UE gets configured with IP address (px_MCX_InitialConfigServer_IPAddress) and port number (px_MCX_InitialConfigServer_Port) of the server from which it retrieves the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
It depends on UE implementation how the IP address and port are configured at the UE but the UE is expected to support this to be configured. 
As there is no requirement by the core specs the HTTP connection is not security protected.

-	IdMS AUTH Endpoint:
The UE retrieves IP address (px_MCX_IdMS_auth_IPAddress) and port number (px_MCX_IdMS_auth_Port) from the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
The connection shall be TLS protected.

-	IdMS TOKEN Endpoint:
The UE retrieves IP address (px_MCX_IdMS_token_IPAddress) and port number (px_MCX_IdMS_token_Port) from the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
The connection shall be TLS protected.

-	HTTP Proxy:
The UE retrieves IP address (px_MCX_HTTP_Proxy_IPAddress) and port number (px_MCX_HTTP_Proxy_Port) from the MCS UE initial configuration document (TS 24.484 [14] clause 7.2).
The connection shall be TLS protected.

Design considerations and working assumptions:

-	Domain Name Server 
DNS is not needed for address resolution as in the URIs of the network entities in the MCS UE initial configuration document the IP address of the respective entity can be used as host name.

-	Initial UE configuration
The UE is able to retrieve the MCS UE initial configuration document (TS 24.484 [14] clause 7.2) via an HTTP connection to an Initial Configuration Server. Other means of initial configuration are out of scope for conformance testing.

-	HTTP signalling between UE and KMS
The UE is expected to support HTTP signalling with the KMS via the HTTP proxy using a transport key (TrK) according to TS 33.180 [43] clause 4.3.3 and clause 5.3.2.

-	Use of TLS
All HTTP traffic between the UE and the HTTP proxy and between the UE and the IdMS is protected using HTTPS (TS 33.180 [43] clause 5.1.3.1, clause 6.1.1 and B.12).
NOTE: Even though NOTE 2 of TS 24.482 [12] clause 6.2.1 might be interpreted as if the HTTP signalling to the authorization endpoint may not be TLS secured, 33.180 [43] mandates TLS to be used.

-	IP Addresses and port numbers
IP addresses and port numbers of the network entities as shown in figure 7.2.1-1 are given as PIXIT (see clause 9). As only restriction the tuple {IP address, port number} needs to be unique for each entity, but there is no restriction no to use one and the same IP address for all entities as long as they use different port numbers.

-	According to TS 24.282 [57] clause 10.2.2.1 the HTTP signalling between the MCData client and the media storage function (MSF) happens via an HTTP proxy according to annex A of TS 24.482 [12]. It is assumed that this is the same proxy as used for signalling with the KMS, CMS and GMS.

-	According to TS 24.282 [57] clause 21.1 the HTTP signalling between the MCData message store client and MCData message store function (MsgSF) happens via an HTTP proxy according to annex A of TS 24.482 [12]. It is assumed that this is the same proxy as used for signalling with the KMS, CMS and GMS.

-	According to RFC 2616 [69] clause 5.1.2 HTTP requests via an HTTP proxy require the Request-URI to be an absolute URI  HTTP requests to KMS, CMS and GMS shall have an absolute URI as Request-URI.

### …

### 9.2.3	MCX Constant PIXIT Definitions

Several parameters for MCX conformance testing can be defined as constants as they are neither preconfigured at the UE nor at the SS. Table 9.2.3-1 lists these constants.

Table 9.2.3-1: MCX Constant PIXIT Definitions

| Constant Name | Constant Type | Value | Description |
| --- | --- | --- | --- |
| tsc_MCX_KMS_Hostname | charstring | "kms." & px_MCX_DomainName_Organization_A | FQDN of the KMS; used in initial UE configuration as domain name for the 'kms' URI in the App-Server-Info. |
| tsc_MCX_CMS_Hostname | charstring | "cms." & px_MCX_DomainName_Organization_A | FQDN of the CMS; used in initial UE configuration as domain name for the 'cms' URI in the App-Server-Info. |
| tsc_MCX_GMS_Hostname | charstring | "gms." & px_MCX_DomainName_Organization_A | FQDN of the GMS; used in initial UE configuration as domain name for the 'gms' URI in the App-Server-Info. |
| tsc_MCX_KMS_CertUri | charstring | "certificate1." & tsc_MCX_KMS_Hostname | Name of the KMS certificate sent to the UE during MCX user authentication |
| tsc_MCX_IdMS_auth_UriPath | charstring | "/idms/auth" | URI path to address the IdMS authorization endpoint |
| tsc_MCX_IdMS_token_UriPath | charstring | "/idms/token" | URI path to address the IdMS token endpoint |
| tsc_MCX_IdMS_userauth_UriPath | charstring | "/idms/userauth" | relative URI at IdMS used for user authentication |
| tsc_MCX_KMS_init_UriPath | charstring | "/keymanagement/identity/v1/init" | Path of the Request-URI for "KMS Initialize" request according to TS 33.180 [43] D.2.3 |
| tsc_MCX_KMS_keyprov_UriPath | charstring | "/keymanagement/identity/v1/keyprov" | Path of the Request-URI for "KMS KeyProvision" request according to TS 33.180 [43] D.2.4 |
| tsc_MCX_GMSURI | charstring | "subscription-proxy." & tsc_MCX_GMS_Hostname | used for <GMS-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9; sub-domain of the GMS's hostname |
| tsc_MCX_CMSXCAPRootURI | charstring | "http://xcap." & tsc_MCX_CMS_Hostname | used for <CMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9C |
| tsc_MCX_GMSXCAPRootURI | charstring | "http://xcap." & tsc_MCX_GMS_Hostname | used for <GMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9B |
| tsc_MCX_MC_ID_User_A | charstring | px_MCX_User_A_username | According to TS 24.482 [12] clause 6.3.1 the MC ID set to the user name |
| tsc_MCPTT_PublicServiceId_A | charstring | "sip:" & "mcptt-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCPTT Server which is simulated by the SS |
| tsc_MCVideo_PublicServiceId_A | charstring | "sip:" & "mcvideo-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCVideo Server which is simulated by the SS |
| tsc_MCData_PublicServiceId_A | charstring | "sip:" & "mcdata-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCData Server which is simulated by the SS |
| tsc_MCPTT_PublicServiceId_B | charstring | "sip:" & "mcptt-server-B@" & px_MCX_DomainName_Organization_A | The URI of a second MCPTT Server which is implemented in the DUT used in MCPTT Server testing. |
| tsc_MCPTT_PublicServiceId_PF_A | charstring | "sip:" & "participating-mcptt-function-A@" & px_MCX_DomainName_Organization_A | The URI of the participating MCPTT function which configures the location reporting at the UE according to TS 24.379 [9] clause 13.2.2. |
| tsc_MCVideo_PublicServiceId_PF_A | charstring | "sip:" & "participating-mcvideo-function-A@" & px_MCX_DomainName_Organization_A | The URI of the participating MCVideo function which configures the location reporting at the UE according to TS 24.281 [55] clause 18.2.2. |
| tsc_MCData_PublicServiceId_PF_A | charstring | "sip:" & "participating-mcdata-function-A@" & px_MCX_DomainName_Organization_A | The URI of the participating MCData function which configures the location reporting at the UE according to TS 24.282 [57] clause 17.2.2. |
| tsc_MCX_SessionID_B | charstring | "sip:" & "sessionB@cf." & px_MCX_DomainName_Organization_A | The URI of the MCPTT session B identity. Ref. TS 24.483 [13]. SIP URI according to TS 24.379 [9] clause 4.5. |
| tsc_MCPTT_SessionId | charstring | "sip:" & "mcptt-session-A@cf." & px_MCX_DomainName_Organization_A | Service Id for MCPTT calls as used as Contact URI of the MCPTT server |
| tsc_MCVideo_SessionId | charstring | "sip:" & "mcvideo-session-A@cf." & px_MCX_DomainName_Organization_A | Service Id for MCVideo calls as used as Contact URI of the MCVideo server |
| tsc_MCData_SessionId | charstring | "sip:" & "mcdata-session-A@cf." & px_MCX_DomainName_Organization_A | Service Id for MCData calls as used as Contact URI of the MCData server |
| tsc_MCData_MSF_Hostname | charstring | "msf." & px_MCX_DomainName_Organization_A | Hostname of the media storage function (MSF) |
| tsc_MCData_MSF_URI | charstring | "http://" & tsc_MCData_MSF_Hostname & "/userA/files" | Absolute URI of the media storage function (MSF) |
| tsc_MCData_MsgSF_Hostname | charstring | "msgsf." & px_MCX_DomainName_Organization_A | Hostname of the message store function (MsgSF) |
| tsc_MCData_MsgSF_BaseURL | charstring | "/MsgStoreRoot/nms/v1/mcc160/UserA" | Base URL of the message store function (MsgSF) according to clause 5.2 of RESTful Network API for NMS [71] |
| tsc_MCData_MsgSF_AbsoluteBaseURL | charstring | "http://" & tsc_MCData_MsgSF_Hostname & tsc_MCData_MsgSF_BaseURL | Absolute Base URL of the message store function (MsgSF) |
| tsc_MCPTT_Group_A_ProSeLayer2GroupID | octetstring | '00000A'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |
| tsc_MCPTT_Group_D_ProSeLayer2GroupID | octetstring | '00000D'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |
| tsc_MCVideo_VSF_Hostname | charstring | "vsf." & px_MCX_DomainName_Organization_A | Hostname of the video storage function (VSF) where video files are stored |
| tsc_MCVideo_VSF_URI | charstring | "http://" & tsc_MCVideo_VSF_Hostname & "/userA/files" | Absolute URI of the message storage function (MSF) where video files are stored |

<END OF CHANGES>