3GPP TSG RAN WG5#104	R5-245873

# Maastricht, Netherlands, August 19 – August 23 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0867 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case 5.2.1.1.2, 1Rx FDD FR1 PDSCH performance for eRedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 5.2.1.1.2 is missing in 38.521-4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 5.2.1.1.2 in 38.521-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eRedCap work item cannot be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522 CR 0471 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | MU and TT in R5-245109 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of R5-245104 and the result of 1 revision |  |  |  |  |  |  |  |  |

#### << Start of changes >>

Table 5.2.1.1.1.5-1: Test Requirements for Rank 1

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD R.PDSCH. 1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x1 Low | 70 | 3.7 + 0.9 |
| 1-2 | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x1 Low | 70 | 12.2 + 0.9 |
| 1-3 | R.PDSCH.1-3.5 FDD R.PDSCH. 1-1.3 HD-FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x1 Low | 70 | 16.5 + 1.0 |
| 1-4 | R.PDSCH.1-4.2 FDD R.PDSCH. 1-1.4 HD-FDD | 10 / 15 | 256QAM, 0.67 | TDLA30-10 | 2x1 Low | 70 | 25.3 + 1.0 |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

##### 5.2.1.1.2	1Rx FDD FR1 PDSCH performance for eRedCap

Editor's Note: This test cases is incomplete in following aspects:

- SNR in test requirements table is within square brackets.

- Test state and generic procedure are TBD in 38.508-1

5.2.1.1.2.1	Test Purpose

To verify the PDSCH performance mapping Type A under 1 receive antenna conditions with different channel models and MCSs for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput.

5.2.1.1.2.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR eRedCap.

5.2.1.1.2.3	Minimum conformance requirements

The performance requirements are specified in Table 5.2.1.1.2.3-3, with the addition of test parameters in Table 5.2.1.1.2.3-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.1.1.2.3-1.

Table 5.2.1.1.2.3-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 1 receive antenna conditions and with different channel models and MCSs for eRedCap UE with reduced baseband bandwidth in FR1. | 1-1, 1-2, 1-3 |
| Verify the PDSCH mapping Type A normal performance under 1 receive antenna conditions and with different channel models and MCSs for eRedCap without reduced baseband bandwidth in FR1. | 2-1, 2-2, 2-3 |

Table 5.2.1.1.2.3-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD / HD-FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 and Test 2-1 2 for other tests |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Test 1-1 and Test 2-1, 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Table 5.2-1 |
|  | CSI-RS offset | Slots | Table 5.2-1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.1.1.2.3-3: Minimum performance for Rank 1

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x1 Low | 70 | [4.4] |
| 1-2 | R.PDSCH.1-12.2 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x1 Low | 70 | [12.3] |
| 1-3 | R.PDSCH.1-12.4 FDD R.PDSCH.1-4.1 HD-FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x1 Low | 70 | [16.1] |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

Table 5.2.1.1.2.3-4: Minimum performance for Rank 1 without reduced baseband bandwidth.

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-1.1 FDD R.PDSCH.1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x1 Low | 70 | [3.9] |
| 2-2 | R.PDSCH.1-12.3 FDD R.PDSCH.1-2.4 HD-FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x1 Low | 70 | [12.5] |
| 2-3 | R.PDSCH.1-12.4 FDD R.PDSCH.1-4.1 HD-FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x1 Low | 70 | [16.1] |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.1.1.2.

5.2.1.1.2.4	Test description

5.2.1.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.0 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2A-1 to Table 5.2A-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.2.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.1.1.2.4.3.

5.2.1.1.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.1.1.2.3-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 5.2.1.1.2.5-1 or Table 5.2.1.1.2.5-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G.1.5.

4.	Repeat steps from 1 to 3 for each test points in Table 5.2.1.1.2.5-1 or Table 5.2.1.1.2.5-2 as appropriate.

5.2.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2 with the following exceptions:

Table 5.2.1.1.2.4.3-1: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | n4, n2 | n4 for test 1-1 and test 2-1 | test 1-1, test 2-1 |
|  | Not present | n2 will be used by default | test point other than test 1-1 and test 2-1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.1.1.2.4.3-2: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots20 | 10 (for CSI-RS resources 1 and 2) 11 (for CSI-RS resources 3 and 4) | CSI-RS offset: 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 CSI-RS periodicity: 20 slots |  |
| } |  |  |  |

5.2.1.1.2.5	Test Requirement

Table 5.2.1.1.2.5-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 and A.3.2.3 for each throughput test shall meet or exceed the specified value in Table 5.2.1.1.2.5-1 the specified SNR including test tolerances for all throughput tests.

Table 5.2.1.1.2.5-1: Test Requirements for Rank 1

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x1 Low | 70 | [4.4] + 0.9 |
| 1-2 | R.PDSCH.1-12.2 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x1 Low | 70 | [12.3] + 0.9 |
| 1-3 | R.PDSCH.1-12.4 FDD R.PDSCH.1-4.1 HD-FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x1 Low | 70 | [16.1] + 1.0 |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. Note 2: Test index 1-1, 1-2, 1-3 is only applicable for UE with reduced baseband bandwidth. |  |  |  |  |  |  |  |

Table 5.2.1.1.2.5-2: Test Requirements for Rank 1

without reduced baseband bandwidth.

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-1.1 FDD R.PDSCH.1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x1 Low | 70 | [3.9] + 0.9 |
| 2-2 | R.PDSCH.1-12.3 FDD R.PDSCH.1-2.4 HD-FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x1 Low | 70 | [12.5] + 0.9 |
| 2-3 | R.PDSCH.1-12.4 FDD R.PDSCH.1-4.1 HD-FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x1 Low | 70 | [16.1] + 0.9 |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. Note 2: Test index 2-1, 2-2, 2-3 is only applicable for UE without reduced baseband bandwidth, supporting eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |

#### << End of changes >>