3GPP TSG-RAN WG5 Meeting #104	R5-245942

Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0550 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NR PRS-based measurement requirements in annex based on the updated TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding TT analysis and updating annex C for NR RSTD, UE Rx-Tx time difference, PRS-RSRP, PRS-RSRPP measurement test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the TC 14.3.2 maximum test system uncertainty and also updated the test case numbers referenced by the remaining RSTD test cases. - Updated annex C for NR RSTD, UE Rx-Tx time difference, PRS-RSRP, PRS-RSRPP measurement test cases - Added Table C.4.8: Derivation of Test Requirements for PRS-RSRPP Measurement tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning test case will be incosistency with layest versionof TS 38.133 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

C.1.6	NR PRS-based measurement requirements

Table C.1.6-1: Maximum Test System Uncertainty for NR PRS-based Measurement Requirements

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = 300ms |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA |  | Same as 14.2.1 |  |  |  |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = 300ms |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA |  | Same as 14.2.3 |  |  |  |
| 14.2.8 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 14.2.3 |  |  |  |
| 14.2.9 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA |  | Same as 14.2.3 |  |  |  |
| 14.2.10 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA |  | Same as 14.2.3 |  |  |  |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA |  | Same as 14.3.1 |  |  |  |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA |  | Same as 14.3.3 |  |  |  |
| 14.3.7 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 14.3.3 |  |  |  |
| 14.3.8 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA |  | Same as 14.3.3 |  |  |  |
| 14.4.3 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA |  | Same as 14.3.3 |  |  |  |
| 14.4.4 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 14.3.3 |  |  |  |
| 14.5.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA |  | Same as 14.3.3 |  |  |  |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |  |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA |  | Same as 15.2.1 |  | Same as 15.2.1 |  |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms |  | PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |  |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA |  | Same as 15.2.3 |  | Same as 15.2.3 |  |
| 15.2.8 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 15.2.3 |  | Same as 15.2.3 |  |
| 15.2.9 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA |  | Same as 15.2.3 |  | Same as 15.2.3 |  |
| 15.2.10 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA |  | Same as 15.2.3 |  | Same as 15.2.3 |  |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 15.3.5 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 15.3.2 |  |  |  |
| 15.3.6 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA |  | Same as 15.3.2 |  |  |  |
| 15.4.3 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA |  | Same as 15.2.3 |  |  |  |
| 15.4.4 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 15.2.3 |  |  |  |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc2 and Ês2 / Noc2 are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms |  | PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |  |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA |  | Same as 16.2.3 |  | Same as 16.2.3 |  |
| 16.2.7 PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR2 SA |  | Same as 16.2.3 |  | Same as 16.2.3 |  |
| 16.2.8 PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA |  | Same as 16.2.3 |  | Same as 16.2.3 |  |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 16.3.2 PRS-RSRP measurement accuracy with PRS in FR2 |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 16.3.4 PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number |  | Same as 16.3.2 |  | Same as 16.3.2 |  |
| 16.4.3 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR2 SA |  | Same as 16.2.3 |  | Same as 16.2.3 |  |
| 16.4.4 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 16.2.3 |  | Same as 16.2.3 |  |
| 16.5.3 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 SA |  | Same as 16.3.2 |  | Same as 16.3.2 |  |
| 16.5.4 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 with reduced sample number |  | Same as 16.3.2 |  | Same as 16.3.2 |  |
| 17.2.1 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 17.2.2 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA |  | Same as 17.2.1 |  | Same as 17.2.1 |  |
| 17.2.3 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA |  | Same as 17.2.1 |  | Same as 17.2.1 |  |
| 17.2.4 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 17.2.5 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA |  | Same as 17.2.4 |  | Same as 17.2.4 |  |
| 17.2.6 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA |  | Same as 17.2.4 |  | Same as 17.2.4 |  |

< Unchanged sections omitted >

C.2.5	NR PRS-based Measurement requirements

Table C.2.5-1: Test Parameter Relaxations for NR PRS-based Measurement requirements

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response time | 300ms |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response time | 300ms |
| 14.2.8 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 14.2.9 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response time | 300ms |
| 14.2.10 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA | Response time | 300ms |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 14.3.1 | Same as 14.3.1 |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Same as 14.3.2 | Same as 14.3.2 |
| 14.3.7 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference | 32 Tc |
| 14.3.8 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA | Cell Timing Difference | 32 Tc |
| 14.4.3 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 14.4.4 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 14.5.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR1 SA | Response time | 300ms |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Cell Timing Difference | 32 Tc |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.3.5 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.3.6 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA | Cell Timing Difference | 32 Tc |
| 15.4.3 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.4.4 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | Response time | 300ms |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 | Response time | 300ms |
| 16.3.2 PRS-RSRP measurement accuracy with PRS in FR2 | Test 1: Reported absolute PRP values Reported relative PRP values Test 2: Reported absolute PRP values Reported relative PRP values | Test 1: Reported absolute PRP values: ±8.5 Reported relative PRP values: ±10 Test 2: Reported absolute PRP values: ±6 Reported relative PRP values: ±7.5 |
| 16.3.4 PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number | Test 1: Reported Cellllabsolute PRP values Reported relative PRP values Reported absolute PRP values Reported relative PRP values | Test 1: Reported absolute PRP values: ±8.5 Reported relative PRP values: ±10 Reported absolute PRP values: ±6 Reported relative PRP values: ±7.5 |
| 16.4.3 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.4.4 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 16.5.3 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 SA | Test 1: Cell1 reported absolute PRP values Cell 1 reported relative PRP values Cell2 reported absolute PRP values Cell 2 reported relative PRP values Test 2: Cell1 reported absolute PRP values Cell 1 reported relative PRP values Cell2 reported absolute PRP values Cell 2 reported relative PRP values | Test 1: Cell 1 reported absolute PRP values: ±5 Cell 1 reported relative PRP values: ±5 Cell 2 reported absolute PRP values: ±8.5 Cell 2 reported relative PRP values: ±10 Test 2: Cell 1 reported absolute PRP values: ±5 Cell 1 reported relative PRP values: ±5 Cell 2 reported absolute PRP values: ±6 Cell 2 reported relative PRP values: ±7.5 |
| 16.5.4 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 with reduced sample number | Same as 16.5.3 | Same as 16.5.3 |
| 17.2.1 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 17.2.2 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Response time | 300ms |
| 17.2.3 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Response time | 300ms |
| 17.2.4 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 17.2.5 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response time | 300ms |
| 17.2.6 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response time | 300ms |

< Unchanged sections omitted >

C.4	Derivation of Test Requirements (This clause is informative)

< Unchanged sections omitted >

Table C.4.5: Derivation of Test Requirements for NR RSTD Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response Time between 1 s and 660 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 660.3 s depending on UE capabilities |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response Time between 2 s and 660 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 660.3 s depending on UE capabilities |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response Time between 6 s and 250 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 250.3 s depending on UE capabilities |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response Time between 11 s and 210 s depending on UE capabilities | 300 ms | Time + TPR: between 11.3 s and 210.3 s depending on UE capabilities |
| 14.2.8 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 2 s and 31 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 31.3 s depending on UE capabilities |
| 14.2.9 NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response Time between 5 s and 25 s depending on UE capabilities | 300 ms | Time + TPR: between 5.3 s and 25.3 s depending on UE capabilities |
| 14.2.10 NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA | Response Time between 11 s and 500 s depending on UE capabilities | 300 ms | Time + TPR: between 11.3 s and 500.3 s depending on UE capabilities |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.7 NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.8 NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.4.3 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response Time between 21 s and 124 s depending on UE capabilities | 300 ms | Time + TPR: between 21.3 s and 124.3 s depending on UE capabilities |
| 14.4.4 NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 6 s and 32 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 32.3 s depending on UE capabilities |
| 14.5.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |

Table C.4.6: Derivation of Test Requirements for UE Rx–Tx time difference Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Response Time between 1 s and 103 s depending on UE capabilities | 300ms | Time + TPR: between 1.3 s and 103.3 s depending on UE capabilities |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA | Response Time between 2.3 s and 53.3 s depending on UE capabilities | 300ms | Time + TPR: between 2.3 s and 53.3 s depending on UE capabilities |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Response Time between 6 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 170.3 s depending on UE capabilities |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA | Response Time between 11 s and 84 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 84.3 s depending on UE capabilities |
| 15.2.8 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 2 s and 21 s depending on UE capabilities | 300ms | Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities |
| 15.2.9 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response Time Sub-test 1: between 3 s and 21 s depending on UE capabilities Sub-test 2: between 6 s and 83 s depending on UE capabilities | 300ms | Sub-test 1: Time + TPR: between 3.3 s and 21.3 s depending on UE capabilities Sub-test 2: Time + TPR: between 6.3 s and 83.3 s depending on UE capabilities |
| 15.2.10 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA | Response Time between 11 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 170.3 s depending on UE capabilities |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 15.3.5 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 15.3.6 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 15.4.3 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response Time between 21 s and 83 s depending on UE capabilities | 300 ms | Time + TPR: between 21.3 s and 83.3 s depending on UE capabilities |
| 15.4.4 UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 6 s and 22 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 22.3 s depending on UE capabilities |

Table C.4.7: Derivation of Test Requirements for PRS-RSRP Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | Response Time between 1 s and 660 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 660.3 s depending on UE capabilities |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | Response Time between 2 s and 660 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 660.3 s depending on UE capabilities |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Response Time between 6 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 170.3 s depending on UE capabilities |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Response Time between 11 s and 84 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 84.3 s depending on UE capabilities |
| 16.2.7 PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR2 SA | Response Time between 2 s and 21 s depending on UE capabilities | 300ms | Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities |
| 16.2.8 PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Response Time Sub-test 1: between 2 s and 21 s depending on UE capabilities Sub-test 2: between 6 s and 83 s depending on UE capabilities | 300ms | Sub-test 1: Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities Sub-test 2: Time + TPR: between 6.3 s and 83.3 s depending on UE capabilities |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 | Absolute power, log: -98 dBm/15Hz PRS Es/Noc Cell#1: -2 PRS Es/Noc Cell#2: -10 Reported PRS-RSRP Cell#1: ±5dB Reported PRS-RSRP Cell#2 Test 1: ±10dB Reported PRS-RSRP Cell#2 Test 2: ±7.5dB | 0 0 0 Via mapping | Absolute power, log: -98 dBm/15Hz PRS Es/Noc Cell#1: -2 PRS Es/Noc Cell#2: -10 Cell#1 PRS-RSRP_51 to RSRP_62 Cell#2 Test 1 PRS-RSRP_38 to RSRP_59 Cell#2 Test 2 PRS-RSRP_41 to RSRP_56 |
| 16.3.2 PRS-RSRP measurement accuracy with PRS in FR2 | Test 1： Reported absolute PRP1 values: ±8.5 Reported relative PRP1 values: ±10 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 Test 2： Reported absolute PRP1 values: ±6 Reported relative PRP1 values: ±7.5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±7.5 | Via mapping | Test 1： Reported absolute PRP1 ：PRS_RSRP_54 to PRS_RSRP_85 Reported relative PRP1 ：DIFFRSRP_22 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_46 to PRS_RSRP_77 Reported relative PRP2 ：DIFFRSRP_20 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_54 to PRS_RSRP_85 Reported relative PRP1 ：DIFFRSRP_24 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_46 to PRS_RSRP_77 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |
| 16.3.4 PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number | Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 | Via mapping | Reported absolute PRP1 ：PRS_RSRP_66 to PRS_RSRP_76 Reported relative PRP1 ：DIFFRSRP_25 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_61 to PRS_RSRP_73 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |
| 16.4.3 PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Response Time between 21 s and 83 s depending on UE capabilities | 300ms | Time + TPR: between 21.3 s and 83.3 s depending on UE capabilities |
| 16.4.4 PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Response Time between 6 s and 22 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 22.3 s depending on UE capabilities |
| 16.5.3 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 SA | Test 1： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 Test 2： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±7.5 | Via mapping | Test 1： Reported absolute PRP1 ：PRS_RSRP_59 to PRS_RSRP_94 Reported relative PRP1 ：DIFFRSRP_20 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_52 to PRS_RSRP_86 Reported relative PRP2 ：DIFFRSRP_20 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_61 to PRS_RSRP_93 Reported relative PRP1 ：DIFFRSRP_23 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_53 to PRS_RSRP_85 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |
| 16.5.4 PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 with reduced sample number | Test 1： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±8.5 Reported reletive PRP2 values: ±10 Test 2： Reported absolute PRP1 values: ±5 Reported relative PRP1 values: ±5 Reported absolute PRP2 values: ±6 Reported reletive PRP2 values: ±7.5 | Via mapping | Test 1： Reported absolute PRP1 ：PRS_RSRP_66 to PRS_RSRP_76 Reported relative PRP1 ：DIFFRSRP_25 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_59 to PRS_RSRP_76 Reported relative PRP2 ：DIFFRSRP_20 to DIFFRSRP_30 Test 2： Reported absolute PRP1 ：PRS_RSRP_66 to PRS_RSRP_76 Reported relative PRP1 ：DIFFRSRP_25 to DIFFRSRP_30 Reported absolute PRP2 ：PRS_RSRP_61 to PRS_RSRP_73 Reported relative PRP2 ：DIFFRSRP_23 to DIFFRSRP_30 |

Table C.4.8: Derivation of Test Requirements for PRS-RSRPP Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 17.2.1 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA | Config 1and 2: Noc: -98dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Config 3: Noc: -95dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Response Time between 1 s and 31 s depending on UE capabilities | Config 1and 2: 0dB 0dB 0dB Config 3: 0dB 0dB 0dB 300 ms | Config 1and 2: Noc: -98dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Config 3 Noc: -95dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Time + TPR: between 1.3 s and 31.3 s depending on UE capabilities |
| 17.2.2 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA | Config 1and 2: Noc: -98dBm/15kHz PRS Ês1 / Noc: 2.23dB PRS Ês2 / Noc: 1.72dB Config 3: Noc: -95dBm/15kHz PRS Ês1 / Noc: 2.23dB PRS Ês2 / Noc: 1.72dB Response Time between 1 s and 8 s depending on UE capabilities | Config 1and 2: -4.6dB 0.75dB 0.85dB Config 3: -4.6 dB 0.75dB 0.85dB 300 ms | Config 1and 2: Noc: -102.6dBm/15kHz PRS Ês1 / Noc: 2.98dB PRS Ês2 / Noc: -0.87dB Config 3 Noc: -99.6dBm/15kHz PRS Ês1 / Noc: 2.98dB PRS Ês2 / Noc: -0.87dB Time + TPR: between 1.3 s and 8.3 s depending on UE capabilities |
| 17.2.3 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA | Config 1and 2 sub-test 1: Noc: -98dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Config 3 sub-test 1: Noc: -95dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Config 1and 2 sub-test 2: Noc: -98dBm/15kHz PRS Ês1 / Noc: 2.23dB PRS Ês2 / Noc: 1.72dB Config 3 sub-test 2: Noc: -95dBm/15kHz PRS Ês1 / Noc: 2.23dB PRS Ês2 / Noc: 1.72dB Response Time Sub-test 1: between 1 s and 31 s depending on UE capabilities Sub-test 2: between 1 s and 8 s depending on UE capabilities | Config 1and 2 sub-test 1: 0dB 0dB 0dB Config 3 sub-test 1: 0dB 0dB 0dB Config 1and 2 sub-test 2: -4.6dB 0.75dB 0.85dB Config 3 sub-test 2: -4.6 dB 0.75dB 0.85dB 300ms | Config 1and 2 sub-test 1: Noc: -98dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Config 3 sub-test 1: Noc: -95dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Config 1and 2 sub-test 2: Noc: -102.6dBm/15kHz PRS Ês1 / Noc: 2.98dB PRS Ês2 / Noc: -0.87dB Config 3 sub-test 2: Noc: -99.6dBm/15kHz PRS Ês1 / Noc: 2.98dB PRS Ês2 / Noc: -0.87dB Sub-test 1: Time + TPR: between 1.3 s and 31.3 s depending on UE capabilities Sub-test 2: Time + TPR: between 1.3 s and8.3 s depending on UE capabilities |
| 17.2.4 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA | Noc: -98dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Response Time between 6 s and 83 s depending on UE capabilities | -1.4dB 0dB 0dB 300 ms | Noc: -99.4dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Time + TPR: between 6.3 s and 83.3 s depending on UE capabilities |
| 17.2.5 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA | Noc: -98dBm/15kHz PRS Ês1 / Noc: 2.23dB PRS Ês2 / Noc: -1.73dB Response Time between 2 s and 21 s depending on UE capabilities | -8.45dB 4.85dB 7dB 300 ms | Noc: -106.45dBm/15kHz PRS Ês1 / Noc: 7.08dB PRS Ês2 / Noc: 5.27dB Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities |
| 17.2.6 PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA | Sub-test 1: Noc: -98dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Sub-test 2: Noc: -98dBm/15kHz PRS Ês1 / Noc: 2.23dB PRS Ês2 / Noc: -1.73dB Response Time Sub-test 1: between 6 s and 83 s depending on UE capabilities Sub-test 2: between 2 s and 21 s depending on UE capabilities | Sub-test 1: -1.4dB 0dB 0dB Sub-test 2: -8.45dB 4.85dB 7dB 300ms | Sub-test 1: Noc: -99.4dBm/15kHz PRS Ês1 / Noc: -2.00dB PRS Ês2 / Noc: -10.00dB Sub-test 2: Noc: -106.45dBm/15kHz PRS Ês1 / Noc: 7.08dB PRS Ês2 / Noc: 5.27dB Sub-test 1: Time + TPR: between 6.3 s and 83.3 s depending on UE capabilities Sub-test 2: Time + TPR: between 2.3 s and 21.3 s depending on UE capabilities |