Title:	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA

Source:	CATT

# 1. Introduction

This document relates to PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA in TS 37.571-1. These test cases are:

- 17.2.1	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA

- 17.2.2	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA

- 17.2.3	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test cases in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v17.13.0.

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap

<< Some clauses skipped >>

Table A.6.6.16.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.16.3.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 20: NRB,c = 106 |  |
|  |  | 2 | 20: NRB,c = 106 |  |
|  |  | 3 | 50: NRB,c = 133 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| PPW configuration |  | 1,2,3 | PPW.1 | As defined in A.3.33 |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | - set to the UE reported capability for receive time difference threshold if the UE reported value is < 3µs - 3µs otherwise | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 0 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | Same as time offset |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | ms | 1, 2, 3 | 10 | In this test UE is configured to measure single PFL within the configured PPW. |

Table A.6.6.16.3.1-3: Cell specific test parameters T2

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sub-test 1 | Sub-test 2 | Sub-test 1 | Sub-test 2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2.41 | 0 | -12.12 | -6 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -2 | 2.23 | -10 | -1.72 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -95.77 | -108 | -99.73 |
|  |  | 2 | -100 | -95.77 | -108 | -99.73 |
|  |  | 3 | -97 | -92.77 | -105 | -96.73 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -100 | -100 | -104 | -104 |
|  |  | 2 | -100 | -100 | -104 | -104 |
|  |  | 3 | -97 | -97 | -101 | -101 |
| Io | dBm/19.08 MHz | 1 | -64.57 | -61.71 | -64.57 | -61.71 |
|  | dBm/19.08 MHz | 2 | -64.57 | -61.71 | -64.57 | -61.71 |
|  | dBm/47.88 MHz | 3 | -60.59 | -57.73 | -60.59 | -57.73 |
| Propagation Condition |  | 1, 2, 3 | Two-tap channel defined in 38.101-4 Annex B.2.4, a = 1, τd=0.45 µs and fD=5 Hz |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

For PRS-RSRPP, the core requirements for PRS-RSRPP measurements are specified in TS 38.133 clause 10.1.38, with side conditions in Annex B.2.14.

# 3. Discussion

There are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell.

The test consists of two sub-tests; Sub-test 1 is for Nsample = 4 and sub-test 2 is for Nsample = 1. Sub-test 2 is only conducted when UE also supporting supportedDL-PRS-ProcessingSamples-RRC-CONNECTED.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-AoD assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first PRS processing window instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are selectively copied from A.6.6.16.3.1-2, A.6.6.16.3.1-3 in TS 38.133, which are subcarrier spacing, channel bandwidth, RBs in channel bandwidth, Noc, SS-RSRP, Cell 1 PRS Es/Noc and Cell 2 PRS Es/Noc. All other parameters such as Io, PRS Es/Iot and PRS-RSRP are derived, and not independently settable by the Test Equipment.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet.

c) Uncertainties

The SS provides AWGN and 2 intra-frequency cells to the UE under AWGN conditions in FR1 in standalone scenario. We propose to control the following parameters:

- AWGN absolute power on carrier frequency, Noc uncertainty: ±1.5 dB

- Ratio of cell 1 signal / AWGN, PRS Ês1 / Noc ±0.3 dB averaged over PRS BWConfig

- Ratio of cell 2 signal / AWGN, PRS Ês2 / Noc ±0.3 dB averaged over PRS BWConfig

- LPP Response Time, 0.3 seconds

The LPP Response Time uncertainty has been chosen to be in-line with all other LPP test cases defined in TS 37.571-1 and there is no necessity to derive the test tolerance in the spreadsheet. LPP Response Time is as defined in TS 37.571-1 clause 4.16.7.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

- UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty.

The stimulus requirements defined in TS 38.133 clause 10.1.38 are reproduced below. The stimulus requirements defined in TS 38.133 clause 10.1.38 indicate that the PRS-RSRPP measurement accuracy for all measured cells and the Io range.

<< Extracts from TS 38.133 v17.13.0 >>

10.1.38	PRS-RSRPP Measurements

The absolute accuracy requirements for PRS-RSRPP measurement for FR1 defined in Table 10.1.38.2.1-1 and Table 10.1.38.2.1-3 are valid under the following conditions:

-	Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

-	PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

…

The absolute accuracy requirements for PRS-RSRPP measurement defined in Table 10.1.38.2.1-3 and Table 10.1.38.2.1-4 apply for the UE supporting supportedDL-PRS-ProcessingSamples [34].

<< Some text skipped >>

Table 10.1.38.2.1-1: PRS-RSRPP absolute accuracy for FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±4.1 | ±8.6 | ≥-3 | ≥24 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±5.8 | ±10.3 | ≥-13 | 24 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±4.9 | ±9.4 |  | BW > 52 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the DL-TDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

<< Some text skipped >>

Table 10.1.38.2.1-3: PRS-RSRPP absolute accuracy for FR1 for reduced number of samples

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±3.9 | ±8.4 | ≥0 | ≥48 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±4.2 | ±8.7 | ≥-6 | 48 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±4.1 | ±8.6 |  | BW >52 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the DL-TDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 48 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

<< Some text skipped >>

The side conditions are defined in TS 38.133 Annex B.2.14 and are reproduced below.

B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

<< Some text skipped >>

Table B.2.14-1: Conditions for NR PRS-based measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  |  | PRS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  |  | dB |  |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz | SCSPRS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 Note2 -13 Note3 -3 Note4 | -6 Note5 -3 Note6 0 Note7 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 |  |  |
|  | NR_TDD_FR1_C | -126 | -123 | -120 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 |  |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 3: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 4: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. NOTE 6: PRS Ês/Iot for RSTD measurement reference cell PRS resource when performed with reduced number of samples. NOTE 7: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. |  |  |  |  |  |  |

Band group NR_FDD_FR1_H gives the highest PRP side condition of -123.5 dBm/15kHz and -120.5 dBm/30kHz.

In summary, during T2 the following conditions shall be satisfied:

For Config 1 sub-test 1:

-	PRS Ês/Iot of Cell 1 at baseband shall be no less than -3dB.

-	PRS Ês/Iot of Cell 2 at baseband shall be no less than -13dB,

-	PRP of Cell 1 shall be no less than -123.5 dBm/15kHz.

-	PRP of Cell 2 shall be no less than -123.5 dBm/15kHz.

-	Io shall be no less than -123.5dBm/Ch BW and no larger than -50dBm/Ch BW.

For Config 1 sub-test 2:

-	PRS Ês/Iot of Cell 1 at baseband shall be no less than 0dB.

-	PRS Ês/Iot of Cell 2 at baseband shall be no less than -6dB,

-	PRP of Cell 1 shall be no less than -123.5 dBm/30kHz.

-	PRP of Cell 2 shall be no less than -123.5 dBm/30kHz.

-	Magnitude of difference between the Cell 1 SS-RSRP and the Cell 2 PRS-RSRP is within 6 dB.

-	Io shall be no less than -123.5dBm/Ch BW and no larger than -50dBm/Ch BW.

For Config 3 sub-test 1:

-	PRS Ês/Iot of Cell 1 at baseband shall be no less than -3dB.

-	PRS Ês/Iot of Cell 2 at baseband shall be no less than -13dB,

-	PRP of Cell 1 shall be no less than -120.5 dBm/30kHz.

-	PRP of Cell 2 shall be no less than -120.5 dBm/30kHz.

-	Io shall be no less than -120.5dBm/Ch BW and no larger than -50dBm/Ch BW.

For Config 3 sub-test 2:

-	PRS Ês/Iot of Cell 1 at baseband shall be no less than 0dB.

-	PRS Ês/Iot of Cell 2 at baseband shall be no less than -6dB,

-	PRP of Cell 1 shall be no less than -120.5 dBm/30kHz.

-	PRP of Cell 2 shall be no less than -120.5 dBm/30kHz.

-	Magnitude of difference between the Cell 1 SS-RSRP and the Cell 2 PRS-RSRP is within 6 dB.

-	Io shall be no less than -120.5dBm/Ch BW and no larger than -50dBm/Ch BW.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE reporting delay (PRS-RSRP measurements)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For Cell 1, they are the PRS Es/Iot range and the PRP power range for the reference cell over which the UE is expected to provide PRS-RSRP measurements. For Cell 2, they are the PRS Es/Iot range and the PRP power range for the neighbour cell over which the UE is expected to provide PRS-RSRP measurements. The Io power shall also be within the range over which the UE is expected to provide PRS-RSRP measurements.

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 PRP, so the sensitivity factor is 1.000 for all tests. However the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Es/Noc has an effect on Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases the sensitivity factor value between 0 and 1 result. It is important to calculate these correctly to obtain the overall uncertainty.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

For Config 1 sub-test 1, we observe that the side conditions are satisfied.

For Config 1 sub-test 2, we observe that the PRS Es/Iot of Cell 1 and PRS Es/Iot of Cell 2 are outside the allowed range, and the magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is beyond 6 dB.

So the PRS Es/Noc of Cell 1 is therefore increased by an amount sufficient to achieve Cell 1 PRS Es/Iot >= 0dB. PRS Es/Noc of Cell 2 is therefore increased by an amount sufficient to achieve Cell 1 PRS Es/Iot >= -6dB. The Noc is decreased by an amount sufficient to achieve║Cell 1 SS-RSRP -Cell 2 PRS-RSRP║ <= 6 dB.

For Config 3 sub-test 1, For Config 1 sub-test 1, we observe that the side conditions are satisfied..

For Config 3 sub-test 2, we observe that the PRS Es/Iot of Cell 1 and PRS Es/Iot of Cell 2 are outside the allowed range, and the magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is beyond 6 dB.

So the PRS Es/Noc of Cell 1 is therefore increased by an amount sufficient to achieve Cell 1 PRS Es/Iot >= 0dB. PRS Es/Noc of Cell 2 is therefore increased by an amount sufficient to achieve Cell 1 PRS Es/Iot >= -6dB. The Noc is decreased by an amount sufficient to achieve║Cell 1 SS-RSRP -Cell 2 PRS-RSRP║ <= 6 dB.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances.

For Config 1 sub-test 1:

No offset is needed.

For Config 1 sub-test 2:

The Noc is decreased by an amount sufficient to achieve║Cell 1 SS-RSRP -Cell 2 PRS-RSRP║ <= 6. The actual offset required is 4.6dB.

The PRS Es/Noc of Cell 1 is increased by an amount sufficient to achieve PRS Es/Iot >=0dB. The actual offset required is 0.75dB for cell 1.

The PRS Es/Noc of Cell 2 is increased by an amount sufficient to achieve PRS Es/Iot >=-6dB.The PRS Es/Noc of Cells 2 is increased by 0.85dB.

For Config 3 sub-test 1:

No offset is needed.

For Config 3 sub-test 2:

The Noc is decreased by an amount sufficient to achieve║Cell 1 SS-RSRP -Cell 2 PRS-RSRP║ <= 6. The actual offset required is 4.6dB.

The PRS Es/Noc of Cell 1 is increased by an amount sufficient to achieve PRS Es/Iot >=0dB. The actual offset required is 0.75dB for cell 1.

The PRS Es/Noc of Cell 2 is increased by an amount sufficient to achieve PRS Es/Iot >=-6dB.The PRS Es/Noc of Cells 2 is increased by 0.85dB

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, the stimulus requirements are met.

# 5. Calculation of the response time

The response time is determined by the core requirements in the following extract from TS 38.133:

9.9.3.6	Measurement Period Requirements without Measurement Gaps

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements as defined in TS 38.215 [4] without measurement gap, on configured positioning frequency layer i, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i if any of the positioning frequency layers are in Case 1, or

TPRS-RSRP, total=maxTPRS-RSRP,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2,

Where,

i is the index of positioning frequency layer, and

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i, and

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TPRS-RSRP, total.

A positioning frequency layer is in Case 1 if UE reports ppw-durationOfPRS-Processing1-r17 for the band containing the positioning frequency layer, and a positioning frequency layer is in Case 2 if UE reports ppw-durationOfPRS-Processing2-r17 for the band containing the positioning frequency layer.

TPRS-RSRP,i is the measurement period for PRS-RSRP measurement in positioning frequency layer i as specified below.

TPRS-RSRP,i=NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1. If positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i,

only unmuted PRS resource instances that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered, if positioning frequency layer i is in Case 1, or

only the PRS resources unmuted and fully or partially overlapped with the first (PPWL-T2) ms of PPW are considered, if positioning frequency layer i is in Case 2, where PPWL is the PPW length and T2 corresponds to ppw-durationOfPRS-ProcessingSymbolsT2.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponding to ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast  is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time.

If positioning frequency layer i is in Case 1 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = Ti +PPWL, else

if positioning frequency layer i is in Case 2 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = PPWL;

otherwise, Tlast,i = Ti + Tavailable_PRS,i.

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to ppw-durationOfPRS-ProcessingSymbolsT  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponds to the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2

Tavailable_PRS,i= LCMTPRS,i,PPWRPi is the least common multiple between TPRS,i and PPWRPi,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

PPWRPi is the PRS processing window repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. If bitmap b1  for higher-layer parameter DL-Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

5.1 LPP response time calculation procedure for case 1

Since the test case is for single positioning frequency layer, i = 1 and L = 1, so TPRS-RSRP, total=TPRS-RSRP, 1.

TPRS-RSRP,i=NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Analysing the equation for TPRS-RSRP, 1:

NRxBeam,i = 1

NPRS,islot=2

LavailablePRS,i=1010*1*14*4=0.285ms

Nsample= 4.

N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN

ppw-durationOfPRS-ProcessingSymbolsN-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms4,

ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32,ms35,

ms40, ms45, ms50 },

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17	SEQUENCE {

scs15-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

...

},

Therefore, the first part of the equation:

1*2N'0.285N*4-1

Based on the value of N’:

| N’ | 2N' |
| --- | --- |
| n1 | 2 |
| >=n2 | 1 |

Based on the value of N:

| N | 0.285N |
| --- | --- |
| msDot125 | 3 |
| msDot25 | 2 |
| >=msDot5 | 1 |

/

Tlast,i = Ti +PPWL

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Tavailable_PRS,i= LCMTPRS,i,PPWRPi; where Tprs = 160 ms, and the repetition periodicity PPWRPi=160ms. Therefore, Tavailable_PRS,i=160

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz

Table A.3.31.1.1-1: PRS.1 FR1: PRS Pattern 1 for SSB SCS=15 KHz

| PRS Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.3 FR1 |  | PRS.1.4 FR1 |  |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 |
| SCS | 15kHz |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  |
| Repetion factor | 2 | 1 | 2 |  | 1 |  |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  |
| RB numbers containing PRS within channel BW Note 1 | 0-23 | 0-103 | 0-23 |  | 0-103 |  |
| PRS Start PRB | 0 |  |  |  |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |  |  |

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz

Table A.3.31.1.2-1: PRS.2 FR1: PRS Pattern 2 for SCS=30 KHz

| PRS Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.3 FR1 |  | PRS.2.4 FR1 |  |
| Resource index in resource set | 0 | 0 | 0 | 1 | 0 | 1 |
| PRS periodicity | 160ms |  |  |  |  |  |
| PRS Resource set slot offset Note 1 | 10 ms |  |  |  |  |  |
| PRS Resource slot offset (slot) Note 1 | 0 | 4 | 0 |  | 4 |  |
| PRS RE offset Note 1 | 0 |  | 0 | 1 | 0 | 1 |
| SCS | 30kHz |  |  |  |  |  |
| PRS comb size | 2 | 4 | 2 |  | 4 |  |
| Number of PRS symbol | 4 | 4 | 4 |  | 4 |  |
| Repetion factor | 2 | 1 | 2 |  | 1 |  |
| PRS resource time gap (slot) | 1 | 1 | 1 |  | 1 |  |
| RB numbers containing PRS within channel BW Note 1 | 0-23 | 0-131 | 0-23 |  | 0-131 |  |
| PRS Start PRB | 0 |  |  |  |  |  |
| Note 1: Unless otherwise specified in the test case |  |  |  |  |  |  |

A.3.33	PRS Processing Window (PPW) configurations

Table A.3.33-1: Reference PPW configuration

| PPW Parameters | Unit | Values |
| --- | --- | --- |
| Reference PPW |  | PPW.1 |
| Periodicity | slot | Note 1 |
| Offset | slot | Note 2 |
| Length | ms | 10 |
| Type |  | Same as the value reported in prsProcessingType-r17 |
| Priority |  | st1 |
| Note 1: Same as PRS resource set periodicity as used in the test case. Note 2: Same as PRS resource with smallest offset as used in the test case. |  |  |

Ti corresponds to ppw-durationOfPRS-ProcessingSymbolsT in TS 37.355 if positioning frequency layer i is in Case 1

ppw-durationOfPRS-ProcessingSymbolsT-r17

ENUMERATED { ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280 }

}																					OPTIONAL,

Therefore, Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i  and Tlast,i = Ti + Tavailable_PRS,itake following values depending on Ti:

| ppw-durationOfPRS-ProcessingSymbolsT-r17 | Teffect | Tlast |
| --- | --- | --- |
| ms1 | 160 | 161 |
| ms2 | 160 | 162 |
| ms4 | 160 | 164 |
| ms8 | 160 | 168 |
| ms16 | 160 | 176 |
| ms20 | 160 | 180 |
| ms30 | 160 | 190 |
| ms40 | 160 | 200 |
| ms80 | 160 | 240 |
| ms160 | 160 | 320 |
| ms320 | 320 | 480 |
| ms640 | 640 | 800 |
| ms1280 | 1280 | 1440 |

Finally, it results in the following equation:

1*2N'0.285N*4-1*Teffect+Tlast

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

Solving the equation for worst-case results in: (1*2*3*4-1)*1280+1440= 30880ms.

Solving the equation for best-case results in: (1*1*1*4-1)*160+161=641ms.

Given the wide range of response time possibilities, it is not possible to conclude on a unique value for the response time. The SS will have to calculate the response time based on the UE capabilities.

Example of the response time calculation:

For the best-case result of 0.6s, the SS should round up to the next integer (1 seconds) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 30.88s, the SS should round up to the next integer (31 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 31.3s.

5.2 LPP response time calculation procedure for case 2

Since the test case is for single positioning frequency layer, TPRS-RSRP, total=maxTPRS-RSRP,1+Tuncertainty.1.

Analysing the equation for TPRS-RSRP,1:

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TRSTD,Total. There is only one positioning frequency layer, so Tuncertainty,i=0

NRxBeam,i = 1

NPRS,islot=2

LavailablePRS,i=1010*1*14*4=0.285ms

Nsample= 1.

N is ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 if positioning frequency layer i is in Case 2

ppw-durationOfPRS-ProcessingSymbolsN2-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3,ms4,ms5,

ms6, ms8, ms12 },

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17	SEQUENCE {

scs15-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

...

},

Therefore, the first part of the equation:

1*2N'0.285N*4-1

Based on the value of N’:

| N’ | 2N' |
| --- | --- |
| n1 | 2 |
| >=n2 | 1 |

Based on the value of N:

| N | 0.285N |
| --- | --- |
| msDot125 | 3 |
| msDot25 | 2 |
| >=msDot5 | 1 |

Tlast,i = PPWL

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Tavailable_PRS,i= LCMTPRS,i,PPWRPi; where Tprs = 160 ms, and the repetition periodicity PPWRPi=160ms. Therefore, Tavailable_PRS,i=160

Ti corresponds of the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2. So the smallest Ti = 4.125ms, the biggest Ti = 20ms.

ppw-durationOfPRS-ProcessingSymbolsT2-r17

ENUMERATED { ms4, ms5, ms6, ms8 }

ppw-durationOfPRS-ProcessingSymbolsN2-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3,ms4,ms5,

ms6, ms8, ms12 },

Therefore, Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i and Tlast,i = Ti + Tavailable_PRS,itake following values depending on Ti:

| Ti | Teffect | Tlast |
| --- | --- | --- |
| 4.125ms | 160 | 164.125 |
| 20ms | 160 | 180 |

Finally, it results in the following equation:

1*2N'0.035N*4-1*Teffect+Tlast

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

Solving the equation for worst-case results in: (1*2*3*4-1)*160+164.125= 3844.125ms.

Solving the equation for best-case results in: (1*1*1*4-1)*160+180=660ms.

Given the wide range of response time possibilities, it is not possible to conclude on a unique value for the response time. The SS will have to calculate the response time based on the UE capabilities.

Example of the response time calculation:

For the best-case result of 0.66s, the SS should round up to the next integer (1 seconds) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 3.87s, the SS should round up to the next integer (4 seconds) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 4.3s.

For case 1 from the calculation procedure above, the LPP response time range is 1.3s-31.3s.

For case 2 from the calculation procedure above, the LPP response time range is 1.3s-4.3s.

5.3 LPP response time for sub-test 2

The only different for sub-test 2 is Nsample= 1 in the LPP response time calculation procedure.

For case 1 from the calculation procedure above, the LPP response time range is 1.3s-8.3s.

For case 2 from the calculation procedure above, the LPP response time range is 1.3s.

# 6. Treatment of test cases 17.2.1 and 17.2.2

TS 37.571-1 also contain similar test cases:

- 17.2.1	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA

- 17.2.2	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA

For test case 17.2.1, the test parameters are identical with test case 17.2.3 sub-test 1. As the calculations use the values, a separate spread sheet tab is not need to be provided. The LPP response time calculation procedure is provided in clause 7.

For test case 17.2.2, the test parameters are identical with test case 17.2.3 sub-test 2. As the calculations use the values, a separate spread sheet tab is not need to be provided. The LPP response time calculation procedure is provided in clause 8.

# 7. Calculation of the response time for 17.2.1

The response time is determined by the core requirements in the following extract from TS 38.133:

9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

where

i is the index of positioning frequency layer,

L is total number of positioning frequency layers,

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=CSSFi*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioining frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and if positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time, if not all PRS resources to be measured are available in the same measurement gap occasion during Tavailable_PRS,i, otherwise Tlast,i = Ti + MGL,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi is the least common multiple between TPRS,i and MGRPi ,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

MGRPi is the measurement gap repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Since the test case is for single positioning frequency layer, i = 1 and L = 1, so TPRS-RSRP, total=TPRS-RSRP,1.

Analysing the equation for TPRS-RSRP,1:

TPRS-RSRP,i=CSSFi*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Kp,PRS,i = 1

CSSFPRS,i=1

NRxBeam,i = 1

NPRS,islot=2

LavailablePRS,i=1010*1*14*4=0.285ms

Nsample= 4.

N is the parameter durationOfPRS-ProcessingSysmbols from TS 37.355

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

N’ is the parameter maxNumOfDL-PRS-ResProcessedPerSlot from TS 37.355

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...

},

Therefore, the first part of the equation:

1*1*1*2N'0.285N*4-1

Based on the value of N’:

| N’ | 2N' |
| --- | --- |
| n1 | 2 |
| >=n2 | 1 |

Based on the value of N:

| N | 0.285N |
| --- | --- |
| nDot125 | 3 |
| nDot25 | 2 |
| >=nDot5 | 1 |

Tlast,i = Ti + Tavailable_PRS,i

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Tavailable_PRS,i= LCMTPRS,i,MGRPi ; where Tprs = 160 ms, and MGRP is 80 (for GP#24) or 40 (for GP#13) depending on UE capabilities. Therefore, Tavailable_PRS,i=160

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

Ti depends on UE parameter durationOfPRS-ProcessingSymbolsInEveryTms from TS 37.355:

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280}

Therefore, Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i  and Tlast,i = Ti + Tavailable_PRS,itake following values depending on Ti:

| durationOfPRS-ProcessingSymbolsInEveryTms | Teffect | Tlast |
| --- | --- | --- |
| n8 | 160 | 168 |
| n16 | 160 | 176 |
| n20 | 160 | 180 |
| n30 | 160 | 190 |
| n40 | 160 | 200 |
| n80 | 160 | 240 |
| n160 | 160 | 320 |
| n320 | 320 | 480 |
| n640 | 640 | 800 |
| n1280 | 1280 | 1440 |

Finally, it results in the following equation:

1*1*1*2N'0.285N*4-1*Teffect+Tlast

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

Solving the equation for worst-case results in: (2*3*4-1)*1280+1440=30880ms.

Solving the equation for best-case results in: (1*1*4-1)*160+168=648ms.

Given the wide range of response time possibilities, it is not possible to conclude on a unique value for the response time. The SS will have to calculate the response time based on the UE capabilities.

Example of the response time calculation:

For the best-case result of 0.648s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 30.88s, the SS should round up to the next integer (31 seconds) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 31.3s.

# 8. Calculation of the response time for 17.2.2

The response time is determined by the core requirements in the following extract from TS 38.133:

9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

where

i is the index of positioning frequency layer,

L is total number of positioning frequency layers,

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=CSSFi*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioining frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and if positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time, if not all PRS resources to be measured are available in the same measurement gap occasion during Tavailable_PRS,i, otherwise Tlast,i = Ti + MGL,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi is the least common multiple between TPRS,i and MGRPi ,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

MGRPi is the measurement gap repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Since the test case is for single positioning frequency layer, i = 1 and L = 1, so TPRS-RSRP, total=TPRS-RSRP,1.

Analysing the equation for TPRS-RSRP,1:

TPRS-RSRP,i=CSSFi*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Kp,PRS,i = 1

CSSFPRS,i=1

NRxBeam,i = 1

NPRS,islot=2

LavailablePRS,i=1010*1*14*4=0.285ms

Nsample= 1.

N is the parameter durationOfPRS-ProcessingSysmbols from TS 37.355

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

N’ is the parameter maxNumOfDL-PRS-ResProcessedPerSlot from TS 37.355

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...

},

Therefore, the first part of the equation:

1*1*1*2N'0.285N*1-1

Based on the value of N’:

| N’ | 2N' |
| --- | --- |
| n1 | 2 |
| >=n2 | 1 |

Based on the value of N:

| N | 0.285N |
| --- | --- |
| nDot125 | 3 |
| nDot25 | 2 |
| >=nDot5 | 1 |

Tlast,i = Ti + Tavailable_PRS,i

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Tavailable_PRS,i= LCMTPRS,i,MGRPi ; where Tprs = 160 ms, and MGRP is 80 (for GP#24) or 40 (for GP#13) depending on UE capabilities. Therefore, Tavailable_PRS,i=160

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

Ti depends on UE parameter durationOfPRS-ProcessingSymbolsInEveryTms from TS 37.355:

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280}

Therefore, Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i  and Tlast,i = Ti + Tavailable_PRS,itake following values depending on Ti:

| durationOfPRS-ProcessingSymbolsInEveryTms | Teffect | Tlast |
| --- | --- | --- |
| n8 | 160 | 168 |
| n16 | 160 | 176 |
| n20 | 160 | 180 |
| n30 | 160 | 190 |
| n40 | 160 | 200 |
| n80 | 160 | 240 |
| n160 | 160 | 320 |
| n320 | 320 | 480 |
| n640 | 640 | 800 |
| n1280 | 1280 | 1440 |

Finally, it results in the following equation:

1*1*1*2N'0.285N*1-1*Teffect+Tlast

The remaining parameters cannot be univocally solved, since they depend on the UE capabilities.

Solving the equation for worst-case results in: (2*3*1-1)*1280+1440=7840ms.

Solving the equation for best-case results in: (1*1*1-1)*160+168=168ms.

Given the wide range of response time possibilities, it is not possible to conclude on a unique value for the response time. The SS will have to calculate the response time based on the UE capabilities.

Example of the response time calculation:

For the best-case result of 0.168s, the SS should round up to the next integer (1 second) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 1.3s.

For the worst-case result of 7.84s, the SS should round up to the next integer (8 seconds) and configure this value in the LPP Response Time IE for the RequestLocationInformation message. Then, it should add the MU value of 300ms. The reporting delay limit for the evaluation is 8.3s.