3GPP TSG-RAN5 Meeting #104	R5-245956

# Maastricht, Netherland, Aug 19th - 23th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3342 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex F for RedCap RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add TE uncertainties and test tolerance for following RedCap RRM TCs: - FR2 RLM TCs: 17.5.1.3, 17.5.1.5, 17.5.1.6, 17.5.1.7, 17.5.1.8. - FR2 BWP switching TCs: 17.5.3.1.1, 17.5.3.2.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TE uncertainties and test tolerance for RedCap RRM TCs mentioned above are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.903 CR 0786, 0787 TS 38.533 CR 3335, 3336, 3337, 3338, 3339, 3340, 3341 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT TC CRs: R5-244634 - R5-244640, 38.903 CRs: R5-244641, R5-244642 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244689r1 |  |  |  |  |  |  |  |  |

<Start of Change 1>

Table F.1.1.2-10: Maximum test system uncertainty for RRM requirements for SA FR2 test cases for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.2 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.3 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.4 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.3.1.1 NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.2 | Same as 7.3.1.2 |
| 17.3.1.2 NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.3 | Same as 7.3.1.3 |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 | Same as 7.3.2.1.2 |
| 17.3.2.3.1 NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2 | Same as 7.3.2.3 | Same as 7.3.2.3 |
| 17.4.1.1 NR SA FR2 NR UE Transmit Timing Test for FR2 | Same as 7.4.1.1 | Same as 7.4.1.1 |
| 17.5.1.1 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.2 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.3 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.4 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.5 NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.6 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.7 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.8 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.2.3 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 17.5.2.4 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode | Same as 7.5.5.4 | Same as 7.5.5.4 |
| 17.5.2.5 NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Same as 7.5.5.5 | Same as 7.5.5.5 |
| 17.5.3.1.1 NR SA FR2 DCI-based and Timer-based DL active BWP switch with non-DRX | Noc ±5.65dB Ês / Noc ±0.3 dB AWGN and signal flatness: ±3.0 dB | Ês / Noc is the ratio of cell 1 signal / AWGN |
| 17.5.3.2.1 NR SA FR2 RRC-based DL active BWP switch with non-DRX | Same as 17.5.3.1.1 | Same as 17.5.3.1.1 |
| 17.5.4.1.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.1.1 | Same as 7.5.8.1.1 |
| 17.5.4.2.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.2.1 | Same as 7.5.8.2.1 |
| 17.5.5.1.1 NR SA FR2 PCell MAC-CE based spatial relation switch associated with known DL-RS | Same as 7.5.9.1.1 | Same as 7.5.9.1.1 |
| 17.5.5.2.1 NR SA FR2 PCell RRC-based spatial relation switch associated with a known DL-RS | Same as 7.5.9.2.1 | Same as 7.5.9.2.1 |
| 17.6.1.1 NR SA FR2 Event triggered reporting test without gap under non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 17.6.1.2 NR SA FR2 Event triggered reporting test without gap under DRX | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 17.6.1.3 NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 17.6.1.4 NR SA FR2 Event triggered reporting test with per-UE gaps under DRX | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 17.6.2.1 NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 17.6.2.2 NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is used | Same as 7.6.2.2 | Same as 7.6.2.2 |
| 17.6.2.3 NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 17.6.2.4 NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is used | Same as 7.6.2.2 | Same as 7.6.2.2 |
| 17.6.3.1 NR SA FR2 SSB based L1-RSRP measurement when DRX is not used | Same as 7.6.3.1 | Same as 7.6.3.1 |
| 17.6.3.2 NR SA FR2 SSB based L1-RSRP measurement when DRX is used | Same as 7.6.3.1 | Same as 7.6.3.1 |
| 17.6.3.3 NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 17.6.3.4 NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 17.7.1.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.1 | Same as 7.7.1.1 |
| 17.7.1.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 17.7.2.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.1 | Same as 7.7.2.1 |
| 17.7.2.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.2 | Same as 7.7.2.2 |
| 17.7.3.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Same as 7.7.4.1 | Same as 7.7.4.1 |
| 17.7.3.2 NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off | Same as 7.7.4.2 | Same as 7.7.4.2 |
| 17.7.4.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.1 | Same as 7.7.3.1 |
| 17.7.4.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.2 | Same as 7.7.3.2 |

<End of Change 1>

<Start of Change 2>

Table F.1.3.2-8: Derivation of test requirements for NR SA FR2 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx | Same as 7.1.1.1 | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| 17.1.1.2 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case | Same as 7.1.1.2 | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| 17.1.1.3 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.3 | Same as 7.1.1.3 |  | Same as 7.1.1.3 |  |
| 17.1.1.4 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.5 | Same as 7.1.1.5 |  | Same as 7.1.1.5 |  |
| 17.3.1.1 NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.2 | Same as 7.3.1.2 |  | Same as 7.3.1.2 |  |
| 17.3.1.2 NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.3 | Same as 7.3.1.3 |  | Same as 7.3.1.3 |  |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |
| 17.3.2.3.1 NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2 | Same as 7.3.2.3 |  | Same as 7.3.2.3 |  | Same as 7.3.2.3 |
| 17.4.1.1 NR SA FR2 NR UE Transmit Timing Test for FR2 | Same as 7.4.1.1 |  | Same as 7.4.1.1 |  | Same as 7.4.1.1 |
| 17.5.1.1 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 7.5.1.1 |  | Same as 7.5.1.1 |  | Same as 7.5.1.1 |
| 17.5.1.2 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Same as 7.5.1.2 |  | Same as 7.5.1.2 |  | Same as 7.5.1.2 |
| 17.5.1.3 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 7.5.1.1 |  | Same as 7.5.1.1 |  | Same as 7.5.1.1 |
| 17.5.1.4 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Same as 7.5.1.4 |  | Same as 7.5.1.4 |  | Same as 7.5.1.4 |
| 17.5.1.5 NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 7.5.1.5 |  | Same as 7.5.1.5 |  | Same as 7.5.1.5 |
| 17.5.1.6 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode | Same as 7.5.1.6 |  | Same as 7.5.1.6 |  | Same as 7.5.1.6 |
| 17.5.1.7 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 7.5.1.7 |  | Same as 7.5.1.7 |  | Same as 7.5.1.7 |
| 17.5.1.8 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 7.5.1.8 |  | Same as 7.5.1.8 |  | Same as 7.5.1.8 |
| 17.5.2.3 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | Same as 7.5.5.3 |  | Same as 7.5.5.3 |  | Same as 7.5.5.3 |
| 17.5.2.4 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode | Same as 7.5.5.3 |  | Same as 7.5.5.3 |  | Same as 7.5.5.3 |
| 17.5.2.5 NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Same as 7.5.5.1 |  | Same as 7.5.5.1 |  | Same as 7.5.5.1 |
| 17.5.3.1.1 NR SA FR2 DCI-based and Timer-based DL active BWP switch with non-DRX | T1 to T3 Noc: -112 dBm/15kHz Ês / Noc: 18dB |  | 0dB 0dB |  | T1 to T3 Noc: -112 dBm/15kHz Ês / Noc: 18dB |
| 17.5.3.2.1 NR SA FR2 RRC-based DL active BWP switch with non-DRX | Same as 17.5.3.1.1 |  | Same as 17.5.3.1.1 |  | Same as 17.5.3.1.1 |
| 17.5.4.1.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |
| 17.5.4.2.1 NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state | Same as 7.5.8.2.1 |  | Same as 7.5.8.2.1 |  | Same as 7.5.8.2.1 |
| 17.5.5.1.1 NR SA FR2 PCell MAC-CE based spatial relation switch associated with known DL-RS | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |
| 17.5.5.2.1 NR SA FR2 PCell RRC-based spatial relation switch associated with a known DL-RS | Same as 7.5.9.2.1 |  | Same as 7.5.9.2.1 |  | Same as 7.5.9.2.1 |
| 17.6.1.1 NR SA FR2 Event triggered reporting test without gap under non-DRX | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |
| 17.6.1.2 NR SA FR2 Event triggered reporting test without gap under DRX | Same as 7.6.1.2 |  | Same as 7.6.1.2 |  | Same as 7.6.1.2 |
| 17.6.1.3 NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |
| 17.6.1.4 NR SA FR2 Event triggered reporting test with per-UE gaps under DRX | Same as 7.6.1.2 |  | Same as 7.6.1.2 |  | Same as 7.6.1.2 |
| 17.6.2.1 NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |
| 17.6.2.2 NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is used | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |
| 17.6.2.3 NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |
| 17.6.2.4 NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is used | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |
| 17.6.3.1 NR SA FR2 SSB based L1-RSRP measurement when DRX is not used | Same as 7.6.3.1 |  | Same as 7.6.3.1 |  | Same as 7.6.3.1 |
| 17.6.3.2 NR SA FR2 SSB based L1-RSRP measurement when DRX is used | Same as 7.6.3.1 |  | Same as 7.6.3.1 |  | Same as 7.6.3.1 |
| 17.6.3.3 NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used | Same as 7.6.3.3 |  | Same as 7.6.3.3 |  | Same as 7.6.3.3 |
| 17.6.3.4 NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used | Same as 7.6.3.3 |  | Same as 7.6.3.3 |  | Same as 7.6.3.3 |
| 17.7.1.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.1 |  | Same as 7.7.1.1 |  | Same as 7.7.1.1 |
| 17.7.1.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.1.2 |  | Same as 7.7.1.2 |  | Same as 7.7.1.2 |
| 17.7.2.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.1 |  | Same as 7.7.2.1 |  | Same as 7.7.2.1 |
| 17.7.2.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.2.2 |  | Same as 7.7.2.2 |  | Same as 7.7.2.2 |
| 17.7.3.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Same as 7.7.4.1 |  | Same as 7.7.4.1 |  | Same as 7.7.4.1 |
| 17.7.3.2 NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off | Same as 7.7.4.2 |  | Same as 7.7.4.2 |  | Same as 7.7.4.2 |
| 17.7.4.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.1 |  | Same as 7.7.3.1 |  | Same as 7.7.3.1 |
| 17.7.4.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.2 |  | Same as 7.7.3.2 |  | Same as 7.7.3.2 |

<End of Change 2>