3GPP TSG-RAN5 Meeting #104	R5-245980

# Maastricht, Netherlands, 19th – 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.901-5 | CR | 0040 | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Table 2 Reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, FS_UE_5GNR_App_Data_Perf |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In various tables, a Table 2 is listed as entry for CQI-table but 37.901-5 does not have a Table 2 and no reference document is provided |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the IE element entry from Table 2 to table2 and change the IE element from CQI-table to cqi-table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarity |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.3, A.10.1.1.1.3, A.10.1.1.2.3, A.10.1.2.1.3, A.10.1.2.2.3, A.11.1.1.1.3, A.11.1.1.2.3, A.11.1.2.1.3, A.11.1.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-244447 R5-245980: set cqi-table entry from Table 2 to table2 |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

### 5.10.3	Simulation Assumptions

The simulation assumptions are captured in Table 5.10.3-1 for studying feasibility of defining link adaptation absolute physical layer requirements.

Table 5.10.3-1: Simulation assumptions for Absolute Physical Layer Throughput alignment with link adaptation

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| cqi-table |  |  | table2 | table2 | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

# A.10	5G NR /TCP Downlink Throughput /Conducted for Variable Reference Channel (VRC) Scenarios with Fading for SA and NSA

## A.10.1	5G NR /TCP Downlink Throughput /Conducted/Fading/VRC for SA and NSA

### A.10.1.1	5G NR /TCP Downlink Throughput /Conducted/Fading/VRC/2Rx for SA and NSA

#### A.10.1.1.1	5G NR /TCP Downlink Throughput /Conducted/Fading/VRC/2Rx FDD /FR1 PDSCH mapping Type A performance - for SA and NSA

A.10.1.1.1.1	Definition

The UE application layer downlink performance for TCP under fading environment with variable reference channel is determined by the UE application layer TCP throughput.

A.10.1.1.1.2	Test Purpose

To measure the performance of the 5G NR UE while downloading TCP based data in a fading channel environment with variable reference channel under 2 receive antenna conditions for FR1. The duplex mode is FDD.

A.10.1.1.1.3	Test Parameters

The test parameters are specified in Table A.10.1.1.1.3-1.

Table A.10.1.1.1.3-1: FDD FR1 2Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.10.1.1.1.4	Test Description

A.10.1.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. Apply the Initial conditions defined in TS 38.521-4 [3] clause 6.4.2.1_1.4.1 with the following additional steps and/or exceptions:

1.1		Connect an application server to the IP output of the SS.

1.2		For an embedded configuration, ensure that the UE has a client test application available. For a tethered configuration, tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

2.	In Step 2 skip reference to 38.521-4 [3] Table 5.2.2.1.1.0-2 and Table 5.2.2.1.0-3 since test parameters are already defined for this test.

5.	For NSA case, the E-UTRA anchor is configured as per Annex E. Ensure the UE is in RRC_CONNECTED State with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode Off or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode Off for EN-DC. Message contents are as per TS 38.521-4 [3] Clause 6.4.2.1_1.4.3 with the exceptions defined in Annex H.

A.10.1.1.1.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.1.1.3-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC over PUSCH according to parameters set during initial conditions. The purpose of this scheduling is to accommodate for TCP UL ACK/NACK feedback transmissions.

4.	Using the data client, begin TCP downlink data transfer from the application server. Wait for 15 seconds and then start recording the TCP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

5.	Repeat step 4 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

6.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

7.	Record the IP address type (IPv4 or IPv6) used during the TCP data transfers.

8.	Using the values for TCP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.1.1.3-1 to obtain reference Application Layer Throughput value.

#### A.10.1.1.2	5G NR /TCP Downlink Throughput /Conducted/Fading/VRC/2Rx TDD /FR1 PDSCH mapping Type A performance - for SA and NSA

A.10.1.1.2.1	Definition

The UE application layer downlink performance for TCP under fading environment with variable reference channel is determined by the UE application layer TCP throughput.

A.10.1.1.2.2	Test Purpose

To measure the performance of the 5G NR UE while downloading TCP based data in a fading channel environment with variable reference channel under 2 receive antenna conditions for FR1. The duplex mode is TDD.

A.10.1.1.2.3	Test Parameters

The test parameters are specified in Table A.10.1.1.2.3-1.

Table A.10.1.1.2.3-1: TDD FR1 2Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD Slot Configuration |  |  | 7D1S2U S:6D+4G+4U |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 9 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.10.1.1.2.4	Test Description

A.10.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. Apply the Initial conditions defined in TS 38.521-4 [3] clause 6.4.2.2_1.4.1 with the following additional steps and/or exceptions:

1.1		Connect an application server to the IP output of the SS.

1.2		For an embedded configuration, ensure that the UE has a client test application available. For a tethered configuration, tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

2.	In Step 2 skip reference to 38.521-4 [3] Table 6.4.2.2_1.3-1 since test parameters are already defined for this test.

5.	For NSA case, the E-UTRA anchor is configured as per Annex E. Ensure the UE is in RRC_CONNECTED State  with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode Off or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode Off  for EN-DC. Message contents are as per TS 38.521-4 [3] Clause 6.4.2.2_1.4.3 with the exceptions defined in Annex H.

A.10.1.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.1.2.3-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC over PUSCH according to parameters set during initial conditions. The purpose of this scheduling is to accommodate for TCP UL ACK/NACK feedback transmissions.

4.	Using the data client, begin TCP downlink data transfer from the application server. Wait for 15 seconds and then start recording the TCP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

5.	Repeat step 4 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

6.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

7.	Record the IP address type (IPv4 or IPv6) used during the TCP data transfers.

8.	Using the values for TCP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.1.2.3-1 to obtain reference Application Layer Throughput value.

### A.10.1.2	5G NR /TCP Downlink Throughput /Conducted/Fading/VRC/4Rx

#### A.10.1.2.1	5G NR /TCP Downlink Throughput /Conducted/Fading/VRC/4Rx FDD/FR1 PDSCH mapping Type A performance - for SA and NSA

A.10.1.2.1.1	Definition

The UE application layer downlink performance for TCP under fading environment with variable reference channel is determined by the UE application layer TCP throughput.

A.10.1.2.1.2	Test Purpose

To measure the performance of the 5G NR UE while downloading TCP based data in a fading channel environment with variable reference channel under 4 receive antenna conditions for FR1. The duplex mode is FDD.

A.10.1.2.1.3	Test Parameters

The test parameters are specified in Table A.10.1.2.1.3-1.

Table A.10.1.2.1.3-1: FDD FR1 4Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.10.1.2.1.4	Test Description

A.10.1.2.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. Apply the Initial conditions defined in TS 38.521-4 [3] clause 6.4.3.1_1.4.1 with the following additional steps and/or exceptions:

1.1		Connect an application server to the IP output of the SS.

1.2		For an embedded configuration, ensure that the UE has a client test application available. For a tethered configuration, tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

2.	In Step 2 skip reference to 38.521-4 [3] Table 6.4.3.1_1.3-1 since test parameters are already defined for this test.

5.	For NSA case, the E-UTRA anchor is configured as per Annex E. Ensure the UE is in RRC_CONNECTED State with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode Off or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode Off for EN-DC. Message contents are as per TS 38.521-4 [3] Clause 6.4.3.1_1.4.3 with the exceptions defined in Annex H.

A.10.1.2.1.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.2.1.3-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC over PUSCH according to parameters set during initial conditions. The purpose of this scheduling is to accommodate for TCP UL ACK/NACK feedback transmissions.

4.	Using the data client, begin TCP downlink data transfer from the application server. Wait for 15 seconds and then start recording the TCP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

5.	Repeat step 4 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

6.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

7.	Record the IP address type (IPv4 or IPv6) used during the TCP data transfers.

8.	Using the values for TCP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.2.1.3-1 to obtain reference Application Layer Throughput value.

#### A.10.1.2.2	5G NR /TCP Downlink Throughput /Conducted/Fading/VRC/4Rx TDD/FR1 PDSCH mapping Type A performance - for SA and NSA

A.10.1.2.2.1	Definition

The UE application layer downlink performance for TCP under fading environment with variable reference channel is determined by the UE application layer TCP throughput.

A.10.1.2.2.2	Test Purpose

To measure the performance of the 5G NR UE while downloading TCP based data in a fading channel environment with variable reference channel under 4 receive antenna conditions for FR1. The duplex mode is TDD.

A.10.1.2.2.3	Test Parameters

The test parameters are specified in Table A.10.1.2.2.3-1.

Table A.10.1.2.2.3-1: TDD FR1 4Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD Slot Configuration |  |  | 7D1S2U S:6D+4G+4U |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 9 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.10.1.2.2.4	Test Description

A.10.1.2.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. Apply the Initial conditions defined in TS 38.521-4 [3] clause 6.4.3.2_1.4.1 with the following additional steps and/or exceptions:

1.1		Connect an application server to the IP output of the SS.

1.2		For an embedded configuration, ensure that the UE has a client test application available. For a tethered configuration, tether the UE to a laptop configured with FTP client software using the appropriate UE to PC interface Modem or Network Interface Connection (NIC) drivers.

2.	In Step 2 skip reference to 38.521-4 [3] Table 6.4.3.2_1.3-1 since test parameters are already defined for this test.

5.	For NSA case, the E-UTRA anchor is configured as per Annex E. Ensure the UE is in RRC_CONNECTED State with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode Off or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode Off for EN-DC. Message contents are as per TS 38.521-4 [3] Clause 6.4.3.2_1.4.3 with the exceptions defined in Annex H.

A.10.1.2.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.2.2.3-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC over PUSCH according to parameters set during initial conditions. The purpose of this scheduling is to accommodate for TCP UL ACK/NACK feedback transmissions.

4.	Using the data client, begin TCP downlink data transfer from the application server. Wait for 15 seconds and then start recording the TCP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

5.	Repeat step 4 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

6.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

7.	Record the IP address type (IPv4 or IPv6) used during the TCP data transfers.

8.	Using the values for TCP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.2.2.3-1 to obtain reference Application Layer Throughput value.

# A.11	5G NR /UDP Downlink Throughput /Conducted for Variable Reference Channel (VRC) Scenarios for SA and NSA

## A.11.1	5G NR /UDP Downlink Throughput /Conducted/Fading/VRC

### A.11.1.1	5G NR /UDP Downlink Throughput /Conducted/Fading/VRC/2Rx

#### A.11.1.1.1	5G NR /UDP Downlink Throughput /Conducted/Fading/VRC/2Rx FDD/FR1 PDSCH mapping Type A performance - for SA and NSA

A.11.1.1.1.1	Definition

The UE application layer downlink performance for UDP under fading environment with variable reference channel is determined by the UE application layer UDP throughput.

A.11.1.1.1.2	Test Purpose

To measure the performance of the 5G NR UE while downloading UDP based data in a fading channel environment with variable reference channel under 2 receive antenna conditions for FR1. The duplex mode is FDD.

A.11.1.1.1.3	Test Parameters

The test parameters are specified in Table A.11.1.1.1.3-1.

Table A.11.1.1.1.3-1: FDD FR1 2Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.11.1.1.1.4	Test Description

A.11.1.1.1.4.1	Initial Conditions

Same initial conditions as in clause A.10.1.1.1.4.1.

A.11.1.1.1.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.1.1.3-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	Using the data client, begin UDP downlink data transfer from the application server. Wait for 15 seconds and then start recording the UDP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

6.	Record the IP address type (IPv4 or IPv6) used during the UDP data transfers.

7.	Using the values for UDP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.1.1.3-1 to obtain reference Application Layer Throughput value.

#### A.11.1.1.2	5G NR /UDP Downlink Throughput /Conducted/Fading/VRC/2Rx TDD/FR1 PDSCH mapping Type A performance - for SA and NSA

A.11.1.1.2.1	Definition

The UE application layer downlink performance for UDP under fading environment with variable reference channel is determined by the UE application layer UDP throughput.

A.11.1.1.2.2	Test Purpose

To measure the performance of the 5G NR UE while downloading UDP based data in a fading channel environment with variable reference channel under 2 receive antenna conditions for FR1. The duplex mode is TDD.

A.11.1.1.2.3	Test Parameters

The test parameters are specified in Table A.11.1.1.2.3-1.

Table A.11.1.1.2.3-1: TDD FR1 2Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD Slot Configuration |  |  | 7D1S2U S:6D+4G+4U |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 9 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.11.1.1.2.4	Test Description

A.11.1.1.2.4.1	Initial Conditions

Same initial conditions as in clause A.10.1.1.2.4.1.

A.11.1.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.1.2.3-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	Using the data client, begin UDP downlink data transfer from the application server. Wait for 15 seconds and then start recording the UDP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

6.	Record the IP address type (IPv4 or IPv6) used during the UDP data transfers.

7.	Using the values for UDP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.1.2.3-1 to obtain reference Application Layer Throughput value.

### A.11.1.2	5G NR /UDP Downlink Throughput /Conducted/Fading/VRC/4Rx

#### A.11.1.2.1	5G NR /UDP Downlink Throughput /Conducted/Fading/VRC/4Rx FDD/FR1 PDSCH mapping Type A performance - for SA and NSA

A.11.1.2.1.1	Definition

The UE application layer downlink performance for UDP under fading environment with variable reference channel is determined by the UE application layer UDP throughput.

A.11.1.2.1.2	Test Purpose

To measure the performance of the 5G NR UE while downloading UDP based data in a fading channel environment with variable reference channel under 4 receive antenna conditions for FR1. The duplex mode is FDD.

A.11.1.2.1.3	Test Parameters

The test parameters are specified in Table A.11.1.2.1.3-1.

Table A.11.1.2.1.3-1: FDD FR1 4Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.11.1.2.1.4	Test Description

A.11.1.2.1.4.1	Initial Conditions

Same initial conditions as in clause A.10.1.2.1.4.1.

A.11.1.2.1.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.2.1.3-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	Using the data client, begin UDP downlink data transfer from the application server. Wait for 15 seconds and then start recording the UDP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

6.	Record the IP address type (IPv4 or IPv6) used during the UDP data transfers.

7.	Using the values for UDP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.2.1.3-1 to obtain reference Application Layer Throughput value.

#### A.11.1.2.2	5G NR /UDP Downlink Throughput /Conducted/Fading/VRC/4Rx TDD/FR1 PDSCH mapping Type A performance - for SA and NSA

A.11.1.2.2.1	Definition

The UE application layer downlink performance for UDP under fading environment with variable reference channel is determined by the UE application layer UDP throughput.

A.11.1.2.2.2	Test Purpose

To measure the performance of the 5G NR UE while downloading TCP based data in a fading channel environment with variable reference channel under 4 receive antenna conditions for FR1. The duplex mode is TDD.

A.11.1.2.2.3	Test Parameters

The test parameters are specified in Table A.11.1.2.2.3-1.

Table A.11.1.2.2.3-1: TDD FR1 4Rx Test point

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR1 |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Duplex Mode |  |  | TDD |
| TDD Slot Configuration |  |  | 7D1S2U S:6D+4G+4U |
| SNR |  | dB | 20 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | ULA Low 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |
|  | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| cqi-table |  |  | table2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 9 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  | CodebookSubsetRestriction |  | Not configured |
|  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 1 |
| Test metric |  |  | [T]% of max throughput at target SNR. |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement |  |  |  |

A.11.1.2.2.4	Test Description

A.11.1.2.2.4.1	Initial Conditions

Same initial conditions as in clause A.10.1.2.2.4.1.

A.11.1.2.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, reference channel, the propagation condition, antenna configuration, antenna correlation, Codebook configuration, Beamforming Model, RI configuration and SNR according to Table A.10.1.2.2.4-1 as appropriate.

2.	SS shall send PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the UE reported CQI (wideband CQI), PMI and RI. The SS sends downlink MAC padding bits on the DL RMC.

3.	Using the data client, begin UDP downlink data transfer from the application server. Wait for 15 seconds and then start recording the UDP throughput result (This is iteration 1). Continue data transfer for the test duration outlined in Table A.1-14.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput across three iterations. Additionally, count and record the overall number of ACK and NACK/DTX on the PUSCH/PUCCH during the test interval purely for information purposes.

6.	Record the IP address type (IPv4 or IPv6) used during the UDP data transfers.

7.	Using the values for UDP in Table 5.4.4-2 (for IPv6) and Table 5.4.4-3 (for IPv4), determine the reduction from PHY reference fractional throughput value listed in Table A.10.1.2.2.4-1 to obtain reference Application Layer Throughput value.

<<< END OF CHANGES >>>