3GPP TSG-RAN5 Meeting #104	R5-246012

# Maastricht, Netherlands, Aug 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2925 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test case Reference sensitivity power level for SUL (4CC) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There’s no applicable REFSENS test case for SUL operation with 3 DL CA configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 7.3C.4 Reference sensitivity power level for SUL with 3 DL CA, and adding test requirements for CA_n41C_n79A-n83A Besides, the title of test case 7.3C.3 is updated to align with the other SUL with DL CA test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The REFSENS for CA_n41C_n79A-n83A can’t be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C.3, 7.3C.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No need to update TR 38.905 and MU TT since no test points defined in this new test case. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-244867r2. |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 7.3C.3	Reference sensitivity power level for SUL with 2 DL CA

### <Unchanged Skipped>

7.3C.3.4	Test description

NOTE:	No testing needs to be performed since the testing has been covered in test case 7.3.2 and 7.3C.2.

For band combination CA_nX_nY-nZ, test the REFSENS of SUL configuration or NR band as listed in table 7.3C.3.4-1.

Table 7.3C.3.4-1: Test band combinations and configuration

| Band configuration | Verifying REFSENS of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n1A_n78A-n80A | SUL_n78A-n80A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n1 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n1A_n78A-n84A | SUL_n78A-n84A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n1 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n3A_n78A-n80A | SUL_n78A-n80A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n3 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n28A_n41A-n83A | SUL_n41A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n28 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n79C-n83A | SUL_n79A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
| CA_n28A_n79A-n83A | SUL_n79A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n28 | 7.3.2 | Table 7.3.2.4.1-1 |

### 7.3C.4	Reference sensitivity power level for SUL with 3 DL CA

Editor’s Note:

- No test points defined for Reference sensitivity power level testing for SUL with DL CA. This test case is covered by 7.3A.1 and 7.3C.2.

7.3C.4.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under SUL and 3 DL CA operation and conditions of low signal level, ideal propagation and no added noise.

7.3C.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports SUL operation on the SUL bands with 3DL CA.

7.3C.4.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 7.3C.0.

7.3C.4.4	Test description

NOTE:	No testing needs to be performed since the testing has been covered in test case 7.3.2, 7.3A.1 and 7.3C.2.

For band combination CA_nX_nY-nZ with one NUL band configured with 2DL CA, test the REFSENS of SUL configuration or NR bands as listed in table 7.3C.4.4-1.

Table 7.3C.4.4-1: Test band combinations and configuration

| Band configuration | Verifying REFSENS of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n41C_n79A-n83A | n41C | 7.3A.1 | Table 7.3A.1.4.1-1 |
|  | SUL_n79A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |

7.3C.4.5	Test requirement

Same test requirement as clause 7.3A.1 and 7.3C.2 for each band or band combinations listed in table 7.3C.4.4-1.

### <End of Changes>