3GPP TSG-RAN WG5 Meeting # 104	R5-246041

# Maastricht, Netherlands,  August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2953 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on 6.2D and 6.2H for delta TIB description for UL MIMO transmitter power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_bands_UL_MIMO_PC3_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The texts for how to handle the ΔTIB,c for the cases when the UE supports more than one of band combination have been duplicated for many times in different sub-clauses in the spec. It is suggested to move the corresponding texts to the “General” sub-clause for the test of UL MIMO transmitter power. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the text for how to handle the ΔTIB,c for the cases when the UE supports more than one of band combination to the “General” sub-clause (new added) for UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The texts for how to handle the ΔTIB,c for the cases when the UE supports more than one of band combination will be redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1.5, 6.2D.1_1.5, 6.2D.2.5, 6.2D.2_1.5, 6.2D.3.5, 6.2D.3_1.5, 6.2D.4.5, 6.2D.4_1.5, 6.2H.0 (new), 6.2H.1.2.5, 6.2H.1.3.5, 6.2H.1.4.5, 6.2H.3.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: The general test requirement for ΔTIB,c is referred to newly added clause 6.0 instead of 6.2D.0. The new clause 6.0 is introduced by another CR 2949. |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2D	Transmitter power for UL MIMO

6.2D.1	UE maximum output power for UL MIMO

Editor’s Note:

- No test points are defined for 2-layer UL MIMO since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- No test points are defined for Power Class 1.5 non-FWA UEs supporting UFPTx since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- TP analysis needs to be updated regarding Note 3 in Table 6.2D.1.4.1-2

6.2D.1.1	Test purpose

To verify that the error of the UE maximum output power for UL MIMO does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2D.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 FWA UEs, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n2 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n5 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n8 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n24 |  |  |  |  |  |  | 23 | +2/-41 |  |  |
| n25 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n28 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n38 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n40 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n41 |  |  | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n70 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n97 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: 1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.1.3-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UEs supporting uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1.3-1 shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1.3-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [9]. NOTE 3: For PUSCH configured with ULFPTx Mode set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2 apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling with the following exception: for UEs indicating txDiversity-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass.

A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.1.

6.2D.1.4	Test description

6.2D.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1.4.1-1: Test Configuration Table for 2-layer UL MIMO

NOTE:	No test points are defined since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2D.1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx) for Power Class 3, Power Class 2 and Power Class 1.5 FWA UEs

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Only Test IDs 1, 2 and 3 apply to Power Class 2 UEs if TxD is indicated. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.1.4.3.

6.2D.1.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO:

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

Sub-test for ULFPTx

4.	If the UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on the UE supported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

6.2D.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1.5-1.

Table 6.2D.1.5-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-3-TT |  |  |
| n2 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n3 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n5 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n7 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n8 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n24 |  |  |  |  |  |  | 23 | +2+TT/-41-TT |  |  |
| n25 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n28 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n30 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n34 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n38 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n39 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n40 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n41 |  |  | 29 | _+2+TT/-31-TT | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n48 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n66 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n70 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n71 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n77 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n78 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n79 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n97 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: 1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.1.5-2 NOTE 3: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.1_1	UE maximum output power for SUL with UL MIMO

6.2D.1_1.1	Test purpose

Same test purpose as in clause 6.2D.1.1.

6.2D.1_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL full power transmission (ULFPTx) operating on the SUL bands.

6.2D.1_1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1_1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1_1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1_1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n84 |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n95 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n97 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n98 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n99 |  |  |  |  | 23 | +2/-41 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1_1.3-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1_1.3-1 shall be met with the PUSCH configurations specified in Table 6.2D.1_1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1_1.3-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ULFPTxModes set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2 apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling with the following exception: for UEs indicating txDiversity-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass.

A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.2D.1.

6.2D.1_1.4	Test description

6.2D.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2D.1_1.4.1-1 and Table 6.2D.1_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1_1.4.1-1: Test Configuration Table for 2-layer UL MIMO on SUL

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2D.1_1.4.1-2: Test Configuration Table for uplink full power transmission on SUL(ULFPTx)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for NUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for NUL carrier |  |
| Test SCS as specified in Table 5.5C-1 |  |  | Lowest supported SCS for NUL carrier 15kHz for SUL carrier |  |
| Test Parameters |  |  |  |  |
| Test ID | DL config | UL config | SUL Configuration |  |
|  |  |  | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure 3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2 and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.1_1.4.1-1 and Table 6.2D.1_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.1_1.4.3.

6.2D.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1_1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.1_1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

6.2D.1_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.1_1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1_1.5-1.

Table 6.2D.1_1.5-1: UE Power Class

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n84 |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-3-TT |  |  |
| n95 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n97 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n98 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n99 |  |  |  |  | 23 | +2+TT/-41-TT |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1_1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.2	UE maximum output power reduction for UL MIMO

6.2D.2.1	Test purpose

To verify that the power reduction of UE due to higher order modulations and transmit bandwidth configuration does not exceed the specified maximum power reduction.

6.2D.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed.

6.2D.2.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 for 2Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band and Table 6.2.2.3-2 for 2Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 respectively. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16, the allowed MPR for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 when TxD is indicated and Table 6.2.2.3-2 when TxD is not indicated for PC2 , Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 respectively, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the maximum output power requirement with MPR according to clause 6.2.2.3. When a UE that indicates PC1.5 for a given band is limited to PC2 by the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply.

The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of .

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the corresponding requirements in clause 6.2D.1.3 apply for the power class as indicated by the ue-PowerClass field in capability signalling. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 with MPR according to clause 6.2.2.3 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port.

Table 6.2D.2.3-1: Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6.2D.2.3-2: Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Table 6.2D.2.3-3: Maximum power reduction (MPR) for power class 1.5 FWA with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Inner, outer and edge allocations are as defined in section 6.2.2 except for 	PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes. The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.2.

6.2D.2.4	Test description

6.2D.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.2.4.1-1, Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2, Table 6.2D.2.4.1-2a, Table 6.2D.2.4.1-3 and Table 6.2D.2.4.1-3a. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2.4.1-1: Test Configuration Table for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-1a: Test Configuration Table for power class 3 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 24 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 34 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 44 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2D.2.4.1-2: Test Configuration Table for Power Class 2 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-2a: Test Configuration Table for power class 2 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

Table 6.2D.2.4.1-3: Test Configuration Table for Power Class 1.5 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-3a: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2.4.1-1, Table 6.2D.2.4.1-2 or Table 6.2D.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2.4.3.

6.2D.2.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1, Table 6.2D.2.5-2, Table 6.2D.2.5-2a, Table 6.2D.2.5-3, or Table 6.2D.2.5-4. The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1a, Table 6.2D.2.5-2b, Table 6.2D2.5-2c, Table 6.2D.2.5-3a, or Table 6.2D.2.5-4a.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n24, n25, n28, n30, n34, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79, n97) for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 | （5.02） | 3 (44) | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 (44) | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 (44) | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 (44) | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. NOTE 4: Applicable for band n24. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-1a: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n24, n25, n28, n30, n34, n38, n41, n48, n66, n70, n71, n77, n78, n79) for Power Class 3 supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.5 - TT2） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.5 - TT2） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | (5.02) | 3 (44) | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. NOTE 4: Only applicable for band n24. NOTE 5: For band n24 when transmission bandwidths are not confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2: UE Power Class test requirements (for Bands n1, n3, n41, n66, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO without indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For n1, n3, and Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2a: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO and indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 | （3.02） | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.6 - TT2） |
| 5 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.6 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 9 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 13 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 14 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2b: UE Power Class test requirements (for Bands n1, n3, n41, n66, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and ul-FullPwrMode1-TPMIGroup-r1 and indicating TxD

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 1.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2c: UE Power Class test requirements (for Bands n1, n3, n41, n66, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and, (supporting ul-FullPwrMode1-TPMIGroup-r1 without indicating TxD) or (supporting ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.5 - TT2） |
| 2 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 5 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.0 - TT2） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 9 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 12 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | （14.0- TT2） |
| 13 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 14 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | （14.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.0 - TT2） |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.5 - TT2） |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.5 - TT2） |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.5 - TT2） |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.0 - TT2） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.0 - TT2） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.0 - TT2） |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.0 - TT2） |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | （26.02） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.0 - TT2） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 13 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 15 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 16 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0- TT2） |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.0 - TT3） |
| 2 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.0 - TT3） |
| 3 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.0 - TT3） |
| 4 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 – TT | （20.5 - TT3） |
| 5 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.5 - TT3） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 8 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.5 - TT3） |
| 9 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 11 | 29 | 0 | 4.5 | 0 | （1.53） | 24.5 | （23.03） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.0 - TT3） |
| 12 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | （14.0- TT3） |
| 13 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | （14.0 - TT3） |
| 14 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | （14.0 - TT3） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.5 - TT3） |
| 2 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.5 - TT3） |
| 3 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.5 - TT3） |
| 4 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 – TT | （23.0 - TT3） |
| 5 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.5 - TT3） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.0 - TT3） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.0 - TT3） |
| 8 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.5 - TT3） |
| 9 | 29 | 0 | 1 | 0 | （1.53） | 28.0 | （26.53） | 3.0 |  | 3 | 31.0 + TT | 25.0 - TT | （23.5 - TT3） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 11 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 12 | 29 | 0 | 3 | 0 | （1.53） | 26.0 | （24.53） | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | （21.5 - TT3） |
| 13 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 14 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.0 - TT3） |
| 15 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | （24.03） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.0 - TT3） |
| 16 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0- TT3） |
| 17 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.0 - TT3） |
| 18 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | （22.03） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.5 - TT | （17.0 - TT3） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.2_1	UE maximum output power reduction for SUL with UL MIMO

6.2D.2_1.1	Test purpose

Same test purpose as in clause 6.2D.2.1

6.2D.2_1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1_1 is executed.

6.2D.2_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The minimum requirements are same as in 6.2D.1.3 with following exceptions:

Instead of Table 6.2D.1.3-1  use Table 6.2D.1_1.3-1

6.2D.2_1.4	Test description

6.2D.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.2D.2_1.4.1-1 ~ 6.2D.2_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2_1.4-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2_1.4.1-2: Test Configuration Table for Power Class 2

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2_1.4.1-1 ~ Table 6.2D.2_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2_1.4.3.

6.2D.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2_1.4.1-1 and Table 6.2D.2_1.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

6.2D.2_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2_1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2_1.5-1~6.2D.2_1.5-2.

Table 6.2D.2_1.5-1: UE Power Class test requirements (for Band n80, n84, n95, n97) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2D.2_1.5-1a: UE Power Class test requirements for Band n99 for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 4 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 4 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 4 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 | （5.02） | 4 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 4 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 4 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 4 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 4 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 4 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 4 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-2: UE Power Class test requirements (for Band n80, n84, n95, n97) for Power Class 2

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16 - TT2） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （222） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17 - TT2） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16 - TT2） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14 - TT2） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14 - TT2） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14 - TT2） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16,52） | 5.0 | （5.02） | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 | （5.02） | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 | （5.02） | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80 and n84, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-3: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U to all types of NR Power Class 2 and 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04, NS_47, NS_21, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

The requirements of this test apply in test case 6.5D.3_1.3 Additional Spurious Emissions for network signalling value NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed.

6.2D.3.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the A-MPR values specified in clause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.

6.2D.3.4	Test description

6.2D.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2D.3.4.1-1 to 6.2D.3.4.1-11. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2D.3.4.1-1: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 3 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 4 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 5 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 6 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 9 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 10 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 11 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 12 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 13 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 14 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 16 | Low |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 17 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 18 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 20 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 23 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-1a: Test Configuration table for NS_04 with supporting ULFPTx

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-2: Test Configuration table for NS_35

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_35 |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default | N/A | CP-OFDM QPSK | Outer Full |
| 4 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 7 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 10 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |

Table 6.2D.3.4.1-3: Test Configuration table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 35 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 37 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 38 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 39 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 40 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 41 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 42 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 43 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |
| 44 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 45 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 46 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 47 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 48 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 49 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 50 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 | 37@14 |  | 17@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-4: Test Configuration table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 35 | 1935 | 10 | Default |  | A4 |  | 64 QAM |  | Outer_Full |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 37 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 38 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 39 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 40 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 41 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM |  | Outer_Full |  |  |  |
| 42 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 43 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 44 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 45 | 1950 | 20 | Default |  | A6 |  | 64 QAM |  | Outer_Full |  |  |  |
| 46 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |
| 47 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 48 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 49 | 1935 | 10 | Default |  | A4 |  | 256 QAM |  | Outer_Full |  |  |  |
| 50 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 51 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 52 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 53 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM |  | Outer_Full |  |  |  |
| 54 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 55 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 56 | 1950 | 20 | Default |  | A6 |  | 256 QAM |  | Outer_Full |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-5: Test Configuration table for NS_48

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 5 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 6 | Default | 40 | Default |  |  | QPSK | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 7 | Default | 40 | Default |  |  | QPSK | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 8 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 9 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 10 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 11 | Default | 50 | Default |  |  | QPSK | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 12 | Default | 50 | Default |  |  | QPSK | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 13 | Default | 50 | Default |  |  | QPSK | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 14 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 15 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 16 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 17 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 19 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 20 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 21 | Default | 40 | Default |  |  | 256 QAM | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 22 | Default | 40 | Default |  |  | 256 QAM | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 23 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 24 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 25 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 26 | Default | 50 | Default |  |  | 256 QAM | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 27 | Default | 50 | Default |  |  | 256 QAM | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 28 | Default | 50 | Default |  |  | 256 QAM | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 29 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 30 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-6: Test Configuration table for NS_49

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 5 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 7 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 8 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 10 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 11 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 12 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 14 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 15 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 16 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 17 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 18 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 19 | Default | 50 | Default |  |  | QPSK | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 20 | Default | 50 | Default |  |  | QPSK | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 22 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 23 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 24 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 26 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 27 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 28 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 29 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 31 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 32 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 33 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 34 | Default | 40 | Default |  |  | 256 QAM | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 35 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 36 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 37 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 38 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 39 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 40 | Default | 50 | Default |  |  | 256 QAM | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 41 | Default | 50 | Default |  |  | 256 QAM | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 42 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-7: Test Configuration table for NS_03, NS_03U and NS_100

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_03, NS_03U and NS_100 |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR test cases | CP- OFDM | QPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 7 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-7a: Test Configuration Table for NS_03, NS_03U and NS_100 for UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| Test parameters for NS_03U |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A | DFT-s OFDM | PI/2 BPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 13 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| 16 | Low | Default | Default |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 19 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 22 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 24 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 25 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-8: Test Configuration table for NS_46

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15MHz, 20MHz, 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS |  | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  |  | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 2 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 3 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default | Downlink Configuration | CP-OFDM | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 7 | Default | 50 | Default |  |  | QPSK | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 8 | Default | 50 | Default |  |  | 64QAM | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 9 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 10 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 11 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 12 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-9: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink | Uplink Configuration |  |  |  |  |  |
|  |  |  |  | Configuration | Modulation |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  | (Note 2) |  |  | SCS 15 kHz | SCS 30 kHz |  |
| 1 | Low | Default | Default |  |  | QPSK |  | Edge_1RB_Left |  |  |
| 2 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |  |  |
| 3 | Default | Default | Default |  |  | QPSK |  | Outer_Full |  |  |
| 4 | Default | 10 MHz | Default |  |  | QPSK |  | 4@0 |  | 2@0 |
| 5 | Default | 10 MHz | Default |  |  | QPSK |  | 4@48 |  | 2@22 |
| 6 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |  |
| 7 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |  |  |
| 8 | Default | Default | Default | N/A | CP-OFDM | 16 QAM |  | Outer_Full |  |  |
| 9 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@0 |  | 2@0 |
| 10 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@48 |  | 2@22 |
| 11 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |  |
| 12 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |  |  |
| 13 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |  |  |
| 14 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@0 |  | 2@0 |
| 15 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@48 |  | 2@22 |
| 16 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |  |
| 17 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |  |  |
| 18 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |  |  |
| 19 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@0 |  | 2@0 |
| 20 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@48 |  | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-10: Test Configuration table for NS_44

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | 2602.5 | 25 | Default |  | CP-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 2 | 2602.5 | 25 | Default |  |  | 64 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 3 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 4 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 5 | 2602.5 | 25 | Default | N/A |  | 256 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 6 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 7 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 8 | 2600 | 30 | Default |  |  | 64 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 9 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 10 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 11 | 2600 | 30 | Default |  |  | 256 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 12 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 13 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 14 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 15 | 2595 | 40 | Default |  |  | 64 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 16 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 17 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |
| 18 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 19 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 20 | 2595 | 40 | Default |  |  | 256 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 21 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 22 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11: Test Configuration table for NS_27

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies |  |  |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters. Associated parameters defined in Table 6.2D.3.4.1-9a and 6.2D.3.4.1-9b. |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Refer to test parameters (15, 20, 40 MHz) |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |
| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS | Downlink | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) | Configuration | Modulation |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  | (Note 2) |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 3557.52 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A3) |  |  |
| 2 | 3557.52 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 3 | 3692.49 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A3) |  |  |
| 4 | 3692.49 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 5 | 3562.5 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A4) |  |  |
| 6 | 3562.5 | 15 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 7 | 3687.48 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A4) |  |  |
| 8 | 3687.48 | 15 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 9 | 3560.01 | 20 | Default |  |  | QPSK |  | Edge_1RB_Left (A5) |  |  |
| 10 | 3560.01 | 20 | Default |  |  | QPSK |  | Outer_Full (A5) |  |  |
| 11 | 3690 | 20 | Default |  |  | QPSK |  | Edge_1RB_Right (A5) |  |  |
| 12 | 3690 | 20 | Default |  |  | QPSK |  | Outer_Full (A5) |  |  |
| 13 | 3570 | 20 | Default |  |  | QPSK |  | Edge_1RB_Left (A6) |  |  |
| 14 | 3570 | 20 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 15 | 3679.98 | 20 | Default |  |  | QPSK |  | Edge_1RB_Right (A6) |  |  |
| 16 | 3679.98 | 20 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 17 | 3570 | 40 | Default | N/A |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 18 | 3570 | 40 | Default |  |  | QPSK |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 19 | 3570 | 40 | Default |  |  | QPSK |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 20 | 3570 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 21 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 22 | 3679.98 | 40 | Default |  |  | QPSK |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 23 | 3679.98 | 40 | Default |  |  | QPSK |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 24 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 25 | 3600 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A8) |  |  |
| 26 | 3600 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 27 | 3649.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A8) |  |  |
| 28 | 3649.98 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 29 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A3) |  |  |
| 30 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 31 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A3) |  |  |
| 32 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 33 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A4) |  |  |
| 34 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 35 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A4) |  |  |
| 36 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 37 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A5) |  |  |
| 38 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 39 | 3690 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A5) |  |  |
| 40 | 3690 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 41 | 3570 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A6) |  |  |
| 42 | 3570 | 20 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 43 | 3679.98 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A6) |  |  |
| 44 | 3679.98 | 20 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 45 | 3570 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A7) |  |  |
| 46 | 3570 | 40 | Default |  |  | 16 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 47 | 3570 | 40 | Default |  |  | 16 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 48 | 3570 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A7) |  |  |
| 49 | 3679.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A7) |  |  |
| 50 | 3679.98 | 40 | Default |  |  | 16 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 51 | 3679.98 | 40 | Default |  |  | 16 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 52 | 3679.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A7) |  |  |
| 53 | 3600 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A8) |  |  |
| 54 | 3600 | 40 | Default |  | CP-OFDM | 16 QAM |  | Outer_Full (4.5) |  |  |
| 55 | 3649.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A8) |  |  |
| 56 | 3649.98 | 40 | Default |  |  | 16 QAM |  | Outer_Full (4.5) |  |  |
| 57 | 3557.52 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A3) |  |  |
| 58 | 3557.52 | 15 | Default |  |  | 64 QAM |  | Outer_Full (A3) |  |  |
| 59 | 3692.49 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A3) |  |  |
| 60 | 3692.49 | 15 | Default |  |  | 64 QAM |  | Outer_Full (A3) |  |  |
| 61 | 3562.5 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A4) |  |  |
| 62 | 3562.5 | 15 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 63 | 3687.48 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A4) |  |  |
| 64 | 3687.48 | 15 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 65 | 3560.01 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A5) |  |  |
| 66 | 3560.01 | 20 | Default |  |  | 64 QAM |  | Outer_Full (A5) |  |  |
| 67 | 3690 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A5) |  |  |
| 68 | 3690 | 20 | Default |  |  | 64 QAM |  | Outer_Full (A5) |  |  |
| 69 | 3570 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A6) |  |  |
| 70 | 3570 | 20 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 71 | 3679.98 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A6) |  |  |
| 72 | 3679.98 | 20 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 73 | 3570 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A7) |  |  |
| 74 | 3570 | 40 | Default |  |  | 64 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 75 | 3570 | 40 | Default |  |  | 64 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 76 | 3570 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A7) |  |  |
| 77 | 3679.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A7) |  |  |
| 78 | 3679.98 | 40 | Default |  |  | 64 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 79 | 3679.98 | 40 | Default |  |  | 64 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 80 | 3679.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A7) |  |  |
| 81 | 3600 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A8) |  |  |
| 82 | 3600 | 40 | Default |  |  | 64 QAM |  | Outer_Full (4.5) |  |  |
| 83 | 3649.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A8) |  |  |
| 84 | 3649.98 | 40 | Default |  |  | 64 QAM |  | Outer_Full (4.5) |  |  |
| 85 | 3557.52 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A3) |  |  |
| 86 | 3557.52 | 15 | Default |  |  | 256 QAM |  | Outer_Full (A3) |  |  |
| 87 | 3692.49 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A3) |  |  |
| 88 | 3692.49 | 15 | Default |  |  | 256 QAM |  | Outer_Full (A3) |  |  |
| 89 | 3562.5 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A4) |  |  |
| 90 | 3562.5 | 15 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 91 | 3687.48 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A4) |  |  |
| 92 | 3687.48 | 15 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 93 | 3560.01 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A5) |  |  |
| 94 | 3560.01 | 20 | Default |  |  | 256 QAM |  | Outer_Full (A5) |  |  |
| 95 | 3690 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A5) |  |  |
| 96 | 3690 | 20 | Default |  |  | 256 QAM |  | Outer_Full (A5) |  |  |
| 97 | 3570 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A6) |  |  |
| 98 | 3570 | 20 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 99 | 3679.98 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A6) |  |  |
| 100 | 3679.98 | 20 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 101 | 3570 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A7) |  |  |
| 102 | 3570 | 40 | Default |  |  | 256 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 103 | 3570 | 40 | Default |  |  | 256 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 104 | 3570 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A7) |  |  |
| 105 | 3679.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A7) |  |  |
| 106 | 3679.98 | 40 | Default |  |  | 256 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 107 | 3679.98 | 40 | Default |  |  | 256 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 108 | 3679.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A7) |  |  |
| 109 | 3600 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A8) |  |  |
| 110 | 3600 | 40 | Default |  |  | 256 QAM |  | Outer_Full (4.5) |  |  |
| 111 | 3649.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A8) |  |  |
| 112 | 3649.98 | 40 | Default |  |  | 256 QAM |  | Outer_Full (4.5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11a: Test frequencies for NS_27 (SCS=15 kHz, ΔFRaster = 15kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 79 | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | 3562.5 | 637500 | 3555.39 | 637026 |  |  | 7888 | 637344 | 6 | 0 | 1 (6) | 6 |
|  |  | 3687.48 | 645832 | 3680.37 | 645358 |  |  | 7975 | 645696 | 2 | 2 | 1 (6) | 8 |
|  |  | 3692.49 | 646166 | 3685.38 | 645692 |  |  | 7978 | 645984 | 4 | 2 | 0 (2) | 4 |
| 20 | 106 | 3560.01 | 637334 | 3550.47 | 636698 |  |  | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | 3570 | 638000 | 3560.46 | 637364 |  |  | 7891 | 637632 | 4 | 0 | 0 (2) | 2 |
|  |  | 3679.98 | 645332 | 3670.44 | 644696 |  |  | 7968 | 645024 | 4 | 1 | 1 (6) | 7 |
|  |  | 3690 | 646000 | 3680.46 | 645364 |  |  | 7975 | 645696 | 8 | 1 | 1 (6) | 7 |
| 40 | 216 | 3570 | 638000 | 3550.56 | 636704 |  |  | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | 3600 | 640000 | 3580.56 | 638704 |  |  | 7905 | 638976 | 8 | 0 | 0 (2) | 2 |
|  |  | 3649.98 | 643332 | 3630.54 | 642036 |  |  | 7940 | 642336 | 0 | 3 | 0 (2) | 5 |
|  |  | 3679.98 | 645332 | 3660.54 | 644036 |  |  | 7961 | 644352 | 4 | 0 | 1 (6) | 6 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11b: Test frequencies for NS_27 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 38 | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | 3562.5 | 637500 | 3555.66 | 637044 |  |  | 7888 | 637344 | 12 | 0 | 2 (2) | 4 |
|  |  | 3687.48 | 645832 | 3680.64 | 645376 |  |  | 7975 | 645696 | 8 | 0 | 3 (3) | 6 |
|  |  | 3692.49 | 646166 | 3685.65 | 645710 |  |  | 7978 | 645984 | 10 | 0 | 1 (1) | 2 |
| 20 | 51 | 3560.01 | 637334 | 3550.83 | 636722 |  |  | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | 3570 | 638000 | 3560.82 | 637388 |  |  | 7891 | 637632 | 4 | 0 | 0 (0) | 0 |
|  |  | 3679.98 | 645332 | 3670.8 | 644720 |  |  | 7968 | 645024 | 16 | 0 | 2 (2) | 4 |
|  |  | 3690 | 646000 | 3680.82 | 645388 |  |  | 7975 | 645696 | 20 | 0 | 2 (2) | 4 |
| 40 | 106 | 3570 | 638000 | 3550.92 | 636728 |  |  | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | 3600 | 640000 | 3580.92 | 638728 |  |  | 7905 | 638976 | 8 | 0 | 0 (0) | 0 |
|  |  | 3649.98 | 643332 | 3630.9 | 642060 |  |  | 7940 | 642336 | 12 | 0 | 1 (1) | 2 |
|  |  | 3679.98 | 645332 | 3660.9 | 644060 |  |  | 7961 | 644352 | 4 | 0 | 2 (2) | 4 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-12: Test Configuration table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Low | 5 | 15 | N/A for A-MPR testing | 14 | CP-OFDM | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 2 | Low | 5 | 30 |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | N/A |
| 3 | Low | 5 | Default |  | 6 |  | QPSK |  | Outer_Full | Outer_Full | N/A |
| 4 | Low | 5 | Default |  | 4 |  | QPSK |  | 21@4 | 8@3 | N/A |
| 5 | Low | 5 | Default |  | 4 |  | QPSK |  | 1@4 | 1@3 | N/A |
| 6 | Low | 5 | 15 |  | 14 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 7 | Low | 5 | 30 |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | N/A |
| 8 | Low | 5 | Default |  | 6 |  | 16 QAM |  | Outer_Full | Outer_Full | N/A |
| 9 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 21@4 | 8@3 | N/A |
| 10 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 1@4 | 1@3 | N/A |
| 11 | Low | 5 | 15 |  | 14 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 12 | Low | 5 | 30 |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | N/A |
| 13 | Low | 5 | Default |  | 6 |  | 64 QAM |  | Outer_Full | Outer_Full | N/A |
| 14 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 21@4 | 8@3 | N/A |
| 15 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 1@4 | 1@3 | N/A |
| 16 | Low | 5 | 15 |  | 14 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 17 | Low | 5 | 30 |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | N/A |
| 18 | Low | 5 | Default |  | 6 |  | 256 QAM |  | Outer_Full | Outer_Full | N/A |
| 19 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 21@4 | 8@3 | N/A |
| 20 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 1@4 | 1@3 | N/A |
| 21 | Low | 10 | 15 |  | 12 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 22 | Low | 10 | Highest |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 23 | Low | 10 | Default |  | 8 |  | QPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 24 | Low | 10 | Default |  | 6 |  | QPSK |  | 1@3 | 1@2 | 1@1 |
| 25 | Low | 10 | Default |  | 6 |  | QPSK |  | 43@9 | 19@5 | 8@2 |
| 26 | Low | 10 | Default |  | 4 |  | QPSK |  | 1@35 | 1@17 | 1@8 |
| 27 | Low | 10 | Default |  | 2 |  | QPSK |  | 17@35 | 5@19 | 3@8 |
| 28 | Low | 10 | Default |  | 5 |  | QPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 29 | Low | 10 | 15 |  | 5 |  | QPSK |  | 1@40 | N/A | N/A |
| 30 | Low | 10 | 30 |  | 5 |  | QPSK |  | N/A | 1@20 | N/A |
| 31 | Low | 10 | 15 |  | 3 |  | QPSK |  | 8@44 | N/A | N/A |
| 32 | Low | 10 | 15 |  | 12 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 33 | Low | 10 | Highest |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 34 | Low | 10 | Default |  | 8 |  | 16 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 35 | Low | 10 | Default |  | 6 |  | 16 QAM |  | 1@3 | 1@2 | 1@1 |
| 36 | Low | 10 | Default |  | 6 |  | 16 QAM |  | 43@9 | 19@5 | 8@2 |
| 37 | Low | 10 | Default |  | 4 |  | 16 QAM |  | 1@35 | 1@17 | 1@8 |
| 38 | Low | 10 | Default |  | 2 |  | 16 QAM |  | 17@35 | 5@19 | 3@8 |
| 39 | Low | 10 | Default |  | 5 |  | 16 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 40 | Low | 10 | 15 |  | 5 |  | 16 QAM |  | 1@40 | N/A | N/A |
| 41 | Low | 10 | 30 |  | 5 |  | 16 QAM |  | N/A | 1@20 | N/A |
| 42 | Low | 10 | 15 |  | 3 |  | 16 QAM |  | 8@44 | N/A | N/A |
| 43 | Low | 10 | 15 |  | 12 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 44 | Low | 10 | Highest |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 45 | Low | 10 | Default |  | 8 |  | 64 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 46 | Low | 10 | Default |  | 6 |  | 64 QAM |  | 1@3 | 1@2 | 1@1 |
| 47 | Low | 10 | Default |  | 6 |  | 64 QAM |  | 43@9 | 19@5 | 8@2 |
| 48 | Low | 10 | Default |  | 4 |  | 64 QAM |  | 1@35 | 1@17 | 1@8 |
| 49 | Low | 10 | Default |  | 2 |  | 64 QAM |  | 17@35 | 5@19 | 3@8 |
| 50 | Low | 10 | Default |  | 5 |  | 64 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 51 | Low | 10 | 15 |  | 5 |  | 64 QAM |  | 1@40 | N/A | N/A |
| 52 | Low | 10 | 30 |  | 5 |  | 64 QAM |  | N/A | 1@20 | N/A |
| 53 | Low | 10 | 15 |  | 3 |  | 64 QAM |  | 8@44 | N/A | N/A |
| 54 | Low | 10 | 15 |  | 12 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 55 | Low | 10 | Highest |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 56 | Low | 10 | Default |  | 8 |  | 256 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 57 | Low | 10 | Default |  | 6 |  | 256 QAM |  | 1@3 | 1@2 | 1@1 |
| 58 | Low | 10 | Default |  | 6 |  | 256 QAM |  | 43@9 | 19@5 | 8@2 |
| 59 | Low | 10 | Default |  | 4 |  | 256 QAM |  | 1@35 | 1@17 | 1@8 |
| 60 | Low | 10 | Default |  | 2 |  | 256 QAM |  | 17@35 | 5@19 | 3@8 |
| 61 | Low | 10 | Default |  | 5 |  | 256 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 62 | Low | 10 | 15 |  | 5 |  | 256 QAM |  | 1@40 | N/A | N/A |
| 63 | Low | 10 | 30 |  | 5 |  | 256 QAM |  | N/A | 1@20 | N/A |
| 64 | Low | 10 | 15 |  | 3 |  | 256 QAM |  | 8@44 | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-13: Test Configuration table for NS_50

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column for test frequency, and configuration detail specified in Table 6.2.3.4.1-32a, Table 6.2.3.4.1-32a and Table 6.2.3.4.1-32c) |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1897.5 | 25 | Default | N/A | CP-OFDM | QPSK | Outer_Full (A7) |  |  |  |  |
| 2 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 3 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 4 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |  |  |
| 5 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 6 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 7 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 8 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 9 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A2) |  | 93@13 (A2) |  | 44@7 (A2) |
| 10 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A6) |  | 55@51 (A6) |  | 25@26 (A6) |
| 11 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A5) |  | 17@89 (A5) |  | 6@45 (A5) |
| 12 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 13 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 14 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 15 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 16 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 17 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 18 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 19 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 20 | Low | 40 | Default |  |  | 256 QAM | 191@25 (A2) |  | 93@13 (A2) |  | 44@7 (A2) |
| 21 | Low | 40 | Default |  |  | 256 QAM | 115@101 (A6) |  | 55@51 (A6) |  | 25@26 (A6) |
| 22 | Low | 40 | Default |  |  | 256 QAM | 39@177 (A5) |  | 17@89 (A5) |  | 6@45 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable Table in clause 6.2D.3.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3.4.3.

6.2D.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to applicable table in clause 6.2D.3.4.1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to relevant configuration tables. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

6.2D.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

6.2D.3.4.3.1	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.2D.3.4.3.2	Message contents exceptions for network signalling value "NS_35"

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

6.2D.3.4.3.3	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3.4.3.4	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3.4.3.5	Message contents exceptions for network signalling value "NS_48"

1.	Information element additionalSpectrumEmission is set to NS_48. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_48"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_48) |  |  |

6.2D.3.4.3.6	Message contents exceptions for network signalling value "NS_49"

1.	Information element additionalSpectrumEmission is set to NS_49. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_49"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_49) |  |  |

6.2D.3.4.3.7	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2D.3.4.3.8	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 6.2D.3.4.3.8-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

6.2D.3.4.3.9	Message contents exceptions for network signalling value "NS_03U"

1.	Information element additionalSpectrumEmission is set to NS_03U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.9-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03U) |  | for band n2, n25, n66 |

6.2D.3.4.3.10	Message contents exceptions for network signalling value "NS_46"

1.	Information element additionalSpectrumEmission is set to NS_46. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.10-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_46"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_46) |  | for band n7 |

6.2D.3.4.3.11	Message contents exceptions for network signalling value "NS_21"

1.	Information element additionalSpectrumEmission is set to NS_21. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.11-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

6.2D.3.4.3.12	Message contents exceptions for network signalling value "NS_44"

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.12-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_44"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_44) |  |  |

6.2D.3.4.3.13	Message contents exceptions for network signalling value "NS_27"

1.	Information element additionalSpectrumEmission is set to NS_27. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.13-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

6.2D.3.4.3.14	Message contents exceptions for network signalling value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.14-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2D.3.4.3.15	Message contents exceptions for network signalling value "NS_50"

1.	Information element additionalSpectrumEmission is set to NS_50. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.15-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_50"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_50) |  |  |

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 2 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 3 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 4 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 6 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 7 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 8 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 9 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 10 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 14 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 15 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 16 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 17 | 26 | 4.5 | 0 | 1.52 | 20 | 6 | 3 | 28+TT | 14.0-TT |
| 18 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 19 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 20 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 21 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 22 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 23 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 24 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-1a: UE Power Class 2 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 10 | 26 | 1 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 11 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 12 | 26 | 1 | 0 | 0 | 25 | 3 | 3 | 28+TT | 22.-TT |
| 13 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 16 | 26 | 2 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 17 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 18 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.-TT |
| 19 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 22 | 26 | 2.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 23 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 24 | 26 | 2.5 | 0 | 0 | 23.5 | 3 | 3 | 28+TT | 20.5-TT |
| 25 | 26 | 3.5 | 6.5 | 1.52 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 28 | 26 | 3 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 29 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 30 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.-TT |
| 31 | 26 | 5.5 | 8 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 33 | 26 | 5.5 | 0 | 0 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 34 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 35 | 26 | 5.5 | 0 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 36 | 26 | 5.5 | 0 | 0 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 37 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 38 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 39 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 40 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 41 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 42 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 43 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 44 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 45 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 46 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 47 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 48 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 49 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 50 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 51 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 52 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 53 | 26 | 4.5 | 0 | 1.52 | 20 | 6 | 3 | 28+TT | 14.-TT |
| 54 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 55 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 56 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 57 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 58 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 59 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 60 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 2 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 4 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 7 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 8 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 13 | 23 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 14 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 16 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.0-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 20 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.0-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2a: UE Power Class 3 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 5.5 | 1.52 | 19 | 5 | 3 | 28+TT | 14-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 3 | 3 | 28+TT | 21.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.52 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 17 | 23 | 0 | 1 | 0 | 1.52 | 20.5 | 6 | 3 | 25+TT | 14.5-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 19 | 23 | 0 | 2 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 23 | 23 | 0 | 2 | 0 | 1.52 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 6 | 3 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.52 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 6 | 3 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.52 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 2 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 4 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 5 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 7 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 8 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 9 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 10 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 11 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 12 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-4: UE Power Class 3 test requirements (NS_05) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-5: UE Power Class 3 test requirements (NS_05U) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-6: UE Power Class 3 test requirements (NS_48) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 4 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 9 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 11 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 16 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 17 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 18 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 21 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 27 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 29 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 30 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-7: UE Power Class 3 test requirements (NS_49) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 4 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 5 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 9 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 11 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 16 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 17 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 18 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 20 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 30 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 31 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 32 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 35 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 36 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 37 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 38 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 39 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 40 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 41 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 42 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9a: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_100 | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_100 | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_100 | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_100 | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_100 | 23 | 2 | 2.5 | 1.5 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | NS_100 | 23 | 2 | 2.5 | 0 | 20.5 | 6 | 3 | 25+TT | 14.5-TT |
| 10, 11 | NS_100 | 23 | 2.5 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | NS_100 | 23 | 2.5 | 3 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 13, 14 | NS_100 | 23 | 4.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | NS_100 | 23 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 16, 17 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_100 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 24 | NS_100 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 25, 26 | NS_100 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 27 | NS_100 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10a: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 6 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 12 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 13, 14 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16, 17 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 18 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 21 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 24 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 25, 26 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 27 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | rr | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11a: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 1.5 | 20 | 6 | 3 | 25+TT | 14-TT |
|  | NS_03U | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_03; NS_03U | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_03; NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_03; NS_03U | 23 | 2 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03; NS_03U | 23 | 2 | 3 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 10, 11 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 1.5 | 18 | 5 | 3 | 25+TT | 13-TT |
| 12 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 13, 14 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 1.5 | 16 | 5 | 3 | 25+TT | 11-TT |
| 15 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16, 17 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 25, 26 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 27 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-12: UE Power Class 3 test requirements (NS_46) for band n7

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 3 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 4 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 10 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 11 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 12 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-13: UE Power Class 3 test requirements (NS_21) for band n30

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4,5 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 6,7 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9,10 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 11,12 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14,15 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 16,17 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 18 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19,20 | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-14: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 2 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 3 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 5 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 7 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 8 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 11 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 14 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 16 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 17 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 0 | 6.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 22 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-15: UE Power Class 3 test requirements (NS_27) for band n48

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 7 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 8 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 9 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 12 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 14 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 17 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 18 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 19 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 21 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 22 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 23 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 25 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 27 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 28 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 29 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 32 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 33 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 37 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 39 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 40 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 43 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 44 | 23 | 0 | 3 | 2 | 0 | 20 | 6 | 3 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 49 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 52 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 53 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 56 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 57 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 58 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 59 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 60 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 61 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 62 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 63 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 64 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 65 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 66 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 67 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 68 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 71 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 72 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 73 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 74 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 75 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 76 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 77 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 78 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 79 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 80 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 81 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 82 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 83 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 85 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 86 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 87 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 88 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 89 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 90 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 91 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 92 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 93 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 94 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 95 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 96 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 97 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 98 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 102 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 103 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 104 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 105 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 106 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 107 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 108 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 109 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 110 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

Table 6.2D.3.5-16: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 2 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 3 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 4 | 23 | 3 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 5 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 6 | 23 | 3 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 7 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 8 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 9 | 23 | 3 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15 -TT |
| 10 | 23 | 2 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 11 | 23 | 3.5 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 12 | 23 | 3.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 13 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 14 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 15 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 16 | 23 | 6.5 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 17 | 23 | 6.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 18 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 19 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 20 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 21 | 23 | 3 | 12 | 0 | 9 | 6 | 4 | 25+TT | 5-TT |
| 22 | 23 | 3 | 8 | 0 | 11 | 5 | 4 | 25+TT | 10-TT |
| 23 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 24 | 23 | 1.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 25 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 26 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 27 | 23 | 3 | 2 | 0 | 20 | 2.5 | 4 | 25+TT | 16-TT |
| 28 | 23 | 3 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 29 | 23 | 1.5 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 30 | 23 | 1.5 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 31 | 23 | 3 | 3 | 0 | 20 | 2.5 | 4 | 25+TT | 16-TT |
| 32 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 33 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 34 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 35 | 23 | 2 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 36 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 37 | 23 | 2 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 38 | 23 | 3 | 2 | 0 | 20 | 2.5 | 4 | 25+TT | 16-TT |
| 39 | 23 | 3 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 40 | 23 | 2 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 41 | 23 | 2 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 42 | 23 | 3 | 3 | 0 | 20 | 2.5 | 4 | 25+TT | 16-TT |
| 43 | 23 | 3.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 44 | 23 | 3.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 45 | 23 | 3.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 46 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 47 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 48 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 49 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 4 | 25+TT | 15.5-TT |
| 50 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 4 | 25+TT | 14-TT |
| 51 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 4 | 25+TT | 14-TT |
| 52 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 4 | 25+TT | 14-TT |
| 53 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 4 | 25+TT | 15.5-TT |
| 54 | 23 | 6.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 55 | 23 | 6.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 56 | 23 | 6.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 57 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 58 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 59 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 60 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 61 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 62 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 63 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 64 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-17: UE Power Class 3 test requirements (NS_50) for band n39

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 3 | 25.0 + TT | 14.0 - TT |  |
| 2 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 3 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 3 | 25.0 + TT | 17 - TT |  |
| 4 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 3 | 25.0 + TT | 17 - TT |  |
| 5 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 6 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 3 | 25.0 + TT | 17 - TT |  |
| 7 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 3 | 25.0 + TT | 17 – TT |  |
| 8 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 9 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 4.0 |  | 3 | 25.0 + TT | 14.5 - TT |  |
| 10 | 23 | 0 | 3 | 3.5 | 0 |  | 19.5 |  | 3.5 |  | 3 | 25.0 + TT | 16 - TT |  |
| 11 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 3 | 25.0 + TT | 14.0 - TT |  |
| 12 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 13 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 14 | 23 | 0 | 6.5 | 6 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 15 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 16 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 17 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT4 |  |
| 18 | 23 | 0 | 6.5 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 19 | 23 | 0 | 6.5 | 4..5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 20 | 23 | 0 | 6.5 | 8 | 0 |  | 15 |  | 5.0 |  | 3 | 25.0 + TT | 10.0 - TT |  |
| 21 | 23 | 0 | 6.5 | 3.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 22 | 23 | 0 | 6.5 | 5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2D.3_1	UE additional maximum output power reduction for SUL with UL MIMO

6.2D.3_1.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1_1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3_1.2	Test applicability

The requirements of this test apply in test case 6.5D.2.4.2_1 UTRA Adjacent channel leakage ratio for network signalling values NS_05U to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

The requirements of this test apply in test case 6.5D.3_2.3 Additional Spurious Emissions for network signalling value NS_05, NS_05U and NS_56 to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

NOTE:	Test execution is not necessary if 6.5D.2.4.2_1 and 6.5D.3_2.3 are executed.

6.2D.3_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1_1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1_1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4_1.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.1.

6.2D.3_1.4	Test description

6.2D.3_1.4.1	Test procedure

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in Table 6.2D.3_1.4.1-1 to Table 6.2D.3_1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.3_1.4.1-1: Test Configuration Table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for NUL carrier |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 37 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 38 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 39 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 40 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 41 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 42 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 43 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 45 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 46 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 48 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 49 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 50 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-2: Test Configuration Table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: Use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |  |  |
| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 44 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 45 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 49 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 50 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 52 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 53 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 54 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 55 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| 56 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-3: Test Configuration Table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carriers |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK | Edge_1RB_Left |
| 2 | Low | 5 |  |  | 6 |  | QPSK | Outer_Full |
| 3 | Low | 5 |  |  | 4 |  | QPSK | 21@4 |
| 4 | Low | 5 |  |  | 4 |  | QPSK | 1@4 |
| 5 | Low | 5 |  |  | 14 |  | 16 QAM | Edge_1RB_Left |
| 6 | Low | 5 |  |  | 6 |  | 16 QAM | Outer_Full |
| 7 | Low | 5 |  |  | 4 |  | 16 QAM | 21@4 |
| 8 | Low | 5 |  |  | 4 |  | 16 QAM | 1@4 |
| 9 | Low | 5 |  |  | 14 |  | 64 QAM | Edge_1RB_Left |
| 10 | Low | 5 |  |  | 6 |  | 64 QAM | Outer_Full |
| 11 | Low | 5 |  |  | 4 |  | 64 QAM | 21@4 |
| 12 | Low | 5 |  |  | 4 |  | 64 QAM | 1@4 |
| 13 | Low | 5 |  |  | 14 |  | 256 QAM | Edge_1RB_Left |
| 14 | Low | 5 |  |  | 6 |  | 256 QAM | Outer_Full |
| 15 | Low | 5 |  |  | 4 |  | 256 QAM | 21@4 |
| 16 | Low | 5 |  |  | 4 |  | 256 QAM | 1@4 |
| 17 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 18 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 19 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |
| 20 | Low | 10 |  |  | 6 |  | QPSK | 43@9 |
| 21 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 22 | Low | 10 |  |  | 2 |  | QPSK | 17@35 |
| 23 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 24 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 25 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 26 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 27 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 28 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 29 | Low | 10 |  |  | 6 |  | 16 QAM | 43@9 |
| 30 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 31 | Low | 10 |  |  | 2 |  | 16 QAM | 17@35 |
| 32 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 33 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 34 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 35 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left |
| 36 | Low | 10 |  |  | 8 |  | 64 QAM | Outer_Full |
| 37 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 |
| 38 | Low | 10 |  |  | 6 |  | 64 QAM | 43@9 |
| 39 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 40 | Low | 10 |  |  | 2 |  | 64 QAM | 17@35 |
| 41 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 42 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 43 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 44 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |
| 45 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 46 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 47 | Low | 10 |  |  | 6 |  | 256 QAM | 43@9 |
| 48 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 49 | Low | 10 |  |  | 2 |  | 256 QAM | 17@35 |
| 50 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 51 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 52 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to the applicable tables 6.2D.3_1.4.1-1 to  6.2D.3_1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3_1.4.3.

6.2D.3_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.3_1.4.1-1 to Table 6.2D.3_1.4.1-3. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

6.2D.3_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. All the AdditionalSpectrumEmission in 6.2.3.4.3 are sent in SIB1 as part of supplementaryUplink instead of uplinkConfigCommon.

6.2D.3_1.4.3.1	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3_1.4.3.2	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3_1.4.3.3	Message contents exceptions for network signalling value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2D.3_1.5	Test requirement

The measured maximum output power shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable tables from Table 6.2D.3_1.5-1 to Table 6.2D.3_1.5-3. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2D.1_1.3-1 apply.

Table 6.2D.3_1.5-1: UE Power Class 3 test requirements (NS_05) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-2: UE Power Class 3 test requirements (NS_05U) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-3: UE Power Class 3 test requirements (NS_56) for band n99

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 2 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 3 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 4 | 23 | 3 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 5 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 6 | 23 | 3 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 7 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 8 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 9 | 23 | 3 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 10 | 23 | 2 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 11 | 23 | 3.5 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 12 | 23 | 3.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 13 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 14 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 15 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 16 | 23 | 6.5 | 14 | 0 | 9 | 6 | 4 | 25+TT | 3-TT |
| 17 | 23 | 6.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 18 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 19 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 20 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 21 | 23 | 3 | 12 | 0 | 9 | 6 | 4 | 25+TT | 5-TT |
| 22 | 23 | 3 | 8 | 0 | 11 | 5 | 4 | 25+TT | 10-TT |
| 23 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 24 | 23 | 1.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 25 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 26 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 27 | 23 | 3 | 2 | 0 | 20 | 2.5 | 4 | 25+TT | 15.5-TT |
| 28 | 23 | 3 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 29 | 23 | 1.5 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 30 | 23 | 1.5 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 31 | 23 | 3 | 3 | 0 | 20 | 2.5 | 4 | 25+TT | 15.5-TT |
| 32 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 33 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 34 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 35 | 23 | 2 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 36 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 37 | 23 | 2 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 38 | 23 | 3 | 2 | 0 | 20 | 2.5 | 4 | 25+TT | 15.5-TT |
| 39 | 23 | 3 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 23 | 2 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 41 | 23 | 2 | 5 | 0 | 18 | 4 | 4 | 25+TT | 14-TT |
| 42 | 23 | 3 | 3 | 0 | 20 | 2.5 | 4 | 25+TT | 15.5-TT |
| 43 | 23 | 3.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 44 | 23 | 3.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 45 | 23 | 3.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 46 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 47 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 48 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 4 | 25+TT | 15-TT |
| 49 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 4 | 25+TT | 15.5-TT |
| 50 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 4 | 25+TT | 14-TT |
| 51 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 4 | 25+TT | 14-TT |
| 52 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 4 | 25+TT | 14-TT |
| 53 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 4 | 25+TT | 15.5-TT |
| 54 | 23 | 6.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 55 | 23 | 6.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 56 | 23 | 6.5 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 57 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 58 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 59 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 60 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 61 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 62 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 63 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 64 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n99, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.4	Configured transmitted power for UL MIMO

6.2D.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c for UL MIMO is within the specified bounds.

6.2D.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.4.3	Minimum conformance requirements

For UE supporting UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.4 shall apply to UE supporting UL MIMO, where

PPowerClass, ΔPPowerClass and TC,c are specified in subclause 6.2D.1.3;

MPRc is specified in subclause 6.2D.2.3;

A-MPRc is specified in subclause 6.2D.3.3.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1.3-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2D.4.3-1. The requirements shall be met with UL MIMO configurations specified in Table 6.2D.1.3-2.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the tolerance is specified in Table 6.2D.4.3-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.4.3-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2D.1.3 apply for the power class as indicated by the ue-PowerClass field in capability signalling.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.4.

6.2D.4.4	Test description

6.2D.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Table 6.2D.4.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.4.4.3.

6.2D.4.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.4.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.2D.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO and following exception.

Table 6.2D.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 0 |  |  |

Table 6.2D.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 14 |  |  |

Table 6.2D.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2D.4.4.3-4: ServingCellConfig: ULFPTx

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | ULFPTx Test ID 1, 2 |
|  | 1 |  | ULFPTx Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2D.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2D.4.5-1.

Table 6.2D.4.5-1: PCMAX configured UE output power

| Configured transmitted power |  |
| --- | --- |
| Measured UE output power test point 1 | 0 dBm ± (7+TT) |
| Measured UE output power test point 2 | 14 dBm ± (6+TT) |
| Measured UE output power test point 3 | 20 dBm + (4+TT) / - (6+TT) |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2D.4.5-2. Note 2: In addition note 2 in Table 6.2D.1.3-1 shall apply to the tolerances. |  |

Table 6.2D.4.5-2: Test Tolerance (Configured transmitted power for UL MIMO)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2D.4_1	Configured transmitted power for SUL with UL MIMO

6.2D.4_1.1	Test purpose

Same test purpose as in clause 6.2D.4.1.

6.2D.4_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

This test applies to all types of NR UE release 17 and forward that support SUL and UL full power transmission (ULFPTx) on the SUL bands.

6.2D.4_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2D.4.3.

6.2D.4_1.4	Test description

6.2D.4_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.4_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.4_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |

Table 6.2D.4_1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2 and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.4_1.4.1-1 or 6.2D.4_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.4_1.4.3.

6.2D.4_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.4_1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols.

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.4_1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.2D.4_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO and following exception.

Table 6.2D.4_1.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 0 |  |  |

Table 6.2D.4_1.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 14 |  |  |

Table 6.2D.4_1.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  |  |

6.2D.4_1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2D.4_1.5-1.

Table 6.2D.4_1.5-1: PCMAX configured UE output power

| Configured transmitted power |  |
| --- | --- |
| Measured UE output power test point 1 | 0 dBm ± (7+TT) |
| Measured UE output power test point 2 | 14 dBm ± (6+TT) |
| Measured UE output power test point 3 | 18 dBm ± (5+TT) |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2D.4_1.5-2. Note 2: In addition note 2 in Table 6.2D.1_1.3-1 shall apply to the tolerances. |  |

Table 6.2D.4_1.5-2: Test Tolerance (Configured transmitted power for UL MIMO)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

<< Next changes >>

6.2H	Transmitter power for CA with UL MIMO

6.2H.1	Transmitter power for intra-band UL contiguous CA with UL MIMO

6.2H.1.1	UE maximum output power for intra-band UL contiguous CA with UL MIMO

6.2H.1.1.1	Test purpose

To verify that the error of the UE maximum output power for intra-band UL contiguous carrier aggregation with UL MIMO does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2H.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support intra-band UL contiguous CA with UL MIMO.

NOTE: Testing can’t be performed due to lack of appropriate test points.

6.2H.1.1.3	Minimum conformance requirements

For intra-band UL contiguous CA and UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors and all UL CCs. The period of measurement shall be at least one sub frame (1 ms), as specified in Table 6.2H.1.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2 and 6.2D.1.3-3 for 2 layer configuration and ULFPTx configuration respectively.

Table 6.2H.1.1.3-1: UE Power Class for intra-band UL contiguous CA with UL MIMO in closed loop spatial multiplexing scheme

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41C |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.2A apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.1.1.

6.2H.1.1.4	Test procedure

No test case details are specified.

6.2H.1.2	UE maximum output power reduction for intra-band UL contiguous CA with UL MIMO

Editor’s Note:

- Test tolerance for BW>100M is FFS.

6.2H.1.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified maximum output power with MPR and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2H.1.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support intra-band UL contiguous CA with UL MIMO.

NOTE:	Test execution is not necessary if TC 6.5H.1.2.3 is executed

6.2H.1.2.3	Minimum conformance requirements

For intra-band UL contiguous CA and UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2H.1.1.3-1 is specified in Table 6.2A.2.0.4-1, Table 6.2A.2.0.4-2 for power class 3 CA; Table 6.2A.2.0.4-1b, Table 6.2A.2.0.4-4 for power class 2 CA.

The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1.3-2 and 6.2D.1.3-3 for 2 layer configuration and ULFPTx configuration respectively. For the UE maximum output power modified by MPR, the power limits specified in clause 6.2H.1.4 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.2A.2 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.1.2.

6.2H.1.2.4	Test description

6.2H.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2H.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2H.1.2.4.1-1: Test Configuration Table (contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |
|  | for PCC & SCC | Modulations for all CCs |  |  | RB allocation (NOTE 2) |
| 1 |  |  |  | QPSK | Inner Full |
| 2 |  |  |  | QPSK | Outer Full |
| 3 |  |  |  | 16QAM | Inner Full |
| 4 |  | CP-OFDM |  | 16QAM | Outer Full |
| 5 |  |  |  | 64QAM | Inner Full |
| 6 |  |  |  | 64QAM | Outer Full |
| 7 |  |  |  | 256QAM | Inner Full |
| 8 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.6 for TE diagram and section A.3.2.3.11 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2H.1.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2H.1.2.4.3.

6.2H.1.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2H.1.2.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.2H.1.2.4.1-1 on both PCC and SCC as appropriate. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power from both antenna connectors and all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test.

6.2H.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO with the following exceptions:

Table 6.2H.1.2.4.3-1: FrequencyInfoUL-SIB for Power Class 3 (contiguous RB allocation) for CA_n41C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Test IDs 1, 3 |
|  | 14 |  | Test ID 5 |
|  | 11 |  | Test ID 7 |
|  | 10 |  | Test IDs 2, 4, 6, 8 |

Table 6.2H.1.2.4.3-2: FrequencyInfoUL-SIB for Power Class 2 (contiguous RB allocation) for CA_n41C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 19 |  | Test IDs 1, 3 |
|  | 17 |  | Test ID 5 |
|  | 14 |  | Test ID 7 |
|  | 12 |  | Test IDs 2, 4, 6, 8 |

6.2H.1.2.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2H.1.2.5-1 – Table 6.2H.1.2.5-2.

Table 6.2H.1.2.5-1: UE Output Power for Power Class 3 (contiguous RB allocation) test requirements (for CA_n41C)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 2 | 23 | 8.0 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 3 | 23 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 4 | 23 | 8.0 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 5 | 23 | 5.0 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 6 | 23 | 8.0 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 7 | 23 | 7.0 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 8 | 23 | 8.0 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2H.1.2.5-3. |  |  |  |  |  |  |  |  |

Table 6.2H.1.2.5-2: UE Output Power for Power Class 2 (contiguous RB allocation) test requirements (for CA_n41C)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 4.0 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 2 | 26 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 3 | 26 | 4.0 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 4 | 26 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 5 | 26 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 6 | 26 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 7 | 26 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 8 | 26 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2H.1.2.5-3. |  |  |  |  |  |  |  |  |

Table 6.2H.1.2.5-3: Test Tolerance

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2H.1.3	UE additional maximum output power reduction for intra-band UL contiguous CA with UL MIMO

Editor’s Note:

- Test tolerance for BW>100M is FFS.

6.2H.1.3.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified maximum output power with AMPR and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2H.1.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support intra-band UL contiguous CA with UL MIMO.

NOTE:	Test execution is not necessary if TC 6.5H.1.2.3 is executed

6.2H.1.3.3	Minimum conformance requirements

For intra-band UL contiguous CA and UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in clause 6.2A.3.0 shall apply to the maximum output power specified in Table 6.2H.1.1.3-1. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1.3-2 and 6.2D.1.3-3 for 2 layer configuration and ULFPTx configuration respectively.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2H.1.4.3 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.2A.3.0.1 apply for the power class as indicated by the ue-PowerClass field in capability signalling.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.1.3.

6.2H.1.3.4	Test description

6.2H.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2H.1.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2H.1.3.4.1-1: Test Configuration Table for intra-band contiguous CA for CA_NS_04 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Referring to Test Frequency Column |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Test Frequency | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation for PCC&SCC (NOTE 1) |
| 1 | High range |  | CP-OFDM QPSK | Outer_Full |
| 2 | High range |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Low range |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low range |  | CP-OFDM QPSK | Edge_1RB_Left |
| 5 | Low range |  | CP-OFDM QPSK | Inner_Full |
| 6 | Low range |  | CP-OFDM 16QAM | Inner_Full |
| 7 | Low range |  | CP-OFDM 64QAM | Inner_Full |
| 8 | Low range |  | CP-OFDM 256QAM | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.6 for TE diagram and section A.3.2.3.11 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2H.1.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2H.1.3.4.3.

6.2H.1.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2H.1.3.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.2H.1.3.4.1-1 on both PCC and SCC as appropriate. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power from both antenna connectors and all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test.

6.2H.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO with the following exceptions:

6.2H.1.3.4.3.1	Message contents exceptions (network signalling value "CA_NS_04" on PCC and SCC)

Table 6.2H.1.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_04" on PCC and SCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_04) |  |  |

Table 6.2H.1.3.4.3.1-2: FrequencyInfoUL-SIB for Power Class 3 (contiguous RB allocation) for CA_n41C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 12 |  | Test IDs 5, 6, 7 |
|  | 10 |  | Test IDs 1, 2, 3, 4, 8 |

Table 6.2H.1.3.4.3.1-3: FrequencyInfoUL-SIB for Power Class 2 (contiguous RB allocation) for CA_n41C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 14 |  | Test IDs 5, 6, 7 |
|  | 12 |  | Test IDs 1, 2, 3, 4 |
|  | 11 |  | Test ID 8 |

6.2H.1.3.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2H.1.3.5-1 – Table 6.2H.1.3.5-2.

Table 6.2H.1.3.5-0: Test Tolerance

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

Table 6.2H.1.3.5-1: UE Power Class 3 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 2 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 3 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 4 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 5 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 6 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 7 | 23 | 5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 8 | 23 | 7 | 8 | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |

Table 6.2H.1.3.5-2: UE Power Class 2 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 2 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 3 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 4 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 5 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 6 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 7 | 26 | 5.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 8 | 26 | 7.5 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2H.1.4	Configured transmitted power for intra-band UL contiguous CA with UL MIMO

Editor’s Note:

- Test tolerance for BW>100M is FFS.

6.2H.1.4.1	Test purpose

To verify that the total measured UE configured maximum output power for intra-band UL contiguous CA with UL MIMO is within the specified bounds.

6.2H.1.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support intra-band UL contiguous CA with UL MIMO.

6.2H.1.4.3	Minimum conformance requirements

For UE supporting intra-band UL contiguous CA with UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in clause 6.2A.4 shall apply to UE supporting intra-band UL contiguous CA with UL MIMO, where

-	ΔPPowerClass and ∆TC,c are specified in clause 6.2A.4 unless otherwise stated;

-	PPowerClass,CA is the maximum UE power specified in Table 6.2H.1.1.3-1 without taking into account the tolerance;

-	MPR, AMPR is specified in clause 6.2H.1.2 and 6.2H.1.3;

The measured configured maximum output power PUMAX over all serving cells shall be within the following bounds:

PCMAX_L  –  MAX{TL, T LOW(PCMAX_L)}  ≤  PUMAX  ≤  PCMAX_H  +  T HIGH(PCMAX_H)

where TLOW(PCMAX_L) and THIGH(PCMAX_H) are defined as the tolerance and applies to PCMAX_L and PCMAX_H separately, while TL is the absolute value of the lower tolerance in Table 6.2 H.1.1.3-1 for the applicable operating band.

For UE supporting intra-band UL contiguous CA with UL MIMO, the tolerance is specified in Table 6.2H.1.4.3-1.

Table 6.2H.1.4.3-1: PCMAX tolerance for intra-band UL contiguous CA with UL MIMO

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 < PCMAX ≤ 26 | 3.0 | 2.0 |
| 21 ≤ PCMAX ≤ 23 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.1.4.

6.2H.1.4.4	Test description

6.2H.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2H.1.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2H.1.4.4.1-1: Test Configuration Table (contiguous RB allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |
|  |  | Modulations for all CCs |  | RB allocation (NOTE 2) |
| 1 | N/A | CP-OFDM QPSK |  | Inner Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.6 for TE diagram and section A.3.2.3.11 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2H.1.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2H.1.4.4.3.

6.2H.1.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2H.1.4.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 6.2H.1.4.4.1-1 on both PCC and SCC as appropriate. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power from both antenna connectors and all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test.

6.2H.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

Table 6.2H.1.4.4.3-1: FrequencyInfoUL: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2H.1.4.4.3-1a: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2H.1.4.4.3-2: FrequencyInfoUL: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 7 |  |  |

Table 6.2H.1.4.4.3-2a: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 7 |  |  |

Table 6.2H.1.4.4.3-3: FrequencyInfoUL: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 17 |  | Power Class 3 |

Table 6.2H.1.4.4.3-3a: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 17 |  | Power Class 3 |

Table 6.2H.1.4.4.3-4: FrequencyInfoUL: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-61 FrequencyInfoUL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power Class 2 |

Table 6.2H.1.4.4.3-4a: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power Class 2 |

6.2H.1.4.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2H.1.4.5-1.

Table 6.2H.1.4.5-1: Configured UE Output Power for Intra-band contiguous CA of PC2 and PC3 (contiguous RB allocation))

|  | Maximum output power |  |
| --- | --- | --- |
|  | Lower limit | Upper limit |
| Measured UE output power test point 1 | -14 dBm - TT | 0 dBm + TT |
| Measured UE output power test point 2 | 4 dBm - TT | 16 dBm + TT |
| Measured UE output power test point 3 (Note 2) | 17 dBm - TT | 22.5 dBm + TT |
| Measured UE output power test point 4 (Note 3) | 15.5 dBm - TT | 22.5 dBm + TT |
| Note 1: TT for each intra-band UL CA configuration is in table 6.2H.4.1.5-2. Note 2: Only applicable to Power Class 3. Note 3: Only applicable to Power Class 2. |  |  |

Table 6.2H.1.4.5-2: Test Tolerance for intra-band CA (Configured transmitted power for CA)

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2H.2

6.2H.3	Transmitter power for inter-band UL CA with UL MIMO

6.2H.3.1	UE maximum output power for inter-band UL CA with UL MIMO

6.2H.3.1.1	Test purpose

To verify that the error of the UE maximum output power for inter-band UL CA with UL MIMO does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2H.3.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.2H.3.1.3	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2H.3.1.3-1. The period of measurement shall be at least one sub frame (1 ms). The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2 and 6.2D.1.3-3 for 2-layer configuration and ULFPTx configuration respectively for the component carrier configured with UL MIMO.

Table 6.2H.3.1.3-1: UE Power Class for inter-band UL CA with UL MIMO in one frequency band

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n5A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n8A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n25A-n77A | 293 | +2/-3 |  |  | 23 | +2/-3 |
| CA_n28A-n41A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n28A-n78A |  |  | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n71A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. NOTE 5: Power class 3 is default power class unless otherwise stated. NOTE 6: FWA form factor is targeted unless otherwise stated. |  |  |  |  |  |  |

If a UE supports power class 2 for the band combination listed in Table 6.2H.3.1.3-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else;

–	shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

If a UE supports power class 1.5 for the band combination listed in Table 6.2H.3.1.3-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and

–	if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower;

–	shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2H.3.4;

–	if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is between 23dBm and 26dBm;

–	shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else;

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability 0.5maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one band within the band combination is power class 1.5

–	if the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

–	else if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported;

–	the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability;

–	else

–	the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1;

–	if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.3.1.

6.2H.3.1.4	Test description

6.2H.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2H.3.1.4.1-1 to Table 6.2H.3.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2H.3.1.4.1-1: Inter-band CA Test Configuration Table for 2-layer MIMO (Power Class 2 UEs supporting PC3 in FDD band and PC3 with UL MIMO in TDD band)

NOTE:	No test points are defined since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2H.3.1.4.1-2: Inter-band CA Test Configuration Table for 2-layer MIMO (Power Class 3, Power Class 2 UEs supporting PC2 with UL MIMO in TDD band)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |  |
|  |  | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |  |
|  |  | PCC | SCC | PCC | SCC |
| 1 | N/A for this test | CP-OFDM QPSK | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | CP-OFDM QPSK | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left | Inner 1RB Left |
| 3 |  | CP-OFDM QPSK | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right | Inner 1RB Right |
| 4 |  | CP-OFDM QPSK | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | CP-OFDM QPSK | DFT-s-OFDM QPSK | Inner 1RB Left | Inner 1RB Left |
| 6 |  | CP-OFDM QPSK | DFT-s-OFDM QPSK | Inner 1RB Right | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: PCC is the component carrier configured with UL MIMO in TDD band. |  |  |  |  |  |

Table 6.2H.3.1.4.1-3: Inter-band CA Test Configuration Table for ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  | (NOTE 2) | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left | Inner 1RB Left |
| 3 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: Only Test IDs 1, 2 and 3 apply to Power Class 2 UEs if TxD is indicated. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.7 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2H.3.1.4.1-1 to Table 6.2H.3.1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2H.3.1.4.3.

6.2H.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2H.3.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

Sub-test for 2-Layer codebook based UL-MIMO

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2H.3.1.4.1-2 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 for PCell is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for SCell.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power from all UE antenna connectors and all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

Sub-test for ULFPTx

4.	If the UE supports ULFPTx, repeat test steps 4~6 with UL RMC according to Table 6.2H.3.1.4.1-3. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on the UE supported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for both PCell and SCell.

6.2H.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO for PCell with the following exceptions.

Table 6.2H.3.1.4.3-2: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA |
|  | 23 |  | Power class 2 and Inter-band CA |

6.2H.3.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2H.3.1.5-1 for inter-band UL CA with UL MIMO configurations.

Table 6.2H.3.1.5-1: Maximum Output Power test requirement

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A |  |  | 262 | +2+TT /-3-TT | 23 | +2+TT /-3-TT |
| CA_n5A-n77A |  |  | 262 | +2+TT /-3-TT | 23 | +2+TT /-3-TT |
| CA_n8A-n78A |  |  | 262 | +2+TT /-3-TT | 23 | +2+TT /-3-TT |
| CA_n28A-n41A |  |  | 262 | +2+TT /-3-TT | 23 | +2+TT /-3-TT |
| CA_n28A-n78A |  |  | 262 | +2+TT /-3-TT | 23 | +2+TT /-3-TT |
| CA_n66A-n77A |  |  | 262 | +2+TT /-3-TT | 23 | +2+TT /-3-TT |
| CA_n71A-n77A |  |  | 262 | +2+TT /-3-TT | 23 | +2+TT /-3-TT |
| NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. NOTE 5: Power class 3 is default power class unless otherwise stated. NOTE 6: FWA form factor is targeted unless otherwise stated. NOTE 7: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table 6.2H.3.1.5-2. |  |  |  |  |  |  |

Table 6.2H.3.1.5-2: Test Tolerance (UE maximum output power for CA)

| TT for overall output power (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |
|  |  |  | BW ≤ 40MHz |  | 40MHz < BW ≤ 100MHz |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
|  | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 |
| SCell |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

6.2H.3.2	UE maximum output power reduction for inter-band UL CA with UL MIMO

6.2H.3.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified maximum output power with MPR and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2H.3.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.2H.3.2.3	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, the requirements in clause 6.2D.2 apply for the component carrier configured with UL MIMO and the requirements in clause 6.2.2 apply for the other component carrier.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.3.2.

6.2H.3.2.4	Test description

This test is covered by clauses 6.2.2 UE maximum output power reduction and 6.2D.2 UE maximum output power reduction for UL MIMO.

6.2H.3.2.5	Test requirements

Same test requirement as 6.2D.2 for the band supporting UL MIMO and 6.2.2 for the other band.

6.2H.3.3	UE additional maximum output power reduction for inter-band UL CA with UL MIMO

6.2H.3.3.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

6.2H.3.3.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.2H.3.3.3	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, unless specified in Table 6.2A.3.0.3-1, the requirements in clause 6.2.3 apply only to the indicated carrier.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.3.3.

6.2H.3.3.4	Test description

This test is covered by clauses 6.2.3 UE additional maximum output power reduction and 6.2D.3 UE additional maximum output power reduction for UL MIMO.

6.2H.3.3.5	Test requirements

Same test requirement as 6.2D.3 for the band supporting UL MIMO and 6.2.3 for the other band.

6.2H.3.4	Configured transmitted power for inter-band UL CA with UL MIMO

6.2H.3.4.1	Test purpose

To verify that the total measured UE configured maximum output power PUMAX in two uplink carrier aggregation is within the specified bounds.

6.2H.3.4.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.2H.3.4.3	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, the requirements in clause 6.2A.4.1.3 apply except that:

-	PPowerClass,CA is the maximum UE power specified in Table 6.2H.3.1.3-1 without taking into account the tolerance;

-	MPRc and A-MPRc are specified in clause 6.2D.2 and clause 6.2D.3 respectively for the component carrier configured with UL MIMO.

-	ΔPPowerClass,CA:

–	For a power class 2 UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2.H.3.1, otherwise ΔPPowerClass, CA = 0 dB;

–	For a power class 1.5 UE, it is 6dB when the requirements of default power class are applied as specified in sub-clause 6.2H.3.1; and it is 3dB when the requirements of power class 2 are applied as specified in sub-clause 6.2H.3.1; otherwise ΔPPowerClass, CA = 0 dB;

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2H.3.4.

6.2H.3.4.4	Test description

This test is covered by clauses 6.2.4 Configured transmitted power and 6.2D.4 Configured transmitted power for UL MIMO.

6.2H.3.4.5	Test requirements

Same test requirement as 6.2D.4 for the band supporting UL MIMO and 6.2.4 for the other band.

==============================================================

<< End of changes >>