3GPP TSG-RAN5 Meeting #104	R5-244251r1

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4497 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PDCP TC 7.1.3.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The message format of RRCReconfiguration is incorrect in the specific message contents. According to TS 38.331, the outOfOrderDelivery in Table 7.1.3.5.4.3.3-2 shall be not present. outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. RRCReconfiguration message was corrected. 2. Value of outOfOrderDelivery was updated as Not present. 3. Editorial updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors will exist these cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: put the RRC messages of these MAC TC into brackets (). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

7.1.3.5.4	PDCP reordering / Maximum re-ordering delay below t-Reordering / t-Reordering timer operations

7.1.3.5.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state using RLC-AM }

ensure that {

when { a PDCP PDU is received from the lower layers and the COUNT value of the received PDCP Data PDU is out of the re-ordering window }

then { UE discards the PDCP PDU }

}

(2)

with { UE in RRC_CONNECTED state using RLC-AM }

ensure that {

when { a PDCP PDU is received from the lower layers and the COUNT value of the received PDCP Data PDU is within the re-ordering window }

then { UE stores the resulting PDCP SDU }

}

(3)

with { UE in RRC_CONNECTED state using RLC-AM, and the RX_DELIV is not equal to the COUNT value of the RX_NEXT (there is missing PDCP PDUs) }

ensure that {

when { a PDCP PDU is received from the lower layers and the RCVD_COUNT = RX_DELIV }

then { UE delivers the resulting PDCP SDU and all stored PDCP SDUs with consecutive COUNT value to upper layer, in ascending order }

}

(4)

with { UE in RRC_CONNECTED state using RLC-AM and the associated PDCP t-Reordering timer is running }

ensure that {

when { RX_DELIV >= RX_REORD }

then { UE stops and resets t-Reordering timer }

}

(5)

with { UE in RRC_CONNECTED state using RLC-AM and the associated PDCP t-Reordering timer is running }

ensure that {

when { the t-Reordering timer expires }

then { UE delivers all stored PDCP SDUs to upper layer }

}

(6)

with { UE in RRC_CONNECTED state using RLC-AM and the associated PDCP t-Reordering timer is running }

ensure that {

when { the t-Reordering is reconfigured by upper layers }

then { UE stops and resets t-Reordering timer }

}

7.1.3.5.4.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.323, clause 5.2.2.1, 5.2.2.2 and 5.2.2.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.2.2.1]

In this section, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.3]

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the receiving PDCP entity shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

7.1.3.5.4.3	Test description

7.1.3.5.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 exception  of PDCP parameters according to Table 7.1.3.5.4.3.1-1.

Table 7.1.3.5.4.3.1-1: PDCP parameters

| t-Reordering | ms300 |
| --- | --- |

7.1.3.5.4.3.2	Test procedure sequence

Table 7.1.3.5.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends the PDCP SDU #X D/C field = 1 (PDCP Data PDU) and PDCP SN = X (Note 1) (Note 6) | <-- | (PDCP SDU #X) | - | - |
| 2 | Check: Does the UE transmit a PDCP SDU via the AM RLC entity in the next 1s? | --> | (PDCP SDU) | 1 | F |
| 3 | The SS sends the PDCP SDU #1 D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. The UE starts t-Reordering. | <-- | (PDCP SDU #1) | - | - |
| 4 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN =2. | <-- | (PDCP SDU #2) | - | - |
| 5 | Wait for 100ms (< configured t-Reordering). | - | - | - | - |
| 6 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | (PDCP SDU #0) | - | - |
| 7 | Check: Does the UE transmit the PDCP SDU #0 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 0? (Note 4) | --> | (PDCP SDU #0) | 2, 3 | P |
| 8 | Check: Does the UE transmit the PDCP SDU #1 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 1? (Note 4) | --> | (PDCP SDU #1) | 2, 3 | P |
| 9 | Check: Does the UE transmit the PDCP SDU #2 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 2? (Note 4) | --> | (PDCP SDU #2) | 2, 3 | P |
| 10 | The SS sends the PDCP SDU #4 D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. The UE starts t-Reordering. | <-- | (PDCP SDU #4) | - | - |
| 11 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 12 | The SS sends the PDCP SDU #7 D/C field = 1 (PDCP Data PDU) and PDCP SN = 7. | <-- | (PDCP SDU #7) | - | - |
| 13 | The SS sends the PDCP SDU #3 D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. The UE restarts t-Reordering timer. Note T1 | <-- | (PDCP SDU #3) | - | - |
| 14 | Check: Does the UE transmit the PDCP SDU #3 via the AM RLC entity? (Note 5) | --> | (PDCP SDU #3) | 3 | P |
| 15 | Check: Does the UE transmit the PDCP SDU #4 via the AM RLC entity? (Note 5) | --> | (PDCP SDU #4) | 3 | P |
| 16 | Check 1: Does the UE transmit the PDCP SDU #7 with PDCP SN=5 via the AM RLC entity after t-Reordering expiry? Note T2 Check 2: Is (T2 – T1) > t-Reordering? | --> | (PDCP SDU #7) | 4,5 | P |
| 17 | The SS sends the PDCP SDU #9 D/C field = 1 (PDCP Data PDU) and PDCP SN = 9. The UE starts t-Reordering. | <-- | (PDCP SDU #9) | - | - |
| 18 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 19 | The SS reconfigures the t-Reordering by sending a NR RRCReconfiguration message. The UE restarts t-Reordering timer. (Note 2) Note T3 | <-- | (RRCReconfiguration) | - | - |
| 20 | The UE transmits a NR RRCReconfigurationComplete message. (Note 3) | --> | (RRCReconfigurationComplete) | - | - |
| 21 | Check 1: Does the UE transmit the PDCP SDU #9 with PDCP SN=6 via the AM RLC entity after t-Reordering expiry? Note T4 Check 2: Is (T4 – T3) > t-Reordering? | --> | (PDCP SDU #9) | 6 | P |
| Note 1: The Reordering Window size is 131072 when 18 bit SN length is used. Note 2: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_Embed_RBConfig. Note 3: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 4: PDCP SDUs at steps 7, 8 and 9 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot) Note 5: PDCP SDUs at steps 14 and 15 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). Note 6: X=131072 for pc_supportOfRedCap_r17=false and X=2048 for pc_supportOfRedCap_r17=true. |  |  |  |  |  |

7.1.3.5.4.3.3	Specific message contents

Table 7.1.3.5.4.3.3-1: RRCReconfiguration (step 19, Table 7.1.3.5.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table: 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | Not EN-DC |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | Not EN-DC |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.4.3.3-2: RadioBearerConfig (Table 7.1.3.5.4.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList ::= SEQUENCE (SIZE 1..2)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| t-Reordering | ms750 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]