3GPP TSG-RAN5 Meeting #104	R5-244291r1

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3280 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of FR1 and FR2 PRACH configurations to align with core speces |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New PRACH configurations for LTM test cases has been added in 38.133 Clause A.3.8 from Rel-18.6.0. Hence corresponding updates are required in 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Table A.7.1-1 for PRACH.6 FR1 and add new Table A.7.1-2 for PRACH.5 FR1. Update Table A.7.2-1 for PRACH.6 FR2 and add new Table A.7.2-2 for PRACH.5 FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.1, A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Add missing mark “X” in “Other specs affected” of CR cover. Add missing word “Table” in the title of Table A.7.1-1. |  |  |  |  |  |  |  |  |

<< Start of changes 1>>

A.7	Reference PRACH configurations

This clause provides the typical PRACH configurations used for all RRM test cases defined in this test specification. Parameters not listed in this clause can be derived from the configuration of each test.

A.7.1	PRACH configurations for FR1

Table A.7.1-1 defines the PRACH configurations for FR1. Each of the PRACH configurations defined in Table A.7.1-1 have different applicabilities:

-	PRACH.1 FR1 for SSB-based contention based random access in FR1.

-	PRACH.2 FR1 for SSB-based non-contention based random access in FR1.

-	PRACH.3 FR1 for CSI-RS based non-contention based random access in FR1.

-	PRACH.4 FR1 for CSI-RS based non-contention based random access in FR1 to convey BFR.

-	PRACH.6 FR1 for SSB-based contention free random access in FR1.

The PRACH configuration defined in Table A.7.1-2 has the following applicability:

-	PRACH.5 FR1 for LTM early UL synchronization on candidate cell in FR1.

Table A.7.1-1 Parameters for PRACH Configurations for FR1

| Field | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR1 | PRACH.2 FR1 | PRACH.3 FR1 | PRACH.4 FR1 |  | PRACH.6 FR1 |  |
| prach-ConfigurationIndex | 102 | 102 | 102 | 102 |  | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 | 48 | 48 |  | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | 48 | 48 |  | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 | 0 | 0 |  | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | - | - | - |  | oneFourth, n16 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | - | oneFourth | oneFourth | oneFourth |  | - | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One | One | One |  | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 | N/A | RSRP_51 |  | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A | RSRP_51 | N/A |  | N/A |  |
| ra-ContentionResolutionTimer | sf48 | - | - | - |  | sf48 | 48 sub-frames |
| powerRampingStep | dB2 | dB2 | dB2 | dB2 |  | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  | dBm-120 |  |
| preambleTransMax | n6 | n6 | n6 | n200 |  | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | sl10 | sl10 | sl1 |  | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | 11 | 11 | 11 |  | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 2 | 2 |  | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present | N/A | N/A |  | - | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A |  | - | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | present | N/A | N/A |  | - | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A |  | - | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| csirs-ResourceList | N/A | N/A | present | presnt |  | N/A | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | N/A | 50 | 50 |  | N/A | Associated with CSI-RS configured |
| ra-OccasionList | - | - | 1 | 1 |  | - | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 | N/A | N/A |  | - | PRACH occasion index 1 is allowed |
| NOTE: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |  |

Table A.7.1-2 Parameters for LTM early UL synchronization PRACH Configuration for FR1

| Field | Value | Comment |
| --- | --- | --- |
| PRACH Configuration | PRACH.5 FR1 |  |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

<< End of changes 1>>

<< Start of changes 2>>

A.7.2	PRACH configurations for FR2

Table A.7.2-1 defines the PRACH configurations for FR2. Each of the PRACH configurations defined in Table A.7.2-1 have different applicabilities:

-	PRACH.1 FR2 for SSB-based contention based random access in FR2.

-	PRACH.2 FR2 for SSB-based non-contention based random access in FR2.

-	PRACH.3 FR2 for CSI-RS based non-contention based random access in FR2.

-	PRACH.4 FR2 for CSI-RS based non-contention based random access in FR2 to convey BFR.

-	PRACH.6 FR2 for SSB-based contention free random access in FR2.

The PRACH configuration defined in Table A.7.2-2 has the following applicability:

-	PRACH.5 FR2 for LTM early UL synchronization on candidate cell in FR2.

Table A.7.2-1 Parameters for PRACH Configurations for FR2

| Field | Value |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| PRACH Configuration |  | PRACH.1 FR2 |  | PRACH.2 FR2 | PRACH.3 FR2 | PRACH.4 FR2 | PRACH.6 FR2 |  |
| prach-ConfigurationIndex |  | 190 |  | 190 | 190 | 190 | 190 | Preamble format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing |  | Same as UL carrier SCS |  | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles |  | 48 |  | 48 | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA |  | 48 |  | 48 | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex |  | 0 |  | 0 | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB |  | oneFourth, n48 |  | N/A | N/A | N/A | oneFourth, n16 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion |  | N/A |  | oneFourth | oneFourth | oneFourth | N/A | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM |  | One |  | One | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB |  | RSRP_51 |  | RSRP_51 | N/A | RSRP_51 | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS |  | N/A |  | N/A | RSRP_51 | N/A | N/A |  |
| ra-ContentionResolutionTimer |  | sf48 |  | N/A | N/A | N/A | sf48 | 48 sub-frames |
| powerRampingStep |  | dB2 |  | dB2 | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower |  | dBm-120 |  | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax |  | n6 |  | n6 | n6 | n200 | n6 | Max number of RA preamble transmission performed before declaring a failure |
| ra-ResponseWindow |  | sl10 |  | sl10 | sl10 | sl40 | sl10 |  |
| zeroCorrelationZoneConfig |  | 11 |  | 11 | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index |  | 2 |  | 2 | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList |  | - |  | present | N/A | N/A | - | Associated with SSB index 0 |
| ra-PreambleIndex |  | - |  | 50 | N/A | N/A | - | Associated with SSB index 0 |
| csirs-ResourceList |  | N/A |  | present | present | present | N/A | Associated with CSI-RS configured |
| ra-PreambleIndex |  | N/A |  | 50 | 50 | 50 | N/A | Associated with CSI-RS configured |
| ra-OccasionList |  | - |  | - | 1 | 1 | - | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex |  | - |  | 1 | N/A | N/A | - | PRACH occasion index 1 is allowed |
| NOTE: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |  |

Table A.7.2-2 Parameters for LTM early UL synchronization PRACH Configuration for FR2

| Field | Value | Comment |
| --- | --- | --- |
| PRACH Configuration | PRACH.5 FR2 |  |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6] |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |

<< End of changes 2>>