# 3GPP TSG-RAN5 Meeting #104	R5-244515r2

# Maastricht, NL, Aug 19th - Aug 23th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4552 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a new test case for R16 IDC feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of a TC for R16 IDC feature. As R18 new IDC WI has been initiated to introduce R18 IDC related test cases, to ensure the integrity of the IDC related tests and further support R18 IDC solutions, the IDC solutions prior to R18 is necessary to be introduced also. There was no R17 IDC WI. The earliest NR IDC solutions were introduced by a TEI16 CR (R2-2002175) into TS 38.331 in R16, and it only focus on NR FDM solution. So, the R16 IDC related TC needs to be introduced into TS 38.523-1 also. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a TC for R16 IDC feature based on the R16 IDC core requirements in TS 38.331 to ensure the integrity of IDC related tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IDC related tests lack integrity |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0490 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Update the work item code on the CR cover page. r2: Add more explanations about why this TC is needed in the cover page. Update clause number 8.1.5.10.X to 8.1.5.10.8, and 4.9.X to 4.9.45 |  |  |  |  |  |  |  |  |

<Start of modified section>

8.1.5.10.8	UE Assistance Information / IDC mechanism

8.1.5.10.8.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information and the UE is experiencing IDC problems that it cannot solve by itself and UE did not transmit a UEAssistanceInformation message with idc-Assistance }

then { UE transmits a UEAssistanceInformation message to provide IDC assistance information including IDC-Assistance-r16 }

}

(2)

with { UE in NR RRC_CONNECTED state and UE already transmits a UEAssistanceInformation message with idc-Assistance }

ensure that {

when { UE configured to provide IDC assistance information and the UE is experiencing IDC problems that it cannot solve by itself and the current idc-Assistance information is different from the one indicated in the last UEAssistanceInformation message }

then { UE transmits a UEAssistanceInformation message to provide IDC assistance information including IDC-Assistance-r16 }

}

8.1.5.10.8.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.7.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

……

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

……

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

……

1>	if configured to provide IDC assistance information based on candidateServingFreqListNR included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

……

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2 or 5.3.5.3;

……

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR;

3>	for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include victimSystemType for each UL CA or NR-DC combination included in affectedCarrierFreqCombList;

3>	if the UE sets victimSystemType to wlan or bluetooth:

4>	include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

3>	else:

4>	optionally include affectedCarrierFreqCombList with an entry for each supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

……

8.1.5.10.8.3	Test description

8.1.5.10.8.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40), WLAN AP 1 (Cell 27) as specified in cl. 4.13 TS 38.508-1[4]; and

-	NR Cell 1/NRf1 (2496-2556MHz for band n41), WLAN AP 2 (Cell 27b) as specified in cl. 4.13 TS 38.508-1[4]

UE:

-	None

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 14 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.8.3.2	Test procedure sequence

Table 8.1.5.10.8.3.2-1 for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.5.10.8.3.2-1. The configuration “T0” and “T1” with only NR Cell 14 and WLAN AP 1 (Cell 27) are referred to the configuration “C1” and “C2” which specified in TS 38.508-1 [4] Table 4.13-1.

Table 8.1.5.10.8.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | NR Cell 1 | WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for non-IDC. |
|  | BeaconRSSI | dBm |  | Off |  | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for IDC. |
|  | BeaconRSSI | dBm |  | -80 |  | Off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | -79 |  | Cell configuration for IDC |
|  | BeaconRSSI | dBm |  | -80 |  | -80 |  |

Table 8.1.5.10.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCReconfiguration message including OtherConfig with idc-AssistanceConfig-r16 including CandidateServingFreqListNR-r16 including center frequency for NR cell 14 and NR cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.5.10.8.3.2-1. Note: UE shall detect IDC problem in RRC_IDLE with configuration “T1”. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.45 indicate that the UE is capable of exchanging data with WLAN AP? | - | - | 1, 2 | - |
| 5 | Check: Does the UE transmit an an UEAssistanceInformation message including IDC-Assistance-r16 with at least one affected carrier frequency including the carrier frequency of NR cell 14 in affectedCarrierFreqList? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the SS/PBCH EPRE level according to row "T2" in Table 8.1.5.10.8.3.2-1. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 1? | --> | NR RRC: MeasurementReport | - | - |
| 8 | The SS transmits an RRCReconfiguration message including to order the UE to perform inter-frequency handover to NR Cell 1 | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 1? | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10 | Check: Does the UE transmit an an UEAssistanceInformation message including IDC-Assistance-r16 with at least one affected carrier frequency including the carrier frequency of NR cell 1 in affectedCarrierFreqList? | --> | NR RRC: UEAssistanceInformation | 2 | P |

8.1.5.10.8.3.3	Specific message contents

Table 8.1.5.10.8.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.8.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Measurements configuration |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| idc-AssistanceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| candidateServingFreqListNR-r16 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR { | 2 entries |  |  |
| ARFCN-ValueNR[1] | ARFCN-ValueNR of NR Cell 14 |  |  |
| ARFCN-ValueNR[2] | ARFCN-ValueNR of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-1a: MeasConfig (Table 8.1.5.10.8.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ssbFrequency IE equals the ARFCN for NR Cell 14 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ssbFrequency IE equals the ARFCN for NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-1b: ReportConfigNR-EventA3 (Table 8.1.5.10.8.3.3-1a)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  | EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 6 | 3dB | FR1 |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-2: UEAssistanceInformation (step 5, Table 8.1.5.10.8.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-Assistance-r16 SEQUENCE { |  |  |  |
| affectedCarrierFreqList-r16 SEQUENCE { | 1 enrty |  |  |
| AffectedCarrierFreq-r16 SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueNR of NR Cell 14 |  |  |
| interferenceDirection-r16 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-3: MeasurementReport (step 7, Table 8.1.5.10.8.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | Cell index corresponding to NR Cell 14 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | Physical layer cell identity of NR Cell 14 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-4: RRCReconfiguration (step 8, Table 8.1.5.10.8.3.2-2)

| Derivation Path: TS 38.508-1 [4] 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-4a: CellGroupConfig (Table 8.1.5.10.8.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical cell Id of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-5: UEAssistanceInformation (step 10, Table 8.1.5.10.8.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-Assistance-r16 SEQUENCE { |  |  |  |
| affectedCarrierFreqList-r16 SEQUENCE { | 1 enrty |  |  |
| AffectedCarrierFreq-r16 SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueNR of NR Cell 1 |  |  |
| interferenceDirection-r16 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>