3GPP TSG-RAN5 Meeting #104	R5-244650r1

# Maastricht, Netherland, Aug 19th - 23th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3282 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of default configuration of FeatureCombinationPreambles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - startPreambleForThisPartition-r17 is set to 0, i.e. PRACH preamble index 0~7/0~3 belong to featureCombination preamble set, which fully overlaps with PRACH preamble index set for CBRA in RACH-ConfigCommon. To fix this overlapping, we suggest change startPreambleForThisPartition-r17 = 8 for FR1 and 4 for FR2, i.e. index 8~15/4~7 belong to featureCombination preamble set. - msgA-RSRP-Threshold should be set to Not present because it is only present if both 2-step and 4-step RA type are configured for the concerned feature combination in the BWP according to 38.331. - Default value for Msg.1 repetition should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Issues mentioned above are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: To fix 3GU issue on coversheet |  |  |  |  |  |  |  |  |

<Start of Change 1>

–	FeatureCombinationPreambles

Table 4.6.3-56E: FeatureCombinationPreambles

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 | FeatureCombination-r17 |  |  |
|  | FeatureCombination-r17 with condition MSG1_REP |  | MSG1_REP |
|  | FeatureCombination-r17 with condition MSG3_REP |  | MSG3_REP |
| startPreambleForThisPartition-r17 | 8 |  | FR1 |
|  | 4 |  | FR2 |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 8 |  | FR1 |
|  | 4 |  | FR2 |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| msgA-RSRP-Threshold-r17 | Not present |  |  |
| rsrp-ThresholdSSB-r17 | RSRP-Range |  |  |
| deltaPreamble-r17 | Not present |  |  |
| msg1-RepetitionNum-r18 | n2 |  | MSG1_REP |
|  | Not present |  |  |
| msg1-RepetitionTimeOffsetROGroup-r18 | n16 |  | MSG1_REP |
|  | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MSG1_REP | For Msg.1 repetition tests |
| MSG3_REP | For Msg.3 repetition tests |

<End of Change 1>