33GPP TSG-RAN5 Meeting #104	R5-244721r1

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

Source:	Huawei, HiSilicon

Title:	Discussion to handle IMS for loop back test case

Agenda item:	6.4.7

Document for:	Endorsement

Discussion

Observation 1: When Test mode is activated, according to 36.509 5.3.2.3 IMS SIP message (e.g IMS REGISTER ) shall not be transmitted by UE’s layer2. However the 36.509 does not forbid UE to establish PDU session for IMS. Therefore, normally UE will establish PDU session for IMS but IMS registration failure.

[36.509 clause 5.3.2.3]

When the UE test mode is active, the UE operates normally, except for the following:

1>	the UE shall accept any request to establish a data radio bearer with an associated EPS bearer context, both included in the same RRC message, and within the radio access capabilities of the UE;

1>	optionally, if the UE supports CSG Proximity Indication;

2>	the UE shall ignore any non 3GPP radio signals which are normally used in determining proximity to a CSG (closed subscriber group) cell and delete any previously stored cell information for proximity detection.

NOTE 1:	The UE may provide other means which would guarantee that the UE ignores any non 3GPP radio signals which are normally used in determining proximity to a CSG (closed subscriber group) cell and deletes any previously stored cell information for proximity detection. For such UEs the above mentioned CSG requirements in UE test mode may not apply.

1>	if neither test loop mode A operation, test loop mode B operation, test loop mode C operation, test loop mode D operation, test loop mode E, test loop mode F, test loop mode G, test loop mode H operation, nor test loop mode I operation is ongoing:

2>	if the UE is operating in E-UTRAN or NB-IoT mode, for each PDN connection requested by the UE:

3>	if the UE has not received the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message in response to the PDN CONNECTIVITY REQUEST message; or

3>	if the UE has received the response message with the PDN type IE set to 'IPv4' and an IPv4 address value different from '0.0.0.0' and "IPv4 address allocation via DHCPv4" was not requested by the UE:

4>	the UE shall not transmit any uplink PDCP SDU on any DRB associated with the requested PDN:

4>	the UE may discard any received downlink PDCP SDU on any DRB associated with the requested PDN.

NOTE 2:	If the UE has requested the use of DHCPv4 signalling for allocation of an IPv4 address, the UE may either accept the IPv4 address provided via NAS signalling and not transmit any uplink PDCP SDU on any DRB and may discard downlink PDCP SDUs on any DRB, or the UE may transmit and receive DHCPv4 messages to obtain an IPv4 address, according to the procedure described in TS 36.508 clause 4.5.2A.

3>	if the UE has received the response message with the PDN type IE set to ‘IPv6’ or ‘IPv4v6’ or "IPv4 address allocation via DHCPv4" was requested by the UE , and the UE has not yet obtained its IP address(es);

4> the UE may transmit uplink PDCP SDUs on any DRB associated with the requested PDN, for the sole purpose of obtaining its IP address(es).

3>	if the UE has obtained its IP address(es):

4>	the UE shall not transmit any uplink PDCP SDU on any DRB associated with the requested PDN:

4>	the UE may discard any received downlink PDCP SDU on any DRB associated with the requested PDN.

Observation 2: UE test mode shall be activated before PDU session (DRB) establishment. Since PDU session always happened after test mode activation, it is impossible that UE firstly perform IMS registration and then activate test mode.

[TS 38.509, clause 5.3.4.2.3]

UE test loop mode B provides loopback of SDAP SDUs for bi-directional QoS Flows while UE is operated in standalone NR mode. Prior to closing the UE test loop mode B, thereby requesting the UE to start looping back the received data packets, at least one 5GS test mode bi-directional QoS Flow shall have been established between SS and UE. This implies that before the procedure for establishing the bi-directional QoS Flows takes place the SS needs to activate the UE test mode as specified in subclause 5.2.2

[TS 38.509, clause 5.3.4.1]

Prior to closing the UE test loop mode A, thereby requesting the UE to start looping back the received data packets, at least one 5GS test mode bi-directional data radio bearer shall have been established between SS and UE. This implies that before the procedure for establishing the bi-directional data radio bearer takes place the SS needs to activate the UE test mode as specified in subclause 5.2.2.

Observation 3: For Voice centric UE, if IMS is not available (e.g UE or SS not support IMS, or UE IMS registration failure), UE shall disable N1 mode. If the “manufacturer determined period of time” is very short, that may cause nearly all the NR test case fail.

[TS 24.501, clause 4.3.2]

In the present document, "IMS voice not available" refers to one of the following conditions:

a)	the UE does not support IMS voice;

b)	the UE supports IMS voice, but the network indicates in the REGISTRATION ACCEPT message that IMS voice over PS sessions are not supported; or

c)	the UE supports IMS voice, the network indicates in the REGISTRATION ACCEPT message that IMS voice over PS sessions are supported, but the upper layers:

1)	provide no indication that the UE is available for voice call in the IMS within a manufacturer determined period of time; or

2)	indicate that the UE is not available for voice calls in the IMS.

…

When IMS voice is not available over 3GPP access, if the UE's usage setting is "voice centric", the UE operates in single-registration mode, and the UE:

a)	does not have a persistent PDU session, and:

1)	if the UE is only registered over 3GPP access, or if the UE is registered over both 3GPP access and non-3GPP access and IMS voice is not available over non-3GPP access, the UE shall disable the N1 mode capability for 3GPP access and proceed as specified in subclause 4.9.2 with modifications described below; or

2)	if the UE is registered over both 3GPP access and non-3GPP access and IMS voice is available over non-3GPP access, the UE may disable the N1 mode capability for 3GPP access and proceed as specified in subclause 4.9.2 with modifications described below; or

b)	has a persistent PDU session, then the UE waits until the radio bearer associated with the persistent PDU session has been released. When the radio bearer associated with the persistent PDU session has been released, then:

1)	if the UE is only registered over 3GPP access, or if the UE is registered over both 3GPP access and non-3GPP access and IMS voice is not available over non-3GPP access,the UE shall disable the N1 mode capability for 3GPP access and proceed as specified in subclause 4.9.2 with modifications described below; or

2)	If the UE is registered over both 3GPP access and non-3GPP access and IMS voice is available over non-3GPP access, the UE may disable the N1 mode capability for 3GPP access and proceed as specified in subclause 4.9.2 with modifications described below.

Note: IMS PDU session established in the preamble only contain default Qos flow (non-GBR Qos flow), so UE does not have Persistent PDU session.

Persistent PDU session: either a non-emergency PDU session contains a GBR QoS flow with QoS equivalent to QoS of teleservice 11 and where there is a radio bearer associated with that PDU session over 3GPP access, or an emergency PDU session where there is a radio bearer associated with that PDU session over 3GPP access.

Observation 4: For Data centric UE, UE shall not disable N1 mode because IMS is not available. However, as 38.509 does not limit the high layer behaviour after test mode activation, so there still may have some error case.

- TS 38.523-1 11.3.1a, SS does not support IMS, if UE and PICS setting still support IMS, test case will timeout because SS is waiting for IMS PDU session establishment.

- R5-243592 raised for TS 38.523-1 11.3.12 step5 in last meeting, and TS 38.523-1 11.3.6 step1AN may have same failure.

Proposal

Based on the above discussion, if UE keep on enabling IMS for the loop back test case, the UE cannot transmit any IMS SIP message and IMS registration will be failure, which may also cause some other error case. The followings are proposed:

Proposal 1: Add a Note in TS 38.508-1 to allow UE to set to Data Centric and/or close IMS for the loop back test case.

For example:

- 4.5.1	General

The generic procedures are used by test cases to get UE under test into SWITCHED_OFF, RRC_IDLE/Ipsec SA not established, RRC_INACTIVE, RRC_CONNECTED/Ipsec SA or Out of Coverage established state.

A test case controls the SS by specifying the required RRC state and a set of generic procedure parameters applicable for the intended testing.

The connectivity values EN-DC and E-UTRA/EPC are both a UE connected to the EPC. The connectivity values E-UTRA/5GC, NR, NGEN-DC, NE-DC and NR-DC are all a UE connected to the 5GC.

Table 4.5.1-1: Generic procedure parameters

| Parameter | Values | Description | Parameter condition |
| --- | --- | --- | --- |
| Connectivity | E-UTRA/5GC | E-UTRA connected to 5GC | Mandatory. For value NR see note 1. |
|  | NR | NR connected to 5GC |  |
|  | EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |  |
|  | NGEN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to 5GC |  |
|  | NE-DC | NR E-UTRA Dual Connectivity |  |
|  | NR-DC | NR-NR Dual Connectivity |  |
|  | WLAN | Un trusted non 3GPP access over WLAN |  |
|  | E-UTRA/EPC | E-UTRA connected to EPC |  |
| Bearers | MCG(s) and SCG | MCG and SCG | Mandatory when Connectivity is set to EN-DC, NGEN-DC, NE-DC and NR-DC and when the generic procedures are used by test cases to get UE under test into RRC_CONNECTED state. s=1 if MULTI_PDN= FALSE and s=2 if MULTI_PDN=TRUE. N/A otherwise. |
|  | MCG(s) and split | MCG and split |  |
| Test Mode | On | UE test mode active as specified in TS 38.509 [11], clause 5.2.2. (Note 2) | Optional |
| Test Loop Function | On | UE test mode active with one of the UE test loop modes activated as specified in TS 38.509 [11], clauses 5.2.2 and 5.3.2. (Note 2) | Optional |
| Connected without release | On | Enter RRC_Connected with Ipsec_SA_Established and without any release. | Optional N/A for NR-DC. |
| Interworking without N26 interface supported | On | The NWK sets the REGISTRATION ACCEPT message, IE 5GS network feature support, IWK N26 (octet 3, bit 7) = 1 | Optional, Depends on test case scenario. Default message content for REGISTRATION ACCEPT is set to Interworking without N26 interface NOT supported |
| Unrestricted nr PDN | On | Allow unrestricted numbers of PDNs. | Optional for Connectivity E-UTRA/EPC. N/A otherwise. |
| Sidelink | On | NR sidelink | Optional |
| Cast type | Unicast | To establish unicast sidelink and PC5-RRC connection. | Optional |
| GNSS Sync | On | To use GNSS as the synchronization reference source. | Optional |
| SL MIMO | On | To transmit PSSCH with 2 spatial layers, i.e. SL MIMO | Optional |
| Note 1: The TS 38.331 [6] abbreviation for NR connected to 5GC is NR/5GC. Note 2: IF Test Mode = On OR Test Loop Function = On for a NR test case, UE may disable IMS and/or set to Data Centric. |  |  |  |