3GPP TSG-RAN5 Meeting #104	R5-245070r1

# Maastricht, Netherlands, 19th – 23th August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1821 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test requirement for Inter-band EN-DC FR1 configurations with three bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT, Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some EN-DC FR1 configurations are missing test requirement in the test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Some format updated; - Updated following tables to include requirement for DC_2A-66A-66A_n2A, DC_2A-66A_n2A, and DC_2A-66A_n66A: - Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands) - Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands) - Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands) - Updated Table 5.5B.4.2-1 to include DC_2A-2A-66A_n66A; - Updated Table 7.3B.2.0.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) with DC_2A-66A_n2A - Added MSD exception config DC_2A-66A_n2A into configuration table Table 7.3B.2.3_1.1.4.1-0 and requirement table; - No exception defined for DC_2A-66A-66A_n2A, DC_2A-2A-66A_n66A and DC_2A-66A_n66A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These EN-DC FR1 configurations will be missing test requirement for certification tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.2, 6.2B.4.2.3.2, 7.3B.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis in R5-244773 Changes also covered DC_2A-66A-66A_n2A which under WIC “NR_CADC_NR_LTE_DC_R17-UEConTest”. Some part with core spec alignment with TS38.101-3 V16.20.0 and V17.14.0. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: corrected TIB value for DC_2-66_n2, DC_2-66-66_n2, DC_2-66_n66 from 0.3 to 0.5; set N/A to band 2 of UL FC and UL LCRB in Table 7.3B.2.0.3.5.2-1. |  |  |  |  |  |  |  |  |

## {Start of changes}

## 5.5B	Configuration for DC

<Unchanged Sections Skipped>

### 5.5B.4	Inter-band EN-DC within FR1

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.521-1 [10] and for NR operating bands and CA configurations in TS 38.521-1 [8] and present document.

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A_n41A5 | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A_n77A5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5,14 DC_1A-3C_n78A5 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3C_n78(2A)5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n79A5,14 | DC_1A_n79A14 DC_3A_n79A14 |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-7A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A_n28A5 | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n77A5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n78(2A)5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-19A_n77A5 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A_n78A5,14 DC_1A-19A_n78C5 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n79A5,14 DC_1A-19A_n79C5 | DC_1A_n79A14 DC_19A_n79A14 |
| DC_1A-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A6,11,12 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n77A5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n78A5,14 DC_1A-21A_n78C5 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n79A5,14 DC_1A-21A_n79C5 | DC_1A_n79A14 DC_21A_n79A14 |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-41A_n41A | DC_1A_n41A |
| DC_1A-41A_n77A | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n78A | DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n77A10,11 DC_1A-42C_n77A10,11 DC_1A-42D_n77A10,11 | DC_1A_n77A |
| DC_1A-42A_n78A10,11,14 DC_1A-42A_n78C10,11 DC_1A-42C_n78A10,11 DC_1A-42C_n78C10,11 DC_1A-42D_n78A10,11 DC_1A-42E_n78A10,11 | DC_1A_n78A14 |
| DC_1A-42A_n79A14 DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42E_n79A | DC_1A_n79A14 |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_n78A-n79A14 | DC_1A_n78A14 DC_1A_n79A14 |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-13A_n77A | DC_2A_n77A DC_13A_n77A |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-66A_n2A | DC_2A_n2A2 DC_66A_n2A |
| DC_2A-66A-66A_n2A | DC_66A_n2A |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A_n77A | DC_2A_n77A DC_66A_n77A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_2A_n5A-n77A14 | DC_2A_n5A DC_2A_n77A14 |
| DC_3A_n1A-n78A5 | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n79A5 | DC_3A_n1A DC_3A_n79A |
| DC_3A-5A_n78A5 DC_3A-5A_n78C5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-7A_n5A | DC_3A_n5A DC_7A_n5A |
| DC_3A-7A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A_n78A5 DC_3C-7A_n78A5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n77A5 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n78A5 | DC_3A_n78A DC_8A_n78A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-8A_n78(2A) | DC_3A_n78A DC_8A_n78A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-19A_n77A5 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A_n78A5,14 DC_3A-19A_n78C5 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n79A5,14 DC_3A-19A_n79C5 | DC_3A_n79A14 DC_19A_n79A |
| DC_3A-20A_n1A | DC_3A_n1A DC_20A_n1A |
| DC_3A-20A_n28A5,6,11,12 | DC_3A_n28A DC_20A_n28A |
| DC_3A-20A_n78A5 | DC_3A_n78A DC_20A_n78A |
| DC_3A-21A_n77A5 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A_n78A5,14 DC_3A-21A_n78C5 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n79A5,14 DC_3A-21A_n79C5 | DC_3A_n79A14 DC_21A_n79A14 |
| DC_3A-28A_n78A5 | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5 | DC_3A_n28A DC_3A_n78A |
| DC_3A-40A_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A-41A_n28A5 DC_3A-41C_n28A5 | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A | DC_3A_n41A |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n77(2A) | DC_3A_n77A DC_41A_n77A |
| DC_3A-42A_n77A10,11 DC_3A-42C_n77A10,11 DC_3A-42D_n77A10,11 | DC_3A_n77A |
| DC_3A-42A_n78A10,11,14 DC_3A-42A_n78C10,11 DC_3A-42C_n78A10,11 DC_3A-42C_n78C10,11 DC_3A-42D_n78A10,11 DC_3A-42E_n78A10,11 | DC_3A_n78A14 |
| DC_3A-42A_n79A14 DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42E_n79A | DC_3A_n79A14 |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_3A_n78A-n79A14 | DC_3A_n78A14 DC_3A_n79A14 |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-20A_n1A | DC_7A_n1A DC_20A_n1A |
| DC_7A-20A_n3A | DC_7A_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6,11,12 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-28A_n3A | DC_7A_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 | DC_7A_n5A DC_28A_n5A |
| DC_7A-28A_n78A5 | DC_7A_n78A DC_28A_n78A |
| DC_7A_n28A-n78A5 DC_7C_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n77A | DC_13A_n77A DC_66A_n77A |
| DC_13A_n2A-n77A14 | DC_13A_n2A DC_13A_n77A14 |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |
| DC_18A-41A_n3A | DC_18A_n3A DC_41A_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_19A_n1A-n78A5 | DC_19A_n1A DC_19A_n78A |
| DC_19A_n1A-n79A5 | DC_19A_n1A DC_19A_n79A |
| DC_19A-21A_n77A5 | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A_n78A5,14 DC_19A-21A_n78C5 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n79A5,14 DC_19A-21A_n79C5 | DC_19A_n79A14 DC_21A_n79A14 |
| DC_19A-42A_n77ADC_19A-42C_n77A | DC_19A_n77A |
| DC_19A-42A_n78A14 DC_19A-42C_n78A DC_19A-42A_n78C DC_19A-42C_n78C | DC_19A_n78A14 |
| DC_19A-42A_n79A14 DC_19A-42C_n79A DC_19A-42A_n79C DC_19A-42C_n79C | DC_19A_n79A14 |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A_n78A-n79A14 | DC_19A_n78A14 DC_19A_n79A14 |
| DC_20A_n28A-n78A5,6,11,12 | DC_20A_n28A DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_21A_n1A-n78A5 | DC_21A_n1A DC_21A_n78A |
| DC_21A_n1A-n79A5 | DC_21A_n1A DC_21A_n79A |
| DC_21A-42A_n77A10,11 DC_21A-42C_n77A10,11 | DC_21A_n77A |
| DC_21A-42A_n78A10,11,14 DC_21A-42A_n78C10,11 DC_21A-42C_n78A10,11 DC_21A-42C_n78C10,11 | DC_21A_n78A14 |
| DC_21A-42A_n79A14 DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C | DC_21A_n79A14 |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A_n78A-n79A14 | DC_21A_n78A14 DC_21A_n79A14 |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A-42A_n78A10,11 | DC_28A_n78A |
| DC_41A-42A_n78A10,11 | DC_41A_n78A |
| DC_66A_n2A-n77A14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A_n5A-n77A14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A-(n)71AA | DC_66A_n71A DC_(n)71AA |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported. NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured PCell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 7: Void. NOTE 8: Reserved. NOTE 9: Reserved. NOTE 10: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 12: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 13: Reserved. NOTE 14: Minimum requirements for PC2 are applicable for this uUplink EN-DC configuration in this downlink/uplink to EN-DC configuration. |  |

<Unchanged Sections Skipped>

### 6.2B.4	Configured Output Power for DC

<Unchanged Sections Skipped>

#### 6.2B.4.2	ΔTIB,c for DC

<Unchanged Sections Skipped>

##### 6.2B.4.2.3	Inter-Band EN-DC within FR1

###### 6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

<Unchanged Sections Skipped>

###### 6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
| DC_1-3_n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n41 | 0.33 |
| DC_1-3_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
| DC_1-5_n78 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
| DC_1-7_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_1-8_n77 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_1-8_n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18_n77 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_1-20_n28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-20_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-21_n77 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
| DC_1-21_n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_1-21_n79 | 1 | 0.3 |
|  | 21 | 0.3 |
| DC_1-28_n3 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |
| DC_1-28_n78 DC_1_n28-n78 | 1 | 0.3 |
|  | 28 or n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | n28 | 0.3 |
| DC_1-41_n28 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
| DC_1-41_n41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
| DC_1-41_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-41_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-42_n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-42_n78 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-42_n79 | 1 | 0.3 |
|  | 42 | 0.8 |
| DC_1_n77-n79 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78-n79 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_2-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |
|  | 2 | 0.6 |
| DC_2_n5-n77 | n5 | 0.3 |
|  | n77 | 0.8 |
|  | 2 | 0.6 |
| DC_2-13_n77 | 13 | 0.5 |
|  | n77 | 0.8 |
| DC_2-14_n2 | 2 | 0.3 |
|  | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | n66 | 0.5 |
| DC_2-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_2-66_n2 | 2 | 0.5 |
| DC_2-66-66_n2 | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-66_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_2-66_n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-66_n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
|  | 2 | 0.6 |
| DC_2-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |
| DC_2-(n)71 | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_3-5_n78 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |
| DC_3-7_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n77 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_3-8_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_3-18_n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_3-18_n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_3-19_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_3-19_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_3-19_n79 | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_3-20_n1 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.3 |
| DC_3-20_n28 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_3-20_n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-21_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-21_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-21_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_3-28_n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_3_n28-n78 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| DC_3-40_n1 | 3 | 0.5 |
|  | 40 | 0.5 |
|  | n1 | 0.5 |
| DC_3-41_n28 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n28 | 0.3 |
| DC_3-41_n41 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3-41_n77 | 3 | 0.6 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n77 | 0.8 |
| DC_3-42_n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-42_n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-42_n79 | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_3_n77-n79 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78-n79 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_5-7_n78 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_5-30_n66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_7-20_n28 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_7-20_n1 | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |
| DC_7-20_n3 | 7 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |
| DC_7-20_n78 | 7 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_7-28_n3 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |
| DC_7-28_n78 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_7_n28-n78 | 7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
|  | 13 | 0.3 |
| DC_13_n2-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
|  | 13 | 0.3 |
| DC_13-66-n2 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | 13 | 0.5 |
| DC_13-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |
| DC_14-66_n2 DC_14-66-66_n2 | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_14-66_n66 | 14 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_18-41_n3 | 18 | 0.3 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n3 | 0.5 |
| DC_18-41_n77 | 18 | 0.3 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n77 | 0.8 |
| DC_18-41_n78 | 18 | 0.3 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n78 | 0.8 |
| DC_19-21_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_19-21_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_19-21_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
| DC_19-42_n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-42_n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-42_n79 | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_19_n77-n79 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78-n79 | 19 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_20_n28-n78 | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_20_n76-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_21-42_n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-42_n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-42_n79 | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21_n77-n79 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78-n79 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_28_n7-n78 | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |
| DC_28-42_n78 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_41-42_n78 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | 66 | 0.6 |
| DC_66_n2-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
|  | 66 | 0.6 |
| DC_66_n5-n77 | n5 | 0.3 |
|  | n77 | 0.8 |
| DC_66-(n)71 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

###### 6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

<Unchanged Sections Skipped>

###### 6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands

<Unchanged Sections Skipped>

###### 6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands

<Unchanged Sections Skipped>

### 7.3B.2	Reference sensitivity for DC

#### 7.3B.2.0	Minimum Conformance Requirements of Reference sensitivity for DC

<Unchanged Sections Skipped>

##### 7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.0.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) and uplink/downlink configurations due to UL harmonic from a PC3 aggressor UL band (low) for either single band uplink or PC3 or PC2 EN-DC are specified in Table 7.3B.2.0.3.1-1. For these exceptions, only the listed test points in Table 7.3B.2.0.3.1-1 need to be tested.

Table 7.3B.2.0.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 1 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| 2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 2 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| 2 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 2 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 2 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| 3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 3 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| 3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| 5 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| 5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 5 | n778 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 5 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| 5 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 8 | n41 | 5 | 15 | 8 (RBstart=0) | 10 | 13 | NOTE 3 | UL3/DL1 direct-hit |
| 8 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.5 | NOTE 3 | UL3/DL1 direct-hit |
| 8 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| 8 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 8 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| 8 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 8 | n79 | 5 | 15 | 10 (RBstart=0) | 10 | 12.0 | NOTE 5 | UL5/DL1 direct-hit |
| 8 | n79 | 5 | 15 | 25 (RBstart=0) | 100 | 4.4 | NOTE 5 | UL5/DL1 direct-hit |
| 12 | n66 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| 12 | n66 | 5 | 15 | 20 (RBstart=0) | 40 | 2.4 | NOTE 3 | UL3/DL1 direct-hit |
| 12 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 12 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 12 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 12 | n78 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 13 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 13 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 14 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 14 | n77 | 5 | 15 | 15 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 18 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 18 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 18 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 18 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 19 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 19 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 19 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| 19 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 19 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| 19 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 20 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| 20 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 20 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| 20 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| 26 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 10.3 | NOTE 3 | UL3/DL1 direct-hit |
| 26 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 2.7 | NOTE 3 | UL3/DL1 direct-hit |
| 26 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| 26 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 1.0 | NOTE 4 | UL4/DL1 direct-hit |
| 26 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| 26 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.0 | NOTE 4 | UL4/DL1 direct-hit |
| n28 | 1 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | 1 | 5 | 15 | 25 (RBstart=0) | 20 | 5.3 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | 2112 | NA | NA | NA | NA | NA | NOTE 2 | UL2/DL1 direct-hit |
| n28 | 2112 | NA | NA | NA | NA | NA | NOTE 2 | UL2/DL1 direct-hit |
| 28 | n51 | 5 | 15 | 12 (RBstart=0) | 5 | 27.8 | NOTE 2 | UL2/DL1 direct-hit |
| 28 | n51 | 5 | 15 | 25 (RBstart=0) | 5 | 1.9 | NOTE 6 | UL2/DL1 near-miss |
| 28 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 28 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 28 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| 28 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| 66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 66 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| 66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 66 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| 71 | n29 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n29 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| 71 | n210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 25 (RBstart=0) | 5 | 4.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 5 | 1.7 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 29 | 5 | 15 | 25 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 210 | 5 | 15 | 8 (RBstart=0) | 20 | 0.6 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | 7 | 5 | 15 | 8 (RBstart=0) | 5 | 14.6 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | 7 | 5 | 15 | 25 (RBstart=0) | 20 | 9 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL EARFCN or NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 11: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). NOTE 12: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-2: Void

Table 7.3B.2.0.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n46 | 482,4 | 22.6 | 19.5 | 17.8 | 16.6 |  |  |  |  |  |  |  |
| NOTE 1: FFS NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: FFS NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-4: Void

Table 7.3B.2.0.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |

7.3B.2.0.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) and uplink/downlink configurations due to UL harmonic from a PC3 aggressor UL band (high) for either single band uplink or PC3 or PC2 EN-DC are specified in Table 7.3B.2.0.3.2-1.

Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor UL for PC2 EN-DC are specified in Table 7.3B.2.3.2-1a.

For these exceptions, only the listed test points in Table 7.3B.2.3.2-1 and Table 7.3B.2.3.2-1a need to be tested.

Table 7.3B.2.0.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 2 | n71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| 2 | n71 | 20 | 15 | 50 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | 71 | 20 | 15 | 50 (RBstart=0) | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | 26 | 15 | 15 | 75 (RBstart=0) | 15 | 22.5 | NOTE 4 | UL1/DL3 |
| n77 | 2 | 10 | 15 | 25 (RBstart=0) | 5 | 6.1 |  |  |
| n77 | 2 | 20 | 15 | 100 (RBstart=0) | 20 | 3.7 |  |  |
| n77 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| n77 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n77 | 12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 12 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 28 | 10 | 15 | 25 (RBstart=0) | 5 | 28 | NOTE 2 | UL1/DL5 |
| n77 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 22 | NOTE 2 | UL1/DL5 |
| n77 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n77 | 41 | 10 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 |  |  |
| n78 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 |  |  |
| n78 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 7.2 |  |  |
| n78 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 3.8 |  |  |
| n78 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 8 | UL2/DL3 |
| n78 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 10.4 | NOTE 8 | UL2/DL3 |
| n79 | 11 | 10 | 15 | 25 (RBstart=0) | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 11 | 15 | 15 | 75 (RBstart=0) | 15 | 34.5 | NOTE 4 | UL1/DL3 |
| n79 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 29.5 | NOTE 2 | UL1/DL5 |
| n79 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 24.7 | NOTE 2 | UL1/DL5 |
| n79 | 21 | 10 | 15 | 25 (RBstart=0) | 5 | 39.3 | NOTE 4 | UL1/DL3 |
| n79 | 21 | 15 | 15 | 75 (RBstart=0) | 15 | 34.5 | NOTE 4 | UL1/DL3 |
| n79 | 26 | 10 | 15 | 25 (RBstart=0) | 5 | 27 | NOTE 2 | UL1/DL5 |
| n79 | 26 | 15 | 15 | 75 (RBstart=0) | 15 | 22.2 | NOTE 2 | UL1/DL5 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE 12: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | 2 | 10 | 15 | 25 (RBstart=0) | 5 | 9.1 |  |  |
| n77 | 2 | 20 | 15 | 100 (RBstart=0) | 20 | 6.7 |  |  |
| n77 | 3 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 |  |  |
| n77 | 3 | 20 | 15 | 100 (RBstart=0) | 20 | 5.7 |  |  |
| n77 | 12 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 12 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 13 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | 14 | 10 | 15 | 50 (RBstart=0) | 10 | 31 | NOTE 1 | UL1/DL5 |
| n77 | 28 | 10 | 15 | 25 (RBstart=0) | 5 | 32 | NOTE 1 | UL1/DL5 |
| n77 | 28 | 20 | 15 | 100 (RBstart=0) | 20 | 25 | NOTE 1 | UL1/DL5 |
| n77 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 19.4 | NOTE 4 | UL2/DL3 |
| n77 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 19.4 | NOTE 4 | UL2/DL3 |
| n78 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 9.8 |  |  |
| n78 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 5.8 |  |  |
| n79 | 19 | 10 | 15 | 25 (RBstart=0) | 5 | 32.5 | NOTE 1 | UL1/DL5 |
| n79 | 19 | 15 | 15 | 75 (RBstart=0) | 15 | 27.7 | NOTE 1 | UL1/DL5 |
| n79 | 21 | 10 | 15 | 25 (RBstart=0) | 5 | 42.3 | NOTE 3 | UL1/DL3 |
| n79 | 21 | 15 | 15 | 75 (RBstart=0) | 15 | 37.5 | NOTE 3 | UL1/DL3 |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: Void. NOTE 3: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-2: Void

7.3B.2.0.3.3	Void

7.3B.2.0.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.4-1 and Table 7.3B.2.0.3.4-1a.

Table 7.3B.2.0.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | 3 | 1922.5 | 5 | 15 | 25 (RBSTART=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n1 | 3 | 1945 | 50 | 15 | 128 (RBSTART=0) | 1877.5 | 5 | 19.7 | ACLR1 |
| 1 | n3 | 1930 | 20 | 15 | 100 (RBSTART=0) | 1877.5 | 5 | [7.9] | >ACLR2 |
| 1 | n3 | 1922.5 | 5 | 15 | 25 (RBSTART=0) | 1877.5 | 5 | 3 | >ACLR2 |
| 1 | n40 | 1970 | 20 | 15 | 100 (RBSTART=0) | 2302.5 | 5 | 6.6 | >ACLR2 |
| 1 | n41 | 1970 | 20 | 15 | 100 (RBSTART=0) | 2501 | 10 | 6.1 | >ACLR2 |
| 3 | n41 | 1775 | 20 | 15 | 50 (RBSTART=50) | 2501 | 10 | 0.7 | >ACLR2 |
| 3 | n51 | 1720 | 20 | 15 | 50 (RBSTART=0) | 1429.5 | 5 | 6.4 | >ACLR2 |
| 3 | n51 | 1712.5 | 5 | 15 | 25 (RBSTART =0) | 1429.5 | 5 | 6.4 | >ACLR2 |
| n5 | 28 | 834 | 20 | 15 | 20 (RBSTART=0) | 800.5 | 5 | 17.5 | ACLR2 |
| 30 | n66 | 2310 | 10 | 15 | 25 (RBSTARTt=0) | 2197.5 | 5 | 8.3 | >ACLR2 |
| n40 | 1 | 2350 | 100 | 30 | 270 (RBSTART=0) | 2167.5 | 5 | [21.9] | ACLR2 |
| n41 | 1 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2167.5 | 5 | 9.1 | >ACLR2 |
| n41 | 2 | 2546 | 100 | 30 | 270 (RBSTART=0) | 1987.5 | 5 | 0.6 | >ACLR2 |
| n41 | 3 | 2546 | 100 | 30 | 270 (RBSTART=0) | 1877.5 | 5 | 0.6 | >ACLR2 |
| n41 | 25 | 2546 | 100 | 30 | 270 (RBSTART=0) | 1992.5 | 5 | 0.6 | >ACLR2 |
| n41 | 661 | 2546 | 100 | 30 | 270 (RBSTART=0) | 2197.5 | 5 | 3.5 | >ACLR2 |
| 41 | n77 | 2680 | 20 | 15 | 100 (RBSTART=0) | 3305 | 10 | 8.3 | >ACLR2 |
| 41 | n78 | 2680 | 20 | 15 | 100 (RBSTART=0) | 3305 | 10 | 8.3 | >ACLR2 |
| n46 | 48 | 5190 | 80 | 30 | 216 (RBSTART=0) | 3697.5 | 5 | 13.3 | >ACLR2 |
| 48 | n46 | 3690 | 20 | 15 | 100 (RBSTART=0) | 5160 | 20 | 7 | >ACLR2 |
| n77 | 411 | 3350 | 100 | 30 | 270 (RBSTART=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | 71 | 3350 | 100 | 30 | 270 (RBSTART=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | 38 | 3350 | 100 | 30 | 270 (RBSTART=0) | 2617.5 | 5 | 3.3 | >ACLR2 |
| n78 | 411 | 3350 | 100 | 30 | 270 (RBSTART=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | 46 | 3750 | 100 | 30 | 270 (RBSTART=3) | 5160 | 20 | 13.5 | >ACLR2 |
| n79 | 426 | 4550 | 100 | 30 | 270 (RBSTART=0) | 3597.5 | 5 | 2.6 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: Void. NOTE 4: The NR DL band should be configured using the lowest SCS that is compatible with the specified DL CBW. NOTE 5: Void. NOTE 6: The requirements only apply for UEs supporting inter-band DC_42_n79 ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. NOTE 7: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-1a: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| 3 | n41 | 1775 | 20 | 15 | 50 (RBstart=50) | 2501 | 10 | 0.7 | >ACLR2 |
| n41 | 1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 12.4 | >ACLR2 |
| n41 | 3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 2.3 | >ACLR2 |
| n77 | 2 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.0 | >ACLR2 |
| n77 | 411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 6.5 | >ACLR2 |
| n77 | 66 | 3350 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 1.0 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-2: Void

7.3B.2.0.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.0.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex M  the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [5] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [5] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [5], with parameters specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
| DC_1A_SUL_n77A- |  |  |  |  |  |  |  |
| n84A | n77, n78 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  |  |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n77A DC_2A_n77(2A) DC_2A-2A_n77A | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
| DC_2A-2A_n77(2A) |  |  |  |  |  |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_2A_n78A | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
| DC_3_n5 | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_3C_n7A | n7 | 2535 | 10 | 50 | 2655 | 10.25 | IMD4 |
| DC_3A_n8A | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_n41A | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | IMD4 |
| DC_3A_SUL_n41A-n80A, | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_SUL_n41A-n80A | n41 | 2657.5 | 10 | 52 | 2657.5 | N/A | N/A |
| DC_3A_n77A | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_5A_n77A8 DC_5A_n77(2A)8 | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n66A 4DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A_n77A | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
| DC_8A_n3A | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_8A_n41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_SUL_n41A- | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
| n81A | n41 | 2685 | 10 | 52 | 2685 | N/A | N/A |
| DC_8A_n77A DC_8A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | H4 |
| DC_12A_n77A DC_12A_n77(2A) | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_12_n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
| DC_14A_n77A DC_14A_n77(2A) | 14 | 795.5 | 5 | 15 | 765.5 | 5.5 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_20A_n3A | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_8A_n79A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
| DC_8A-SUL_n79A-n81A | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_18A_n77A DC_18A_n78A | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_12A_n77A | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_19A_n78A | 19 | 836.5 | 5 | 25 | 881.5 | 13.6 | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_20_n7 | 20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | IMD3 |
| DC_20A_n77A DC_20A_n78A, | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
| DC_20A-SUL_n78A-n82A | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD54 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n28A7 | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD3 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD 4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 8 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_26A_n77A, | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
| DC_26A_n78A | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_30A_n77A DC_30A_n77(2A) | 30 | 2310 | 5 | 25 | 2355 | 8.0 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_48A_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_66A_n2A | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
| DC_66A_n25A | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) DC_66A-66A-66A_n77A | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2130 | 5.0 | IMD5 |
|  | n78 | 3660 | 5 | 50 | 3660 | N/A | N/A |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Void NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_2A_n77A DC_2A-2A_n77A DC_2A_n77C DC_2A-2A_n77C | 2 | 1855 | 5 | 25 | 1935 | 32.10 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 19.10 | IMD41 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n77A3 DC_5A_n77C3 | 5 | 844 | 5 | 25 | 889 | 18.60 | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_12A_n77A | 12 | 702 | 5 | 20 | 732 | 11.7 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_13A_n77A DC_13A_n77C | 13 | 782 | 5 | 20 | 751 | 15.37 | IMD5 |
|  | n77 | 3879 | 10 | 50 | 3879 | N/A | N/A |
| DC_14A_n77A | 14 | 795.5 | 5 | 15 | 765.5 | 11.7 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_18A_n77A5 | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_19A_n78A | 19 | 836.5 | 5 | 25 | 881.5 | 25.3 | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 33.4 | IMD3 |
|  | n79 | 4420.5 | 10 | 50 | 4420.5 | N/A | N/A |
| DC_28A_n77A | 28 | 705.5 | 5 | 25 | 760.5 | 19.2 | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | 1775 | 5 | 25 | 2175 | 34.33 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Void. NOTE 3: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 5: There is no IMD4/5 products in band n18 downlink for n77 operating in 3520 – 3560 MHz, 3700 – 3800MHz and 4000 - 4100MHz frequency range. |  |  |  |  |  |  |  |

7.3B.2.0.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.0.3.5.2-0: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 |  | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |  |

Editor’s Note:

- In 38.101-3 (V18.3.0) Table 7.3B.2.3.5.2-1 (MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)), ENDC band combination DC_2A-13A_n77A, and DC_2A-66A_n77A have UL/DL bandwidth 5 MHz for n77, while there is no bandwidth definition of 5MHz for n77 in 38.101-1 (V18.3.0) Table 5.3.5-1 (Channel bandwidths for each NR band). Therefore in following Table 7.3B.2.0.3.5.2-1 corresponding cases, n77 UL/DL bandwidth of 10 MHz and UL CLRB of 50 are used instead.

Table 7.3B.2.0.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-7A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 2145 | N/A | N/A |
|  | 8 | N/A | 5 | N/A | 955 | 3.3 | IMD5 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2140 | 14.4 | IMD3 |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |
| DC_1A-18A_n77A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 19 | N/A | 5 | N/A | 880 | 5.1 | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2154.6 | 3.6 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3647 | 10 | 50 | 3647 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | N/A | 5 | N/A | 1500 | 31.5 | IMD2 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | IMD5 |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | IMD5 |
| DC_1A-41A_n78A | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1955 | 5 | 25 | 2145 | 8.7 | IMD4 |
|  | 41 | 2507.5 | 10 | 50 | 2507.5 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
| DC_2A_n5A-n77A | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | IMD3 |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 2 | N/A | 5 | N/A | 1944 | 16.0 | IMD3 |
| DC_2A-13A_n77A | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_2A-66A_n2A | 2 | N/A | 5 | N/A | 1980 | 20 | IMD3 |
|  | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 22.6 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |
|  | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | N/A | 5 | N/A | 2115 | 29.2 | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | N/A | 5 | N/A | 2140 | 10.4 | IMD4 |
| DC_2A-66A_n77A | n77 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | M/A | N/A |
|  | 66 | N/A | 5 | N/A | 2175 | 4.0 | IMD5 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
|  | 2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD2X |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
| DC_3A-7A_n28A DC_3C-7A_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
| DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 8 | N/A | 5 | N/A | 955 | 9.7 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-18A_n77A DC_3A-18A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-19A_n78A | 3 | N/A | 5 | N/A | 1850 | 17.3 | IMD3 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | [18.5] | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 5.5 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 3 | 1733 | 5 | 25 | 1828 | 9.4 | IMD4 |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
| DC_3A-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1862.5 | 30.8 | IMD2 |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | N/A | N/A |
|  | n78 | 3322 | 10 | 50 | 3322 | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |
|  | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A-41A_n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |
| DC_3A-SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD34 |
| DC_7A-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 | 814 | N/A | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 |
|  | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
| DC_7A-20A_n28A | 20 | 842 | 5 | 25 | 801 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 7 | 2520 | 10 | 50 | 2640 | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
|  | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_7A-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3421 | N/A | N/A |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | N/A |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 768 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1896 | 5 | 25 | 1976 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3460 | 17.3 | IMD3 |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 16.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
| DC_13A-66A_n2A | 66 | N/A | 5 | N/A | 2156 | 7..2 | IMD4 |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
| DC_13A-66A_n77A | 66 | N/A | 5 | N/A | 2156 | 17.1 | IMD3 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | 16.0 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
|  | 19 | N/A | 5 | N/A | 882.5 | 13.2 | IMD4 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3468.7 | 10 | 50 | 3468.7 | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_28A_n7A-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_28A-42A_n79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |
| DC_20A_n28A-n78A DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |
|  | n2 | N/A | 5 | N/A | 1960 | 32.1 | IMD2 |
| DC_66A_n2A-n77A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_66A_n5A-n77A | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3460 | 16.6 | IMD39 |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 3: This band is subject to IMD3 also which MSD is not specified. NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 5 to NOTE 8: FFS NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.2-1a: MSD test points for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1807.5 | 37.2 | IMD21 |
| DC_1A_3A-n78A | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2125 | 17.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2140 | 24.6 | IMD5 |
| DC_1A-3A_n79A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 10 | 50 | 4860 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2130 | 26.7 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
| DC_1A-19A_n78A | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 19 | N/A | 5 | N/A | 880 | 18.5 | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | N/A | 5 | N/A | 882.5 | 33.3 | IMD35 |
| DC_1A-19A_n79A | n79 | 4782.5 | 10 | 50 | 4782.5 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2140 | 26.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 10 | 50 | 4652.5 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2154.6 | 36.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2154.6 | 16.2 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
| DC_1A-21A_n78A | n78 | 3647 | 10 | 50 | 3647 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | N/A | 5 | N/A | 1500 | 37.5 | IMD2 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | N/A | 5 | N/A | 1500 | 14.9 | IMD5 |
|  | n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-21A_n79A7,8 | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
| DC_1A-42A_n79A | 42 | N/A | 5 | N/A | 3490 | 25.8 | IMD5 |
|  | n79 | 4420 | 10 | 50 | 4420 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
| DC_1A_n78A-n79A | n79 | N/A | 10 | N/A | 4870 | 24.9 | IMD31 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3490 | 22.6 | IMD5 |
|  | n79 | 4670 | 10 | 50 | 4670 | N/A | N/A |
|  | 2 | N/A | 5 | N/A | 1944 | 24.2 | IMD3 |
| DC_2A-13A_n77A | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n77 | 3510 | 10 | 50 | 3510 | N/A | N/A |
|  | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | N/A | 5 | N/A | 2115 | 34.7 | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | M/A | N/A |
|  | 66 | N/A | 5 | N/A | 2140 | 21.1 | IMD41 |
| DC_2A-66A_n77A | n77 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 2 | N/A | 5 | N/A | 1960 | 37.6 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 2 | N/A | 5 | N/A | 1940 | 19.8 | IMD41,2 |
|  | 66 | 1775 | 5 | 25 | 2195 | N/A | N/A |
|  | n77 | 3385 | 10 | 50 | 3385 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1850 | 26.3 | IMD3 |
| DC_3A-19A_n78A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | N/A | 5 | N/A | 885 | 27.5 | IMD35 |
| DC_3A-19A_n79A | n79 | 4435 | 40 | 50 | 4435 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1877.5 | 16.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 50 | 4420 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1862.5 | 36.6 | IMD2 |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | N/A | N/A |
|  | n78 | 3322 | 10 | 50 | 3322 | N/A | N/A |
| DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 23.2 | IMD4 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | N/A | 5 | N/A | 1503.5 | 9.5 | IMD5 |
|  | n78 | 3403 | 10 | 50 | 3403 | N/A | N/A |
|  | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |
| DC_3A-21A_n79A7 | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1869.2 | 32.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 10 | 50 | 4770 | N/A | N/A |
|  | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-42A_n79A9 | 42 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| DC_3A_n78A-n79A | n79 | N/A | 10 | N/A | 4910 | 25.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3710 | 25.2 | IMD5 |
|  | n79 | 4510 | 10 | 50 | 4510 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
| DC_13A_n2A-n77A | n2 | N/A | 5 | N/A | 1960 | 25.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | N/A | 5 | N/A | 2156 | 25.3 | IMD3 |
| DC_13A-66A_n77A | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 13 | N/A | 5 | N/A | 750 | 23.4 | IMD32 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 19 | N/A | 5 | N/A | 882.5 | 27.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
| DC_19A-21A_n78A | n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
|  | 19 | N/A | 5 | N/A | 882.5 | 25.2 | IMD4 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3468.7 | 10 | 50 | 3468.7 | N/A | N/A |
|  | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_19A-21A_n79A7 | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | N/A | 5 | N/A | 1500 | 24.8 | IMD5 |
|  | n79 | 4850 | 10 | 50 | 4850 | N/A | N/A |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_19A-42A_n79A10 | 42 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
| DC_19A_n78A-n79A | n79 | N/A | 40 | N/A | 4515 | 35.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3715 | 34.8 | IMD2 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_21A-42A_n79A10 | 42 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
| DC_21A_n78A-n79A | n79 | N/A | 10 | N/A | 4873 | 36.1 | IMD25 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3487 | 38.8 | IMD2 |
|  | n79 | 4940 | 10 | 50 | 4940 | N/A | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 37.6 | IMD2 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
| DC_66A_n2A-n77A | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 21.1 | IMD41,2 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: Void NOTE 5: This band is subject to IMD4 also which MSD is not specified. NOTE 6: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 7: The frequency range in band n79 is restricted for this band combination to 4400 - 4900 MHz for both the UL and the DL. NOTE 8: The frequency range in band 1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130 - 2150 MHz for the DL. NOTE 9: The frequency range in band n79 is restricted for this band combination to 4500 - 5000 MHz for both the UL and the DL NOTE 10: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz for both the UL and the DL NOTE 11: Void |  |  |  |  |  |  |  |

<Unchanged Sections Skipped>

##### 7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)

Editor's note:	The test requirements section of this test case contains some EN-DC configurations that have no test points defined meaning these cannot be tested.

Editor's note:	For EN-DC configurations with >1NR CC, test points in Table 7.3B.2.3_1.1.4.1-0 only need to be tested if the configuration is a max NR CC of the UE, where detailed execution rules are FFS.

<Unchanged Sections Skipped>

7.3B.2.3_1.1.4	Test description

7.3B.2.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.1.4.1-0 and 7.3B.2.3_1.1.4.1-1.

Table 7.3B.2.3_1.1.4.1-0: Test Configuration Table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | DC_XA_nYA-nZA Mid range for MCG and SCG DC_XA_nYC: Mid range for XA Low range, High Range for nXC DC_(n)XCA: Low range, High Range DC_XA_nY(2A): TBD |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB" columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | E-UTRA |  |  |  | EUTRA/NR |  |  |  |  | NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range |  | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc | UL MOD | DL MOD |  | UL/DL Ch BW | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | X | default |  |  |  | nX | default |  |  |
|  | QPSK | QPSK | Highest | All RBs / REFSENS_LTE | N/A | QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA-XA_nXA/ DC_XA_(n)XAA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  |  | nY | default |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| 2 | X | Mid |  |  | nZ | default |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Note 1: LTE anchor agnostic configuration Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: LTE anchor agnostic configuration with LTE CA where LTE CA fallback to 1CC is sufficient to test Note 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 5: For UE supporting multiple EN-DC configurations with the same NR CA combination, only one EN-DC configuration is tested. Note 6: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions  (three bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “NR NRB” and “E-UTRA NRB” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | E-UTRA |  |  |  |  |  |  | EUTRA/NR |  |  |  |  |  |  |  |  |  |  |  |  |  | NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  |  |  | Band |  | Range |  |  |  | NRB |  |  |  |  |  |  |  | Band |  |  | Range |  | NRB |  |  |  |
|  | UL MOD | DL MOD |  | CH BW |  | DLalloc/UL alloc |  | UL MOD |  | DL MOD |  |  |  | UL/DL Ch BW |  |  |  |  |  | DLalloc/ ULalloc |  | UL MOD |  |  | DL MOD |  | UL/DL Ch BW |  | DLalloc/UL alloc |  |
| Default Test Settings for a DC_XA_nYC, DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_1A-3A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1975 / DL 2165 MHz |  |  |  |  |  | 3 |  | DL 1818.5 MHz |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs |  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / All RBs |  |  |
| 2 | 3 | UL 1780 / DL 1875 MHz |  |  |  |  |  | 1 |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs |  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / 0 |  | DFT-s- OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / All RBs |  |  |
| Test Settings for DC_1A-7A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1935 / DL 2125 MHz |  |  |  |  |  | 7 |  | DL 2653 MHz |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 718 / DL 773 MHz |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs |  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-3A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  | 3 |  | DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3757.5 MHz |  |  |  |  |  |
|  | QPSK |  | QPSK |  | 5 MHz | All RBs / All RBs |  | N/A |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  | 3 |  | DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3980 MHz |  |  |  |  |  |
|  | QPSK |  | QPSK |  | 5 MHz | All RBs / All RBs |  | N/A |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 1 |  | DL 2140 MHz |  |  |  |  | 3 |  | UL 1775 MHz / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3915 MHz |  |  |  |  |  |
|  | N/A |  | QPSK |  | 5 MHz | All RBs / 0 |  | QPSK |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-3A_n78A Configuration (PC3 & PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 3 |  |  | DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3757.5 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 211 | 1 | DL 2125 MHz |  |  |  |  | 3 |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3725 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-3A_n79A Configuration (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2140 MHz |  |  |  |  | 3 |  |  | UL 1750 / DL 1845 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4860 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-3A_n79A Configuration (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2140 MHz |  |  |  |  | 3 |  |  | UL 1750 / DL 1845 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4860 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-5A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2122 MHz |  |  |  |  | 5 |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3780 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1975 / DL 2165 MHz |  |  |  |  | 5 |  |  | DL 885 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3405 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1977.5 / DL 2167.5 MHz |  |  |  |  | 7 |  |  | DL 2627.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3305 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | DL 2140 MHz |  |  |  |  | 7 |  |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3580 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 10MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-18A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2120 MHz |  |  |  | 18 |  |  | UL 825 / DL 870 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3770 MHz |  |  |  |  |  |  |
|  | N/A |  | QPSK | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  | All RBs / All RBs |
| Test Settings for DC_1A-19A_n78A Configuration (PC2 & PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 1 |  | DL 2130 MHz |  |  |  | 19 |  |  | UL 832.5 / DL 877.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3795 MHz |  |  |  |  |  |  |
|  | N/A |  | QPSK | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  | All RBs / All RBs |
| 211 | 1 |  | UL 1940 / DL 2130 MHz |  |  |  | 19 |  |  | DL 880 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3350 MHz |  |  |  |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  | All RBs / All RBs |
| Test Settings for DC_1A-19A_n79A Configuration (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  | 19 |  |  | DL 882.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4782.5 MHz |  |  |  |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  | All RBs / All RBs |
| 2 | 1 |  | DL 2140 MHz |  |  |  | 19 |  |  | UL 837.5 / DL 882.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4652.5 MHz |  |  |  |  |  |  |
|  | N/A |  | QPSK | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  | All RBs / All RBs |
| Test Settings for DC_1A-19A_n79A Configuration (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  | 19 |  |  | DL 882.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4782.5 MHz |  |  |  |  |  |  |
|  | QPSK |  | QPSK | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  | All RBs / All RBs |
| 2 | 1 |  | DL 2140 MHz |  |  |  | 19 |  |  | UL 837.5 / DL 882.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4652.5 MHz |  |  |  |  |  |  |
|  | N/A |  | QPSK | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  | All RBs / All RBs |
| Test Settings for DC_1A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1925/ DL 2115 MHz |  |  |  |  | 20 |  |  | DL 805 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2120 MHz |  |  |  |  | 20 |  |  | UL 835 / DL 794 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3790 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3330 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-21A_n78A Configuration (PC2 & PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 1 | DL 2154.6 MHz |  |  |  |  | 21 |  |  | UL1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3605 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 211 | 1 | DL 2154.6 MHz |  |  |  |  | 21 |  |  | UL1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3647 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 311 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 21 |  |  | DL 1500 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3450 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 411 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | 21 |  |  | DL 1500 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3675 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-28A_n3A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | DL 2139 MHz |  |  |  |  | 28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A_n28A-n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n78 |  |  | 3416 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 733 / DL 788 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n28 |  |  | DL 790 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3320 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-28A_n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | UL 740 / DL 795 MHz |  |  |  |  | 1 |  |  | DL 2150 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3630 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | UL 1970 / DL 2160 |  |  |  |  | 28 |  |  | DL 794 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3352 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-41A_n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1970 / DL 2160 MHz |  |  |  |  | 41 |  |  | 2510 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3400 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 1 | DL 2140 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3710 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 1 | UL 1930 / DL 2120 MHz |  |  |  |  | 41 |  |  | 2510 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 4150 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-41A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1935 / DL 2125 MHz |  |  |  |  | 41 |  |  | 2653 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 718 / DL 773 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-42A_n79A (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1977.5 / DL 2167.5 MHz |  |  |  |  | 42 |  |  | DL 3490 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A-42A_n79A (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1977.5 / DL 2167.5 MHz |  |  |  |  | 42 |  |  | DL 3490 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A_n78A-n79A (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n78 |  |  | 3410 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4870 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| 2 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n78 |  |  | 3490 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4670 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_1A_n78A-n79A (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n78 |  |  | 3410 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4870 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / 0 |  |
| 2 | 1 | UL 1950 / DL 2140 MHz |  |  |  |  | n78 |  |  | 3490 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4670 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A_n5A-n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1880 / DL 1960 MHz |  |  |  |  | n5 |  |  | UL 830 / DL 875 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3540 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| Test Settings for DC_2A-13A_n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 2 | DL 1944 MHz |  |  |  |  | 13 |  |  | UL 783 / DL 752 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3510 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-66A_n2A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1900 / DL 1980 MHz |  |  |  |  | 66 |  |  | UL 1730 / DL 2130 MHz |  |  |  |  |  |  |  |  |  |  | n2 |  |  | UL 1855 / DL 1935 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-66A_n5A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1900 / DL 1980 MHz |  |  |  |  | 66 |  |  | UL 1740 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 830 / DL 875 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-66A_n41A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | UL 1860 / DL 1940 MHz |  |  |  |  | 66 |  |  | UL 1715 / DL 2115 MHz |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2685 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-66A_n77A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 2 | UL 1855 / DL 1935 MHz |  |  |  |  | 66 |  |  | DL 2115 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3970 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 211 | 2 | UL 1880 / DL 1960 MHz |  |  |  |  | 66 |  |  | DL 2140 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3500 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 2 | UL 1860 / DL 1940 MHz |  |  |  |  | 66 |  |  | DL 2175 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3915 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 411 | 2 | DL 1960 MHz |  |  |  |  | 66 |  |  | UL 1760 / DL 2160 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3720 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n5A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1780/ DL 1875 MHz |  |  |  |  | 7 |  |  | DL 2625 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 845 / DL 890 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n8A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1780/ DL 1875 MHz |  |  |  |  | 7 |  |  | DL 2670 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 890 / DL 935 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1712.5 / DL 1807.5 MHz |  |  |  |  | 7 |  | DL 2682 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |
|  | QPSK | QPSK |  |  | 5 MHz | All RBs / All RBs | N/A |  | QPSK |  |  | 10 MHz |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2543 / DL 2663 MHz |  |  |  |  | 3 |  | DL 1832.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |
|  | QPSK | QPSK |  |  | 10 MHz | All RBs / All RBs | N/A |  | QPSK |  |  | 5 MHz |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1820 MHz |  |  |  |  | 7 |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1820 MHz |  |  |  |  | 7 |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3475 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-8A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1820 MHz |  |  |  |  | 8 |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3640 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-19A_n78A Configuration (PC2 & PC3)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 3 | DL 1850 MHz |  |  |  |  | 8 |  |  | UL 835 / DL 880 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3520 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-19A_n79A Configuration (PC2 & PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 3 | UL 1775 / DL 1870 MHz |  |  |  |  | 19 |  |  | DL 885 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4435 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| 211 | 3 | DL 1877.5 MHz |  |  |  |  | 19 |  |  | UL 842.5 / DL887.5 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4420 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1860 MHz |  |  |  |  | 20 |  |  | UL 840 MHz / DL 799 |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 900 / DL 945 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1828 MHz |  |  |  |  | 20 |  |  | UL 852 / DL 811 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1820 MHz |  |  |  |  | 20 |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3510 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-21A_n78A Configuration (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1767.5 / DL 1862.5 MHz |  |  |  |  | 21 |  |  | DL 1507.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3795 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1862.5 |  |  |  |  | 21 |  |  | UL 1459.5 / DL 1507.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3322 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-21A_n78A Configuration (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1767.5 / DL 1862.5 MHz |  |  |  |  | 21 |  |  | DL 1507.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3795 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1862.5 |  |  |  |  | 21 |  |  | UL 1459.5 / DL 1507.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3322 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 3 | UL 1767.5 / DL 1862.5 MHz |  |  |  |  | 21 |  |  | DL 1503.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3403 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-21A_n79A Configuration (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1869.2 MHz |  |  |  |  | 21 |  |  | UL 1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4770 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-21A_n79A Configuration (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1869.2 MHz |  |  |  |  | 21 |  |  | UL 1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4770 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | UL 740 / DL 760 MHz |  |  |  |  | 3 |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3350 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1750/ DL 1845 MHz |  |  |  |  | n78 |  |  | 3764 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-40A_n1A Configuration – Note 3, 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1735 / DL 1830 MHz |  |  |  |  | 40 |  |  | DL 2380 MHz |  |  |  |  |  |  |  |  |  |  | n1A |  |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-41A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | DL 1832.5 MHz |  |  |  |  | 41 |  |  | 2543 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 3 | UL 1780 / DL 1875 MHz |  |  |  |  | 41 |  |  | 2518 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 738 / DL 793 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 3 | 3 | UL 1715 / DL 1810 MHz |  |  |  |  | 41 |  |  | 2687 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A-41A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1720 / DL 1815 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3900 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 3 | DL 1840 MHz |  |  |  |  | 41 |  |  | 2620 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3400 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A_n78A-n79A (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1770 / DL 1865 MHz |  |  |  |  | n78 |  |  | 3340 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4910 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| 2 | 3 | UL 1770 / DL 1865 MHz |  |  |  |  | n78 |  |  | 3710 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4510 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_3A_n78A-n79A (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | UL 1770 / DL 1865 MHz |  |  |  |  | n78 |  |  | 3340 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4910 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / 0 |  |
| 2 | 3 | UL 1770 / DL 1865 MHz |  |  |  |  | n78 |  |  | 3710 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4510 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_5A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 | UL 844 / DL 889 MHz |  |  |  |  | 7 |  |  | DL 2645 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3489 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 5 | DL 879 MHz |  |  |  |  | 7 |  |  | UL 2550 / DL 2670 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3429 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 5 | DL 875 MHz |  |  |  |  | 7 |  |  | UL 2525 / DL 2645 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3350 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-8A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2530 / DL 2650 MHz |  |  |  |  | 8 |  |  | DL 940 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1735 / DL 1830 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2670 MHz |  |  |  |  | 8 |  |  | UL 890 / DL 935 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n1A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2510 / DL 2630 MHz |  |  |  |  | 20 |  |  | 800 MHz |  |  |  |  |  |  |  |  |  |  | n1 |  |  | UL 1940 / DL 2130 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2543 / DL 2663 MHz |  |  |  |  | 20 |  |  | DL 806 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1737 / DL 1832 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 10 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2630 MHz |  |  |  |  | 20 |  |  | UL 855 / DL 896 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 10 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2565 / DL 2685 MHz |  |  |  |  | 20 |  |  | DL 795 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 885 / DL 930 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2640 MHz |  |  |  |  | 20 |  |  | UL 840 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 900 / DL 945 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 3 | 7 | DL 2624 MHz |  |  |  |  | 20 |  |  | UL 857 / DL 816 MHz |  |  |  |  |  |  |  |  |  |  | n8 |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | UL 842/ DL 801 MHz |  |  |  |  | 7 |  |  | DL 2640 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2560 / DL 2680 MHz |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3370 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2560 / DL 2680 MHz |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3435 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 7 | DL 2675 MHz |  |  |  |  | 20 |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3520 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-28A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2543 / DL 2663 MHz |  |  |  |  | 28 |  |  | DL 796 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | 1842 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2685 MHz |  |  |  |  | 28 |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  | n3 |  |  | 1810 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-28A_n5A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2540 / DL 2660 MHz |  |  |  |  | 28 |  |  | DL 776 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | DL 2630 MHz |  |  |  |  | 28 |  |  | UL 730 / DL 785 MHz |  |  |  |  |  |  |  |  |  |  | n5 |  |  | UL 840 / DL 874 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2567.5 / DL 2687.5 MHz |  |  |  |  | 28 |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3350 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2567.5 / DL 2687.5 MHz |  |  |  |  | 28 |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3460 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| 3 | 7 | DL 2650 MHz |  |  |  |  | 28 |  |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3390 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_7A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | UL 2565/ DL 2685 MHz |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 7 | UL 2565/ DL 2685 MHz |  |  |  |  | n28 |  |  | DL 800 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3365 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_2A-14A_n66A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | DL 1954 MHz |  |  |  |  | 14 |  |  | UL 793 / DL 763 MHz |  |  |  |  |  |  |  |  |  |  | n66 |  |  | UL 1770 / DL 2170 MHz |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_13A_n2A-n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | UL 782 / DL 751 MHz |  |  |  |  | n2 |  |  | UL 1896 / DL 1976 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3460 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| 211 | 13 | UL 782 / DL 751 MHz |  |  |  |  | n2 |  |  | DL 1960 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3524 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_13A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 13 | UL 782 / DL 751 MHz |  |  |  |  | 66 |  |  | DL 2156 MHz |  |  |  |  |  |  |  |  |  |  | n2 |  |  | UL 1860 / DL 1940 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_13A-66A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 13 | UL 782 / DL 751 MHz |  |  |  |  | 66 |  |  | DL 2156 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3720 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_14A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 | DL 763 MHz |  |  |  |  | 66 |  |  | UL 1762 / DL 2162 MHz |  |  |  |  |  |  |  |  |  |  | n2 |  |  | UL 1874 / DL 1954 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_18A-41A_n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 18 | DL 865 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3527.5 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| Test Settings for DC_18A-41A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 18 | DL 865 MHz |  |  |  |  | 41 |  |  | 2640 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3527.5 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| Test Settings for DC_19A-21A_n78A Configuration (PC2 & PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 19 | DL 882.5 MHz |  |  |  |  | 21 |  |  | UL 1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3783.3 |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| 211 | 19 | DL 882.5 MHz |  |  |  |  | 21 |  |  | UL 1450.4 / DL 1498.4 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3468.7 |  |  |  |  |  |  |
|  | N/A | QPSK |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| Test Settings for DC_19A-21A_n79A Configuration (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | UL 837.5 / DL 882.2 MHz |  |  |  |  | 21 |  |  | DL 1500 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4850 |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40MHz |  | All RBs / All RBs |  |
| Test Settings for DC_19A-21A_n79A Configuration (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | UL 837.5 / DL 882.2 MHz |  |  |  |  | 21 |  |  | DL 1500 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4850 |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40MHz |  | All RBs / All RBs |  |
| Test Settings for DC_19A_n78A-n79A (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | UL 835/ DL 880 MHz |  |  |  |  | n78 |  |  | 3680 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4515 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / 0 |  |
| 2 | 19 | UL 835/ DL 880 MHz |  |  |  |  | n78 |  |  | 3715 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4550 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_19A_n78A-n79A (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 19 | UL 835/ DL 880 MHz |  |  |  |  | n78 |  |  | 3680 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4515 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / 0 |  |
| 2 | 19 | UL 835/ DL 880 MHz |  |  |  |  | n78 |  |  | 3715 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4550 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_20A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | UL 857 / DL 816 MHz |  |  |  |  | n78 |  |  | 3314 MHz |  |  |  |  |  |  |  |  |  |  | n28 |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 20 | UL 837 / DL 796 MHz |  |  |  |  | n28 |  |  | UL 744 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10MHz |  | All RBs / All RBs |  |
| Test Settings for DC_20A_n28A-n75A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Mid |  |  |  |  | n28 |  |  | High |  |  |  |  |  |  |  |  |  |  | n75 |  |  | DL 1476MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 20MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 | N/A |  |  | CP-OFDM QPSK |  |  | 20MHz |  | All RBs / N/A |  |
| 2Note 8 | 20 | Mid |  |  |  |  | n28 |  |  | Low |  |  |  |  |  |  |  |  |  |  | n75 |  |  | High |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 20MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 | N/A |  |  | CP-OFDM QPSK |  |  | 20MHz |  | All RBs / N/A |  |
| Test Settings for DC_21A_n78A-n79A (PC2) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | UL 1453/ DL 1501 MHz |  |  |  |  | n78 |  |  | 3420 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4873 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| 2 | 21 | UL 1453/ DL 1501 MHz |  |  |  |  | n78 |  |  | 3487 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4940 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_21A_n78A-n79A (PC3) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | UL 1453/ DL 1510 MHz |  |  |  |  | n78 |  |  | 3420 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4873 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | DFT-s-OFDM QPSK |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / 0 |  |
| 2 | 21 | UL 1453/ DL 1510 MHz |  |  |  |  | n78 |  |  | 3487 MHz |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4940 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 40 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_28A_n7A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | UL 745 / DL 8005 MHz |  |  |  |  | n78 |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  | n7 |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |  |
| 2 | 28 | UL 745 / DL 8005 MHz |  |  |  |  | n7 |  |  | DL 2650 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3390 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_66A_n2A-n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 111 | 66 | UL 1760 / DL 2160 MHz |  |  |  |  | n2 |  |  | DL 1960 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3720 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |  |
| Test Settings for DC_66A_n5A-n77A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | UL 1770 / DL 2170 MHz |  |  |  |  | n5 |  |  | UL 845 / DL 890 MHz |  |  |  |  |  |  |  |  |  |  | n77 |  |  | 3460 MHz |  |  |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / All RBs | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs | N/A |  |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / 0 |  |
| Note 1: Void Note 2: Void Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for to UEs not supporting UE capability singleUL-Transmissionn for the correspondent uplink configuration. Note 7: NR CA fallback to 1CC is sufficient to test, unless both NR cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 8: Test ID with UL harmonic exception avoided. Note 9: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 10: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE 11: This test ID is for both UL PC3 and PC2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.1.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with Note 3 in Table 7.3B.2.3_1.1.4.1-1, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3_1.1.5	Test requirement

Reference sensitivity test requirements for EN-DC configurations affected by 3 band 2UL intermodulation interference, are specified in Table 7.3B.2.3_1.1.5-1 and Table 7.3B.2.3_1.1.5-2 with uplink configuration specified in Table 7.3B.2.3_1.1.4.1-1.

Reference sensitivity test requirements for test points in Table 7.3B.2.3_1.1.4.1-1, are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.1.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for PC3 EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 3 | N/A | -92.3 | - | - | - |  | IMD5 |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 1 | N/A | -88.3 | - | - | - |  | IMD4 |  |
| DC_1A-7A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 7 | N/A | - | -64.3 | - | - |  | IMD2 |  |
| DC_1A-3A_n77A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -64.8 | - | - | - |  | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -87.8 | - | - | - |  | IMD4 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -68.3 | - | - | - | FDD | IMD2 |  |
|  |  | 3 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-3A_n78A DC_1A-3C_n78A |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -65.1 | - | - | - |  | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -96.5 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-3A_n79A | 1 | 1 | N/A | -95.7 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_1A-5A_n78A | 1 | 1 | N/A | -91.0 | - | - | - | FDD | IMD4 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | -81.2 | - | - | - | FDD | IMD3 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -94.2 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-7A_n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 7 | N/A | -88.2 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -90.6 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | - | REFSENS | - |  | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-18A_n77A | 1 | 1 | N/A | -82.9 | - | - | - | FDD | IMD3 |  |
|  |  | 18 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-19A_n78A | 1 | 1 | N/A | -81.5 | - | - | - | FDD | IMD3 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -94.2 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-19A_n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -81.0 | - | - | - | FDD | IMD3 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 1 | N/A | -91.2 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_1A-20A_n8A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -87.8 | - | - | - | FDD | IMD4 |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-20A_n78A | 1 | 1 | N/A | -79.0 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-20A_n78A | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -93.3 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-21A_n78A | 1 | 1 | N/A | -68.7 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -95.7 | - | - | - | FDD | IMD5 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -67.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -96.4 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-28A_n3A | 1 | 28 | N/A | REFSENS |  | - | - | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS |  | - | - | FDD | N/A |  |
|  |  | 1 | N/A | -88.3 |  | - | - | FDD | IMD4 |  |
| DC_1A_n28A-n78A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -79.1 | - | - | TDD | IMD3 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n28 | 15 | -94.5 | - | - | - | FDD | IMD5 |  |
| DC_1A-28A_n78A | 1 | 1 | N/A | -83.6 | - | - | - | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -93.6 | - | - |  | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-41A_n28A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | - | -64.3 | - | - | TDD | IMD2 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-41A_n77A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -86.3 | - | - | - | TDD | IMD4 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -90.0 | - | - | - | FDD | IMD4 |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -93.7 | - | - | - | TDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A-42A_n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  |  | 42 | N/A | -93.2 | - | - | - | TDD | IMD5 |  |
| DC_1A_n78A-n79A | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -73.7+TT | TDD | IMD3 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -91.2+TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A_n5A-n77A |  | n5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | -78.3 | - | - | TDD | IMD3 |  |
|  | 1 | 2 | N/A | -81.3 | - | - | - | FDD | IMD3 |  |
| DC_2A-13A_n77A |  | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_2A-14A_n66A | 1 | 2 | N/A | -90.1 | - | - | - | FDD | IMD4 |  |
|  |  | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n66 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A-66A_n2A | 1 | 2 | N/A | -77.3 |  |  |  | FDD | IMD3 |  |
|  |  | 66 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | n2 | 15 | REFSENS |  |  |  | FDD | N/A |  |
| DC_2A-66A_n5A | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.6 | - | - | - | FDD | IMD4 |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_2A-66A_n41A | 1 | 2 | N/A | -74.4 | - | - | - | FDD | IMD4 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n41 | 15 | REFSENS | - | - | - | TDD | N/A |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -69.6 | - | - | - | FDD | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -88.4 | - | - | - | FDD | IMD4 |  |
| DC_2A-66A_n77A |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -94.8 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 2 | N/A | -65.2 | - | - | - | FDD | IMD2 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-5A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -70.3 | - | - | - | FDD | IMD2 |  |
|  |  | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 3 | N/A | -88.3 | - | - | - | FDD | IMD4 |  |
|  |  |  |  | [TBD] |  |  | 10.75 |  |  |  |
| DC_3A-7A_n5A | 1 | 3 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 7 | N/A | - | - 64.3 | - | - | FDD | IMD2XX |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_3A-7A_n28A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 7 | N/A | - | -77.4 | - | - |  | IMD3 |  |
|  | 2 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - |  | N/A |  |
|  |  | 3 | N/A | -70.3 | - | - | - |  | IMD2 |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A | 1 | 3 | N/A | -78.7 | - | - | - | FDD | IMD3 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -87.7 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-8A_n78A | 1 | 3 | N/A | -79.8 | - | - | - | FDD | IMD3 |  |
|  |  | 8 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-19A_n78A | 1 | 3 | N/A | -82.0 | - | - | - | FDD | IMD3 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-19A_n79A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -80.8 | - | - | - | FDD | IMD3 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 3 | N/A | -90.8 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A-20A_n28A | 1 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -86.9 | - | - | - | FDD | IMD4 |  |
| DC_3A-20A_n78A | 1 | 3 | N/A | -79.0 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-21A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -90.5 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -65.5 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-21A_n79A | 1 | 3 | N/A | -78.5 | - | - | - | FDD | IMD3 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A_n28A-n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -90.3 | - | - | TDD | IMD5 |  |
| DC_3A-28A_n78A | 1 | 3 | N/A | -79 | - | - | - | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A-40A_n1A | 1 | n1 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 40 | N/A | -91.3 | - | - | - | TDD | IMD5 |  |
| DC_3A-41A_n28A | 1 | 3 | N/A | -70.3 | - | - | - | FDD | IMD2 |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n28 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -69.9 | - | - | - | TDD | IMD2 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | TDD | N/A |  |
|  | 3 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 41 | N/A | -81.4 | - | - | - | TDD | IMD3 |  |
|  |  | n28 | 15 | REFSENS | - | - | - | TDD | N/A |  |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | -92 | - | - | - | TDD | IMD5 |  |
|  | 2 | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 3 | N/A | -79.9 | - | - | - | FDD | IMD3 |  |
| DC_3A_n78A-n79A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -73.3+TT | TDD | IMD3 |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -91.6+TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_5A-7A_n78A | 1 | 5 | N/A | -89.0 | - | - | - | FDD | IMD4 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 5 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 7 | N/A | -67.2 | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 5 | N/A | -67.1 | - | - | - | FDD | IMD2 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 5 | N/A | -94.0 | - | - | - | FDD | IMD5 |  |
|  |  | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A-8A_n3A | 1 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 8 | N/A | -78.3 | - | - | - | FDD | IMD3 |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | - | -65.3 | - | - | FDD | IMD2+ IMD3YY |  |
|  |  | 8 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n1A | 1 | 7 |  |  | REFSENS |  |  | FDD | N/A |  |
|  |  | 20 |  |  | -88.8 |  |  | FDD | IMD5 |  |
|  |  | n1 |  | REFSENS |  |  |  | FDD | N/A |  |
| DC_7A-20A_n3A | 1 | 7 | N/A | - | REFSENS | - | - | FDD | N/A |  |
|  |  | 20 | N/A | - | -82.8 | - | - | FDD | IMD2 |  |
|  |  | n3 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | - | -68.3 | - | - | FDD | IMD2 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n3 | 15 | - | REFSENS | - | - | FDD | N/A |  |
| DC_7A-20A_n8A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -78.9 | - | - | - | FDD | IMD3 |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | -76.2 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 3 | 7 | N/A | -78.5 | - | - | - | FDD | IMD3 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n8 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n28A | 1 | 7 | N/A | - | -91.4 | - | - | FDD | IMD5 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-20A_n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -65.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 20 | N/A | -93.3 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 7 | N/A | -66.5 | - | - | - | FDD | IMD2 |  |
|  |  | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A-28A_n3A | 1 | 7 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | 28 | N/A | -77.8 |  |  |  | FDD | IMD2 |  |
|  |  | n3 | 15 | REFSENS |  |  |  | FDD | N/A |  |
|  | 2 | 7 | N/A | -79.3 |  |  |  | FDD | IMD3 |  |
|  |  | 28 | N/A | REFSENS |  |  |  | FDD | N/A |  |
|  |  | n3 | 15 | REFSENS |  |  |  | FDD | N/A |  |
| DC_7A-28A_n5A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -93.4 | - | - | - | FDD | IMD5 |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 2 | 7 | N/A | -91.9 | - | - | - | FDD | IMD5 |  |
|  |  | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_7A-28A_n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -89.5 | - | - | - |  | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 28 | N/A | -94.8 | - | - | - |  | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 7 | N/A | -66.8 | - | - | - | FDD | IMD2 |  |
|  |  | 28 | N/A | REFSENS | - | - | - |  | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_7A_n28A-n78A | 1 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -65.1 | - | - | TDD | IMD2 |  |
|  | 2 | 7 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | -69.0 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A_n2A-n77A |  | n77 | 15 | - | -77 | - | - | TDD | IMD3 |  |
|  | 2 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | -81 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n2A |  | 66 | N/A | -91.3 | - | - | - | FDD | IMD4 |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n77A |  | 66 | N/A | -81.4 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | FDD | N/A |  |
| DC_18A-41A_n77A DC_18A-41C_n77A | 1 | 18 | N/A | -95.9 | - | - | - | FDD | IMD5 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 1 | 18 | N/A | -95.9 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | 41 | N/A | REFSENS | - | - | - | TDD | N/A |  |
| DC_19A-21A_n78A | 1 | 19 | N/A | -80.6 | - | - | - | FDD | IMD3 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 19 | N/A | -86.1 | - | - | - | FDD | IMD4 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_19A-21A_n79A | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -95.5 | - | - | - | FDD | IMD5 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_19A_n78A-n79A | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -60.3+TT | TDD | IMD2 |  |
|  | 2 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -67.0+TT | - | - | TDD | IMD2 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_20A_n28A-n78A | 1 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n28 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -86.1 | - | - | TDD | IMD4 |  |
|  | 2 | 20 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n28 | 15 | -89.1+TT | - | - | - | FDD | IMD4 |  |
| DC_21A_n78A-n79A | 1 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  |  | n79 | 15 | - | - | - | -59.5+TT | TDD | IMD2 |  |
|  | 2 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -66.0+TT | - | - | TDD | IMD2 |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_28A_n7A-n78A | 1 | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n7 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -65.1 | - | - | TDD | IMD2 |  |
|  | 2 | 28 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n7 | 15 | -66.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_14A-66A_n2A | 1 | 14 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -91.2 | - | - | - | FDD | IMD4 |  |
|  |  | n2 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  | 1 | n2 | 15 | -65.2 | - | - | - | FDD | IMD2 |  |
| DC_66A_n2A-n77A |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_66A_n5A-n77A |  | n5 | 15 | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | -77.7 | - | - | TDD | IMD39 |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 6: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the Band 46. The reference sensitivity should only be verified when this is not the case (the requirements for Band 46 specified in the CA_7A-46A in clause 7.3.1 of TS 36.101 [5] apply). NOTE 7: This band is subject to IMD3 also which MSD is not specified. NOTE 8: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.5-1a: Reference sensitivity exceptions for Scell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 3 | N/A | -59.1 | - | - | - | FDD | IMD2 |  |
| DC_1A-3A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -81.5 | - | - | - | FDD | IMD5 |  |
|  |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | -74.7 | - | - | - | FDD | IMD5 |  |
| DC_1A-3A_n79A |  | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | -72.6 | - | - | - | FDD | IMD3 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-19A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -80.8 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -66.0 | - | - | - | FDD | IMD3 |  |
| DC_1A-19A_n79A |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -73.2 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | -62.7 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 1 | N/A | -83.1 | - | - | - | FDD | IMD5 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-21A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -61.8 | - | - | - | FDD | IMD2 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -84.4 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_1A_n78A-n79A |  | n79 | 15 | - | +TT | - | - | TDD | IMD3 |  |
|  | 2 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | +TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 1 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_1A-42A_n79A |  | 42 | N/A | -72.2 | - | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 2 | N/A | -98.0 +TT+24.2 | - | - | - | FDD | IMD3 |  |
| DC_2A-13A_n77A |  | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -99.5 +TT+34.7 | - | - | - | FDD | IMD2 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 2 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 66 | N/A | -99.5 +TT+21.1 | - | - | - | FDD | IMD43 |  |
| DC_2A-66A_n77A |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 4 | 2 | N/A | -98.0 +TT+37.6 | - | - | - | FDD | IMD2 |  |
|  |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | -70.0 | - | - | - | FDD | IMD3 |  |
| DC_3A-19A_n78A |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 19 | N/A | -71.8 | - | - | - | FDD | IMD3 |  |
| DC_3A-19A_n79A |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
|  | 2 | 3 | N/A | -80.1 | - | - | - | FDD | IMD4 |  |
|  |  | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | - | - | REFSENS | TDD | N/A |  |
| DC_3A-21A_n78A | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -76.1 | - | - | - | FDD | IMD4 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 3 | N/A | -59.7 | - | - | - | FDD | IMD2 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 3 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | 21 | N/A | -89.8 | - | - | - | FDD | IMD5 |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | -66.5 | - | - | - | FDD | IMD3 |  |
| DC_3A-21A_n79A |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_3A_n78A-n79A |  | n79 | 15 | - | -70.5+TT | - | - | TDD | IMD3 |  |
|  | 2 | 3 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -70.6+TT | - | - | TDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_13A_n2A-n77A | 2 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n2 | 15 | -98.0 +TT+25.0 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 13 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_13A-66A_n77A |  | 66 | N/A | -99.5+ TT+25.3 | - | - | - | FDD | IMD3 |  |
|  |  | n77 | 15 | - | REFSENS | - | - | FDD | N/A |  |
|  | 1 | 19 | N/A | -71.6 | - | - | - | FDD | IMD3 |  |
|  |  | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_19A-21A_n78A |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 2 | 19 | N/A | -74.1 | - |  |  | FDD | IMD4 |  |
|  |  | 21 | N/A | REFSENS | - |  |  | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS |  |  | TDD | N/A |  |
|  | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
| DC_19A-21A_n79A |  | 21 | N/A | -74.5 | - | - | - | FDD | IMD5 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_19A_n78A-n79A |  | n79 | 15 | - | - | - | -60.5+TT | TDD | IMD2 |  |
|  | 2 | 19 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | - | - | -61.0+TT | TDD | IMD2 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| DC_21A_n78A-n79A |  | n79 | 15 | - | -59.7+TT | - | - | TDD | IMD2 |  |
|  | 2 | 21 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n78 | 15 | - | -57.0+TT | - | - | TDD | IMD2 |  |
|  |  | n79 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | 1 | n2 | 15 | -98.0+ TT+37.6 | - | - | - | FDD | IMD2 |  |
| DC_66A_n2A-n77A |  | 66 | N/A | REFSENS | - | - | - | FDD | N/A |  |
|  |  | n77 | 15 | - | REFSENS | - | - | TDD | N/A |  |
| NOTE 1: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |  |

Test tolerance is the same as given in Table 7.3B.2.3.5-2.

<Unchanged Sections Skipped>

#### 7.3B.3.3	ΔRIB,c for Inter-band EN-DC within FR1

<Unchanged Sections Skipped>

##### 7.3B.3.3.2	ΔRIB,c for EN-DC in three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | n28 | 0.2 |
| DC_1-3_n41 | n41 | 03 |
| DC_1-3_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n3-n78 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28 | n28 | 0.2 |
| DC_1-7_n78 | 1 | 0.2 |
|  | 7 or n7 | 0.2 |
|  | n78 | 0.5 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_1-8_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_1-8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18_n77 | n77 | 0.5 |
| DC_1-19_n77 | n77 | 0.5 |
| DC_1-19_n78 | n78 | 0.5 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-20_n78 | n78 | 0.5 |
| DC_1-21_n77 | n77 | 0.5 |
| DC_1-21_n78 | 1 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28_n78 DC_1_n28-n78 | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
| DC_1-41_n28 | n28 | 0.2 |
| DC_1-41_n41 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |
| DC_1-41_n77 | n77 | 0.5 |
| DC_1-41_n78 | n78 | 0.5 |
| DC_1-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-42_n78 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-42_n79 | 42 | 0.5 |
| DC_1_n77-n79 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78-n79 | n78 | 0.5 |
| DC_2-5_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.2 |
| DC_2_n5-n77 | 2 | 0.2 |
|  | n77 | 0.5 |
| DC_2-13_n66 DC_2-2-13_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | 2 | 0.2 |
| DC_2-13_n77 | 13 | 0.2 |
|  | n77 | 0.5 |
| DC_2-29_n260 | 2 | 0 |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.4 |
|  | 30 | 0.5 |
| DC_2-30_n66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_2-66_n2 | 2 | 0.3 |
| DC_2-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_2-66_n5 DC_2A-2A-66A_n5A DC_2-66-66_n5 DC_2A-2A-66A-66A_n5A | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-66_n41 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_2-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-66_n71B | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 2 | 0.3 |
| DC_2-66_n77 | 66 | 0.3 |
|  | n77 | 0.5 |
| DC_2-66_n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |
| DC_3_n1-n77 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n1-n78 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n8 | n8 | 0.2 |
| DC_3-7_n78 DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 or n7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n28 | 8 | 0.2 |
|  | n28 | 0.1 |
| DC_3-8_n77 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_3-8_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-18_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-18_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-19_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n28 | 20 | 0.1 |
|  | n28 | 0.1 |
| DC_3-20_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 28 | 0.1 |
|  | n5 | 0.1 |
|  | n41 | 01/0.52 |
| DC_3-28_n78 DC_3_n28-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-41_n28 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0 |
| DC_3-41_n41 | 41 | 03 |
|  |  | 0.54 |
|  | n41 | 03 |
|  |  | 0.54 |
| DC_3-41_n77 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| DC_3-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.52 |
| DC_5-30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_7-8_n3 | 8 | 0.2 |
| DC_7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_7-20_n78 | n78 | 0.5 |
| DC_7-28_n78 | n78 | 0.5 |
| DC_7_n28-n78 | n78 | 0.5 |
| DC_13_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| DC_13-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | 13 | 0.3 |
| DC_13-66_n77 | 66 | 0.3 |
|  | n77 | 0.5 |
| DC_14-66_n2 DC_14-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_18-41_n77 | n77 | 0.5 |
| DC_18-41_n78 | n78 | 0.5 |
| DC_19-21_n77 | n77 | 0.5 |
| DC_19-21_n78 | n78 | 0.5 |
| DC_19-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-42_n79 | 42 | 0.5 |
| DC_19_n77-n79 | n77 | 0.5 |
| DC_19_n78-n79 | n78 | 0.5 |
| DC_20_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_20-38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_20_n76-n78 | n78 | 0.5 |
| DC_21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-42_n79 | 42 | 0.5 |
| DC_21_n77-n79 | n77 | 0.5 |
| DC_21_n78-n79 | n78 | 0.5 |
| DC_28_n7-n78 | n78 | 0.5 |
| DC_28-41_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |
| DC_28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_66_n5 | 66 | 0.4 |
|  | n5 | 0.5 |
| DC_41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
|  | 66 | 0.3 |
| DC_66_n2-n77 | n2 | 0.3 |
|  | n77 | 0.5 |
| DC_66_n5-n77 | 66 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

## {End of changes}