3GPP TSG RAN WG5#104	R5-245109r1

# Maastricht, Netherlands, August 19 – August 23 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0872 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MU and TT for new eRedCap test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | MU and TT is missing for new eRedCap test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added MU and TT for test case 5.2.1.1.2, 5.2.1.2.2., 6.2.1.1.1.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eRedCap test cases cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.1.3, F.1.3.2, F.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Solving corrupted file |  |  |  |  |  |  |  |  |

#### << Start of changes >>

### F.1.1.2	Measurement of Demod Performance requirements

This clause defines the maximum test system uncertainty for Demod Performance requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.2-1.

Table F.1.1.2-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | ±2.0 | Same as in LTE |
| Signal to noise ratio uncertainty | dB | ±0.3 | Same as in LTE |
| Signal to noise ratio variation | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 1Tx | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 2Tx | dB | ±0.7 | Same as in LTE |
| Fading profile power uncertainty for 4Tx | dB | ±0.7 | Same as in LTE |

The maximum test system uncertainty for test cases defined in section 5 is defined in Table F.1.1.2-2.

Table F.1.1.2-2: Maximum test system uncertainty for FR1 demodulation performance test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |  |
| 5.2.1.1.2 1 Rx FDD FR1 PDSCH performance for eRedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |  |
| 5.2.1.2.1 1Rx TDD FR1 PDSCH performance for RedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |  |
| 5.2.1.2.2 1 Rx FDD FR1 PDSCH performance for eRedCap | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler | Same as 5.2.2.1.1_1 |  |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler ± 0.6 dB for test 1-6 ± 0.9 dB for test 1-7 | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) + SNR uncertainty due to finite test time2 Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB For test point 1-6, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For test point 1-7, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB |  |
| #### << Rows skipped here >> |  |  |  |

### F.1.1.3	Measurement of Channel State Information reporting

This clause defines the maximum test system uncertainty for channel state information reporting requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.3-1.

Table F.1.1.3-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio uncertainty | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio variation | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 1Tx | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 2Tx | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 4Tx | dB | Same as in table F.1.1.2-1 |  |

The maximum test system uncertainty for test cases defined in section 6 is defined in Table F.1.1.3-2.

Table F.1.1.3-2: Maximum test system uncertainty for FR1 channel state information reporting test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Same as 6.2.2.1.1.1 | Same as 6.2.2.1.1.1 |
| 6.2.1.1.1.2 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | Same as 6.2.2.1.1.1 | Same as 6.2.2.1.1.1 |
| 6.2.1.1.2.1 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Same as 6.2.2.1.2.1 | Same as 6.2.2.1.2.1 |
| #### << Rows skipped here >> |  |  |

#### << Clauses skipped here >>

### F.1.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 5 is defined in Table F.1.3.2-1.

Table F.1.3.2-1: Derivation of Test Requirements (FR1 demodulation performance tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.1.1.2 1 Rx FDD FR1 PDSCH performance for eRedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.1.2.1 1Rx TDD FR1 PDSCH performance for RedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.1.2.2 1Rx TDD FR1 PDSCH performance for eRedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.6 dB for test 1-6 0.9 dB for test 1-7 | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| << Rows skipped here >> |  |  |  |

### F.1.3.3	Measurement of Channel State Information reporting

The derivation of the test requirements for the test cases in section 6 is defined in Table F.1.3.3-1.

Table F.1.3.3-1: Derivation of Test Requirements (FR1 channel state information reporting tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.1.1.2 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.1.2.1 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.1.2.1.1 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| #### << Rows skipped here >> |  |  |  |

#### << End of changes >>