3GPP TSG-RAN5 Meeting #104	R5-245312r1

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0699 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eRedCap capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | eRedCap capabilities is needed for eRedCap test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added RedCap capabilities in A.4.3.x Updated A.4.3.2, A.4.3.3 and A.4.3.4 to capture eRedCap capability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eRedCap capabilities will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2 A.4.3.3 A.4.3.4 A.4.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.522 CR 0477, 0679 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: removed changes in A.4.3.x eRedCap capabilities to avoid confliction with R5-244731 Updated coversheet by adding the other spec affected CR |  |  |  |  |  |  |  |  |

<Start of changes>

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | CY |  | Mandatory for non-RedCap and non-eRedCap UEs and optional for RedCap and eRedCap UEs. |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 | No |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 | No |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 | No |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols | 38.306, 4.2.7.10 | Rel-15 | N/A | Yes | Yes | pdsch-MappingTypeA is purely mandatory |
| 6 | Support receiving PDSCH using PDSCH mapping type B | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeB | Yes |  |  |
| 7 | Support resource allocation Type 0 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_ra_Type0_PUSCH | No |  |  |

<skipped>

A.4.3.3	PDCP Implementation Capabilities

Table A.4.3.3-1: UE PDCP Implementation Capabilities

| Item | UE PDCP Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support 12 bit length of PDCP sequence number | 38.306, 4.2.4 | Rel-15 | pc_shortSN | Yes | Yes (for RedCap UE) | This PICS shall always be true for RedCap and eRedCap UE. |
| 1A | Support of 18 bit length of PDCP sequence number | 38.822, 4.2, 38.306, 4.2.21.3 | Rel-15 | pc_longSN | No | Yes (for non-RedCap UE) | This PICS shall always be true for non-RedCap and non-eRedCap UE. |
| 2 | Supports Out of order delivery of data to upper layers by PDCP | 38.306, 4.2.4 | Rel-15 | pc_outOfOrderDelivery | No |  |  |
| 3 | Support CA-based PDCP duplication over MCG or SCG DRB | 38.306, 4.2.4 | Rel-15 | pc_pdcp_DuplicationMCG_OrSCG_DRB | No |  |  |

<skipped>

A.4.3.4	RLC Implementation Capabilities

Table A.4.3.4-1: UE RLC Implementation Capabilities

| Item | UE RLC Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1A | Support RLC AM with 18 bit length of RLC sequence number | 38.306, 4.2.21.4 | Rel-15 | pc_am_WithLongSN | No | Yes (for non-RedCap UE) | This PICS shall always be true for non-RedCap and non-eRedCap UE. |
| 1 | Support RLC AM with 12 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_am_WithShortSN | Yes | Yes (for RedCap UE) | This PICS shall always be true for RedCap and eRedCap UE. |
| 2 | Support RLC UM with 12 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_um_WithLongSN | Yes |  |  |
| 3 | Support RLC UM with 6 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_um_WithShortSN | Yes |  |  |

<End of changes>