3GPP TSG-RAN5 Meeting #104	R5-244115

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3225 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add IEs SL-ServingCellInfo and SL-SourceIdentity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add and align the IEs SL-ServingCellInfo and SL-SourceIdentity with core specification updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IEs SL-ServingCellInfo and SL-SourceIdentity have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be fully applicable and aligned with the core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	SL-SDAP-Config

Table 4.6.6-30: SL-SDAP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-SDAP-Config-r16 ::= SEQUENCE { |  |  |  |  |
| sl-SDAP-Header-r16 |  | present |  |  |
| sl-DefaultRB-r16 |  | true |  |  |
| sl-MappedQoS-Flows-r16 |  | Not present | All PC5 QoS flows are mapped to default SL DRB |  |
| sl-CastType-r16 |  | Unicast |  |  |
| } |  |  |  |  |

#### –	SL-ServingCellInfo

Table 4.6.6-30AA: SL-ServingCellInfo

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ServingCellInfo-r17 ::= SEQUENCE { |  |  |  |  |
| sl-PhysCellId-r17 |  | PhysCellId |  |  |
| sl-CarrierFreqNR-r17 |  | ARFCN-ValueNR with condition DL_SSB |  |  |
| } |  |  |  |  |

#### –	SL-SourceIdentity

Table 4.6.6-30AB: SL-SourceIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-SourceIdentity-r17 |  | FFS |  |  |

#### –	SL-SRAP-Config

Table 4.6.6-30A: SL-SRAP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-SRAP-Config-r17 ::= SEQUENCE { |  |  |  |  |
| sl-LocalIdentity-r17 |  | 1 |  |  |
| sl-MappingToAddModList-r17 |  | Not present |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SL-SRB0 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| srb-Identity-r17 |  | 0 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SL-SRB0 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition SL-SRB0 |  | L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SL-SRB1 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| srb-Identity-r17 |  | 1 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SL-SRB1 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition SL-SRB1 |  | L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SL-SRB2 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| srb-Identity-r17 |  | 2 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SL-SRB2 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition SL-SRB2 |  | L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SL-DRB1 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| drb-Identity-r17 |  | DRB-Identity using condition DRB1 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SL-DRB1 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition SL-DRB1 |  | L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17[1] SEQUENCE { |  | 1 entry |  | SL-DRB2 |
| sl-RemoteUE-RB-Identity-r17 CHOICE { |  |  |  |  |
| drb-Identity-r17 |  | DRB-Identity using condition DRB2 |  |  |
| } |  |  |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelUu-r17 |  | Uu-RelayRLC-ChannelID using condition SL-DRB2 |  | L2RelayUE |
| sl-EgressRLC-ChannelPC5-r17 |  | Not present |  |  |
| sl-EgressRLC-ChannelPC5-r17 |  | SL-RLC-ChannelID using condition SL-DRB2 |  | L2RemoteUE |
| } |  |  |  |  |
| sl-MappingToReleaseList-r1 |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL-SRB0 | Establishment of Sidelink SRB0 |
| SL-SRB1 | Establishment of Sidelink SRB1 |
| SL-SRB2 | Establishment of Sidelink SRB2 |
| SL-DRB1 | Establishment of Sidelink DRB1 |
| SL-DRB2 | Establishment of Sidelink DRB2 |
| L2RelayUE | For L2 U2N Relay UE test cases |
| L2RemoteUE | For L2 U2N Remote UE test cases |

<End of modified section>