3GPP TSG-RAN5 Meeting #104	R5-244163

# Maastricht, Netherlands, 19th Aug 2024 - 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0774 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC5 FR2 MU definition in 38.903 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn257_Bn258-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress has been made on MU and TT analysis for PC5 in discussion R5-244162. TR 38.903 needs to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined MU and editorial clean up for the following test cases: -EVM -Spurious Emissions up to 87GHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MU and TT for PC5 FR2 will remain undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.12, B.18, B.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

# B.12	Error Vector Magnitude

Following tables summarize the MU threshold for Error Vector Magnitude (EVM) measurements. The origin MU values for different test setups can be found in following subclauses.

Table B.12-1: MU threshold for beam peak measurement for Frequency error

| Power Class | Frequency | MBW | Power | Threshold MU value for NTC and ETC (NOTE1) |
| --- | --- | --- | --- | --- |
| PC3 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | PUSCH: Table B.12.2-1. Otherwise: FFS |
|  | 32.125GHz < f <= 40.8GHz |  |  | PUSCH: Table B.12.2-2. Otherwise: FFS |
| PC1 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | PUSCH: Table B.12.2-5 Otherwise: FFS |
|  | 32.125GHz < f <= 40.8GHz |  |  | FFS |
| PC5 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | PUSCH: Table B.12.2-7 Otherwise: FFS |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in section B.10.2 |  |  |  |  |

## B.12.1	Uncertainty budget format and assessment for DFF

FFS

## B.12.2	Uncertainty budget format and assessment for IFF

Table B.12.2-1: Measurement Uncertainty (MU) for PUSCH, PC3, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.31% | 4.50% | 6.36% | 11.21% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.60% | 4.73% | 6.68% | 11.21% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.01% | 7.08% | 9.99% | 15.84% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.60% | 4.73% | 6.68% | 11.89% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.71% | 4.99% | 7.07% | 11.89% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.31% | 8.91% | 12.59% | 21.13% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.31% | 8.91% | 12.59% | 21.13% |

Table B.12.2-2: Measurement Uncertainty (MU) for PUSCH, PC3, FR2b (32.125GHz < f <= 40.8GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.54% | 6.26% | 8.91% | 18.06% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.09% | 7.19% | 10.15% | 18.06% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.06% | 11.38% | 16.09% | 25.50% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.09% | 7.19% | 10.15% | 19.13% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.39% | 7.61% | 10.75% | 19.13% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.14% | 14.33% | 20.25% | 34.01% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.14% | 14.33% | 20.25% | 34.01% |

-	The uncertainty assessment has been derived for the case of Quiet zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz, 40.8GHz}, P = [Maximum output power].

- 	Values above are calculated considering a TE noise floor assumption of -10.6 dBm/400 MHz for FR2a and -8.5 dBm/400 MHz for FR2b and additional TE measurement uncertainty (not related to TE noise floor) as shown in Table B.12-2-3 and Table B.12-2-4 added quadratically.

Table B.12.2-3: Additional TE EVM MU (not related to TE noise floor) for PUSCH, PC3, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.50% | 3.44% | 4.86% | 6.87% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.50% | 3.26% | 4.61% | 6.58% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.50% | 3.44% | 4.86% | 6.87% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.50% | 3.26% | 4.61% | 6.58% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.50% | 3.29% | 4.65% | 6.45% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 2.50% | 3.01% | 4.25% | 6.45% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.50% | 3.43% | 4.84% | 9.12% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 2.88% | 4.07% | 5.75% | 9.12% |
| 9 | CP-OFDM QPSK | Inner_Full | 2.50% | 3.01% | 4.25% | 6.85% |
| 10 | CP-OFDM QPSK | Outer_Full | 2.50% | 3.16% | 4.48% | 6.85% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 2.50% | 3.43% | 4.84% | 9.12% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 2.50% | 3.43% | 4.84% | 9.12% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.63% | 5.12% | 7.25% | 12.16% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.63% | 5.12% | 7.25% | 12.16% |

Table B.12.2-4: Additional TE EVM MU (not related to TE noise floor) for PUSCH, PC3, FR2b (32.125GHz < f <= 40.8GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.00% | 4.00% | 5.75% | 8.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.00% | 4.00% | 5.75% | 8.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.00% | 4.00% | 5.75% | 8.00% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.00% | 4.00% | 5.75% | 8.00% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.00% | 4.00% | 5.75% | 10.93% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.08% | 4.35% | 6.14% | 10.93% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.10% | 5.79% | 8.19% | 15.44% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 4.87% | 6.88% | 9.73% | 15.44% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.08% | 4.35% | 6.14% | 11.58% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.26% | 4.60% | 6.50% | 11.58% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.10% | 5.79% | 8.19% | 15.44% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.10% | 5.79% | 8.19% | 15.44% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.12% | 8.66% | 12.25% | 20.59% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.12% | 8.66% | 12.25% | 20.59% |

Table B.12.2-5: Measurement Uncertainty (MU) for PUSCH, PC1, FR2a (23.45GHz <= f <= 32.125GHz)

| 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- |
| 2.48% | 3.50% | 4.95% | 7.00% |

-	The uncertainty assessment has been derived for the case of Quiet zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz}, P = [Maximum output power].

- 	Values above are calculated considering a TE noise floor assumption of -10.6 dBm/400 MHz for FR2a and additional TE measurement uncertainty (not related to TE noise floor) as shown in Table B.12-2-3 added quadratically.

Table B.12.2-6: Additional TE EVM MU (not related to TE noise floor) for PUSCH, PC1, FR2a (23.45GHz <= f <= 32.125GHz)

| 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- |
| 2.40% | 3.39% | 4.79% | 6.68% |

Table B.12.2-7: Measurement Uncertainty (MU) for PUSCH, PC5, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.55% | 3.08% | 4.37% | 6.17% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.61% | 3.18% | 4.51% | 6.52% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.55% | 3.08% | 4.37% | 6.17% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.61% | 3.18% | 4.51% | 6.52% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.66% | 3.26% | 4.61% | 7.10% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 2.72% | 3.36% | 4.76% | 7.10% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.88% | 3.61% | 5.12% | 8.05% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 3.02% | 3.84% | 5.43% | 8.05% |
| 9 | CP-OFDM QPSK | Inner_Full | 2.72% | 3.36% | 4.76% | 7.23% |
| 10 | CP-OFDM QPSK | Outer_Full | 2.74% | 3.40% | 4.81% | 7.23% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 2.88% | 3.61% | 5.12% | 8.05% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 2.88% | 3.61% | 5.12% | 8.05% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.28% | 4.25% | 6.02% | 9.34% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.28% | 4.25% | 6.02% | 9.34% |

-	The uncertainty assessment has been derived for the case of Quiet zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz}, P = [Maximum output power].

- 	Values above are calculated considering a TE noise floor assumption of -10.6 dBm/400 MHz for FR2a and additional TE measurement uncertainty (not related to TE noise floor) as shown in Table B.12-2-8 added quadratically.

Table B.12.2-8: Additional TE EVM MU (not related to TE noise floor) for PUSCH, PC5, FR2a (23.45GHz <= f <= 32.125GHz)

| 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- |
| 2.50% | 3.00% | 4.25% | 6.00% |

## <<< Skip unchanged sections >>>

# B.18	Spurious emissions

Editor’s Note:

-	MU value analysis and offset value analysis for PC 2 and 4 are not complete.

-	MU offset value analysis for PC2 and 4 are not complete.

-	MU value analysis for various test setups in clause B.18.x is not complete for above 87 GHz.

Test procedure of general spurious emission comprises 2 stages: coarse TRP measurement and fine TRP measurement BW. Coarse TRP measurement is introduced to reduce the measurement time by applying sparser grids and/or wider measurement BW than fine TRP measurement while having offset dB more stringent test requirement in order not to cause additional misjudgement risk. For the frequency ranges for which coarse TRP measurement does not PASS, the measurement is continued with fine TRP measurement procedure.

Tables B.18-1,  B.18-1a, B.18-1b summarizes the MU threshold for fine TRP measurements for General spurious emissions, spurious emission band UE co-existence and additional spurious emission, respectively. The origin MU values for fine TRP measurement for different test setups can be found in following subclauses.

Table B.18-1: MU threshold for TRP measurement for general spurious emission

| Power Class | Diversity | Frequency | In-band BW | In-band Power (NOTE2) | Threshold MU value [dB] (NOTE1) |
| --- | --- | --- | --- | --- | --- |
| PC3 | SISO, MIMO | 6 GHz <= f <=12.75 GHz | BW <= 400MHz | P = Max Output Power | 5.29 |
|  |  | 12.75 GHz <= f <= 23.45 GHz |  |  | 5.25 |
|  |  | 23.45 GHz <= f <= 40.8 GHz |  |  | 5.41 |
|  |  | 40.8 GHz <= f <= 66 GHz |  |  | 7.42 |
|  |  | 66 GHz <= f <= 80 GHz |  |  | 7.72 |
|  |  | 80 GHz < f <= 87 GHz |  |  | 8.14 |
| PC1 | SISO, MIMO | 6 GHz <= f <=12.75 GHz | BW <= 400MHz | P = Max Output Power | 5.28 |
|  |  | 12.75 GHz <= f <= 23.45 GHz |  |  | 5.91 |
|  |  | 23.45 GHz <= f <= 40.8 GHz |  |  | 6.07 |
|  |  | 40.8 GHz <= f <= 66 GHz |  |  | 8.09 |
|  |  | 66 GHz <= f <= 80 GHz |  |  | 7.71 |
| PC5 |  | 6 GHz <= f <=12.75 GHz | BW <= 400MHz | P = Max Output Power | 5.28 |
|  |  | 12.75 GHz <= f <= 23.45 GHz |  |  | 5.24 |
|  |  | 23.45 GHz <= f <= 40.8 GHz |  |  | 5.40 |
|  |  | 40.8 GHz <= f <= 66 GHz |  |  | 7.42 |
|  |  | 66 GHz <= f <= 80 GHz |  |  | 7.71 |
|  |  | 80 GHz < f <= 87 GHz |  |  | 8.13 |
| NOTE 1: Total EIRP Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.18.2-3 to Table B.18.2-11 for PC3 UEs and in Table B.18.2-12 to Table B.18.2-16 for PC1 UEs. NOTE 2: Max output power level for device with corresponding power class. |  |  |  |  |  |

Table B.18-1a: MU threshold for TRP measurement for spurious emission band UE co-existence

| Power Class | Diversity | Frequency | In-band BW | In-band Power (NOTE2) | Threshold MU value [dB] (NOTE1) |
| --- | --- | --- | --- | --- | --- |
| PC3 | SISO, MIMO | n257, n260, n261 | BW <= 400MHz | P = Max Output Power | 6.00 |
|  | SISO, MIMO | 23.6 GHz < f <= 24.0 GHz |  |  | 6.00 |
|  | SISO, MIMO | 36 GHz <= f <= 37 GHz |  |  | 6.00 |
|  | SISO, MIMO | 57 GHz <= f <= 66 GHz |  |  | 8.01 |
| PC1 | SISO, MIMO | n257, n261 | BW <= 400MHz | P = Max Output Power | 7.32 |
|  | SISO, MIMO | n260 |  |  | 7.32 |
|  | SISO, MIMO | 23.6 GHz < f <= 24.0 GHz |  |  | 7.32 |
|  | SISO, MIMO | 57 GHz <= f <= 66 GHz |  |  | 8.00 |
| PC5 |  | n260 | BW <= 400MHz | P = Max Output Power | 5.98 |
|  |  | 23.6 GHz < f <= 24.0 GHz |  |  | 5.98 |
|  |  | 57 GHz <= f <= 66 GHz |  |  | 8.00 |
| NOTE 1: Total EIRP Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.18.2-3 to Table B.18.2-11 for PC3 UEs and in Table B.18.2-12 to Table B.18.2-16 for PC1 and PC5 UEs. NOTE 2: Max output power level for device with corresponding power class. |  |  |  |  |  |

Table B.18-1b: MU threshold for TRP measurement for additional spurious emission

| Power Class | Diversity | Frequency | In-band BW | In-band Power (NOTE2) | Threshold MU value [dB] (NOTE1) |
| --- | --- | --- | --- | --- | --- |
| PC3 | SISO, MIMO | 6 GHz <= f <=12.75 GHz NS_202 | BW <= 400MHz | P = Max Output Power | 5.29 |
|  |  | 12.75 GHz <= f <= 23.45 GHz NS_202 |  |  | 5.84 |
|  |  | 23.45 GHz <= f <= 40.8 GHz NS_202, NS_203 |  |  | 6.00 |
|  |  | 40.8 GHz <= f <= 2nd harmonic of the upper frequency edge of the UL operating band NS_202 |  |  | 8.01 |
| PC1 | SISO, MIMO | 6 GHz <= f <=12.75 GHz NS_202 | BW <= 400MHz | P = Max Output Power | 5.28 |
|  |  | 12.75 GHz <= f <= 23.45 GHz NS_202 |  |  | 7.16 |
|  |  | 23.45 GHz <= f <= 40.8 GHz NS_202, NS_203 |  |  | 7.32 |
|  |  | 40.8 GHz <= f <= 2nd harmonic of the upper frequency edge of the UL operating band NS_202 |  |  | 9.34 |
| PC5 |  | 6 GHz <= f <=12.75 GHz NS_202 | BW <= 400MHz | P = Max Output Power | 5.28 |
|  |  | 12.75 GHz <= f <= 23.45 GHz NS_202 |  |  | 5.82 |
|  |  | 23.45 GHz <= f <= 40.8 GHz NS_202, NS_203 |  |  | 5.98 |
|  |  | 40.8 GHz <= f <= 2nd harmonic of the upper frequency edge of the UL operating band NS_202 |  |  | 8.00 |
| NOTE 1: Total EIRP Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.18.2-3 to Table B.18.2-11 for PC3 UEs and in Table B.18.2-12 to Table B.18-2.16 for PC1 and PC5 UEs. NOTE 2: Max output power level for device with corresponding power class. |  |  |  |  |  |

## <<< Skip unchanged sections >>>

Table B.18.2-16: Uncertainty assessment for TRP measurement (f= 66 GHz to 87 GHz, Quiet Zone size ≤ 30 cm) for PC1 and PC5 UEs

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.02 | Normal | 2.00 | 0.01 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.60 | Actual | 1.00 | 0.60 |
| 4 | Mismatch | 2.30 | Actual | 1.00 | 2.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | Uncertainty of the RF power measurement equipment | 4.00 | Normal | 2.00 | 2.00 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 3.00 | Normal | 2.00 | 1.50 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.09 | U-shaped | 1.41 | 0.06 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Influence of TRP measurement grid (NOTE 1) | 0.25 | Actual | 1 | 0.25 |
| 14 | Influence of beam peak search grid (NOTE 2) | N/A | Actual | 1 | N/A |
| 15 | Multiple measurement antenna uncertainty (NOTE 5) | 0.15 | Actual | 1 | 0.15 |
| 16 | DUT repositioning | 0.00 | Rectangular | 1.73 | 0.00 |
| 17 | Misalignment of DUT due to change of DUT orientation | 0.10 | Actual | 1 | 0.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 18 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 19 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 20 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 21 | Uncertainty of the Network Analyzer | 1.70 | Normal | 2.00 | 0.85 |
| 22 | Uncertainty of the absolute gain of the calibration antenna | 1.70 | Normal | 2.00 | 0.85 |
| 23 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.05 | Rectangular | 1.73 | 0.03 |
| 24 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 25 | Quality of quiet zone for calibration process (NOTE 4) | 0.60 | Actual | 1.00 | 0.60 |
| 26 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 27 | Influence of the calibration antenna feed cable | 0.28 | Normal | 2.00 | 0.14 |
| 28 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Expanded uncertainty (1.96σ - confidence interval of 95 %) |  |  |  | Value |
|  | TRP Expanded uncertainty (66 GHz < f <= 87 GHz) [dB] (a) |  |  |  | 7.30 |
|  | Systematic uncertainties (NOTE 3) |  |  |  | Value |
| 29 | Systematic error due to TRP calculation/quadrature (NOTE 1) (b) |  |  |  | 0 |
| 30 | General spurious emissions Influence of noise (c1) (66 GHz < f <= 80 GHz) |  |  |  | 0.41 |
| 30 | General spurious emissions Influence of noise (c2) (80 GHz < f <= 87 GHz) (PC5) |  |  |  | 0.83 |
| 31 | Systematic error related to beam peak search (NOTE 2) |  |  |  | N/A |
| Total measurement uncertainty |  |  |  |  | Value |
| General spurious emissions Total measurement uncertainty (a)+(b)+(c1) [dB] (66 GHz < f <= 80 GHz) |  |  |  |  | 7.71 |
| General spurious emissions Total measurement uncertainty (a)+(b)+(c1) [dB] (80 GHz < f <= 87 GHz) (PC5) |  |  |  |  | 8.13 |
| NOTE 1: This contributor shall only be considered for TRP measurements. NOTE 2: This contributor shall only be considered for EIRP measurements. NOTE 3: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 4: Value based on procedure defined in clause D.2 of TR 38.810 for Quiet Zone size of less or equal to 30 cm. NOTE 5: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 6: The analysis is valid for SISO and MIMO. |  |  |  |  |  |

NOTE: MU assessment for additional spurious in Table B.18.2-3 to Table B.18.2-16 is based on the following relaxations:

Table B.18.2-17: Transmitter Spurious emissions relaxation considered in MU assessment (Quiet Zone size ≤ 30 cm)

| Power Class | Frequency | Relaxation |
| --- | --- | --- |
| PC1 | 6 GHz < f <= 12.75 GHz | 0 dB |
|  | 12.75 GHz < f <= 23.45 GHz | 0 dB |
|  | 23.45GHz <= f <= 40.8GHz | 0 dB |
|  | 40.8 GHz < f <= 66 GHz | 0 dB |
|  | 66 GHz < f <= 80 GHz | 0 dB |
| PC2 | 6 GHz < f <= 12.75 GHz | FFS |
|  | 12.75 GHz < f <= 23.45 GHz | FFS |
|  | 23.45GHz <= f <= 40.8GHz | FFS |
|  | 40.8 GHz < f <= 66 GHz | FFS |
|  | 66 GHz < f <= 87 GHz | FFS |
| PC3 | 6 GHz < f <= 12.75 GHz | 0 dB |
|  | 12.75 GHz < f <= 23.45 GHz | 0 dB |
|  | 23.45GHz <= f <= 40.8GHz | 0 dB |
|  | 40.8 GHz < f <= 66 GHz | 0 dB |
|  | 66 GHz < f <= 87 GHz | 0 dB |
| PC4 | 6 GHz < f <= 12.75 GHz | FFS |
|  | 12.75 GHz < f <= 23.45 GHz | FFS |
|  | 23.45GHz <= f <= 40.8GHz | FFS |
|  | 40.8 GHz < f <= 66 GHz | FFS |
|  | 66 GHz < f <= 80 GHz | FFS |
| PC5 | 6 GHz < f <= 12.75 GHz | 0 dB |
|  | 12.75 GHz < f <= 23.45 GHz | 0 dB |
|  | 23.45GHz <= f <= 40.8GHz | 0 dB |
|  | 40.8 GHz < f <= 66 GHz | 0 dB |
|  | 66 GHz < f <= 87 GHz | 0 dB |

## <<< Skip unchanged sections >>>

# B.25	Receiver spurious emissions

Editor’s Note:

-	MU value analysis and offset value analysis for PC 2 and 4 are not complete.

-	MU value analysis for various test setups in subsection B.25.x is not complete for above 87 GHz.

-	Offset value analysis is not complete as it is derived from MU value analysis for above 87 GHz.

Test procedure of general spurious emission comprises 2 stages: coarse TRP measurement and fine TRP measurement BW. Coarse TRP measurement is introduced to reduce the measurement time by applying sparser grids and/or wider measurement BW than fine TRP measurement while having offset dB more stringent test requirement in order not to cause additional misjudgement risk. For the frequency ranges for which coarse TRP measurement does not PASS, the measurement is continued with fine TRP measurement procedure.

Table B.25-1 summarizes the MU threshold for fine TRP measurements for General spurious emissions. The origin MU values for fine TRP measurement for different test setups can be found in following subclauses.

Table B.25-1: MU threshold for TRP measurement for Rx spurious emission

| Power Class | Frequency | In-band BW | In-band Power (NOTE2) | Threshold MU value [dB] (NOTE1) |
| --- | --- | --- | --- | --- |
| PC3 | 6 GHz <= f <=12.75 GHz | BW <= 400MHz | P = Max Output Power | 5.64 |
|  | 12.75 GHz <= f <= 23.45 GHz |  |  | 5.60 |
|  | 23.45 GHz <= f <= 40.8 GHz |  |  | 6.11 |
|  | 40.8 GHz <= f <= 66 GHz |  |  | 7.65 |
|  | 66 GHz <= f <= 80 GHz |  |  | 7.95 |
|  | 80 GHz < f <= 87 GHz |  |  | 8.31 |
| PC1,PC5 | 6 GHz <= f <=12.75 GHz | BW <= 400MHz | P = Max Output Power | 5.63 |
|  | 12.75 GHz <= f <= 23.45 GHz |  |  | 5.59 |
|  | 23.45 GHz <= f <= 40.8 GHz |  |  | 6.10 |
|  | 40.8 GHz <= f <= 66 GHz |  |  | 7.64 |
|  | 66 GHz <= f <= 80 GHz |  |  | 7.95 |
| PC5 | 80 GHz < f <= 87 GHz | BW <= 400MHz | P = Max Output Power | 8.31 |
| NOTE 1: Total EIRP Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.25.2-3 to Table B.25.2-11 for PC3 UEs and in Table B.25.2-12 to Table B.25.2-16 for PC1 and PC5 UEs. |  |  |  |  |

## <<< Skip unchanged sections >>>

Table B.25.2-16: Uncertainty assessment for TRP measurement (f= 66 GHz to 87 GHz, Quiet Zone size ≤ 30 cm) for PC1 and PC5 UEs

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.02 | Normal | 2.00 | 0.01 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch | 2.30 | Actual | 1.00 | 2.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | Uncertainty of the RF power measurement equipment | 4.0 | Normal | 2.00 | 2.0 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 3.0 | Normal | 2.00 | 1.50 |
| 9 | Random uncertainty | 0.5 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.09 | U-shaped | 1.41 | 0.064 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Influence of TRP measurement grid (NOTE 1) | 0.25 | Actual | 1 | 0.32 |
| 14 | Influence of beam peak search grid (NOTE 2) | N/A | Actual | 1 | N/A |
| 15 | Multiple measurement antenna uncertainty (NOTE 5) | 0.15 | Actual | 1 | 0.15 |
| 16 | DUT repositioning | 0.00 | Rectangular | 1.73 | 0.00 |
| 17 | Misalignment of DUT due to change of DUT orientation | 0.10 | Actual | 1 | 0.10 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 18 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 19 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 20 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 21 | Uncertainty of the Network Analyzer | 1.7 | Normal | 2.00 | 0.85 |
| 22 | Uncertainty of the absolute gain of the calibration antenna | 1.70 | Normal | 2.00 | 0.85 |
| 23 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.05 | Rectangular | 1.73 | 0.03 |
| 24 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 25 | Quality of quiet zone for calibration process (NOTE 4) | 0.60 | Actual | 1.00 | 0.60 |
| 26 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 27 | Influence of the calibration antenna feed cable | 0.28 | Normal | 2.00 | 0.14 |
| 28 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Expanded uncertainty (1.96σ - confidence interval of 95 %) |  |  |  | Value |
|  | TRP Expanded uncertainty ( 66 GHz < f <= 80 GHz) [dB] (a) |  |  |  | 7.31 |
|  | Systematic uncertainties (NOTE 3) |  |  |  | Value |
| 29 | Systematic error due to TRP calculation/quadrature (NOTE 1) (b) |  |  |  | 0.00 |
| 30 | Influence of noise (66 GHz < f <= 80 GHz) (c1) |  |  |  | 0.64 |
| 30 | Influence of noise (80 GHz < f <= 87 GHz) (c2) (PC5) |  |  |  | 1.00 |
| 31 | Systematic error related to beam peak search (NOTE 2) |  |  |  | N/A |
| Total measurement uncertainty |  |  |  |  | Value |
| Total measurement uncertainty (a)+(b)+(c1) [dB] (66 GHz < f <= 80 GHz) |  |  |  |  | 7.95 |
| Total measurement uncertainty (a)+(b)+(c2) [dB] (80 GHz < f <= 87 GHz) (PC5) |  |  |  |  | 8.31 |
| NOTE 1: This contributor shall only be considered for TRP measurements. NOTE 2: This contributor shall only be considered for EIRP measurements. NOTE 3: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 4: Value based on procedure defined in clause D.2 of TR 38.810 for Quiet Zone size of less or equal to 30 cm. NOTE 5: Applies to the system which has a structure of mechanical feed antenna positioning. |  |  |  |  |  |

## <<< END OF CHANGES >>>