3GPP TSG-RAN5 Meeting #104	R5-244172

# Maastricht, Netherlands, 19th Aug 2024 - 23th Aug 2024

Agenda item:	5.3.11.9

Source:	Keysight Technologies

Title:	Discussion on Influence of noise for PC6 in FR2 test cases

Document for:	Discussion and Endorsement

# Introduction

Support to FR2 PC6 is being added into RAN5 specs under Rel-17 WI NR_HST_FR2-UEConTest. There are certain MU factors that are still undefined in TR 38.903 [2]. Proposal for those related to antenna array assumption are being covered in [1]. This contribution focuses then on the remaining MU factor for PC6, that is the influence of noise, looking for leveraging definition already made for other power classes.

# Discussion

Influence of noise is a MU factor that needs to be calculated per test case, band and power class considering the expected UE power level, typically the lowest expected value, and the assumed test system noise floor in those conditions.

In this section, the minimum requirements described in TS 38.101-2 [3] for PC6 will be analysed and compared to those applicable to other power classes. Considering the associated RAN5 WI is Rel-17, the latest Rel-17 version of TS 38.101-2 [3] will be used in this analysis.

## Frequency range

As it can be observed in TS 38.101-2 [3], requirements for PC6 are only defined for FR2a bands.

## Maximum output power

Looking into section 6.2.1.6 in [3],  PC6 requirements for minimum peak EIRP, max TRP, max EIRP and multi-band relaxation are the same as those defined for PC5 FR2a bands in section 6.2.1.5 in [3].

However, the requirement there for Min EIRP over UE spherical coverage is a bit different between PC5 and PC6 given the evaluation areas are defined a bit different per power class. The lowest value defined for PC5 in FR2a is 22dBm while the lowest value defined for PC6 is 20dBm, what drives into a slight SNR reduction.

For MOP – Spherical test case, we have evaluated the influence of noise for PC6 in our test system and concluded that value defined for PC5 in TR 38.903 [2] can be reused for PC6.

## Maximum output power reduction

Maximum output power reduction (MPR) requirements and additional maximum output power reduction (A-MPR) requirements defined for in TS 38.101-2 [3], sections 6.2.2 and 6.2.3, are exactly the same as defined there for PC5.

It must be noted that in many test cases the UE is set to maximum output power and there the MOP or MPR power will apply depending on the required UL BW and RB allocation to test.

## Output power dynamics

Minimum output power requirements are jointly defined for PC5 and PC6 in section 6.3.1.2 in [3], that is, they are the same.

Regarding OFF power requirement defined in section 6.3.2 in [3],  no difference per power class is made so the same requirement applies to all of them.

Regarding power control requirement defined in section 6.3.4 in [3], again no difference per power class is made so the same requirement applies in any case.

## Other UE power requirements

For carrier leakage and in-band emissions requirements defined in TS 38.101-2 [3], sections 6.4.2.2 and 6.4.2.3 respectively, the specific requirements for PC6 are defined pointing directly to the requirements defined there for PC5.

## UL MIMO

All observations made in sections 2.2 to 2.5 for UL SISO are observed in equivalent UL MIMO subsections “D” in TS 38.101-2 [3] that in summary means that in terms of UL power, requirements for PC6 are the same as those defined for PC5 except for MOP-Spherical, where PC6 power can be up to 2dB lower, but in such a case still the influence of noise defined for PC5 can be assumed for PC6 in our test system.

## Carrier Aggregation

In Rel-17 core specs TS 38.101-2 [3] there are no power requirements for PC6 operating in carrier aggregation. As a consequence, CA for PC6 is out of scope in Rel-17 RAN5 WI NR_HST_FR2-UEConTest.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. There are only PC6 requirements defined for FR2a bands.

Observation 2. CA is out of scope in associated Rel-17 RAN5 WI NR_HST_FR2-UEConTest.

Observation 3. For UL SISO and UL MIMO test cases, UL power requirements for PC6 are the same as PC5 except for MOP-spherical.

Observation 4. For test cases other than MOP-spherical, influence of noise for PC6 must be the same defined for PC5.

Observation 5. For MOP-spherical, for our test system, influence of noise defined for PC5 can be reused for PC6.

Proposal 1. Based on all observations above, for PC6 define influence of noise equal to the one defined for PC5 in all FR2a, UL SISO and UL MIMO single carrier cases.

# References

- R5-244448, “PC6 MUs based on antenna array assumptions”, Keysight, RAN5#104

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v18.3.0 (2024-06)

- TS 38.101-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v17.14.0 (2024-06)