3GPP TSG-RAN5 Meeting #104	R5-244276

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4522 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ATSSS TC 10.5.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, ATSSS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause number 10.5.2.1.3.1 is in wrong place. The NR RRC messages shall be removed in step 10 in Table 10.5.2.1.3.2-1 and step 1 in Tale 10.5.2.1.3.2-2 as the non-3GPP leg of the MA PDU session is connected to 5GC via WLAN. The step and table reference in Table 10.5.2.1.3.3-7 is incorrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clause 10.5.2.1.3.1 was updated. 2. NR RRC messages were removed. 3. Step and table reference in Table 10.5.2.1.3.3-7 were updated. 4. Editorial updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors will exist in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

10.5.2.1	UE-requested user-plane resource(s) release / ATSSS / Accepted

10.5.2.1.1	Test Purpose (TP)

(1)

with { UE is in BEARER CONTEXT ACTIVE state and the MA PDU session has user-plane resources establishment with 3GPP access connected to EPC and non-3GPP access connected to 5GCN }

ensure that {

when { UE is triggered to disconnect from the 3GPP leg of the MA PDU session }

then { UE transmits PDN DISCONNECT REQUEST message and the user-plane resources on 3GPP access is released }

}

(2)

with { UE is in PDU SESSION ACTIVE state and the MA PDU session has user-plane resources established on Non-3GPP access}

ensure that {

when { UE is triggered to release from the non-3GPP leg of the MA PDU session }

then { UE transmits PDU SESSION RELEASE REQUEST message and the whole MA PDU session was released }

}

10.5.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.193, clause 5.3.4. Unless otherwise stated these are Rel-17 requirements.

[TS 24.193, clause 5.3.4]

In order to release an MA PDU session which has a PDN connection established as a user-plane resource:

a)	to release the PDN connection established as a user-plane resource of the MA PDU session:

1)	the network shall:

i)	initiate the network initiated detach procedure as specified in clause 5.5.2.3 of 3GPP TS 24.301 [10] by sending the DETACH REQUEST message to the UE if EMM-REGISTERED without PDN connection is not supported by the UE and the MME and the PDN connection is the last PDN connection; or

ii)	initiate the EPS bearer context deactivation procedure as specified in clause 6.4.4 of 3GPP TS 24.301 [10] by sending the DEACTIVATE EPS BEARER CONTEXT REQUEST message to the UE if EMM-REGISTERED without PDN connection is supported by the UE and the MME or the PDN connection is not the last PDN connection; or

2)	the UE shall:

i)	initiate the UE initiated detach procedure as specified in clause 5.5.2.2 of 3GPP TS 24.301 [10] by sending the DETACH REQUEST message to the network if EMM-REGISTERED without PDN connection is not supported by the UE and the MME and the PDN connection is the last PDN connection; or

ii)	initiate the UE requested PDN disconnect procedure as specified in clause 6.5.2 of 3GPP TS 24.301 [10] by sending the PDN DISCONNECT REQUEST message to the network if EMM-REGISTERED without PDN connection is supported by the UE and the MME or the PDN connection is not the last PDN connection; and

b)	to release the user-plane resources, if any, established on non-3GPP access of the MA PDU session:

1)	the network shall initiate the network-requested PDU session release procedure as specified in clause 6.3.3.2 of 3GPP TS 24.501 [6] by sending the PDU SESSION RELEASE COMMAND message with the Access type IE indicating "Non-3GPP access" or without including the Access type IE to the UE; or

2)	the UE shall initiate the UE-requested PDU session release procedure as specified in clause 6.4.3.2 of 3GPP TS 24.501 [6] by sending the PDU SESSION RELEASE REQUEST message to the network.

10.5.2.1.3	Test description

10.5.2.1.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to TS 38.508-1 [4].

-	E-UTRA Cell 1 is configured according to TS 36.508 [7].

-	E-UTRA Cell 1 and WLAN Cell 27 belong to the same PLMN. Both E-UTRA Cell 1 and WLAN Cell 27 are set to ''Serving cell''.

UE:

-	UE is registered over both 3GPP access and non-3GPP access in the same PLMN.

Preamble:

-	The UE is brought to state 3W-A according to TS 38.508-1 [4] and Generic RB Established (state 3) according to TS 36.508 [7].

10.5.2.1.3.2	Test procedure sequence

Table 10.5.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to request to establish an MA PDU session by MMI or AT command. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 2-4 below the events specified in Table 10.5.2.1.3.2-2 may take place. | - | - | - | - |
| 2 | The UE transmits a PDN CONNECTIVITY REQUEST message and sets the request type to "initial request" and set the ATSSS request PCO parameter according to the ATSSS functionality and steering mode that UE supports on E-UTRA Cell 1. | --> | NAS: PDN CONNECTIVITY REQUEST | - | - |
| 3 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message with the extended protocol configuration options IE with the ATSSS response. Note: the SS allocates a PDN address of a PDN type which is compliant with the PDN type requested by the UE. | <-- | NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| 4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message as specified. | --> | NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 5 | Cause the UE to request disconnection from the 3GPP leg of the MA PDU session. | - | - | - | - |
| 6 | Check: Does the UE transmit a PDN DISCONNECT REQUEST message as specified (to disconnect from the additional PDN)? | --> | PDN DISCONNECT REQUEST | 1 | P |
| 7 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST message included in an RRCConnectionReconfiguration message. | <-- | DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 8 | The UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | - |
| 9 | Cause the UE to request release from the Non-3GPP leg of the MA PDU session. | - | - | - | - |
| 10 | Check: Does the UE transmit a PDU SESSION RELEASE REQUEST message? | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE REQUEST | 2 | P |
| 11 | The UE performs PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4]. | - | - | - | - |

Table 10.5.2.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and sets the request type to "MA PDU request" in the UL NAS TRANSPORT message and sets the ATSSS-ST bits in the 5GSM capability IE of the PDU SESSION ESTABLISHMENT REQUEST message according to the ATSSS functionality and steering mode that UE supports on WLAN Cell 27. Note: PDU SESSION ESTABLISHMENT REQUEST is included in UL NAS transport. | --> | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 2 | The SS transmits an PDU SESSION ESTABLISHMENT ACCEPT on WLAN Cell 27. | <-- | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |

10.5.2.1.3.3	Specific message contents

Table 10.5.2.1.3.3-1: PDN CONNECTIVITY REQUEST (Step 2, Table 10.5.2.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol configuration options |  | Only the values indicated below are checked |  |  |
| protocol identifier |  |  |  |  |
| length of the protocol identifier contents of the unit |  |  |  |  |
| protocol identifier contents |  |  |  |  |
| Additional parameters list |  |  |  |  |
| container identifier |  | ‘0030‘H | ATSSS request |  |
| container identifier contents |  | Any allowed value according to 3GPP TS 24.193 [45] subclause 6.1.6.2 that UE supports. |  |  |
| container identifier |  | ‘001A‘H | PDU session ID |  |
| container identifier contents |  | Any allowed value | PDU session ID |  |

Table 10.5.2.1.3.3-2: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Step 3, Table 10.5.2.1.3.2-1)

| Derivation path: TS 36.508 [7] Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| extended protocol configuration options | Only the values indicated below are checked |  |  |
| protocol identifier |  |  |  |
| length of the protocol identifier contents of the unit |  |  |  |
| protocol identifier contents |  |  |  |
| Additional parameters list |  |  |  |
| container identifier | ‘0030‘H | ATSSS response |  |
| container identifier contents | Any allowed value according to 3GPP TS 24.193 [45] subclause 6.1.6.2. |  |  |

Table 10.5.2.1.3.3-3: REGISTRATION ACCEPT (Preamble, Table 10.5.2.1.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS network feature support |  |  |  |
| ATS-IND | '1'B | ATSSS supported |  |

Table 10.5.2.1.3.3-4: PDU SESSION ESTABLISHMENT REQUEST (Step 1, Table 10.5.2.1.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | Same value as in PDN CONNECTIVITY REQUEST of step 2 in Table 10.5.2.1.3.2-1 |  |  |
| 5GSM capability |  |  |  |
| All octets with the exception of octet 3, bits 4 to 7 | Any allowed value |  |  |
| ATSSS-ST(octet 3, bits 4 to 7) | Any allowed value according to 3GPP TS 24.193 [45] subclause 6.1.6.2. |  |  |

Table 10.5.2.1.3.3-5: UL NAS Transport (Step 1, Table 10.5.2.1.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | ‘011’B | MA PDU request |  |
| S-NSSAI | Not Present |  |  |

Table 10.5.2.1.3.3-6: PDN DISCONNECT REQUEST (Step 6, Table 10.5.2.1.3.2-1)

| Derivation Path: TS 36.508 [7] Table 4.7.3-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | ‘0000’ | "no EPS bearer identity assigned" |  |
| Procedure transaction identity | PTI-1 | UE assigns a particular PTI not yet used between 1 and 254 |  |
| Linked EPS bearer identity | 6 |  |  |

Table 10.5.2.1.3.3-7: PDU SESSION RELEASE COMMAND (Step 11, Table 10.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.2-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same ID as the ID of PDU session which UE request in step 1 in Table 10.5.2.1.3.2-2 |  |  |
| Access type | '01'B | Non-3GPP access |  |

[End of change]