3GPP TSG-RAN5 Meeting #104	R5-244285

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3275 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum requirements for 6.3.4.0 - FR1 LTM Pcell switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add mobility enhancement related abbreviations and minimum requirements for FR1 LTM Pcell switch TCs |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Mobility enhancement related abbreviations - Minimum requirements for FR1 LTM Pcell switch TCs are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AoA	Angle of Arrival

AoD	Angle of Departure

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BS	Base Station

BWP	Bandwidth Part

CBD	Candidate Beam Detection

CDF	Cumulative Distribution Function

CC	Component Carrier

CCA	Clear Channel Assessment

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

E-UTRA	Evolved UTRA

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA – NR Dual Connectivity

FR	Frequency Range

FR1	Frequency Range 1

FR2	Frequency Range 2

GEO	Geostationary Earth Orbit

HARQ	Hybrid Automatic Repeat Request

HO	Handover

IMR	Interference Measurement Resource

L1-RSRP	Layer 1 RSRP

LEO	Low Earth Orbit

LMF	Location Management Function

LPP	LTE Positioning Protocol

LTM	L1/L2 triggered mobility

MAC	Medium Access Control

MCG	Master Cell Group

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR/5GC	NR connected to 5GC

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

NSA	Non-Standalone

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

PRS	Positioning Reference Signal

PSCell	Primary Secondary Cell

PSS	Primary Synchronization Signal

pTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RedCap	Reduced Capabilities

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMC	Reference Measurement Channel

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRM	Radio Resource Management

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SA	Standalone

SAB	Satellite Access Band

SAN	Satellite Access Node

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCPAC	Subsequent conditional PSCell Addition or Change

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS	System Simulator

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TN	Terrestrial Network

TTI	Transmission Time Interval

UL	Uplink

VIL	Visible Interruption Length

VIRP	Visible Interruption Repetition Period

<< Skip unchanged text >>

6.3.4	LTM PCell Switch

6.3.4.0	Minimum conformance requirements

6.3.4.0.1	Minimum conformance requirements for LTM PCell Switch

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When the target cell and the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, the LTM cell switch delay is defined as:

DLTM = Tcmd  + TLTM-interrupt

Where:

Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement as specified in TS 38.213[8].

TLTM-interrupt is as stated in TS 38.133[6] section 6.3.1.3.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in TS 38.133[6] clause 9.2 for intra-frequency cell and in TS 38.133[6] clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in TS 38.133[6] clause 9.2 for intra-frequency cell and in TS 38.133[6] clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

[-	The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or]

[-	The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

-	LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

-	The target DL/UL TCI state remains detectable during the LTM cell switching period

-	The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

The interruption time TLTM-interrupt is the time between the end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd stated in TS 38.133[6] section 6.3.1.2.

TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

TLTM-RRC-processing = 0, if the UE supports [earlyDecodingAndValidityCheck] capability, and at least one of the following conditions is met:

-	The number of candidate cells in the LTM candidate cell configuration does not exceed [number of candidate cells for early ASN.1 decoding and validity check],

-	UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and the number of candidate cells with TCI state(s) in LTM candidate cell active TCI state list does not exceed [number of candidate cells for early ASN.1 decoding and validity check],

-	UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3]. [FFS further conditions]

-	[FFS: SCell is not part of the cell switch.]

Otherwise TLTM-RRC-processing = 10 ms.

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

If the UE supports [faster LTM processing] capability, the value of TLTM-processing equals to

[faster intra-FR processing delay (10 ms or 15 ms)] for FR1 to FR1 and FR2 to FR2 LTM cell switch.

[faster inter-FR processing delay (20 ms or 30 ms)] for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Otherwise, the value of TLTM-processing equals to

-	20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

-	40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Editor’s note: FFS whether a smaller value can be considered in some scenarios or under certain conditions, when the UE does not support [faster LTM processing] capability.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

TRS-proc is the time for SSB processing.

Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

-	The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or

-	The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and

-	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in TS 38.133[6] section 8.x, and

- 	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms], or

-	the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or

-	The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

-	The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and

-	the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in TS 38.133[6] section 8.x, and not more than TCI state activation delay stated in TS 38.133[6] section 8.x + [160 ms], or

-	The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].

Otherwise,

Tfirst-RS is the time to the first SSB transmission on the target cell [after Tcmd].

Editor’s note: FFS whether TRS transmission is also considered.

TRS-proc = 2 ms.

TLTM-IU is the interruption uncertainty during LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

<< End of changes >>